3GPP TSG RAN WG3 #129		  R3-255746

Bengaluru, India, Aug. 25th – 29th, 2025

Title:	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO without RLF Report

Agenda item:	10.2

Source:	Google, Ericsson, NEC, Lenovo, CATT

Document for:	Discussion and Decision

# Introduction

In some cases, there may not be the LTM MRO RLF Report provided from the UE after LTM connection failure. In this paper, we discuss possible cases and provide potential solutions on LTM MRO without RLF Report.

# Discussion

## No RLF report generated for MCG LTM failure

Currently, there are the pre-Rel-19 UEs in the real network. For these UEs, they are unable to generated LTM RLF report when detecting LTM connection failure during LTM procedure.

Furthermore, according to the running CR[1] in RAN2 on UE capability definition in TS 38.306, for a Rel-19 UE, if it doesn’t support RLF-Report for MCG LTM, the UE cannot generate MCG LTM RLF report when detecting LTM connection failure during LTM procedure.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| onDemandSI-Report-r17 Indicates whether the UE supports delivery of on-Demand SI information upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| pscell-MHI-Report-r17 Indicates whether the UE supports the storage of PSCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. | UE | No | No | No |
| rach-Report-r16 Indicates whether the UE supports delivery of RA report upon request from the network. | UE | No | No | No |
| rlfReportCHO-r17 Indicates whether the UE supports RLF-Report for conditional handover. | UE | No | No | No |
| rlfReportCondHandoverWithCandSCG-Change-r19 Indicates whether the UE supports RLF-Report for conditional handover with candidate SCG. A UE supporting this feature shall also support rlfReportCHO-r17. | UE | No | No | No |
| rlfReportDAPS-r17 Indicates whether the UE supports RLF-Report for DAPS handover. | UE | No | No | No |
| rlfReport-LTM-r19 Indicates whether the UE supports RLF-Report for MCG LTM. | UE | No | No | No |
| spr-Report-r18 Indicates whether the UE supports the storage and delivery of Successful PScell Change/Addition Report upon request from the network. | UE | No | No | No |

With the above in mind, it is noticed that:

Observation 1: There are cases for LTM failure without RLF report, e.g., for pre-Rel-19 UEs and Rel-19 UE without capability to support RLF Report for MCG LTM.

## Additional information for LTM MRO without RLF Report

According to the LTM procedure described in the TS 38.331, when the UE detects connection failure with the source or target LTM cell, the UE behaviors are defined below:

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

1>	else:

[……]

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

### Case a: Reestablishment without RLF Report

When the UE detects LTM connection failure, it may select a cell which belongs to the LTM candidate cells list while the attemptLTM-Switch is not configured. According to the descriptions on UE behavior in the previous section, the UE falls back to legacy reestablishment procedure with the selected LTM candidate cell.

Observation 2: If the LTM attempt is not configured, the UE performs legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

Taking into consideration the pre-Rel-19 UEs and Rel-19 UEs without LTM RLF report capability, the UE is unable to provide the LTM RLF Report to the CU after reestablishment procedure.

It is known that Rel-18 MCG LTM was introduced for intra-CU case. Despite the lack of RLF report, the CU can still perform initial analysis, e.g., based on the information gathered in the reestablishment attempt, including failure cell PCI, failure C-RNTI and reestablishment cell CGI. For the case that the reestablishment cell is one of the LTM candidates, the CU determines that the LTM failure is not due to inappropriate LTM candidate cell preparation at CU side. Subsequently, the CU forwards the failure related additional information to the source DU.

Observation 3: For reestablishment without RLF report, the CU can perform initial analysis and identify whether the LTM failure is due to wrong selection of prepared LTM cells, e.g., whether the reestablishment cell belongs to LTM candidate cells list.

In previous L3 handover related MRO scheme, RAN3 has supported the reestablishment case without RLF report. In this case, the RAN node receiving the reestablishment request initiates a failure indication to the failure RAN node where the connection failure was detected. Correspondingly, the failure RAN node performs UE verification and perform a thorough analysis of the failure. The failure indication for reestablishment without RLF report defined in TS 38.423 can include the failure information as shown below:

9.1.3.16	FAILURE INDICATION

This message is sent by NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  | YES | reject |
| >RRC Reestab |  |  |  |  |  |  |
| >>CHOICE RRC Reestab Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Reestab Reporting without RLF Report |  |  |  |  |  |  |
| >>>>Failure cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | Physical Cell Identifier | – |  |
| >>>>Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>>>C-RNTI | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the c-RNTI contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>ShortMAC-I | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the shortMAC-I contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>RRC Conn Reestab Indicator | O |  | ENUMERATED (reconfigurationFailure, handoverFailure, otherFailure, ...) | Corresponds to information provided in the reestablishmentCause contained in the RRCReestablishmentRequest message as defined in TS 38.331 [10] or in the RRCConnectionReestablishmentRequest message as defined in TS 36.331 [14]. | YES | ignore |

The similar principle can be applied for the LTM MRO in case of reestablishment case without RLF report. When the RAN node (i.e., the CU) receives reestablishment attempt from the UE, it first verifies the UE and determines the failure type. If needed, the CU can forward the source cell, target cell, reestablishment cell, source C-RNTI and RLF Report Failure Type to the source DU for root cause analysis to optimize the LTM related parameters.

### Case b: LTM Recovery without RLF Report

For CHO and LTM mobility, when the UE detects connection failure and selects a CHO or LTM candidate cell with attempt configured, the UE can perform CHO or LTM recovery with the selected candidate cell.

For CHO, the UE decides the CHO target cell if the CHO conditions are satisfied and executes CHO with the target cell. This is UE-based decision. If connection failure occurs, the UE selects a CHO cell and initiates CHO recovery. The UE sends the RRC Reconfiguration Complete message to the selected candidate cell. If the UE does not provide the RLF report to the CHO recovery cell, the CHO recovery cell is unable to identify whether the access is a normal CHO execution or a CHO recovery from CHO failure. Consequently, the CHO recovery cell will not initiate a failure indication to the failure RAN node where the connection failure was detected.

Observation 4: Without RLF report, the CHO recovery RAN node cannot identify whether there was a connection failure prior to the CHO access.

The situation changes for LTM. Although the CU configures the candidate LTM cells to UE, the LTM cell switch decision is still initiated by the source DU based on the L1 measurement results. This is NW-driven decision. The source DU selects the target LTM cell and informs the CU of the target LTM cell information via DU-CU CELL SWITCH NOTIFICATION message. After a successful LTM cell switch, the target LTM cell sends an ACCESS SUCCESS message to CU, indicating the (first) access cell where the UE has successfully accessed for LTM. If LTM procedure fails and the UE performs LTM recovery with a candidate LTM cell, the LTM recovery cell also sends an ACCESS SUCCESS message to the CU to indicate the (second) access cell.

For example, for too late LTM, the DU didn’t initiate the LTM cell switch command. The CU will not receive DU-CU CELL SWITCH NOTIFICATION message. The UE performs LTM recovery after the RLF in source cell. Subsequently, the CU will receive ACCESS SUCCESS message from the LTM recovery cell. The possible flow is illustrated in figure 1.

Fig.1 Too late LTM procedure

For too early LTM or LTM to wrong cell, the DU initiated LTM cell switch command. The CU received DU-CU CELL SWITCH NOTIFICATION message, e.g., including the LTM target cell 1.

In case of HOF, the UE fails to complete LTM procedure with target LTM cell and performs LTM recovery with a candidate cell, e.g., the source cell 0 or a different cell 2. The CU receives an ACCESS SUCCESS message including access cell (recovery one). The CU will determine the LTM failure type based on the LTM recovery cell and realize there was a HOF during LTM procedure. The possible flow is illustrated in figure 2.

Fig.2 procedures of HOF case for too early LTM/LTM to wrong cell

In case of RLF, the UE successfully completed LTM procedure with the target LTM cell. The CU will receive an ACCESS SUCCESS message from the target one. If the UE detects RLF with target LTM cell shortly after, the UE performs LTM recovery. Correspondingly, the CU receives a second ACCESS SUCCESS message from the recovery cell. The CU can determine the LTM failure type and understand there was an RLF in LTM target cell shortly before the LTM recovery procedure. The possible flow is illustrated in figure 3.

Fig.3 procedures of RLF case for too early LTM/LTM to wrong cell

As shown in the previous figures, the CU receives different messages and acquires different cell information for different failure cases. Consequently, if RLF report is not provided after successful LTM recovery, the CU still has the ability to detect the connection failure during LTM procedure.

Observation 5: For LTM recovery without RLF report, the CU can perform initial analysis and identify whether there was a connection failure during LTM procedure.

The additional information for LTM recovery without RLF report can follow the principle for reestablishment without RLF report case. For LTM recovery case, there may be recovery cell during the procedure. Based on the UE context at CU side and the information provided by the source/target/recovery DU, the CU can derive source cell information, LTM target cell information, LTM recovery cell information, source C-RNTI and RLF Report Failure Type.

To assist source DU for LTM performance optimization, it is highly beneficial for the CU to forward the additional information to the source DU.

Based on the aforesaid discussions, we prefer to summarize the additional information from the CU to the DU for both reestablishment and LTM recovery cases without RLF Report:

Proposal 1: For Reestablishment and LTM recovery cases without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment/Recovery cell information

- RLF Report Failure Type

Based on the previous discussion, a TP for LTM MRO on TS 38.473 is provided in the annex.

# Conclusion

In this contribution, we discussed the cases for LTM MRO without RLF Report. The related observations and proposals are summarized below:

Observation 1: There are cases for LTM failure without RLF report, e.g., for pre-Rel-19 UEs and Rel-19 UE without capability to support RLF Report for MCG LTM.

Observation 2: If the LTM attempt is not configured, the UE performs legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

Observation 3: For reestablishment without RLF report, the CU can perform initial analysis and identify whether the LTM failure is due to wrong selection of prepared LTM cell.

Observation 4: Without RLF report, the CHO recovery RAN node cannot identify whether there was a connection failure prior to the CHO access.

Observation 5: For LTM recovery without RLF report, the CU can perform initial analysis and identify whether there was a connection failure during LTM procedure.

Proposal 1: For both Reestablishment and LTM recovery cases without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment/Recovery cell information

- RLF Report Failure Type

# Reference

- R2-2505208, Introduction of SONMDT UE Capabilities 38.306, CATT

# Annex - TP for LTM MRO for TS 38.473

Start of Change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| RRC Reestab/Recovery Reporting without RLF report |  |  |  |  |  |  |
| >Source cell PCI | M |  | INTEGER (0..1007) | Physical Cell Identifier | - |  |
| >Target cell PCI | O |  | INTEGER (0..1007) | Physical Cell Identifier | - |  |
| >Re-establishment or recovery cell CGI | M |  | 9.3.1.12 |  | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| [……] |  |  |  |  |  |  |

Next Change

### 9.4.4	PDU Definitions

<SNIP>

F1U-PathFailure,

LTMResetInformation,

CuBeamFailureRecoveryInformation,

DuBeamFailureRecoveryInformation，

LTMFailurewithoutRLFReport

<SNIP>

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-CuBeamFailureRecoveryInformation,

id-DuBeamFailureRecoveryInformation,

id-LTMFailurewithoutRLFReport

<SNIP>

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-CuBeamFailureRecoveryInformation			CRITICALITY ignore 	TYPE CuBeamFailureRecoveryInformation  		PRESENCE optional }|

{ ID id-LTMFailurewithoutRLFReport					CRITICALITY ignore 	TYPE LTMFailurewithoutRLFReport  		PRESENCE optional },

...

}

Next Change

### 9.4.5	Information Element Definitions

<SNIP>

CuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { CuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

CuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMFailurewithoutRLFReport	::= SEQUENCE {

sourceCellPCI			INTEGER (0..1007),

targetCellPCI			INTEGER (0..1007)		OPTIONAL,

reestRecoveryCGI			NRCGI,

c-RNTI				C-RNTI			 	OPTIONAL,

rLFReportFailureType		RLFReportFailureType		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTMFailurewithoutRLFReport-ExtIEs} } OPTIONAL,

...

}

LTMFailurewithoutRLFReport-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Change