3GPP TSG-RAN WG3 #129	          R3-255747

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell, ZTE Corporation, Qualcomm, CATT, China Telecom, CMCC, Samsung

Title:	Response to R3-255513

Document for:	Discussion and Decision

# 1	Introduction

This contribution responses to R3-255513.

# 2	Discussion on S&F mode transition

R3-255513 discusses the S1 S&F mode transition, and have following proposals:

- RAN3 to agree that during the S&F mode transition (i.e., from the S&F mode to the normal mode or vice versa), the MME is not changed for its served UEs (for all connected/inactive/idle UEs). - RAN3 to agree the stage 2 TP to capture the above and some further clean-ups given this is last R19 WI meeting (see Annex). - RAN3 to send an LS to SA2 to inform the agreements as described in Proposal 1 in LS R3-255516

In our view, R3-255513 has a misunderstanding, and does not align with SA2 who is responsible for MME change.

TS 23.401 defines following for transition between S&F mode and non S&F mode:

4.13.9.2 Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

It is clear that SA2 TS 23.401 does allow the MME change for following reasons:

- Via existing NAS procedure, the MME change is supported.

- The “in inter MME mobility” implies a MME change.

The MME change is actually needed in some scenarios. For example, in non S&F mode (normal mode), the eNB can serve both S&F UE and non S&F UE (normal UE). The eNB setup additional S1 with the MME on ground (or a normal MME). When a new UE just power up and initiates the attach, the eNB cannot know whether it is a S&F UE or a normal UE before the eNB select MME for the 1st S1AP message to MME. The eNB can only know the UE type later, e.g. via a UE capability transfer procedure. So the eNB may randomly select a MME, either the MME-onboard (used for S&F), or a normal MME on ground. In this a normal MME is selected and the UE is a S&F UE, MME change is needed, in order to relocate the UE context to MME onboard in both split MME architecture and full CN on board architecture.

Observation 1: SA2 TS 23.401 supports the change of MME during the mode transition between S&F mode and non S&F mode.

Observation 2: there is benefit to use the normal MME-ground after transition to non S&F mode in certain scenarios.  So MME change is needed in some scenarios.

The MME change procedure does not have RAN2/3 impact. RAN2 agreed “RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change”. RAN2 agreement actually mean the S&F mode changes do not imply a MME is always changed or not changed, and both MME change and no MME change can happen.

In addition, the MME change is more related to NAS, rather to AS. Even in case MME is changed, it is transparent to RAN3, and has no impact to RAN3. This is different to MME relocation for RRC CONNECTED UE.

Observation 3: In case MME is changed during transition, it is transparent to RAN3 and have no RAN3 impact.

Based on above analysis, we propose no need to add any restriction on the MME change during the transitioning between S&F mode and non S&F mode.

Even in case SA2 agree no MME change, it is still transparent to RAN3.

Due to above reasons, there is no need to add any text in RAN3 specification for no MME change in S&F transition.

Proposal 1: there is no need to add any text in RAN3 specification for no MME change in S&F transition.

# 3	Conclusion

In this contribution, we discussed the proposals from R3-255513. Our proposals are:

Observation 1: SA2 TS 23.401 supports the change of MME during the mode transition between S&F mode and non S&F mode.

Observation 2: there is benefit to use the normal MME-ground after transition to non S&F mode in certain scenarios.  So MME change is needed in some scenarios.

Observation 3: In case MME is changed during transition, it is transparent to RAN3 and have no RAN3 impact.

Proposal 1: there is no need to add any text in RAN3 specification for no MME change in S&F transition.

# References

- R3-255513, (TP for TS 36.300) Discussion on S&F mode transition

- TS 23.401