3GPP TSG-RAN WG3 Meeting #129	R3-255748

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	15.2

Source:	ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm, CATT, China Telecom, CMCC, Samsung

Title:	Response to R3-255514

Document for:	Discussion

- Introduction

This contribution responses to R3-255514.

2	Discussion

In R3-255514[1], the issue of S&F mode indication of the neighbour cells has been discussed, and the corresponding proposal is given as below.

- RAN3 to agree to introduce an OAM-provision the S&F mode indication of the neighbour cells pending to RAN2 decision.

The main reason from R3-255514 is to support following RAN2 WA reached in RAN2#130[2].

WA: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

However, the above statement is only the WA rather than the final agreement. And companies in RAN2 have not reached the consensus on the corresponding UE behaviour with the indication. If there is no further consensus in the August meeting, the above WA may be withdrawn. Therefore, it is too early for us to discuss RAN3’s impact based on a WA from RAN2.

Observation: RAN3 shall discuss the issue of the S&F mode indication based on the final agreement from RAN2 rather than the WA from RAN2.

In addition, even though the WA turns to the agreement in RAN2, there may be no impact on RAN3’s specification for the following reasons:

- It is clear that no provision is needed for the eNB’s S&F mode, or transition between S&F mode and non-S&F mode for the neighbour cell on the same satellite. The reason is that the eNB can determine its transition between the S&F mode and non-S&F mode, by determining when it needs to setup feeder link or release the feeder link, based on the satellite ephemeris information, location of NTN GW, etc.

- The eNB/satellite is configured with the information of neighbouring satellites via OAM or NTN control function, e.g. to generate SystemInformationBlockType32, SystemInformationBlockType33, etc.

- Similarly, the eNB/satellite can also determine when the neighbouring eNB/satellite transitions between S&F mode and non-S&F mode, e.g. based on the neighbouring satellite’s ephemeris information, location of NTN GW, etc.

In a summary, based on the existing information, the eNB/satellite can determine when it transitions between S&F mode and non-S&F mode, and can also determine when neighbouring eNB/satellite transitions between S&F mode and non-S&F mode. At the same time, the neighboring eNB may transition between S&F mode and non S&F mode. In this case, the S&F mode information is not a “static” information, it is a “semi-static” information that can be changed. So, there is no need to provision the S&F indication of neighbor cell(s).

Proposal: There is no need to provision S&F indication of neighbor cell(s).

3	Summary

Observation: RAN3 shall discuss the issue of the S&F mode indication based on the final agreement from RAN2 rather than the WA from RAN2.

Proposal: There is no need to provision S&F indication of neighbor cell(s).

4	Reference

[1] R3-255514, (TP for TS 36.300) Provision of S&F Mode Indication of neighbour cell list, Huawei, Ericsson, Jio Platforms

[2] R2-2504673, Report from Break-out session on NR-NTN and IoT-NTN, RAN2#130