3GPP TSG-RAN WG3 Meeting #129	R3-255802

Bengaluru, India, August 25th–29th, 2025

Title:	(TP to TS 38.401) AI/ML support

Source:	Ericsson, Deutsche Telekom, Jio Platforms, BT, ZTE Corporation,CATT, Ofinno, Samsung, LG Electronics, NEC, Nokia

Agenda item:	11.2

Document Type:	Discussion and agreement

# 1. Introduction

In this paper, we present a TP to TS 38.401 for Support of AI/ML for NG-RAN (in the case of split architecture) based on the agreements reached in the previous meetings.

The TP aims at covering support for al use cases developed in Rel-19 and for that reason it does not fall under any specific use case section.

# 2. Discussion

In the following we gather the agreements made in the previous meetings regarding the support for AI/ML for NG-RAN in the case of the split architecture.

From RAN3#125-bis we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Introducing separate time information for future coverage state and predicted CCO issue.

R18 leftovers Split Architecture Support:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

From RAN3#126 we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

R18 leftovers Split Architecture Support:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

From RAN3#127 we note the following agreements:

R18 leftovers Split Architecture Support:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

From RAN3#127-bis we note the following agreements:

R18 leftovers Split Architecture Support:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

To the following WA:

WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

From RAN3#128 we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

Adopt Opt2 (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU)

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

R18 leftovers Split Architecture Support:

Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Introduce the start and stop indication over F1 for Delay measurement.

Introduce Data Collection ID in UE associated messages over E1

Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Based on the above, we propose RAN3 to agree to the TP for TS 38.401 that can be found in the Annex.

Proposal 1:  RAN3 to agree to the TP for TS 38.401 that can be found in the Annex.

# 3. Conclusions

Based on the discussion in this paper, the following proposal was made.

Proposal 1:  RAN3 to agree to the TP for TS 38.401 that can be found in the Annex.

# 4. Annex I

# TP to TS 38.401 - Support of AI/ML for NG-RAN

-------------------------------------- START OF CHANGES --------------------------------------

## 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

### 7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

### 7.11.x	Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

-	Energy Cost (EC),

-	UE performance feedback.

The collection of EC is configured through the Resource Status Reporting Initiation procedure while the reporting is performed through the Resource Status Reporting procedure. Packet delay measurement collection for UE Performance from gNB-DU is triggered through the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages and the packet delay measurements collected by the gNB-DU are reported from gNB-DU to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame, specified in TS 38.425 [33].

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of UE performance feedback from a gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the reporting is performed through the Data Collection Reporting procedure.

Signaling by a split gNB of the RLC Average UE UL/DL Throughput over the Xn interface is not supported in this version of the specifications.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted Coverage and Capacity Optimisation (CCO) assistance information, including a predicted CCO issue or an indication that the predicted CCO issue is cancelled, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.   The gNB-DU may also notify the gNB-CU in the GNB-DU CONFIGURATION UPDATE message that a previously notified non executed coverage state change has been cancelled.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration as derivedpredicted by the source gNB. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause, The gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously notified predicted CCO issue and its associated future coverage state changes have been cancelled.

-------------------------------------- END OF CHANGES --------------------------------------