3GPP TSG-RAN WG3 Meeting #129	     R3-255840

Bengaluru, India, 25 – 29 August, 2025

Title:	[Draft] LS on MBS Communication Service Type

Response to:	S4-251096/R3-255028

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	Nokia [to be RAN3]

To:	SA4

Cc:	RAN2, SA5

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA4 for their reply LS on MBS Communication Service Type in S4-251096/R3-255028.

Related to the definition of the value ‘all’ of @communicationServiceType, SA4 indicates in their answer 1: “Option 1 is the right interpretation.  It refers to all communication service types (not limited to MBS communication services).” (Option 1: Value “all” means that the UE shall conduct QoE measurements on broadcast, multicast and unicast.)

Question A: In the CR to TS 26.247 agreed by SA4 in S4-251095, in the description of the @communicationServiceType attribute, it is stated that “When present, this attribute indicates a list of communication service type(s) for which the QoE collection and reporting of QoE metrics is requested and shall contain one or more of the following values:”. Regarding the “one or more” part, RAN3 would like to point out that, in a QMC configuration, only one attribute value is possible at a time (“broadcast”, “multicast”, “unicast”). This effectively means that the “or more” should be deleted from the Description field of the attribute. RAN3 kindly asks SA4 to confirm this.

Question B: In line with the above, a given QoE Reference can, in RAN3’s specification, only point to a single communication service type, and any combination of communication service types is therefore not supported in RAN3’s specification. This means that also the value “all” should be deleted from the @communicationServiceType attribute. RAN3 kindly asks SA4 to confirm this.

Additionally, RAN3 observes in SA4’s answer 2 that ““OD” is revised into “O” in TS 26.247”, however in the attached CR the sentence following sentence can be found: “When absent, quality metrics collection is requested for all communication service types.”

Question C: Whether the @communicationServiceType can have a default value even though the Use is “O”? If the answer is “no”, RAN3 kindly asks SA4 to remove the following sentence from the Description field of the attribute: “When absent, quality metrics collection is requested for all communication service types.”. Furthermore, in line with question B, the combination of all communication service types is not supported in RAN3’s specification. RAN3 kindly asks SA4 to confirm whether the default value as such, or the combination “all communication service types”, can be removed from SA4’s specification.

In addition, if the above sentence quoted in CR to TS 26.247 agreed by SA4 in S4-251095TS 26.247 is the common understanding in SA4, RAN3 assumes there’s potential backward compatibility issue from end-to-end perspective. More specifically, RAN3 assumes that a Rel-17 UE not receiving the attribute considers the quality metrics collection is requested for Unicast only, rather than for all communication service types.

Question DC: If the answer to Question C is yes, does SA4 expect different default behaviours for Rel-17 UE and Rel-18 UE?

2. Actions:

To SA4: 	RAN3 kindly asks SA4 to provide answers to the questions above, and, if feasible update their specifications accordingly.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129-bis			13 - 17 October 2025			Prague, CZ

TSG-RAN WG3 Meeting #130			17 - 21 	November 2025			Dallas, US