3GPP TSG-RAN3 Meeting #128	R3-255848

Bengaluru, IN, 25-29 Aug 2025

Title:	(TP to TS 38.401 BL CR) Introduction of SBFD

Source:	ZTE Corporation, Ericsson, China Telecom, Huawei

Agenda Item:	19.2

Document for:	Other

- 1  	Introduction

This TP captures the RAN3 agreement:

- For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.

- 2  TP to TS 38.401 BL CR

=============================Start of change==============================

7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

In Sub-band full duplex (SBFD) operation, gNB-to-gNB Cross Link Interference (CLI) may also be present. In case of split gNB architecture, the gNB-DU reports gNB-to-gNB CLI related information of its serving cells to the gNB-CU if CLI is detected. The gNB-CU forwards gNB-to-gNB CLI related information received from served gNB-DUs and from neighboring gNBs to each concerned gNB-DU. The gNB-DU should evaluate the received information and may mitigate interference if necessary. For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.

=============================End of change==============================