3GPP TSG-RAN WG3 Meeting #129	R3-255935

Bengaluru, India, 25 – 29 August, 2025

Title:	Reply LS on AI/ML Positioning Case 3a

Response to:	R3-255023 - S2-2505943

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	SA2

CC :

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their LS reply with feedback to RAN3 solutions on case 3a (NW-sided model, AI/ML-assisted positioning).

RAN3 will proceed with solution 1, as described below, in Rel-19.

Solution 1: Positioning Information collection for UEs under positioning

- The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures.

- The gNB(s) determines that data collection is needed for the UE being positioned.

- The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

  - Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. The gNB may use the NRPPa MEASUREMENT REPORT message to indicate that data collection is needed.

- The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed.

RAN3 kindly asks SA2 to add support of related LMF procedures in TS 23.273, as needed.

2. Actions:

To SA2: 	RAN3 kindly asks SA2 to take the above into account and introduce support for solution 1 in TS 23.273, as needed.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129-bis			  	13 - 17 	October 2025							Prague, CZ

TSG-RAN WG3 Meeting #130				  	17 - 21 	November 2025						Dallas, US