3GPP TSG-RAN WG3 Meeting #129	R3-255940

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	12.3

Source:	Huawei, Ericsson, Qualcomm, Nokia

Title:	(TP for Femto BL CRs for TS 38.300) Support of NG mobility for NR Femto

Document for:	Approval

1	Introduction

This TP is to reflect the following agreements in stage 2 specification for the NG mobility for NR Femto:

For the routing of HANDOVER REQUEST message from target NR Femto GW to correct target Femto node in case of NG-based HO, the AMF includes the target global gNB ID in the HANDOVER REQUEST message before sending it to the target NR Femto GW.

Annex ——TP for BL CR on TS 38.300

Start of Change

4.X.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [xx], that the identity used by the NR Femto node is valid;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [xx], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [xx], that the indicated cell identity is valid.

-	In case of NG handover towards an NR Femto node connected to an NR Femto GW, including the Global RAN Node ID of the target NR Femto node in the HANDOVER REQUEST message.

End of Change