3GPP TSG-RAN3 Meeting #129	R3-256050

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Wireless Access Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell, NTTDOCOMO, Samsung, LG Electronics, Huawei, China Telecom, NTT DOCOMO, Airbus, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-09-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support for Wireless Access and Backhaul (WAB) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 3.1: Added the “WAB” abbreviation. Clause 3.2: Added references to the definitions of WAB-related terms. Clause 4.x: Defined the WAB functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No overall description for Wireless Access and Backhaul. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0439 TS 38.413 CR 1263 TS 38.423 CR 1487 TS 38.455 CR 0189 TS 38.305 draft CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev0: Initial version based on R3-255254 Rev1: Update of cover page. Change of user name to “Author” |
| --- | --- |

--- Begin of Changes ---

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

>>>>> SKIP

VR	Virtual Reality

V2X	Vehicle-to-Everything

WAB				Wireless Access Backhaul

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

XR	eXtended Reality

>>>>> SKIP

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

>>>>> SKIP

Uu Relay RLC channel: an RLC channel between L2 U2N Relay UE or MP Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network Relay or for indirect path in case of MP.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

WAB-gNB: as defined in TS 38.401 [4].

WAB-MT: as defined in TS 38.401 [4].

WAB-node: as defined in TS 38.401 [4].

Xn: network interface between NG-RAN nodes.

>>>>> SKIP

## 4.X	Wireless Access Backhaul

Wireless Access Backhaul (WAB) is supported by means of the WAB-node, which provides NR access links to UEs via a gNB functionality, referred to as the WAB-gNB. The WAB-node further supports a WAB-MT function, which terminates an NR Uu interface to a stationary gNB that is used for the backhauling of the WAB-gNB’s traffic. The WAB-node may be stationary or mobile.

Further details of WAB are defined in TS 23.501 [3] and TS 38.401 [4].

--- End of Changes ---