3GPP TSG-RAN WG3 #129	R3-255858

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	11.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # 15_AIRAN_NRDC

Document for:	Approval

# Introduction

CB: # 15_AIRAN_NRDC

- Check remaining issues for NR-DC including UE performance granularity

(Nokia - moderator)

# For the Chairman’s Notes

Agree to the following proposals:

Proposal 1: Reporting of UL/DL Average RLC Level Throughput over Xn for a split gNB is not supported in this version of the specification.

Proposal 2: Agree to the following note: It is up to implementation how MN derives overall UL/DL Average Throughput in case of split bearer.

Proposal 3: Agree that UE Performance is reported from SN to MN per UE granularity (no stage 3 impact).

Proposal 4: Reflect in TS 37.340 the following note:

Note: It is up to implementation how MN derives overall UL/DL Average Throughput in case of split bearer.

Proposal 5: Reflect in TS 38.401 the following:

Reporting of UL/DL Average RLC Level Throughput over Xn for a split gNB is not supported in this version of the specification.

# Discussion

In this CB discussion, the moderator would like to focus on topics that in moderator’s view are critical to complete the aspects related to AI/ML Mobility Optimization in NR-DC in the WI successfully, which is related to UE Performance Reporting from SN to MN in NR-DC.

NR-DC use case can be considered in both split and non-split scenario.

Split Architecture

In case of split architecture, the SN aggregates per DRB measurement information it receives over E1 in order to create a per UE measurement to send to MN. Note that in split architecture, we have agreed that UL/DL Average Throughput measurements are not supported:

DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Therefore, the only measurements that are supported in split architecture scenario correspond to UL/DL Average Packet Loss and UL/DL Average Packet Delay (which are agreed to be measured at PDCP level). So, only those measurements (excluding UL/DL Average UE throughput) can subsequently be aggregated by the SN and reported towards the MN.

P1: Split gNB cannot report RLC level Throughput over Xn

Non-split Architecture

In RAN3 we agreed to measure UE Performance at PDCP layer. We think that this agreement should also hold in case of non-split architecture.  One of the motivations of this approach, besides the benefits of providing of service level performance, is that it simplified our signalling to request and receive UE Performance in some scenarios such as in split architecture. In our view the same benefit is observed in case of NR-DC in that if the measurements are collected at PDCP layer, then the measurements can always be collected from a single layer and single node, i.e., from PDCP Layer in Master Node in case of MN-Terminated MCG, SCG or Split bearer and from PDCP Layer in Secondary Node in case of SN-Terminated MCG, SCG or Split bearer.

In addition, in order for a node to determine whether it is better to enable NR-DC or not it needs to evaluate UE Performance of itself (in single connectivity) and in Dual Connectivity. However, in case of split bearer, it will not be feasible to aggregate the different RLC components of the throughput measurement at the MN, making the comparison not possible.

Q2: Exclude UL/DL Average Throughput reporting from SN to MN in case of split bearer in Rel-19?

CATT, E///: throughput can be also measured in case of split bearer.  SN knows the timing and configuration.

Common understanding of the group is that it can be done by implementation.

P2: Agree to the following Note: MN derives overall UL/DL Average Throughput from the SN by implementation.

P3: Agree that UE Performance is reported from SN to MN per UE granularity (no stage 3 impact).

Q4:  Stage 2 TPs?

- TS 38.401

- TS 37.340: Capture Note

P4: Reflect in TS 37.340 the following note:

Note: MN derives overall UL/DL Average Throughput from the SN by implementation.

P5: Reflect in TS 38.300 the following: Split gNB cannot report RLC Level Throughput over Xn.

# Conclusion, Recommendations [if needed]

If needed

# References