3GPP TSG RAN WG3 #129		  R3-255902

Bengaluru, India, Aug. 25th – 29th, 2025

Title:	(TP for SON BL CR for TS 38.473) Support for LTM MRO without RLF Report

Agenda item:	10.2

Source:	Google, Ericsson, NEC, Lenovo, CATT

Document for:	Discussion and Decision

# TP for LTM MRO for TS 38.473

Start of Change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Reporting without RLF report |  | 0..1 |  |  |  |  |
| >NR CGI | M |  | 9.3.1.12 | NR CGI for the re-establishment or recovery cell. | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| [……] |  |  |  |  |  |  |

Next Change

### 9.4.4	PDU Definitions

<SKIP>

F1U-PathFailure,

LTMResetInformation,

CuBeamFailureRecoveryInformation,

DuBeamFailureRecoveryInformation，

LTMFailurewithoutRLFReport

<SKIP>

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-CuBeamFailureRecoveryInformation,

id-DuBeamFailureRecoveryInformation,

id-LTMFailurewithoutRLFReport

<SKIP>

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-CuBeamFailureRecoveryInformation			CRITICALITY ignore 	TYPE CuBeamFailureRecoveryInformation  		PRESENCE optional }|

{ ID id-LTMFailurewithoutRLFReport					CRITICALITY ignore 	TYPE LTMFailurewithoutRLFReport  		PRESENCE optional },

...

}

Next Change

### 9.4.5	Information Element Definitions

<SKIP>

CuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { CuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

CuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMFailurewithoutRLFReport	::= SEQUENCE {

sourceCellPCI			INTEGER (0..1007),

targetCellPCI			INTEGER (0..1007)		OPTIONAL,

reestRecoveryCGI			NRCGI,

c-RNTI				C-RNTI			 	OPTIONAL,

rLFReportFailureType		RLFReportFailureType		OPTIONAL, -- this IE may need to be revised

iE-Extensions		ProtocolExtensionContainer { {LTMFailurewithoutRLFReport-ExtIEs} } OPTIONAL,

...

}

LTMFailurewithoutRLFReport-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Change