3GPP TSG-RAN WG3 Meeting #129	R3-256021

Bengaluru, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0156 | rev | 11 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of XR enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, BT, T-Mobile USA, Charter Communications, Qualcomm, Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC, ZTE, CATT, Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support for L4S in NR-DC, and enhancements for XR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates for the support of XR enhancements and L4S in NR-DC - Support for DL PSI based discarding. - Add the support for BSSize, TTNB, and available bitrate report. - Enhancements to support timely RLC retransmissions and Unnecessary RLC Retransmission Avoidance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | XR and L4S in NR-DC is not supported. DL PSI based discarding not supported. XR enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4.1.1, 5.4.3.1, 5.5.2.1, 5.5.2.3, 5.5.3.a-5.5.3.r (all new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1269 TS 37.340 Draft CR TS 38.473 CR 1485 TS 38.300 Draft CR TS 38.413 CR 1282 TS 37.483 CR 0171 TS 38.415 CR 0056 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: update text Rev 2: update cover page Rev 3: implementation of R3-244768 for support of DL PSI based discarding Rev 4: resubmission to RAN3#125bis Rev 5: update the title of the CR. Rev 6: resubmission to RAN3#127bis Rev 7: implementation of R3-252388 Rev 8: resubmission to RAN3#128 Rev 9: implementation of R3-253930 Rev 10: resubmission to RAN3#129 Rev 11: implementation of R3-255861 and R3-255863 |  |  |  |  |  |  |  |  |

-------------------------------------- CHANGES START HERE --------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSSize	Burst Size

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MBS	Multicast/Broadcast Service

MR-DC	Multi-RAT Dual Connectivity

MRB	MBS Radio Bearer

PTM	Point To Multipoint

TTNB				Time To Next Burst

NEXT CHANGE

### 5.4.1	Transfer of Downlink User Data

      - 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. The corresponding node may stop transmission or retransmission of the NR PDCP PDU as indicated in the DL discard NR PDCP PDU SN, or stop transmission or retransmission of one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The node hosting the NR PDCP entity can indicate to the corresponding node to either retransmit all NR PDCP PDUs up to and including a defined DL retransmission NR PDCP PDU SN or retransmit one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

The node hosting the NR PDCP entity can indicate to the corresponding node to either trigger polling for all NR PDCP PDUs up to and including a defined DL polling NR PDCP PDU SN or trigger polling for one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

NEXT CHANGE

### 5.4.3	Transfer of Assistance Information

#### 5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or it shall take this information into account when sending the UL Congestion Information and/or DL Congestion information  to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

The ASSISTANCE INFORMATION DATA frame may include the DL PSI Based Discard Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or deactivate DL PSI based PDCP discard. The node hosting the NR PDCP entity may take this information into account to decide whether to activate or deactivate DL PSI based PDCP discard.

The ASSISTANCE INFORMATION DATA frame may include UL Available Bitrate information and/or DL Available Bitrate information generated by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account and report to UPF as specified in TS 23.501 [8].

Figure 5.4.3.1-1: Successful Transfer of Assistance Information

NEXT CHANGE

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |  |
| PDU Type (=0) |  |  |  |  |  |  |  | Spare |  | DL Discard Blocks |  | DL Flush |  |  | Report polling | 1 |
| New IE Flag | BSSI |  |  | TTNBI |  | Request OutofSeq Report |  | Report Delivered |  | User data existence flag |  | Assistance Info. Report Polling Flag |  |  | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| BSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| New IE Flag 7(E) |  | New IE Flag 6 |  |  | New IE Flag 5 |  | New IE Flag 4 |  | New IE Flag 3 |  | New IE Flag 2 |  | New IE Flag 1 | New IE Flag 0 |  | 0 or 1 New IE Flags Octet |
| DL retransmission NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL retransmission Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL retransmission NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmission Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL retransmission NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmission Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL polling NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL polling Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL polling NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Polling Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL polling NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Polling Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

The New IE Flag in bit 7 of 2nd octet in DL USER DATA (PDU Type 0) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL retransmission NR PDCP PDU SN is present (1) or not (0).

Bit 1 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL retransmission Number of blocks, DL retransmission NR PDCP PDU SN start and Retransmission Block size are present (1) or not (0).

Bit 2 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL polling NR PDCP PDU SN is present (1) or not (0).

Bit 3 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL polling Number of blocks, DL polling NR PDCP PDU SN start and Polling Block size are present (1) or not (0).

NEXT CHANGE

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  |  | PDCP Dupl. Ind. | Assistance Info. Ind. | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  | UL Available Bitrate Ind | DL Available Bitrate Ind | DL PSI Based Discard Ind. | DL PSI Based Discard Activation Suggestion | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Bitrate |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Available Bitrate |  |  |  |  |  |  |  |  | 0 or 4 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

NEXT CHANGE

### 5.5.3	Coding of information elements in frames

NOTE 1:	In this section, information elements are also applicable to E-UTRA PDCP unless specified otherwise. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP unless specified otherwise.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 5.5.3.58	Feedback Delay Result

Description: This parameter indicates the feedback delay time measured at the corresponding node in milliseconds from the time of reception of the Polling Frame that triggered transmission of the DL DATA DELIVERY STATUS to the time of transmission of the DL DATA DELIVERY STATUS. It is encoded as an Unsigned32 binary integer value.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.a	DL PSI Based Discard Indication

Description: This field indicates the presence of the DL PSI Based Discard Activation Suggestion. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL PSI Based Discard Activation Suggestion not present, 1= DL PSI Based Discard Activation Suggestion present}.

Field length: 1 bit.

#### 5.5.3.b	DL PSI Based Discard Activation Suggestion

Description: This parameter indicates the suggestion given by the corresponding node on whether DL PSI based PDCP discard should be activated or not. This information element is not applicable to E-UTRA PDCP.

Value range: {0= Not activate, 1= Activate}.

Field length: 1 bit.

#### 5.5.3.c	UL Available Bitrate Ind

Description: This parameter indicates the presence of UL Available Bitrate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= UL Available Bitrate not present, 1= UL Available Bitrate present}.

Field length: 1 bit.

#### 5.5.3.d	DL Available Bitrate Ind

Description: This parameter indicates the presence of DL Available Bitrate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL Available Bitrate not present, 1= DL Available Bitrate present}.

Field length: 1 bit.

#### 5.5.3.e	UL Available Bitrate

Description: This field indicates the value of uplink available bitrate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is kbps.

Value range: {0.. 4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.f	DL Available Bitrate

Description: This field indicates the value of downlink available bitrate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is kbps.

Value range: {0.. 4,000,000,000}.

Field length: 4 octets.

#### 5.5.3.g	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.h	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.i	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.3.j	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

#### 5.5.3.k	DL retransmission NR PDCP PDU SN

Description: This parameter indicates the downlink NR PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.l	DL retransmission NR PDCP PDU SN start

Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.m	DL retransmission Number of blocks

Description: This parameter indicates the number of NR PDCP PDU blocks to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.n	Retransmission Block size

Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {1..255}.

Field length: 1 octet.

#### 5.5.3.o	DL polling NR PDCP PDU SN

Description: This parameter indicates the downlink NR PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.p	DL polling NR PDCP PDU SN start

Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.q	DL polling Number of blocks

Description: This parameter indicates the number of NR PDCP PDU blocks to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.r	Polling Block size

Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {1..255}.

Field length: 1 octet.

CHANGES END