3GPP TSG RAN3 meeting #129-bis													R3-25xxxx

Prague, CZ, 13 - 17 October 2025

Agenda Item:			4

Source:				ETSI MCC

Title:					(draft) RAN3 #129 Meeting Report

Bengaluru, India, 25/08/2025 to 29/08/2025

Document for:		Approval

Contents:

1	Opening of the meeting	3

2	Reminders	3

2.1	IPR Declaration	3

2.2	Statement of Antitrust Compliance	3

2.3	Consensus Principles Reminder	3

2.4	Responsible IT Behavior	3

2.5	Additional reminders	3

3	Approval of the Agenda	3

4	Approval of the minutes from previous meetings	3

5	Documents for immediate consideration	4

6	Organizational topics	4

7	General, protocol principles and issues	4

8	Incoming LSs	4

8.1	New Incoming LSs	4

8.2	LSin received during the meeting	15

8.3	Left over LSs / pending actions	16

9	Corrections to Rel-18 or earlier releases	18

9.1	LTE	18

9.2	NR	20

10	Data Collection for SON_MDT in NR standalone and MR-DC WI (RAN3-led)	28

10.1	General	28

10.2	MRO Enhancements	32

10.3	SON/MDT for NTN and Slicing	39

10.3.1	Intra-NTN Mobility	39

10.3.2	Network Slicing	42

10.4	R18 leftovers	43

11	AI/ML for NG-RAN WI (RAN3-led)	43

11.1	General	43

11.2	AI/ML enabled Slicing	48

11.3	AI/ML enabled Coverage and Capacity Optimization	50

11.4	R18 leftovers	56

12	Additional topological enhancements for NR WI (RAN3-led)	63

12.1	General	63

12.2	Wireless Access Backhaul (WAB)	66

12.3	5G Femto	71

13	NR Mobility Enhancements WI	73

13.1	General	73

13.2	Support for inter-CU LTM	77

13.3	Support for Conditional LTM	85

14	NR NTN Enhancements WI	87

14.1	General	87

14.2	Support MBS broadcast service	88

14.3	Support of Regenerative payload	88

15	IoT NTN Enhancements WI	94

15.1	General	94

15.2	Support Full eNB as Regenerative Payload	95

16	Ambient IoT WI	99

16.1	General	99

16.2	RAN Architecture and Procedures	101

16.3	Location Report	108

17	Network ES Enhancements WI	108

17.1	General	108

17.2	Support on-demand SSB SCell operation	111

17.3	Support on-demand SIB1 for UEs	112

17.4	Others	116

18	NR Low Power WUS/WUR WI	117

18.1	General	117

18.2	Support LP-WUS Indicating Paging Monitoring	119

19	Evolution of Duplex Operation WI	124

19.1	General	124

19.2	Support CLI Handling	127

20	AI/ML for Air Interface WI	133

20.1	General	133

20.2	Support Positioning Accuracy Enhancements	136

21	NR XR Enhancements WI	142

21.1	General	142

21.2	Support XR in DC	146

21.3	Others	146

22	NR Sidelink Multi-hop Relay WI	151

22.1	General	151

22.2	Support multi-hop Layer-2 UE-to-Network relay	153

23	LTE-based 5G Broadcast WI	155

23.1	General	155

23.2	Signalling Support	155

31	Corrections and Enhancements to Rel-19	156

31.1	Corrections	156

31.2	Enhancements	157

31.3	Endorsed TEI19 CRs Review	157

32	Any other business	166

33	Closing of the meeting	166

## 1	Opening of the meeting

3GPP TSG RAN3 Chair Mr. Sean Kelley (Nokia) opened the f2f meeting RAN3 #129 in Bengaluru, India on Monday August 25th, 2025 at 9am.

## 2	Reminders

### 2.1	IPR Declaration

I draw your attention to your obligations under the 3GPP Partner Organizations’ IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

Delegates are asked to take note that they are thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become, essential in respect of the work of 3GPP.

to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (see: http://ipr.etsi.org/).

### 2.2	Statement of Antitrust Compliance

I also draw your attention to the fact that 3GPP activities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG/SWG meeting including the Chair and Vice Chairs. In case of question I recommend that you contact your legal counsel.

The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP.

Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG/SWG meetings is important to allow for full and fair consideration of such matters.

### 2.3	Consensus Principles Reminder

The attention of the delegates to the meeting is drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members are encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership will conduct the present meeting in a manner whereby informal methods of reaching consensus are encouraged, whilst ensuring that well justified concerns are taken into account.

### 2.4	Responsible IT Behavior

We all share meeting IT resources with one another. Delegates should restrict their IT usage to things which are essential for the meeting, and they:

- shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws.

- shall not engage in non-work-related activities that consume excessive bandwidth or cause significant network performance degradation.

And most importantly:

1. DON’T place your WiFi device in ad-hoc mode;

2. DON’T set up a personal hotspot in the meeting room;

3. DO try 802.11a if your device supports it;

4. DON’T manually allocate an IP address;

5. DON’T stream video, play online games, or download huge files;

6. DON’T use packet probing software (e.g., packet sniffers or port scanners) which clogs the local network.

### 2.5	Additional reminders

This is an ordinary face-to-face meeting with 1-way remote access.

1. All agreed TDocs must be provided during the meeting week, i.e., BEFORE the end of the meeting. In order to continue with the principle of “agreed unseen”, please ensure that all such TDocs are uploaded in time and reflect exactly the agreed changes.

2. During physical meetings, prefer face-to-face offline discussion to e-mail discussion.

3. When a CB is set up, e.g.:

CB: # 1_Name

- topics of the offline discussion

(Company Owner - moderator)

Rev in R3-xxxxxx

Summary of offline disc R3-xxxxxy

- Create a folder in “Inbox/Drafts/1_Name” with the assigned CB number (1) and name;

- Upload all drafts, corrections, revisions, etc. in the same folder “Inbox/Drafts/1_Name”;

- Avoid sending drafts via e-mail or on the reflector!

- When sending e-mails, do not attach any document, and please minimize e-mail discussion (e.g. it is enough to announce start of discussion, availability of drafts on server, support for a document, discussion conclusion).

- It is highly beneficial if the summary of offline discussion contains proposals for “official” group conclusions, e.g. “propose to agree R3-xxxxxx”, “propose to agree that….”, “no agreement”, “to be continued”, etc.

4. To encourage the use of pCRs, if there are discussion papers and pCRs from the same company on the same topic, only the pCRs will be treated.

5. Papers submitted to the wrong AI will not be treated.

6. When subsections are available, please do not submit papers to the “top level” AI. If you think none of the available subsections fits your contribution, then it should go to the “Others” subsection. Any papers submitted to the “top level” AIs should not be expected to be treated.

7. To save time, incoming LSs which have no action for RAN3 will not be treated unless they are flagged to the Chair before the start of the meeting.

8. QUOTAS – When a quota is indicated for an Agenda Item, each company may submit up to that number of contributions in total across all its sub-Agenda Items. Please refer to the example at the end of this document. Quota rules are to be maintained R3-221096 (revised from R3-200133) and continue to be the basis for working with quotas in RAN3.

9. RAN3 breakout room: Only a CB moderator or SI/WI rapporteur may request the RAN3 breakout room (via MCC), and should follow these guidelines to ensure fair access:

- The offline session time slot should be announced via the RAN3 email reflector.

- The maximum total duration of all CBs for a given SI/WI is 2 hours.

- 1-way remote access may be provided via GoToWebinar (GTW), on a best-effort basis.

Some suggestions for better RAN3 meetings can also be found here.

## 3	Approval of the Agenda

R3-255001	RAN3#129 Meeting Agenda

Type: agenda		For: Approval
					Source: RAN3 Chair

Decision: 		The document was approved.

## 4	Approval of the minutes from previous meetings

R3-255002	RAN3#128 Meeting Report

Type: report		For: Approval
					Source: ETSI-MCC

Decision: 		The document was approved.

## 5	Documents for immediate consideration

R3-255003	Guidelines for RAN3 Meetings

Type: discussion		For: Endorsement
					Source: RAN3 Chair, RAN3 Vice-Chairs

Decision: 		The document was noted.

## 6	Organizational topics

R3-255141	TDoc Quota Rules

Type: discussion		For: Decision
					Source: RAN3 Chair

Discussion:

Continue to be the basis for working with TDoc quotas in RAN3

Decision: 		The document was noted.

## 7	General, protocol principles and issues

R3-255004	TR 30.531 v1.59.0 Work Plan and Working Procedures - RAN WG3

Type: draft TR		For: Endorsement
					30.531 v1.58.0
					Source: ETSI-MCC

Decision: 		The document was endorsed.

## 8	Incoming LSs

### 8.1	New Incoming LSs

[7 MHz channel bandwidth]

R3-255017	Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth

Type: LS in		For: Discussion
					Original outgoing LS: R4-2508088, to RAN2, cc RAN3
					Source: RAN4(T-Mobile)

Decision: 		The document was noted.

R3-255339	Signalling for 7 MHz Channel Bandwidth

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was noted.

R3-255340	Introduction of 7 MHz Channel Bandwidth

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1505  Cat: B (Rel-19)

					Source: Huawei

Discussion:

-	Add Nokia, ERICSSON, ZTE, Qualcomm, Samsung, Jio Platforms

Decision: 		The document was revised to R3-255752.

R3-255752	Introduction of 7 MHz Channel Bandwidth

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1505  rev 1 Cat: B (Rel-19)

					Source: Huawei, Nokia, Ericsson, ZTE, Qualcomm, Samsung, China Telecom, Jio Platforms

(Replaces R3-255340)

Decision: 		The document was agreed.

R3-255341	Introduction of 7 MHz Channel Bandwidth

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1580  Cat: B (Rel-19)

					Source: Huawei

Abstract:

Technically Endorsed

Decision: 		The document was revised to R3-255753.

R3-255753	Introduction of 7 MHz Channel Bandwidth

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1580  rev 1 Cat: B (Rel-19)

					Source: Huawei, Nokia, Ericsson, ZTE, Qualcomm, Samsung, China Telecom, Jio Platforms

(Replaces R3-255341)

Decision: 		The document was agreed.

R3-255342	[draft] Reply LS on Signalling for 7 MHz Channel Bandwidth

Type: LS out		For: Approval
					to RAN4, cc RAN2
					Source: Huawei

Decision: 		The document was not treated.

R3-255144	Introduction of 7MHz channel bandwidth

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1501  Cat: B (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255145	Introduction of 7MHz channel bandwidth

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1576  Cat: B (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255146	[Draft] Reply LS on Signalling for 7 MHz Channel Bandwidth

Type: LS out		For: Approval
					to RAN4, RAN2
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255303	Discussion on 7MHz channel bandwidth

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-255304	TP (BL CR TS38.473) Support for 7MHz transmission bandwidth

Type: other		For: Agreement
					38.473 v..
					Source: Nokia, T-Mobile USA

Decision: 		The document was not treated.

R3-255308	TP (BL CR TS38.423) Support for 7MHz transmission bandwidth

Type: other		For: Agreement
					38.423 v..
					Source: Nokia, T-Mobile USA

Decision: 		The document was not treated.

Introduce a new value of 35 RB to the NR Transmission Bandwidth IE in XnAP and the Transmission Bandwidth IE in F1AP, according to the newly approved value in TS 38.104.

The introduction of 35 PRB transmission bandwidth over network interfaces starts from R19.

RAN3 to send reply LS to RAN4 to inform the introduced new value of transmission bandwidth over Xn interface and F1 interface, with RAN plenary in the CC.

Nokia, ZTE: CRs looks agreeable

ERICSSON: Should we wait to agree to the CRs after RAN4 agreement?

QUALCOMM: How do we handle early implementation (pre-R19 UEs)?

ERICSSON: We do not have magic sentence in RAN3 for early implementation.

[Emergency call back and paging]

R3-255022	Reply LS on emergency call back and paging

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505938, to RAN3, cc RAN2, CT1, TSG RAN
					Source: SA2(Qualcomm)

Decision: 		The document was noted.

R3-255552	Discussion on the LS from SA2 on emergency call back and paging

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was noted.

R3-255553	Clarification on emergency call back and UE RRC state

Type: draftCR		For: Endorsement
					38.300 v17.13.0
					Source: Ericsson

Decision: 		The document was revised to R3-255765.

R3-255765	Clarification on emergency call back and UE RRC state

Type: draftCR		For: Endorsement
					38.300 v17.13.0
					Source: Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO

(Replaces R3-255553)

Decision: 		The document was endorsed.

R3-255554	Clarification on emergency call back and UE RRC state

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson

Decision: 		The document was revised to R3-255766.

R3-255766	Clarification on emergency call back and UE RRC state

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO

(Replaces R3-255554)

Decision: 		The document was endorsed.

R3-255555	[DRAFT] Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc RAN2, CT1, RAN
					Source: Ericsson

Abstract:

Att: xx CRs

Decision: 		The document was not treated.

R3-255222	Handling of Emergency call back and Paging

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-255223	Response LS on Emergency call back and paging

Type: LS out		For: Approval
					to SA2
					Source: Nokia

Decision: 		The document was not treated.

R3-255235	Emergency call back and paging

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was not treated.

R3-255236	[draft] Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc RAN2, CT1
					Source: Huawei

Decision: 		The document was not treated.

R3-255259	Further discussion on emergency call back and paging

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255260	[Draft] Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc CT1, RAN2, TSG RAN
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255309	[Draft] Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc CT1, RAN2, RAN
					Source: Qualcomm India Pvt Ltd

Decision: 		The document was revised to R3-255769.

R3-255769	Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc RAN2, CT1, TSG RAN
					Source: RAN3(Qualcomm)

(Replaces R3-255309)

Decision: 		The document was revised to R3-255883.

R3-255883	Reply LS on emergency call back and paging

Type: LS out		For: Approval
					to SA2, cc RAN2, CT1, TSG RAN
					Source: RAN3(Qualcomm)

(Replaces R3-255769)

Decision: 		The document was approved.

Q1：Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session?  YES, with further explanation

Q2: If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF?  NO, with further explanation

Nokia: The wording in ERICSSON CR is still incomplete, all RRC states are allowed.

QUALCOMM: Release to RRC_IDLE does not seem sensible

ZTE: CR does not seem needed, it is all up to implementation

HUAWEI: CR seems to overspecify

SAMSUNG: CR is fine since it aligns with RAN2 LS

CB # 1_EmergencyCallBack

- Further discussion whether clarifying the Note is agreeable

- Reply LS, YES/NO with suitable clarifications

- Mirror note in 36.300 in R3-255557 from Rel-17

(Qualcomm)

[MBS communication service type]

R3-255028	Draft LS to RAN3 on MBS Communication Service Type

Type: LS in		For: Discussion
					Original outgoing LS: S4-251096, to RAN3, cc RAN2, SA5
					Source: SA4(Nokia)

Abstract:

Att: S4-251095

Decision: 		The document was noted.

R3-255166	Discussion and Draft Reply LS on MBS Communication Service Type for QMC

Type: discussion		For: Agreement
					Source: Ericsson

Decision: 		The document was revised to R3-255756.

R3-255756	Discussion and Draft Reply LS on MBS Communication Service Type for QMC

Type: discussion		For: Agreement
					Source: Ericsson

(Replaces R3-255166)

Decision: 		The document was noted.

R3-255167	MBS Communication Service Type for QMC

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1287  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-255755.

R3-255755	MBS Communication Service Type for QMC

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1287  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R3-255167)

Decision: 		The document was revised to R3-255819.

R3-255819	MBS Communication Service Type for QMC

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1287  rev 2 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R3-255755)

Decision: 		The document was noted.

R3-255168	MBS Communication Service Type for QMC

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1502  Cat: F (Rel-18)

					Source: Ericsson

Decision: 		The document was revised to R3-255754.

R3-255754	MBS Communication Service Type for QMC

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1502  rev 1 Cat: F (Rel-18)

					Source: Ericsson

(Replaces R3-255168)

Decision: 		The document was noted.

R3-255584	Discussion on MBS Communication Service Type for QMC

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-255585	[Draft] Reply LS on MBS Communication Service Type for QMC

Type: LS out		For: Approval
					to SA4, RAN2, cc SA5
					Source: Nokia

Decision: 		The document was revised to R3-255840.

R3-255840	Reply LS on MBS Communication Service Type for QMC

Type: LS out		For: Approval
					to SA4, cc RAN2, SA5
					Source: RAN3(Nokia)

(Replaces R3-255585)

Decision: 		The document was revised to R3-255884.

R3-255884	Reply LS on MBS Communication Service Type for QMC

Type: LS out		For: Approval
					to SA4, cc RAN2, SA5
					Source: RAN3(Nokia)

(Replaces R3-255840)

Discussion:

-	Add “RAN3 has several follow up questions.” at end of 1st paragraph

-	removed word “additionally”

Decision: 		The document was revised to R3-255896.

R3-255896	Reply LS on MBS Communication Service Type

Type: LS out		For: Approval
					to SA4, cc RAN2, SA5
					Source: RAN3(Nokia)

(Replaces R3-255884)

Decision: 		The document was approved.

R3-255676	Discussion on MBS Communication Service Type

Type: discussion		For: Discussion
					Source: Huawei

Decision: 		The document was noted.

R3-255677	Clarification on MBS communication service type for QMC configuration

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1309  Cat: F (Rel-18)

					Source: Huawei

Decision: 		The document was not treated.

R3-255678	Clarification on MBS communication service type for QMC configuration

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1507  Cat: F (Rel-18)

					Source: Huawei

Decision: 		The document was not treated.

R3-255690	Discussion on QoE MBS communication service types

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-255687	Correction on stage 2 QoE communication service type description

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255688	Correction on QoE communication service type

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1310  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255689	Correction on QoE communication service type

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1508  Cat: F (Rel-18)

					Source: ZTE Corporation

Decision: 		The document was revised to R3-255837.

R3-255837	Correction on QoE communication service type

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1508  rev 1 Cat: F (Rel-18)

					Source: ZTE Corporation

(Replaces R3-255689)

Decision: 		The document was noted.

R3-255686	[Draft]Reply LS to SA4 on MBS Communication Service Type

Type: LS out		For: Approval
					to SA4, cc RAN2,SA5
					Source: ZTE Corporation

Decision: 		The document was not treated.

Ericsson: two new codepoints (“unicast” and “all”)

Align the codepoints of MBS Communication Service Type with the codepoints of @communicationServiceType in TS 26.247.

ENUMERATED (broadcast, multicast,..., unicast, all)

Send an LS asking SA4 to clarify:

•	Whether the text in TS 26.247 implies that the @communicationServiceType can have more than one value at the same time.

•	Whether the @communicationServiceType can have a default value even though the Use is “O”.

Nokia: RAN2 involvement needed

Send a reply LS to RAN2, SA4 cc SA5 describing solution where the gNB receives additional configuration information from the UE so that QMC sessions applicable to any combination of communication service types can be supported.

In the LS, ask feedback from RAN2 whether such solution is feasible, including in which 3GPP release.

Huawei: no new codepoints

The current RAN3 spec already defined a mandatory IE representing service type of unicast.

The current RAN3 spec already support to perform QMC task for all the service types towards the same UE.

ZTE: one new codepoint (“all”)

Introduce new codepoint “all” for MBS Communication Service Type IE.With this “all” for MBS Communication Service Type IE, the QoE measurement may be triggered when UE is using either unicast, multicast, or broadcast service for one QoE configuration.

Update semantic description for MBS Communication Service Type IE, without the presence of this IE, the QoE measurement can only be triggered by unicast service.

CB # 2_MBScommType

- Further clarification needed from SA4 and/or RAN2?

(Nokia)

[UE radio capability for paging]

R3-255025	Reply LS on Handling of UE Radio Capability for Paging

Type: LS in		For: Discussion
					Original outgoing LS: S2-2506082, to RAN3, cc RAN2
					Source: SA2(Ericsson)

Decision: 		The document was noted.

R3-255712	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1313  Cat: F (Rel-16)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

Decision: 		The document was revised to R3-255820.

R3-255820	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1313  rev 1 Cat: F (Rel-16)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

(Replaces R3-255712)

Discussion:

-	Change source to TSG to “R3”

Decision: 		The document was revised to R3-255885.

R3-255885	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1313  rev 2 Cat: F (Rel-16)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson, Nokia, Jio Platforms

(Replaces R3-255820)

Decision: 		The document was agreed.

R3-255713	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1314  Cat: A (Rel-17)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

Decision: 		The document was revised to R3-255821.

R3-255821	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1314  rev 1 Cat: A (Rel-17)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

(Replaces R3-255713)

Discussion:

-	Change source to TSG to “R3”

-	Add “and” before “use it for subsequent RAN paging”

Decision: 		The document was revised to R3-255886.

R3-255886	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1314  rev 2 Cat: A (Rel-17)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson, Nokia, Jio Platforms

(Replaces R3-255821)

Decision: 		The document was agreed.

R3-255714	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1315  Cat: A (Rel-18)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

Decision: 		The document was revised to R3-255822.

R3-255822	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1315  rev 1 Cat: A (Rel-18)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson

(Replaces R3-255714)

Discussion:

-	Change source to TSG to “R3”

-	Add “and” before “use it for subsequent RAN paging”

Decision: 		The document was revised to R3-255887.

R3-255887	Correction on UE radio capability for paging information

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1315  rev 2 Cat: A (Rel-18)

					Source: CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson, Nokia, Jio Platforms

(Replaces R3-255822)

Decision: 		The document was agreed.

R3-255521	Handling of Radio Capability for Paging

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was noted.

R3-255524	Correction of handling UE radio capability for paging

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1297  Cat: F (Rel-16)

					Source: Nokia

Decision: 		The document was noted.

R3-255525	Correction of handling UE radio capability for paging

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1298  Cat: A (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R3-255526	Correction of handling UE radio capability for paging

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1299  Cat: A (Rel-18)

					Source: Nokia

Decision: 		The document was not treated.

R3-255522	Response LS on handling of UE radio capability for paging

Type: LS out		For: Approval
					to SA2, cc RAN2
					Source: Nokia

Abstract:

Att: R3-255524, R3-255525, R3-255526

Decision: 		The document was not treated.

R3-255651	Handling of UE Radio Capability for Paging

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was noted.

RAN3 to agree that when the AMF receives the new UE Radio Capability for Paging IE in the UE RADIO CAPABILITY INFO INDICATION message, the AMF replaces the currently stored IE by the new one.

Nokia: delta with CMCC is “including all the RATs supported by the UE known by the gNB”

HUAWEI: gNB should report all the RATs that the UE supports

ERICSSON, QUALCOMM, SAMSUNG, CATT: Same view as Huawei

CB # 3_UEradioCapPaging

- Check text for RRC_INACTIVE case

- Check release (R16 or R17)

(CMCC)

[PWS over satellite NG-RAN]

R3-255018	Reply LS from RAN on removal of support of PWS over satellite NG-RAN in Rel-17 and 18

Type: LS in		For: Discussion
					Original outgoing LS: RP-251859, to CT1, SA1, CT, SA, RAN3, RAN2, cc SA2, CT4
					Source: TSG RAN(AALYRIA)

Decision: 		The document was noted.

R3-255019	Reply LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505538, to RAN2, CT1, cc SA3, RAN3, SA1
					Source: SA2(Samsung)

Abstract:

Att: S2-2505537

Decision: 		The document was noted.

R3-255538	PWS Support in NTN

Type: discussion		For: Discussion
					Source: Ericsson, Thales, Huawei

Decision: 		The document was noted.

R3-255142	Pre-release Feature Removal

Type: discussion		For: (not specified)
					Source: ZTE  Corporation

Decision: 		The document was noted.

R3-255143	pCR on Pre-release feature removal process

Type: pCR		For: Endorsement
					30.531 v1.58.0
					Source: ZTE  Corporation

Decision: 		The document was not treated.

Regardless of the specific topic at hand, for future discussions RAN3 should consider that any normative reference to functionality owned by other WGs, unless strictly needed, will create unnecessary dependencies and be detrimental to specification maintainability.

RAN3 should wait for CT1 decision before taking any further action.

[Other]

R3-255007	LS on Rel-19 higher layers parameters list Post RAN1#121

Type: LS in		For: Discussion
					Original outgoing LS: R1-2503242, to RAN WG2, RAN WG3, cc RAN WG4
					Source: RAN1(Ericsson)

Abstract:

Att: R1-2503243

Decision: 		The document was noted.

R3-255012	Reply LS on Number of UEs in RRC_INACTIVE state with data transmission

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504746, to RAN3, SA5, cc -
					Source: RAN2(chinatelecom)

Abstract:

Att: R2-2504742

Decision: 		The document was noted.

R3-255741	Consideration on Muting of always-on signals in 5G broadcast [5GB_CASMuting]

Type: discussion		For: Approval
					Source: Huawei, Qualcomm Incorporated, EBU

Decision: 		The document was noted.

R3-255742	Support of Muting of always-on signals in 5G broadcast [5GB_CASMuting]

Type: CR		For: Agreement
					36.443 v18.0.0	  CR-0134  Cat: B (Rel-19)

					Source: Huawei, Qualcomm Incorporated, EBU

Decision: 		The document was revised to R3-255773.

R3-255773	Support of Muting of always-on signals in 5G broadcast [5GB_CASMuting]

Type: CR		For: Agreement
					36.443 v18.0.0	  CR-0134  rev 1 Cat: B (Rel-19)

					Source: Huawei, Qualcomm Incorporated, EBU

(Replaces R3-255742)

Decision: 		The document was agreed.

MCE makes decision on the values of the K_CAS and the N_CAS, and informs to the eNBs via M2AP.

Include the CAS Muting Parameters IE (includes sub-IEs K_CAS and N_CAS) in the MBSFN Area Configuration Item Ies IE in the M2AP: MBMS SCHEDULING INFORMATION message.

Define the new K_CAS IE as INTEGER (4..63, ...), and define the new N_CAS IE as ENUMERATED (n2, n4, n8, n16, ...).

CB # 4_5GB_CASMuting

- Check status in RAN2

- Check alignment of M2AP CR with RRC CR

(Huawei)

### 8.2	LSin received during the meeting

R3-255881	Reply LS on security parameter in paging request message

Type: LS in		For: discussion
					Original outgoing LS: R2-2506465, to SA3, cc RAN3, SA2
					Source: RAN2(Huawei)

Decision: 		The document was noted.

### 8.3	Left over LSs / pending actions

[UAV]

R3-255352	Further discussion on UAV altitude information reporting

Type: discussion		For: Agreement
					Source: Huawei, Nokia

Decision: 		The document was noted.

R3-255353	Support Aerial UE Flight Information Reporting to CN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1290  Cat: B (Rel-19)

					Source: Huawei, Nokia

Decision: 		The document was not treated.

R3-255191	[DRAFT] Reply LS on clarification of UAV regulation from SA2

Type: LS out		For: Approval
					to SA2
					Source: Nokia, Huawei

Decision: 		The document was not treated.

R3-255582	Support Aerial UE Flight Information Reporting to CN

Type: discussion		For: (not specified)
					Source: Ericsson, CATT

Decision: 		The document was noted.

R3-255520	Support Aerial UE Flight Information Failure Reporting in FAILURE INDICATION message

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1296  Cat: B (Rel-19)

					Source: Ericsson, CMCC, CATT

Decision: 		The document was revised to R3-255817.

R3-255817	Support Aerial UE Flight Information Failure Reporting in FAILURE INDICATION message

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1296  rev 1 Cat: B (Rel-19)

					Source: Ericsson, CMCC, CATT

(Replaces R3-255520)

Discussion:

-	Capitalize words in IE name “Aerial UE flight information ongoing reporting failed”

Decision: 		The document was revised to R3-255898.

R3-255898	Support Aerial UE Flight Information Failure Reporting in FAILURE INDICATION message

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1296  rev 2 Cat: B (Rel-19)

					Source: Ericsson, CMCC, CATT, LG Electronics, ZTE, Huawei, Nokia

(Replaces R3-255817)

Decision: 		The document was agreed.

R3-255618	Support Aerial UE Flight Information Failure Reporting in Filure Indicator message [UAS_Ph3]

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CATT, Ericsson, CMCC

Decision: 		The document was revised to R3-255818.

R3-255818	Support Aerial UE Flight Information Reporting

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CATT, ZTE, Ericsson, CMCC, Nokia, China Telecom, Huawei, LG Electronics

(Replaces R3-255618)

Decision: 		The document was endorsed.

R3-255695	Discussion on UAV remaining issues

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-255693	Support Aerial UE Flight Information Reporting to CN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1311  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was merged.

R3-255707	Discussion on UAV information reporting

Type: discussion		For: Discussion
					Source: CMCC, CATT

Decision: 		The document was noted.

Solution A: Huawei, Nokia

Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message.

Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped.

Solution B: Ericsson, CATT, CMCC

Add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message

Solution C: ZTE, Samsung

Introduce a new cause value for Cause IE to indicate NG-RAN node cannot report the aerial UE information.

LGE: Can there be multiple configurations for Aerial UE Flight Information Reporting?

ERICSSON: Cause value does not have receiver behavior defined.

Agree on either solution B or solution C.

R3-255815	Summary of offline discussion on UAV

Type: discussion		For: Decision
					Source: Ericsson

Discussion:

- Downselect Solution B or Solution C

- Can there be multiple configurations for Aerial UE Flight Information Reporting, and if so how to handle?

- Resolve any other open issues?

Decision: 		The document was noted.

R3-255816	Reply LS on clarification of UAV regulation from SA2

Type: LS out		For: Approval
					to SA2
					Source: RAN3(CMCC)

Discussion:

-	Align IE name with NGAP CR

Decision: 		The document was revised to R3-255899.

R3-255899	Reply LS on clarification of UAV regulation from SA2

Type: LS out		For: Approval
					to SA2
					Source: RAN3(CMCC)

(Replaces R3-255816)

Decision: 		The document was approved.

## 9	Corrections to Rel-18 or earlier releases

### 9.1	LTE

R3-255325	Correction on RRC establishment Cause for CIoT

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1970  Cat: F (Rel-18)

					Source: Huawei, ZTE, Nokia, Qualcomm Incorporated

Decision: 		The document was revised to R3-255751.

R3-255751	Correction on RRC establishment Cause for CIoT

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1970  rev 1 Cat: F (Rel-18)

					Source: Huawei, ZTE, Nokia, Qualcomm Incorporated, Ericsson

(Replaces R3-255325)

Decision: 		The document was agreed.

R3-255386	Clarification on E-RAB management procedures

Type: CR		For: Agreement
					36.413 v16.14.0	  CR-1971  Cat: F (Rel-16)

					Source: Google

Decision: 		The document was noted.

R3-255387	Clarification on E-RAB management procedures

Type: CR		For: Agreement
					36.413 v17.6.0	  CR-1972  Cat: A (Rel-17)

					Source: Google

Decision: 		The document was not treated.

R3-255388	Clarification on E-RAB management procedures

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1973  Cat: A (Rel-18)

					Source: Google

Decision: 		The document was not treated.

R3-255556	Clarification on BL CE UE in RRC_INACTIVE

Type: draftCR		For: Endorsement
					36.300 v16.9.0
					Source: Ericsson

Decision: 		The document was not treated.

R3-255557	Clarification on BL CE UE in RRC_INACTIVE

Type: draftCR		For: Endorsement
					36.300 v17.10.0
					Source: Ericsson

Decision: 		The document was revised to R3-255767.

R3-255767	Clarification on BL CE UE in RRC_INACTIVE

Type: draftCR		For: Endorsement
					36.300 v17.10.0
					Source: Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO

(Replaces R3-255557)

Decision: 		The document was endorsed.

R3-255558	Clarification on BL CE UE in RRC_INACTIVE

Type: draftCR		For: Endorsement
					36.300 v18.5.0
					Source: Ericsson

Decision: 		The document was revised to R3-255768.

R3-255768	Clarification on BL CE UE in RRC_INACTIVE

Type: draftCR		For: Endorsement
					36.300 v18.5.0
					Source: Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO

(Replaces R3-255558)

Decision: 		The document was endorsed.

R3-255720	Correction to Criticality of Retrieve UE Context Request

Type: CR		For: Agreement
					36.423 v15.14.0	  CR-1813  Cat: F (Rel-15)

					Source: CATT

Discussion:

X2AP rapporteur will include in R19 rapporteur corrections

Decision: 		The document was noted.

R3-255721	Correction to Criticality of Retrieve UE Context Request

Type: CR		For: Agreement
					36.423 v16.13.0	  CR-1814  Cat: A (Rel-16)

					Source: CATT

Decision: 		The document was not treated.

R3-255722	Correction to Criticality of Retrieve UE Context Request

Type: CR		For: Agreement
					36.423 v17.7.0	  CR-1815  Cat: A (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R3-255723	Correction to Criticality of Retrieve UE Context Request

Type: CR		For: Agreement
					36.423 v18.4.0	  CR-1816  Cat: A (Rel-18)

					Source: CATT

Decision: 		The document was not treated.

### 9.2	NR

[PEI and emergency PDU session]

R3-255559	Discussion on emergency call back and paging with PEIPS

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was noted.

R3-255233	Correction of PEI and emergency PDU session

Type: draftCR		For: Endorsement
					38.300 v17.13.0
					Source: Huawei, Qualcomm, Ericsson, China Telecom, Nokia, CATT

(Replaces R3-253408)

Discussion:

Resp in R3-255749

Decision: 		The document was not treated.

R3-255234	Correction of PEI and emergency PDU session

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Huawei, Qualcomm, Ericsson, China Telecom, Nokia, CATT

(Replaces R3-253409)

Discussion:

Resp in R3-255749

Decision: 		The document was not treated.

R3-255749	Response to R3-255233/255234

Type: discussion		For: Discussion
					Source: ZTE Corporation, CMCC, Pengcheng Laboratory

Decision: 		The document was not treated.

HUAWEI: postpone pending further CT1 progress?

ERICSSON: LS to SA2 to revert removal of the restriction in Rel-19?

R3-255875	Summary of CB: #6_PEIandEmergencyPDUsess

Type: discussion		For: Decision
					Source: ZTE Coroporation

Decision: 		The document was noted.

R3-255876	LS on compatibility issue for PEI and emergency PDU session

Type: LS out		For: discussion
					to SA2, cc CT1, RAN2
					Source: RAN3(ZTE)

Discussion:

-	Change last paragraph to “Some companies think that solution 1 may be the simplest. The final decision is up to SA2.”

Decision: 		The document was revised to R3-255906.

R3-255906	LS on compatibility issue for PEI and emergency PDU session

Type: LS out		For: discussion
					to SA2, cc CT1, RAN2
					Source: RAN3(ZTE)

(Replaces R3-255876)

Decision: 		The document was approved.

[LTM]

R3-255613	Correction on Intra-CU LTM shared preamble index limit per DU

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1586  Cat: F (Rel-18)

					Source: LG Electronics Inc., Huawei, Samsung, Google, NEC, ZTE, CATT, Qualcomm, CMCC, China Telecom, Ericsson, Nokia, Lenovo

Abstract:

NBC

Decision: 		The document was agreed.

R3-255319	Support Cell Level Measurements signaling from CU to DU for L3 measurement based R18 LTM

Type: discussion		For: Agreement
					Source: Qualcomm Inc, Apple Inc, NTT DOCOMO, AT&T, Boost Mobile Networks, JIO Platforms, Charter Communications, Rakuten, CEWiT, Vivo, Tejas Networks, Xiaomi.

Discussion:

HUAWEI, ERICSSON, ZTE: not essential correction for Rel-18

Decision: 		The document was noted.

R3-255322	Cell level measurements in F1-AP signalling for L3 measurements-based LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1579  Cat: F (Rel-18)

					Source: Qualcomm Inc, Apple Inc, NTT DOCOMO, AT&T, Boost Mobile Networks, JIO Platforms, Charter Communications, Rakuten, CEWiT, Vivo, Tejas Networks, Xiaomi.

Decision: 		The document was noted.

R3-255535	Rel-18 LTM Correction for L2 reset

Type: discussion		For: (not specified)
					Source: Ericsson, CATT, Jio Platforms

Discussion:

Add a new Old Serving Cell ID IE in the CU-DU CELL SWITCH NOTIFICATION message to indicate the ID of the cell serving the UE before LTM cell switch execution.

Update stage 2 (TS 38.401) to update the Cell Switch Notification message where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message including the ID of the cell serving the UE before LTM cell switch execution.

ZTE: Need to check RAN2 progress about inter-DU cases

Lenovo: Nothing seems needed

NEC: For Inter-DU LTM, Layer 2 reset is always performed

Nokia: same view as NEC

QUALCOMM: support the CR

RAN3 understanding is that for inter-DU LTM, Layer 2 reset is always performed

CB # 7_LTM_Layer2Reset

- LS to RAN2 to describe RAN3 understanding that for inter-DU LTM, Layer 2 reset is always performed

(Ericsson)

Decision: 		The document was noted.

R3-255843	draft LS to RAN2(*CB:#7)

Type: LS out		For: Approval
					to RAN2
					Source: RAN3(Ericsson)

Discussion:

-	remove “draft”, change source to RAN3

Decision: 		The document was revised to R3-255907.

R3-255907	LS on the handling of inter-DU L2 reset for LTM

Type: LS out		For: Approval
					to RAN2
					Source: RAN3(Ericsson)

(Replaces R3-255843)

Decision: 		The document was approved.

R3-255536	F1AP correction for Layer 2 reset at inter-gNB-DU LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1581  Cat: F (Rel-18)

					Source: Ericsson, CATT, Jio Platforms

Decision: 		The document was not treated.

R3-255537	Stage 2 correction for Layer 2 reset at inter-gNB-DU LTM

Type: draftCR		For: Endorsement
					38.401 v18.6.0
					Source: Ericsson, CATT, Jio Platforms

Decision: 		The document was not treated.

R3-255718	Essential corrections to TCI states for LTM

Type: discussion		For: (not specified)
					Source: Ericsson, Lenovo, Google, Jio Platforms, CATT, Ofinno, Verizon Wireless

Discussion:

Issue is acknowledged, solution needs further discussion (e.g., semantics part?)

CB # 8_LTM_TCIstates

- Further discuss the F1AP CR

(Ericsson)

Decision: 		The document was treated.

R3-255859	TCI States for LTM

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0479  Cat: F (Rel-18)

					Source: Nokia, Ericsson, Qualcomm

Discussion:

-	Add ZTE, NEC, Huawei

-	Add impact statement

Decision: 		The document was revised to R3-255908.

R3-255908	TCI States for LTM

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0479  rev 1 Cat: F (Rel-18)

					Source: Nokia, Ericsson, Qualcomm, LG Electronics, Samsung, Jio Platforms, Lenovo, Google, CATT, Ofinno, Verizon Wireless, ZTE, NEC, Huawei

(Replaces R3-255859)

Decision: 		The document was agreed.

R3-255719	Essential corrections to TCI states for LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1587  Cat: F (Rel-18)

					Source: Ericsson, Lenovo, Google, Jio Platforms, CATT, Ofinno, Verizon Wireless

Decision: 		The document was revised to R3-255841.

R3-255841	Essential corrections to TCI states for LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1587  rev 1 Cat: F (Rel-18)

					Source: Ericsson, Lenovo, Google, Jio Platforms, CATT, Ofinno, Verizon Wireless, ZTE, LG Electronics, Nokia, Samsung, Huawei, NEC, Qualcomm

(Replaces R3-255719)

Decision: 		The document was agreed.

[AI/ML for NG-RAN]

R3-255460	On packet loss metric in UE performance

Type: discussion		For: Approval
					Source: Ericsson, Deutsche Telekom, Jio Platforms

Decision: 		The document was noted.

R3-255461	Correction to exclude Packet Loss as a UE Performance metric

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1449  rev 4 Cat: F (Rel-18)

					Source: Ericsson, Deutsche Telekom, Jio Platforms

(Replaces R3-253475)

Decision: 		The document was noted.

R3-255448	Correction of the granularity of the Average Packet Loss DL IE

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1506  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was noted.

Whether to correct packet loss metric in UE performance in R18?

•	Opt1: Remove the IE

•	Opt2: Introduce new IE for packet loss metric in UE performance to replace the old IE

•	Opt3: Ignore the IE in R18

•	Opt4: Do nothing

ZTE: Rel-18 correction should be handled based on Rel-19 agreement (and Reply LS from SA5)

HUAWEI: Wait for Rel-19 issue to be resolved. Prefer not to NULL the Rel-18 IE.

NEC: NBC CR seems OK

Wait for Reply LS(es) from SA5

To be continued...

[Other]

R3-255147	Discussion on the configuration in case of CHO with CPAC

Type: discussion		For: (not specified)
					Source: ZTE Corporation, CATT, China Telecom, China Unicom

Discussion:

Proposal 1: RAN3 shall clarify that in case of CHO with CPAC configured, whether or not“CHO Information SN Addition” IE can be in the SN addition request message.

Proposal 2: In case of CHO with CPAC configured,

Solution 1: “CHO Information SN Addition” IE is allowed in the SN addition request message, or

Solution 2 “CHO Information SN Addition” IE cannot be in the SN addition request message, or

Solution 3: “CHO Information SN Addition” IE must be in the SN addition request message

Google: Target MN can decide

Nokia: Solution 1

LGE: Solution 3

Decision: 		The document was noted.

R3-255148	Clarification on the configuration in case of CHO with CPAC

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: ZTE Corporation, CATT, China Telecom, China Unicom

Decision: 		The document was not treated.

R3-255187	Discussion on positioning activation and deactivation procedure

Type: discussion		For: Agreement
					Source: Samsung, CATT, ZTE

Discussion:

ERICSSON: doubting the scenario, previously discussed and some aspects were ruled out already. We are OK to further discuss on the basis of minimizing spec impact.

To be continued...

Decision: 		The document was noted.

R3-255186	Correction to Positioning activation and deactivation procedure

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: CATT, Samsung, ZTE

Decision: 		The document was not treated.

R3-255188	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0193  Cat: F (Rel-18)

					Source: Samsung, CATT, ZTE Corporation

Decision: 		The document was not treated.

R3-255189	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1577  Cat: F (Rel-18)

					Source: Samsung, CATT, ZTE Corporation

Decision: 		The document was not treated.

R3-255190	Correction to positioning activation and deactivation procedure

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1503  Cat: F (Rel-18)

					Source: ZTE Corporation, Samsung, CATT

Decision: 		The document was not treated.

R3-255232	Specifying procedure texts for the Core Network Assistance Information for RRC INACTIVE IE

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1288  Cat: F (Rel-18)

					Source: Huawei, Ericsson, Nokia

Decision: 		The document was not treated.

R3-255351	Clarification for propagation of roaming and access restrictions

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Samsung

Decision: 		The document was not treated.

R3-255391	Clarification on PDU session management procedures

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1291  Cat: F (Rel-16)

					Source: Google

Decision: 		The document was not treated.

R3-255392	Clarification on PDU session management procedures

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1292  Cat: A (Rel-17)

					Source: Google

Decision: 		The document was not treated.

R3-255410	Clarification on PDU session management procedures

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1293  Cat: A (Rel-18)

					Source: Google

Decision: 		The document was not treated.

R3-255483	Usage of root cause in MRO

Type: discussion		For: (not specified)
					Source: Huawei, Vodafone, Nokia, Qualcomm, BT, Jio Platforms

Decision: 		The document was not treated.

R3-255484	Usage of root cause in SON/MRO

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: Huawei, Vodafone, Nokia, Qualcomm, BT, Jio Platforms

Decision: 		The document was not treated.

R3-255485	Usage of root cause in SON/MRO

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: Huawei, Vodafone, Nokia, Qualcomm, BT, Jio Platforms

Decision: 		The document was not treated.

R3-255560	Extensions for enumerated type definitions over several Releases - and what can go wrong

Type: discussion		For: Discussion
					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255561	Correcting the extension of FR1-bandwidth type definition

Type: CR		For: Agreement
					38.473 v16.21.0	  CR-1582  Cat: F (Rel-16)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255562	Correcting the extension of FR1-bandwidth type definition

Type: CR		For: Agreement
					38.473 v17.13.0	  CR-1583  Cat: A (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255563	Correcting the IE extension definition for bandwidth SRS

Type: CR		For: Agreement
					38.455 v16.15.0	  CR-0194  Cat: F (Rel-16)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255564	Correcting the IE extension definition for bandwidth SRS

Type: CR		For: Agreement
					38.455 v17.10.0	  CR-0195  Cat: A (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255565	Correcting the IE extension definition for SRS Pos SIB Type

Type: CR		For: Agreement
					38.455 v17.10.0	  CR-0196  Cat: F (Rel-17)

					Source: Ericsson, Jio Platforms, China Telecom, CMCC

Decision: 		The document was not treated.

R3-255566	Addition of PDCP discard timer for scheduling with DSR

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1584  Cat: F (Rel-18)

					Source: Ericsson, Qualcomm Inc., Jio Platforms

Decision: 		The document was not treated.

R3-255737	Clarification for propagation of MDT Configuration in stage2

Type: other		For: Agreement
					Source: ZTE Corporation,China Unicom,China Telecom,CMCC, Huawei

(Replaces R3-253738)

Decision: 		The document was not treated.

R3-255738	Clarify on UE context retrieval

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255242	Discussion on L3-report based SCell activation

Type: discussion		For: Decision
					Source: CATT, China Telecom, CMCC, NTT Docomo, Huawei

(Replaces R3-253300)

Decision: 		The document was not treated.

R3-255192	Withdrawn

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1578  Cat: F (Rel-18)

					Source: Nokia

Decision: 		The document was withdrawn.

## 10	Data Collection for SON_MDT in NR standalone and MR-DC WI (RAN3-led) [NR_ENDC_SON_MDT_Ph4-Core]: RP-234038 (target: RAN #109)

### 10.1	General

R3-255030	(BL CR to 36.300 for SON) Addition of SON enhancements

Type: draftCR		For: Endorsement
					36.300 v18.5.0
					Source: Lenovo

(Replaces R3-253063)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255031	(BL CR to 38.413 for MDT)  Addition of MDT enhancements

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1189  rev 7 Cat: B (Rel-19)

					Source: Nokia

(Replaces R3-253068)

Abstract:

BL CR

Decision: 		The document was revised to R3-255996.

R3-255996	(BL CR to 38.413 for MDT)  Addition of MDT enhancements

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1189  rev 8 Cat: B (Rel-19)

					Source: Nokia

(Replaces R3-255031)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255032	(BL CR to 38.423 for MDT) Addition of MDT enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1412  rev 5 Cat: B (Rel-19)

					Source: CATT

(Replaces R3-253069)

Abstract:

BL CR

Decision: 		The document was revised to R3-255997.

R3-255997	(BL CR to 38.423 for MDT) Addition of MDT enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1412  rev 6 Cat: B (Rel-19)

					Source: CATT

(Replaces R3-255032)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255033	(BL CR to 38.420 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0050  rev 1 Cat: B (Rel-19)

					Source: Samsung, ZTE, CATT, Lenovo, Cybecore

(Replaces R3-253627)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255034	(BL CR to 36.423 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					36.423 v18.4.0	  CR-1790  rev 9 Cat: B (Rel-19)

					Source: CATT, Samsung, Lenovo, Huawei, Ericsson, ZTE, CMCC, Nokia, Jio Platforms

(Replaces R3-253845)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255035	(BL CR to 37.340 for SON) Addition of SON enhancements

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: CMCC

(Replaces R3-253995)

Abstract:

BL CR

Decision: 		The document was revised to R3-255993.

R3-255993	(BL CR to 37.340 for SON) Addition of SON enhancements

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: CMCC

(Replaces R3-255035)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255036	(BL CR to 38.300 for SON) Addition of SON enhancements

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: China Unicom

(Replaces R3-253996)

Abstract:

BL CR

Decision: 		The document was revised to R3-255994.

R3-255994	(BL CR to 38.300 for SON) Addition of SON enhancements

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: China Unicom

(Replaces R3-255036)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255037	(BL CR to 38.401 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0440  rev 8 Cat: B (Rel-19)

					Source: ZTE Corporation, Nokia, Huawei, Lenovo, CATT, Ericsson, Samsung, Jio Platforms

(Replaces R3-253997)

Abstract:

BL CR

Decision: 		The document was revised to R3-255995.

R3-255995	(BL CR to 38.401 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0440  rev 9 Cat: B (Rel-19)

					Source: ZTE Corporation, Nokia, Huawei, Lenovo, CATT, Ericsson, Samsung, Jio Platforms

(Replaces R3-255037)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255038	(BL CR to 38.413 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1286  rev 1 Cat: B (Rel-19)

					Source: Ericsson, Jio Platforms

(Replaces R3-253998)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255039	(BL CR to 38.423 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1413  rev 10 Cat: B (Rel-19)

					Source: ZTE Corporation, Samsung, Nokia, Lenovo, Jio

(Replaces R3-253999)

Abstract:

BL CR

Decision: 		The document was revised to R3-255998.

R3-255998	(BL CR to 38.423 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1413  rev 11 Cat: B (Rel-19)

					Source: ZTE Corporation, Samsung, Nokia, Lenovo, Jio

(Replaces R3-255039)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255040	(BL CR to 38.473 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1519  rev 6 Cat: B (Rel-19)

					Source: Huawei, Nokia, Jio

(Replaces R3-254000)

Abstract:

BL CR

Decision: 		The document was revised to R3-255999.

R3-255999	(BL CR to 38.473 for SON) Addition of SON enhancements

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1519  rev 7 Cat: B (Rel-19)

					Source: Huawei, Nokia, Jio

(Replaces R3-255040)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255320	Workplan for Rel-19 SON_MDT Enhancement

Type: Work Plan		For: Information
					Source: China Unicom, CMCC

Decision: 		The document was noted.

### 10.2	MRO Enhancements

R3-255164	(TP for SON BL CR for TS 38.401) MRO Enhancements for LTM

Type: other		For: Agreement
					38.401 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255165	MRO Enhancements for CHO with Candidate SCG(s)

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R3-255193	(SON TPs to BL CR to TS 38.423 and to TS 37.340) Completion of the signalling for providing UHI to the SN in case of S-CPAC release

Type: other		For: Agreement
					38.423 v..
					Source: Nokia, Huawei, Ericsson

Discussion:

Completion of the signalling for providing UHI to the SN in case of S-CPAC release

Decision: 		The document was revised to R3-255803.

R3-255803	(SON TPs to BL CR to TS 38.423) Completion of the signalling for providing UHI to the SN in case of S-CPAC release

Type: other		For: Agreement
					38.423 v..
					Source: Nokia, Huawei, Ericsson

(Replaces R3-255193)

Decision: 		The document was agreed.

R3-255214	(TP for SON BLCR of 38.473) Discussion on MRO for LTM

Type: other		For: Agreement
					38.473 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255315	MRO enhancements for LTM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255321	Discussion on MRO Enhancements

Type: discussion		For: (not specified)
					Source: China Unicom

Decision: 		The document was not treated.

R3-255393	(TP for SON BLCR for 38.473) Discussion on MRO for LTM

Type: other		For: Agreement
					38.473 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255394	(TP for SON BLCR for 38.423) Discussion on MRO for CHO with candidate SCG(s) and subsequent CPAC

Type: other		For: Agreement
					38.423 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255432	Ping-Pong Mitigation in CU-DU Coordination for LTM Mobility

Type: discussion		For: Decision
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255486	Usage of root cause in MRO

Type: discussion		For: (not specified)
					Source: Huawei, Vodafone, Qualcomm, BT, Jio Platforms

Decision: 		The document was not treated.

R3-255487	(TP for BLCR for SON for 38.401) Usage of root cause in MRO

Type: other		For: Agreement
					38.401 v..
					Source: Huawei, Vodafone, Qualcomm, BT, Jio Platforms

Decision: 		The document was not treated.

R3-255488	LTM ping pong

Type: discussion		For: (not specified)
					Source: Huawei, Qualcomm, CMCC, Jio Platforms

Decision: 		The document was not treated.

R3-255489	(TP for BLCR for SON for 38.473) LTM ping pong

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Qualcomm, CMCC, Jio Platforms

Decision: 		The document was revised to R3-255787.

R3-255787	(TP for BLCR for SON for 38.473) LTM ping pong

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Ericsson, Lenovo, Nokia, ZTE, Samsung, CMCC

(Replaces R3-255489)

Decision: 		The document was agreed.

R3-255490	(TP for BLCR for SON for 38.413) LTM ping pong

Type: other		For: Agreement
					38.413 v..
					Source: Huawei, Qualcomm, CMCC, Jio Platforms

Decision: 		The document was not treated.

R3-255491	LTM Near failure TA

Type: other		For: Agreement
					Source: Huawei, Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255592	Signaling of MRO for LTM between target DU and source DU

Type: discussion		For: (not specified)
					Source: Ericsson, Huawei, Jio Platforms

Decision: 		The document was not treated.

R3-255593	(TP for BL CR to 38.401 for SON) Signaling of MRO for LTM between target DU and source DU

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Huawei, Jio Platforms

Decision: 		The document was not treated.

R3-255594	(TP for BL CR to 38.473 for SON) Signaling of MRO for LTM between target DU and source DU

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Huawei, Jio Platforms

Decision: 		The document was revised to R3-255838.

R3-255838	(TP for BL CR to 38.473 for SON) Signaling of MRO for LTM between target DU and source DU

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Huawei, Jio Platforms

(Replaces R3-255594)

Decision: 		The document was revised to R3-255905.

R3-255905	(TP for BL CR to 38.473 for SON) Signaling of MRO for LTM between target DU and source DU

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Huawei, Jio Platforms, Nokia, Lenovo, ZTE, Samsung, CATT

(Replaces R3-255838)

Decision: 		The document was agreed.

R3-255595	Ping-pong avoidance for LTM

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-255596	(TP for BL CR to 38.473 for SON) Ping-pong avoidance for LTM

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255620	(TP for SON BL CR for TS38.401) MRO for LTM

Type: other		For: Agreement
					38.401 v..
					Source: Samsung

Decision: 		The document was revised to R3-255791.

R3-255791	(TP for SON BL CR for TS38.401) MRO for LTM

Type: other		For: Agreement
					38.401 v..
					Source: Samsung

(Replaces R3-255620)

Decision: 		The document was revised to R3-255882.

R3-255882	(TP for SON BL CR for TS38.401) MRO for LTM

Type: other		For: Agreement
					38.401 v..
					Source: Samsung, Huawei, Nokia, ZTE

(Replaces R3-255791)

Decision: 		The document was agreed.

R3-255621	(TP for SON BL CR for TS38.473) MRO for LTM

Type: other		For: Agreement
					38.473 v..
					Source: Samsung

Decision: 		The document was not treated.

R3-255622	Failure scenarios on MRO for CHO with candidate SCGs

Type: discussion		For: Information
					Source: Samsung, Cybercore, Lenovo

Decision: 		The document was not treated.

R3-255623	(TP for SON BLCR for TS37.340, TS38.300 and TS38.423) MRO for CHO with candidate SCGs

Type: other		For: Agreement
					Source: Samsung, Jio Platforms, Cybercore

Decision: 		The document was revised to R3-255833.

R3-255833	(TP to SON BLCR for TS38.300) MRO for CHO with candidate SCGs'

Type: other		For: Agreement
					38.300 v..
					Source: Samsung, Jio Platforms, Cybercore

(Replaces R3-255623)

Decision: 		The document was revised to R3-255893.

R3-255893	(TP to SON BLCR for TS 38.300) MRO for CHO with candidate SCGs'

Type: other		For: Agreement
					38.300 v..
					Source: Samsung, Jio Platforms, Cybercore, Nokia, ZTE, Lenovo

(Replaces R3-255833)

Decision: 		The document was agreed.

R3-255624	(TP for SON BLCR for TS38.423) MRO for S-CPAC

Type: other		For: Agreement
					38.423 v..
					Source: Samsung, Jio Platforms, Cybercore

Decision: 		The document was not treated.

R3-255641	(TP for 38.473 and 38.401) MRO for Rel-18 LTM

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255642	(TP for 38.423 and 38.300) MRO for CHO with Candidate SCG(s) and S-CPAC

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255734	(TP for SON BLCR for 38.473) LTM Ping Pong left Issues

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255735	(TP for SON BLCR for 38.473 and 38.401) LTM Beam and TA Left Issues

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255736	(TP for SON BLCR for 38.423) MRO for others

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255814.

R3-255814	(TP for SON BLCR for 38.423) MRO for others

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Samsung

(Replaces R3-255736)

Decision: 		The document was agreed.

R3-255746	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO without RLF Report

Type: other		For: Agreement
					38.473 v..
					Source:  Google, Ericsson, NEC, Lenovo, CATT

Abstract:

discussion on LTM failure without RLF report

Decision: 		The document was revised to R3-255902.

R3-255902	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO without RLF Report

Type: other		For: Agreement
					38.473 v..
					Source:  Google, Ericsson, NEC, Lenovo, CATT

(Replaces R3-255746)

Discussion:

-	Add Samsung, CMCC to co-sources

Decision: 		The document was revised to R3-255957.

R3-255957	(TP for SON BL CR for TS 38.473) Support for LTM MRO without RLF Report

Type: other		For: Agreement
					38.473 v..
					Source: Google, Ericsson, NEC, Lenovo, CATT, Samsung, CMCC

(Replaces R3-255902)

Decision: 		The document was agreed.

R3-255777	SoD of MRO for LTM

Type: discussion		For: Decision
					Source: Ericsson

Decision: 		The document was noted.

R3-255779	SoD of MRO for Others

Type: discussion		For: Decision
					Source: Samsung

Decision: 		The document was noted.

R3-255804	(SON TPs to BL CR to TS 37.340) Completion of the signalling for providing UHI to the SN in case of S-CPAC release

Type: other		For: Agreement
					37.340 v..
					Source: Nokia, Huawei, Ericsson

Decision: 		The document was agreed.

Stage-3 (F1AP) for MRO:

[Agreement]

For stage-3 implementation of MRO cases 1, 2 and 3, use a common structure for all cases.

IE types and names:

-	Case 1: SSB Index.

-	Case 2: SSB Index.

-	Case 3: Keep TA Information name and use semantics to describe recovery and re-establishment

Additional IEs:

-	Case 2: TCI State ID? SSB Configuration?

SAMSUNG: signaling only SSB index is enough. Beam information is choice of SSB index and CSI-RS

ERICSSON, CATT, LENOVO, Nokia: CRS-RS is not used in Rel-18.

RAN3 assumes TA is per cell level. Extend case 3 with recovery or re-establishment to a different beam?

Discuss in stage-2 TP rewording

Is it possible for the source DU to identify the cases i.e. whether it is wrong beam or TA issues?

HUAWEI: support this

ERICSSON, ZTE: never to correct it

LENOVO: if TA is cell level, the proposal is fine

Add an “Indication of TA value sent in LTM cell switch command” in CELL SWITCH NOTIFICATION messages

SAMSUNG: to avoid the signlling in case of UE’s problem

CATT, QUALCOMM: source DU has UE context.

Ping-pong and UHI:

[Agreement]

Down-select options 1a and 2b.

CU sends relevant information to the DU whenever CU decides this is necessary

CU only send last consecutive LTM cell switch entries to the DU and an optional Mobility Issue indicator

CATT, ZTE has concern on the indicator. The DU can detect this issue based on the received LTM entries. If the message is sent, it already means the issue detected.

ERICSSON, LENOVO, Nokia, SAMSUNG: better to notify this issue to the DU detected.

HUAWEI: Not convinced it is needed. If have an indicator, it just indicates a potential problem.

QUALCOMM: why not L3 related?

ERICSSON, Samsung, ZTE, CMCC: no need send information to the DU which is not relevant to the DU. L3 measurement is not useful for ping-pong.

Other:

Support LTM MRO failure cases when RLF Report is not available?

HUAWEI, QUALCOMM ERICSSON, NEC, CATT, LENOVO, SAMSUNG: better for legacy UEs. There is risk if not send and failure happens.

ZTE, Nokia: introduce backward compatibility.

CMCC: focus on the existing cases.

Try to have common understanding offline, if agreeable, check TP.

Topics not discussed in offline but that may need conclusion for WI completion:

Support near-failure TA optimization use-case in Rel-19?

Qualcomm/Nokia/Samsung: message 1 can be received anywhere, implementation issue. Don’t think it is necessary.

Nokia: not convinced on the scenario.

ZTE: it is beneficial for the source DU to know the margin.

HUAWEI: it is the cases of almost outdated TA.

CATT/LENOVO: the target DU cannot differentiate it.

Mobility information?

CB # SONMDT_MROLTM

- check TPs

(Ericsson)

MRO for CHO with Candidate SCG(s)

Include CPC failure type as an optional IE in SCG Failure Indication message with codepoint ‘To Wrong PSCell’ and ‘Too late CPC’

MRO for S-CPAC

Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN?

CB # SONMDT_MROothers

- check TPs

(Samsung)

R3-255917	(TP for 38.423) Add CPC failure type in SCG failure indication

Type: other		For: Agreement
					38.423 v..
					Source: CATT, Samsung, Nokia

Decision: 		The document was agreed.

R3-255918	(TP for 37.340) MRO for S-CPC stage 2

Type: other		For: Agreement
					37.340 v..
					Source: CATT

Decision: 		The document was noted.

R3-255894	(TP to SON BLCR for TS 37.340) MRO for CHO with candidate SCGs

Type: other		For: Agreement
					37.340 v..
					Source: Samsung

Decision: 		The document was agreed.

### 10.3	SON/MDT for NTN and Slicing

#### 10.3.1	Intra-NTN Mobility

R3-255194	(SON TP to BL CR to TS 38.423 and MDT TP to BL CR TS 38.413) Solution to avoid restarting MRO in NTN deployments and a proposal for the MDT area definition

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

(Replaces R3-253186)

Discussion:

-	Add Huawei, Ericsson, ZTE, CATT, Samsung to co-sources

-	Correct ASN.1 for Geographical Area of MDT IE (should be CHOICE)

Decision: 		The document was revised to R3-255928.

R3-255928	(MDT TP to BL CR to TS 38.413) A proposal for the MDT area definition

Type: other		For: Agreement
					38.423 v..
					Source: Nokia, Huawei, Ericsson, ZTE, CATT, Samsung

(Replaces R3-255194)

Abstract:

MDT

Decision: 		The document was revised to R3-255958.

R3-255958	(MDT TP to BL CR to TS 38.413) A proposal for the MDT area definition

Type: other		For: Agreement
					38.413 v..
					Source: Nokia, Huawei, Ericsson, ZTE, CATT, Samsung

(Replaces R3-255928)

Decision: 		The document was agreed.

R3-255261	(TPs to BL CR for 38.300, 38.413 and 38.423) Further discussion on SONMDT enhancements for NTN

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255895.

R3-255895	(TP to BL CR for 38.423) MDT Enhancement for NTN

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation

(Replaces R3-255261)

Discussion:

-	Change “indicate area scope for NTN MDT” to “indicate the geographical area scope for NTN MDT”

-	Add Huawei, Ericsson, Nokia, CATT, Samsung to co-sources

Decision: 		The document was revised to R3-255959.

R3-255959	(TP to MDT BL CR for 38.423) MDT Enhancement for NTN

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Huawei, Ericsson, Nokia, CATT, Samsung

(Replaces R3-255895)

Abstract:

MDT

Decision: 		The document was agreed.

R3-255316	MDT enhancements for NTN

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255350	(TP for MDT BLCRs for TS38.413, TS38.423) SON/MDT enhancements for NTN

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255395	Discussion on SON/MDT for NTN

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-255428	Discussion on SONMDT for Intra-NTN mobility

Type: discussion		For: (not specified)
					Source: Huawei, CMCC, Jio Platforms, China Unicom

Decision: 		The document was not treated.

R3-255429	(TP for MDT BLCR for TS38.413): MDT enhancement for NTN

Type: other		For: Agreement
					38.413 v..
					Source: Huawei, CMCC, Jio Platforms, China Unicom

Decision: 		The document was not treated.

R3-255430	(TP for MDT BLCR for TS38.423): MDT enhancement for NTN

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, CMCC, Jio Platforms, China Unicom

Decision: 		The document was not treated.

R3-255431	(TP for MDT BLCR for TS37.320): MDT enhancement for NTN

Type: other		For: Agreement
					37.320 v..
					Source: Huawei, CMCC, Jio Platforms, China Unicom

Discussion:

-	Change text to “for NR NTN, a list of up to 8 geographical areas. If this list is configured, the UE will only log measurements when camping in any cell that belongs to the configured geographical areas”

-	Add ZTE, CATT, Ericsson, Nokia, Samsung to co-sources

Decision: 		The document was revised to R3-255870.

R3-255870	(TP for MDT BLCR for TS37.320): MDT enhancement for NTN

Type: other		For: Agreement
					37.320 v..
					Source: Huawei, CMCC, Jio Platforms, China Unicom, ZTE, CATT, Ericsson, Nokia, Samsung

(Replaces R3-255431)

Decision: 		The document was agreed.

R3-255433	SON MDT for Intra-NTN Mobility

Type: discussion		For: Decision
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255495	MRO for NTN

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-255597	SON-MDT enhancements for NTN

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-255598	(TP for BL CR to 38.413 for SON) Additional cause values and cell types for NTN

Type: other		For: Agreement
					38.413 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255643	(TP for 38.300) Intra-NTN mobility for SON

Type: other		For: Agreement
					38.300 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255644	(TP for 38.413 and 38.423) Intra-NTN mobility for MDT

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255790	Summary of SONMDT for NTN

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-255423	SON MDT for Intra-NTN Mobility

Type: discussion		For: Decision
					Source: Jio Platforms

Decision: 		The document was withdrawn.

MDT for NTN

[Agreement]

The geographical area scope applies to both logged and immediate MDT.

Agree geographical area based MDT should be able to be collected in specified PLMN(s)

[RAN2 agreements: RAN2 intends to specify that the legacy area scope cannot be configured when Rel-19 area scope is configured.]

ERICSSON: need new PLMN list based on RAN2 agreement.

Nokia: define PLMN as backup solution, RAN3 decide it, not based on RAN2.

ZTE: agree RAN2 intention.

HUAWEI: RAN2 agreement means the new measurement is inside the existing MDT configuration.

Nokia: RAN3 conclusion should allow the fallback and should be decided in RAN3.

QUALCOMM: RAN2 decide UE behavior.

Check online: FFS on geographical area based MDT plus cell ID list

SAMSUNG, Nokia, ERICSSON: operator may want to measure one specific cell.

ERICSSON: support this. It is needed in overlapping area of cells.

QUALCOMM Huawei, ZTE: need to understand the use case. How about TAC?

[Agreement] Inside the existing MDT configuration

QUALCOMM CATT, ZTE, HUAWEI: inside, add PLMN ID.

SAMSUNG, Nokia, ERICSSON: outside for future proof.

Agree TP for 38.413, 38.423 and 37.320 when above FFS is solved.

[Agreement] Send LS to SA5 and RAN2 for geographical area scope for NTN MDT

SON for NTN

Whether to add measured RSRP for the SSB beam(s) when RACH based occurs. Whether the measurement results contained in RACH report, SHR and RLF report can be reused.

QUALCOMM SAMSUNG: not clear about the scenario. Failure case, whether report each beam corresponding to the CG configure. There can be 10 beams.

Whether to consider per-UE mobility offset for NTN.

CB # SONMDT_NTN

-  Reflect the agreements in TP

-  LS to SA5 and RAN2

(CATT)

R3-255916	LS on geographical area scope MDT

Type: LS out		For: Approval
					to SA2, SA5
					Source: RAN3(CATT)

Abstract:

Att: R3-255928 next

Decision: 		The document was revised to R3-255960.

R3-255960	LS on geographical area scope MDT

Type: LS out		For: Approval
					to SA2, SA5
					Source: RAN3(CATT)

(Replaces R3-255916)

Decision: 		The document was approved.

R3-255992	(BLCR to 37.320 for MDT) MDT enhancement for NTN

Type: draftCR		For: discussion
					37.320 v18.4.0
					Source: Huawei, CMCC, Jio Platforms, China Unicom, ZTE, CATT, Ericsson, Nokia, Samsung

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

#### 10.3.2	Network Slicing

R3-255195	Discussion on the possibility of requesting a non-registered slice

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R3-255446	Collection of rejected slice information for slice coverage optimisation

Type: discussion		For: Agreement
					Source: Ericsson, BT, Deutsche Telekom, Jio Platforms (JPL), FiberCop, InterDigital

Decision: 		The document was not treated.

R3-255447	(TP for BL CR to 38.413 for SON) Collection of rejected slice information for slice coverage optimisation

Type: other		For: Agreement
					38.413 v..
					Source: Ericsson, BT, Deutsche Telekom, Jio Platforms (JPL), FiberCop, InterDigital

Decision: 		The document was not treated.

R3-255600	Enhanced Rejected Slice Reporting for Dynamic Slice Optimization

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255812	LS on data collection for TAI Slice Support List configuration

Type: LS out		For: Approval
					to SA2, cc SA5
					Source: RAN3(Ericsson)

Decision: 		The document was not treated.

R3-255852	Summary of the discussion on the SON for slicing

Type: discussion		For: Decision
					Source: Nokia

Decision: 		The document was noted.

Check if the LS is needed; if not the proposal shall be noted and the SON for slicing declared completed in RAN3.

SAMSUNG: the issue is out of the SON scope. The issue shoud be firstly discussed in SA2.

CMCC: whether the scenario can be solved by target AMF.

ERICSSON: why SON should not solve this issue? The information of which N-SSAI should be able to serve UE need to be known.

ZTE: the scenario is still not clear.

BT, Deutsche Telekom, Jio: Support sending the LS to SA2. Acknowledge the issue. The information is helpful for planning and optimistion network.

CATT: corner case.

CMCC: slice coverage issue. Can propose this as correction.

HUAWEI: there is no real benefit.

Vodafone: question on the scenario.

### 10.4	R18 leftovers

R3-255492	SON/MDT Leftovers

Type: other		For: Agreement
					Source: Huawei

Discussion:

SAMSUNG: see some benefit. Prefer network based solution

Nokia: agree with stage 3 solution

ERICSSON: introducing complexity.

Decision: 		The document was noted.

## 11	AI/ML for NG-RAN WI (RAN3-led) [NR_AIML_NGRAN_enh-Core]: RP-251245 (target: RAN #109)

### 11.1	General

R3-255041	(BL CR to 37.480) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					37.480 v18.1.0	  CR-0009  rev 2 Cat: B (Rel-19)

					Source: CATT, ZTE Corporation

(Replaces R3-253072)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255042	(BL CR to 37.483) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0170  rev 3 Cat: B (Rel-19)

					Source: Samsung, ZTE Corporation, Nokia

(Replaces R3-253983)

Abstract:

BL CR

Decision: 		The document was revised to R3-255980.

R3-255980	(BL CR to 37.483) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0170  rev 4 Cat: B (Rel-19)

					Source: Samsung, ZTE Corporation, Nokia

(Replaces R3-255042)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255043	(BL CR to 38.401) Enhancement of AI/ML Energy Saving for NGRAN split architecture

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0478  rev 1 Cat: B (Rel-19)

					Source: CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms

(Replaces R3-253985)

Abstract:

BL CR

Decision: 		The document was revised to R3-255982.

R3-255982	(BL CR to 38.401) Enhancement of AI/ML Energy Saving for NGRAN split architecture

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0478  rev 2 Cat: B (Rel-19)

					Source: CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms

(Replaces R3-255043)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255044	(BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1411  rev 9 Cat: B (Rel-19)

					Source: ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

(Replaces R3-253986)

Abstract:

BL CR

Decision: 		The document was revised to R3-255983.

R3-255983	(BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1411  rev 10 Cat: B (Rel-19)

					Source: ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

(Replaces R3-255044)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255045	(BL CR to 38.425) UE performance enhancement for split architecture

Type: CR		For: Agreement
					38.425 v18.1.0	  CR-0157  rev 1 Cat: B (Rel-19)

					Source: Nokia, Samsung, Ericsson, Huawei, ZTE, Deutsche Telekom, FiberCop, Jio Platforms

(Replaces R3-253987)

Abstract:

BL CR

Decision: 		The document was revised to R3-255984.

R3-255984	(BL CR to 38.425) UE performance enhancement for split architecture

Type: CR		For: Agreement
					38.425 v18.1.0	  CR-0157  rev 2 Cat: B (Rel-19)

					Source: Nokia, Samsung, Ericsson, Huawei, ZTE, Deutsche Telekom, FiberCop, Jio Platforms

(Replaces R3-255045)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255046	(BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1532  rev 8 Cat: B (Rel-19)

					Source: Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno

(Replaces R3-253988)

Abstract:

BL CR

Decision: 		The document was revised to R3-255985.

R3-255985	(BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1532  rev 9 Cat: B (Rel-19)

					Source: Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno

(Replaces R3-255046)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255047	(BL CR to 37.340) AIML for NR-DC

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: Huawei, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Huawei, Ofinno, FiberCop, LG Electronics, Jio Platforms

(Replaces R3-254028)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255048	(BL CR to 38.300) Support of enhancements on AI/ML for NG-RAN

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia, Jio Platforms

(Replaces R3-254029)

Abstract:

BL CR

Decision: 		The document was revised to R3-255981.

R3-255981	(BL CR to 38.300) Support of enhancements on AI/ML for NG-RAN

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia, Jio Platforms

(Replaces R3-255048)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255178	Work plan for enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

Type: Work Plan		For: Information
					Source: ZTE Corporation, NEC

Decision: 		The document was noted.

R3-255462	(TP to TS 38.300) AI/ML support

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT

Decision: 		The document was revised to R3-255801.

R3-255801	(TP to TS 38.300) AI/ML support

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT

(Replaces R3-255462)

Discussion:

-	Replace “SSBs” by “beams”

-	Replace “non executed” by “non applied”

-	Add sentence capturing agreement from CB#12

-	Address any other offline comments

Decision: 		The document was revised to R3-255933.

R3-255933	(TP to TS 38.300) AI/ML support

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT, NEC, InterDigital, ZTE Corporation, LG Electronics, Ofinno, Nokia, CATT, Qualcomm, Lenovo, Huawei, Samsung, Rakuten

(Replaces R3-255801)

Decision: 		The document was agreed.

R3-255463	(TP to TS 38.401) AI/ML support

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT

Decision: 		The document was revised to R3-255802.

R3-255802	(TP to TS 38.401) AI/ML support

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT

(Replaces R3-255463)

Decision: 		The document was revised to R3-255946.

R3-255946	(TP to TS 38.401) AI/ML support

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms, BT, ZTE Corporation,CATT, Ofinno, Samsung, LG Electronics, NEC, Nokia, Qualcomm

(Replaces R3-255802)

Decision: 		The document was agreed.

### 11.2	AI/ML enabled Slicing

R3-255170	Remaining issues in AI/ML assisted  Network Slicing

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was treated.

R3-255171	(TP to BL CR to 38.423) Support of AI/ML assisted Network Slicing

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Qualcomm, China Unicom

Discussion:

-	Update with agreements captured in Chair minutes

Decision: 		The document was revised to R3-255760.

R3-255760	(TP to 38.423) AI/ML assisted Network Slicing

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Qualcomm, China Unicom, Lenovo, Nokia, Ericsson, Huawei, NEC, InterDigital, CATT, Jio Platforms, Ofinno, Deutsche Telekom,Samsung

(Replaces R3-255171)

Decision: 		The document was agreed.

R3-255203	(TP to BLCR TS38.423) Clean-up for AI/ML-based slicing

Type: other		For: Agreement
					38.423 v..
					Source: NEC

Decision: 		The document was noted.

R3-255357	Further discussion on per-UE per-slice UE performance in AI/ML-based Network Slicing

Type: discussion		For: Agreement
					Source: Huawei, Jio Platforms, FiberCop, BT

Decision: 		The document was noted.

R3-255358	(TP for AI/ML BLCR to TS 38.423) Per-UE per-slice UE performance in AI/ML-based Network Slicing

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, Ofinno, Jio Platforms, FiberCop, BT

Decision: 		The document was not treated.

R3-255449	(TP to TS 38.423) AI/ML Slicing Remaining Aspects

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

Decision: 		The document was noted.

R3-255450	[Draft] Reply LS on clarification on UE performance measurement at PDCP level

Type: LS out		For: Approval
					to SA5
					Source: Nokia

Decision: 		The document was not treated.

R3-255464	AI/ML enabled Network Slicing

Type: discussion		For: Approval
					Source: Ericsson, Jio Platforms

Decision: 		The document was noted.

R3-255465	(TP for BLCR to 38.423) Support of AI/ML assisted Network Slicing

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255497	(TP on 38.423)Discussion on slice level UE performance

Type: other		For: Agreement
					38.423 v..
					Source: CATT

Decision: 		The document was noted.

R3-255704	(TP to BLCR for TS 38.423) AIML enabled network slice

Type: other		For: Agreement
					38.423 v..
					Source: CMCC

Decision: 		The document was not treated.

R3-255603	Packet loss metric Observability in UE performance

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255661	Discussion on AI/ML enabled slicing

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

[Stage 2 only]

R3-255354	(TP for AI/ML BLCR to TS 38.300) Capturing agreements in Stage 2 RAN specifications

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Jio Platforms, Deutsche Telekom, Ofinno, FiberCop, Qualcomm, InterDigital, Orange

Decision: 		The document was not treated.

R3-255355	(TP for AI/ML BLCR to TS 38.401) Capturing agreements in Stage 2 RAN specifications

Type: other		For: Agreement
					38.401 v..
					Source: Huawei, Jio Platforms, Deutsche Telekom, FiberCop, InterDigital, CATT

Decision: 		The document was not treated.

R3-255356	(TP for AI/ML BLCR to TS 37.340) Capturing agreements in Stage 2 RAN specifications

Type: other		For: Agreement
					37.340 v..
					Source: Huawei, Jio Platforms, Ofinno, Qualcomm, InterDigital, Deutsche Telekom

Decision: 		The document was not treated.

The UL/DL Packet Loss IEs will be revised with a reference to an SA5 definition.

[Agreement]

Update the IE names in Slice Based UE Performance IE (and corresponding ASN.1 names) to align them with the procedural text, namely update them as Slice Average UE Throughput DL IE, Slice Average UE Throughput UL IE, Slice Average Packet Delay IE, Slice Average Packet Loss DL IE and Slice Average Packet Loss UL IE.

It is proposed to align the tabular with ASN.1, i.e. Slice UE Performance IE in tabular is modified to include one PLMN rather than PLMN list.

CB # 11_AIRAN_slicing

- Check TP

- Handling of slice performance reporting in case target cannot report for a specific slice?

(ZTE)

### 11.3	AI/ML enabled Coverage and Capacity Optimization

R3-255204	Clean-up for AI/ML-based CCO

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was noted.

R3-255205	(TP to BLCR TS38.423 and TS38.473) AI/ML-based CCO

Type: other		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-255284	Discussion on remaining issues for AI/ML-based CCO

Type: discussion		For: (not specified)
					Source: LG Electronics Inc.

Decision: 		The document was noted.

R3-255285	(TP for NR_AIML_NGRAN_enh-Core for TS 38.423) Discussion on remaining issues for AIML-based CCO

Type: other		For: Agreement
					38.423 v..
					Source: LG Electronics Inc.

Decision: 		The document was noted.

R3-255286	(TP for NR_AIML_NGRAN_enh-Core for TS 38.473) Discussion on remaining issues for AIML-based CCO

Type: other		For: Agreement
					38.473 v..
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-255359	Discussion on timing aspects for AI/ML-based Coverage and Capacity Optimization

Type: discussion		For: Decision
					Source: Huawei, Jio Platforms, Deutsche Telekom, Orange, BT, Vodafone

Decision: 		The document was noted.

R3-255360	(TP for AI/ML BLCR to TS 38.423) Timing information for predicted CCO issue over Xn

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, Jio Platforms, Deutsche Telekom, Orange, BT, Vodafone

Decision: 		The document was not treated.

R3-255471	UE radio measurements for CCO

Type: discussion		For: Approval
					Source: Ericsson, Jio Platforms, Orange, Deutsche Telekom

Decision: 		The document was not treated.

R3-255472	(TP for TS 38.423) – UE radio measurements for CCO

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Orange, Deutsche Telekom

Decision: 		The document was not treated.

R3-255271	AI/ML enabled Coverage and Capacity Optimization: Cause value in the Neighbour Future Coverage Modification Notification

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255272	(TP for BLCR to 38.473) AI/ML enabled Coverage and Capacity Optimization: Cause value in the Neighbour Future Coverage Modification Notification

Type: other		For: Agreement
					38.473 v..
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255385	Discussion on AI/ML based Coverage and Capacity Optimization

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255434	(TP for BLCR to TS 38.300, 38.401, 38.423, 38.473) Remaining issues for AI/ML-based CCO

Type: other		For: Agreement
					38.300 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255498	Open issues on CCO issue

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255499	(TP for 38.423 and 38.473)Open issues on CCO issue

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was revised to R3-255844.

R3-255844	(TP to BLCR for 38.423) Open issues on CCO

Type: other		For: Agreement
					38.423 v..
					Source: CATT

(Replaces R3-255499)

Decision: 		The document was revised to R3-255929.

R3-255929	(TP to BLCR on 38.423) Mechanism to cancel previously signalled AI/ML-based CCO information

Type: other		For: Agreement
					38.423 v..
					Source: CATT, Ofinno, Ericsson, Huawei, Lenovo, Jio Platforms, NEC, Deutsche Telekom , ZTE,  Rakuten, Nokia, Samsung

(Replaces R3-255844)

Decision: 		The document was agreed.

R3-255662	Discussion on AI/ML enabled CCO

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255705	(TP to BLCR for TS 38.401, TS 38.423, TS 38.473 and TS 38.300) AIML based CCO

Type: other		For: Agreement
					Source: CMCC

Decision: 		The document was not treated.

R3-255729	Discussion on AI/ML assisted Coverage and Capacity Optimization

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255730	[TP to 38.473] Support of AI/ML assisted Coverage and Capacity Optimization

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255361	Discussion on feedback information for AI/ML-based Coverage and Capacity Optimization

Type: discussion		For: Agreement
					Source: Huawei, Jio Platforms, Deutsche Telekom, Orange, Vodafone

Decision: 		The document was not treated.

R3-255362	(TP for AIML BLCR to TS 38.423) Additional feedback information for AI/ML-based Coverage and Capacity Optimization

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, Jio Platforms, Deutsche Telekom, Orange, Vodafone

Decision: 		The document was not treated.

R3-255363	Discussion on prediction update/cancel for AI/ML-based Coverage and Capacity Optimization

Type: discussion		For: Agreement
					Source: Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

Decision: 		The document was not treated.

R3-255364	(TP for AI/ML BLCR to TS 38.473) Replacing a predicted CCO assistance information previously signalled

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

Decision: 		The document was revised to R3-255788.

R3-255788	(TP for AI/ML BLCR to TS 38.473) Mechanism to cancel previously signalled AI/ML-based CCO information

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

(Replaces R3-255364)

Decision: 		The document was revised to R3-255930.

R3-255930	(TP for AI/ML BLCR to TS 38.473) Mechanism to cancel previously signalled AI/ML-based CCO information

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange, Ericsson, Samsung, Ofinno, CATT, ZTE, NEC, Lenovo, Rakuten

(Replaces R3-255788)

Decision: 		The document was agreed.

R3-255365	(TP for AI/ML BLCR to TS 38.423) Replacing a predicted CCO assistance information previously signalled

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

Decision: 		The document was not treated.

R3-255466	Prediction time for CCO issue

Type: discussion		For: Approval
					Source: Ericsson, Jio Platforms, Orange, InterDigital, BT, Deutsche Telekom, FiberCop

Decision: 		The document was not treated.

R3-255467	(TP for TS 38.423) - Prediction time for CCO issue

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Orange, InterDigital, BT, Deutsche Telekom, FiberCop

Decision: 		The document was not treated.

R3-255468	Cancel or update a predicted CCO issue

Type: discussion		For: Approval
					Source: Ericsson, Jio Platforms, Orange, InterDigital, Deutsche Telekom, FiberCop

Decision: 		The document was not treated.

R3-255469	(TP for TS 38.423) - Cancel or update a predicted CCO issue

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Orange, InterDigital, Deutsche Telekom, FiberCop

Decision: 		The document was not treated.

R3-255470	(TP for TS 38.473) - Cancel or update a predicted CCO issue

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Jio Platforms, Orange, InterDigital, Deutsche Telekom, FiberCop

Decision: 		The document was not treated.

R3-255588	AI/ML-Based CCO with Timing Aspects over Xn

Type: discussion		For: (not specified)
					Source: Jio Platforms, Ericsson

Decision: 		The document was not treated.

R3-255589	(TP for AIML BLCR to TS 38.423) AI_ML-Based CCO with timing aspects over XN

Type: other		For: (not specified)
					38.423 v..
					Source: Jio Platforms, Ericsson

Decision: 		The document was not treated.

R3-255273	AI/ML enabled Coverage and Capacity Optimization

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255317	Discussion on remaining issues in AI/ML enabled CCO

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255396	Discussion on AI/ML based CCO

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-255590	Discussion on update/cancel procedures for Predicted CCO issue vs. Detected CCO issue

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255745	Timing Information for AIML based CCO

Type: discussion		For: (not specified)
					Source: Rakuten Mobile, Inc

Decision: 		The document was not treated.

R3-255441	(TP for AI/ML BLCR to TS 38.423) AI/ML-Based CCO with Timing Aspects over Xn

Type: other		For: Decision
					38.423 v..
					Source: Jio Platforms

Decision: 		The document was withdrawn.

R3-255586	AI_ML-Based CCO with Timing Aspects over Xn

Type: discussion		For: Decision
					Source: Jio Platforms

Decision: 		The document was withdrawn.

R3-255587	(TP for AIML BLCR to TS 38.423) AI_ML-Based CCO with timing aspects over XN

Type: other		For: (not specified)
					38.423 v..
					Source: Jio Platforms

Decision: 		The document was withdrawn.

R3-255172	(TP to BL CR to 38.300) Support of AI/ML assisted CCO

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation, Lenovo, China Unicom

Decision: 		The document was not treated.

R3-255206	(TP to BLCR for TS38.300) AI/ML-based CCO

Type: other		For: Agreement
					38.300 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255173	(TP to BL CR to 38.401) Support of AI/ML assisted CCO

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation, Lenovo, China Unicom, China Telecom

Decision: 		The document was not treated.

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1) to update/cancel a prediction?

[Agreement] Add a new “cancel” code-point in the Predicted Coverage Modification Cause IE (XnAP) and Predicted CCO Issue IE (F1AP).

Code-point is received with a list of affected cells and beams, and any previously received predicted CCO issue and future coverage state associated with the same list of affected cells and beams received with the cancel code-point will be cancelled.

Timing information for predicted CCO issue over Xn?

Add a semantic text to clarify the Time for Predicted CCO Issue IE is only present from gNB-CU to gNB-DU in the sending side.

RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

ERICSSON, Huawei, Jio, InterDigital, Rakuten, BT: lack of timing information compromises the use of this feature

Whether/what additional UE performance measurement metrics?

No additional UE performance measurement metric is needed for CCO.

R3-255853	Summary of Offline Discussion on CB: # 12_AIRAN_CCO

Type: discussion		For: Decision
					Source: Huawei

Discussion: - TP to capture the above agreements, including further stage 3 details

Decision: 		The document was noted.

[Agreement]

Over both F1 and Xn, cancelling a predicted CCO issue originates from the CU and it also implies a cancellation of the associated future CCO states (if any) that have not been applied yet for cells/SSBs predicted to be affected by the predicted CCO issue, both at source and at neighbor node.

If the receiving node receives the cancel codepoint without predicted affected cells/SSBs it should be considered as an abnormal condition.

Capture in TPs for F1AP and XnAP BLCRs that cancel codepoint is introduced in the F1AP’s Predicted CCO issue IE (included in the Predicted CCO Assistance Information IE) and in the Future Coverage Modification Cause IE (included in the Future Coverage Modification Notification IE), as well as in the XnAP’s Predicted Coverage Modification Cause IE (included in the Future Coverage Modification List IE).

At any given point in time, there can be only one Predicted CCO Issue for a list of affected cells and beams originated by the source gNB-CU.

### 11.4	R18 leftovers

[Split architecture support]

R3-255698	(TP to BL CR for TS 38.473) Transfor Energy Cost through F1

Type: other		For: Agreement
					38.473 v..
					Source: CMCC, CATT, Huawei, Samsung, ZTE, Nokia, Ericsson

Decision: 		The document was agreed.

R3-255176	(TP to BL CR to 38.473) Remaining issues in F1AP for AI/ML for NG-RAN

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation

Discussion:

Merged to F1AP TP for split architecture support

Decision: 		The document was merged.

R3-255177	(TP to BL CR to 37.483) Remaining issues in E1AP for AI/ML for NG-RAN

Type: other		For: Agreement
					37.483 v..
					Source: ZTE Corporation

Discussion:

For inter-gNB HO, common understanding is Collection Time Duration for UE Performance is the same value as received over Xn from the source gNB

Decision: 		The document was revised to R3-255830.

R3-255830	(TP to BL CR to 37.483) Cleanup on Data Collection procedure

Type: other		For: Agreement
					37.483 v..
					Source: ZTE Corporation

(Replaces R3-255177)

Discussion:

-	Add ERICSSON, Samsung, LG Electronics, InterDigital, Lenovo as co-sources

Decision: 		The document was revised to R3-255932.

R3-255932	(TP to BL CR to 37.483) Cleanup on Data Collection procedure

Type: other		For: Agreement
					37.483 v..
					Source: ZTE Corporation, NEC, CATT, Nokia, Huawei, Ericsson, Samsung, LG Electronics, InterDigital, Lenovo

(Replaces R3-255830)

Decision: 		The document was agreed.

R3-255208	Clean-up for Split architecture support

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was revised to R3-255829.

R3-255829	(TP for BLCR to TS 38.473) UE performance delay monitoring

Type: other		For: Agreement
					38.473 v..
					Source: NEC, CATT, Nokia, Huawei, ZTE

(Replaces R3-255208)

Decision: 		The document was agreed.

R3-255318	Discussion on UE Performance Reporting for Split Architecture

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255368	Remaining open issues on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Type: discussion		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-255369	[Draft] LS reply to LS on energy saving for split gNB

Type: LS out		For: Approval
					to SA5
					Source: Huawei

Decision: 		The document was not treated.

R3-255370	(TP for AI/ML BLCR to TS 38.473) F1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255371	(TP for AI/ML BLCR to TS 37.483) E1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Type: other		For: Agreement
					37.483 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255397	(TP to BLCR 37.483) UE performance collection via E1 interface after intra-CU-UP HO

Type: other		For: Agreement
					37.483 v..
					Source: Lenovo

Decision: 		The document was revised to R3-255831.

R3-255831	(TP to BL CR to 37.483) Introduction of Data Collection ID in Bearer Context Modification procedure

Type: other		For: Agreement
					37.483 v..
					Source: Lenovo, Ericsson, CATT, Nokia, Huawei

(Replaces R3-255397)

Decision: 		The document was agreed.

R3-255454	Open points on UE Performance in Split Architecture

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-255455	(TP to TS 38.473, TP to TS 37.483) UE Performance in Split Architecture

Type: other		For: Agreement
					38.473 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255475	On LS discussions with SA5 on Packet loss measurements

Type: discussion		For: Approval
					Source: Ericsson, Deutsche Telekom, Jio Platforms

Decision: 		The document was not treated.

R3-255476	Introduction of new packet loss metric in UE performance

Type: discussion		For: Approval
					Source: Ericsson, Deutsche Telekom, Jio Platforms

Decision: 		The document was not treated.

R3-255477	(TP to 38.423) On packet loss metric in UE performance

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Deutsche Telekom, Jio Platforms

Decision: 		The document was not treated.

R3-255502	(TP on 37.483 and 38.473)AI/ML for split scenario

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255664	Discusson on AI/ML for NG-RAN in split architecture

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255666	Text proposals on the support of enhancements on AIML for NG-RAN

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255452	(TP to TS 38.425) Stage 2 NR User Plane Updates

Type: other		For: Agreement
					38.425 v..
					Source: Nokia, FiberCop, Deutsche Telekom

Decision: 		The document was revised to R3-255832.

R3-255832	(TP to TS 38.425) Stage 3 NR User Plane Updates

Type: other		For: Agreement
					38.425 v..
					Source: Nokia, FiberCop, Deutsche Telekom, Ericsson, CATT, Huawei, ZTE, NEC, Samsung, Lenovo

(Replaces R3-255452)

Decision: 		The document was agreed.

R3-255699	(TP to BLCR for TS 38.425) Specify Delay for UE Performance Metric_v3

Type: other		For: Agreement
					38.425 v..
					Source: CMCC, Samsung, ZTE

Decision: 		The document was not treated.

R3-255700	(TP to BLCR for TS 38.470) F1-U function for AIML

Type: other		For: Agreement
					38.470 v..
					Source: CMCC

Decision: 		The document was not treated.

R3-255399	(TP to BLCR 38.401) AIML for RAN in split architecture

Type: other		For: Agreement
					38.401 v..
					Source: Lenovo, CATT, ZTE Corporation

Decision: 		The document was not treated.

R3-255451	(TP to TS 38.401) Stage 2 updates for split architecture support

Type: other		For: Agreement
					38.401 v..
					Source: Nokia, Deutsche Telekom, Orange

Decision: 		The document was not treated.

R3-255701	(TP to BLCR for TS 38.401) UE Performance Requirement for AIML in Split Architecture

Type: other		For: Agreement
					38.401 v..
					Source: CMCC

Decision: 		The document was not treated.

[Agreement]

Capture the following exit conditions of termination of data collection in E1AP:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state

[Agreement]

Introduce UE performance Collection Configuration IE in the DATA COLLECTION REQUEST message.

For Packet Loss, align with XnAP TP agreed for slicing

R3-255827	Summary of offline discussion on AIML for split gNB

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

[Agreement]

Take Bearer suspend and release as the exit condition.

Agree to introduce Data Collection ID IE in the BEARER CONTEXT MODIFICATION REQUEST message.

[Mobility optimization for NR-DC]

R3-255175	(TP to BL CR to 38.423) Addition of predicted PSCell ID within DC procedure

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Samsung, Qualcomm, China Unicom, CMCC, CATT, Ofinno

Decision: 		The document was not treated.

R3-255207	Clean-up for NR-DC mobility optimization

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was not treated.

R3-255274	Mobility Optimization for NR-DC

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255275	(TP for BLCR to 38.423) Mobility Optimization for NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255366	Discussion on assistance information for SN configuration in AI/ML-based Mobility Optimization in NR-DC

Type: discussion		For: Agreement
					Source: Huawei, Nokia

Decision: 		The document was not treated.

R3-255367	(TP for AI/ML BLCR to TS 38.423) Discussion on UE performance feedback for AI/ML-based Mobility Optimization in NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255398	(TP to BLCR 37.340) Discussion on UE performance collection in AIML for NR-DC

Type: other		For: Agreement
					37.340 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255453	(TP to TS 38.423) Remaining issues for AI/ML Mobility Optimization in NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255473	AI/ML support for NR-DC

Type: discussion		For: Approval
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255474	(TP to TS 38.423) AI/ML support for NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255500	(TP on 37.340) Discussion on AI/ML in DC scenario

Type: other		For: Agreement
					37.340 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255501	(TP for 38.423) AI/ML in DC scenario

Type: other		For: Agreement
					38.423 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255663	Discussion on AI/ML for mobility in NR-DC

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255696	Discussion on AIML based Mobility Optimization for NR-DC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-255697	(TP to BLCR for TS 38.423) AIML based Mobility Optimization for NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: CMCC

Decision: 		The document was not treated.

R3-255858	SoD to the CB: # 15_AIRAN_NRDC

Type: discussion		For: Decision
					Source: Nokia

Discussion:- Check remaining issues for NR-DC including UE performance granularity

Decision: 		The document was noted.

R3-255919	(TP to TS 37.340) AI/ML for NR-DC

Type: other		For: Agreement
					37.340 v..
					Source: Nokia

Decision: 		The document was noted.

[Agreement]

Reporting of UL/DL Average RLC Level Throughput over Xn for a split gNB is not supported in this version of the specification.

It is up to implementation how MN derives overall UL/DL Average Throughput in case of split bearer.

Agree that UE Performance is reported from SN to MN per UE granularity (no stage 3 impact).

[Continuous MDT collection]

R3-255174	Way forward on Continuous MDT collection

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-255478	(TP to 37.320) - Continuous MDT collection targeting the same UE across RRC states

Type: other		For: Agreement
					37.320 v..
					Source: Ericsson, ZTE Corporation, BT, Deutsche Telekom, InterDigital, Jio Platforms

Decision: 		The document was not treated.

R3-255479	(TP for AI/ML BLCR to TS38.423) Continuous MDT collection targeting the same UE across RRC states

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, ZTE Corporation, BT, Deutsche Telekom, InterDigital, Jio Platform

Decision: 		The document was not treated.

R3-255665	Further Discussion on continuous MDT collection

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255800	Summary of Offline Discussion on AI/ML Stage 2

Type: discussion		For: Decision
					Source: Ericsson

Discussion:

- Check remaining issues for NR-DC

- TP for 38.300, 38.401 37.340, 37.320 with all relevant agreements including Continuous MDT if agreeable

Decision: 		The document was noted.

## 12	Additional topological enhancements for NR WI (RAN3-led) [NR_WAB_5GFemto-Core]: RP-243009 (target: RAN #109)

### 12.1	General

R3-255049	(BL CR to 38.305 for WAB) Support of Location Service Involving WAB-Nodes

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, Lenovo, CATT, Samsung, Huawei, China Telecom

(Replaces R3-253077)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255050	(BL CR to 38.401 for WAB) Support for Wireless Access Backhaul

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0439  rev 9 Cat: B (Rel-19)

					Source: Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms

(Replaces R3-253078)

Abstract:

BL CR

Decision: 		The document was revised to R3-256014.

R3-256014	(BL CR to 38.401 for WAB) Support for Wireless Access Backhaul

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0439  rev 10 Cat: B (Rel-19)

					Source: Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms

(Replaces R3-255050)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255052	(BL CR to 38.413 for WAB) Support for Wireless Access Backhaul

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1263  rev 4 Cat: B (Rel-19)

					Source: Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo

(Replaces R3-253081)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255053	(BL CR to 38.423 for WAB) Support for Wireless Access Backhaul

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1487  rev 2 Cat: B (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Ericsson, ZTE, LG Electronics, Qualcomm, Huawei, China Telecom, Samsung, Lenovo

(Replaces R3-253082)

Abstract:

BL CR

Decision: 		The document was revised to R3-256016.

R3-256016	(BL CR to 38.423 for WAB) Support for Wireless Access Backhaul

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1487  rev 3 Cat: B (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Ericsson, ZTE, LG Electronics, Qualcomm, Huawei, China Telecom, Samsung, Lenovo

(Replaces R3-255053)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255054	(BL CR to 38.455 for WAB) Support of Location Service Involving WAB-Nodes

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0189  rev 3 Cat: B (Rel-19)

					Source: Lenovo, ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, CATT, Samsung, Huawei, China Telecom, Jio Platforms (JPL)

(Replaces R3-253083)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255051	(BL CR to 38.410 for Femto) Introduction of NR Femto in NGAP list of functions

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0052  rev 5 Cat: B (Rel-19)

					Source: ZTE Corporation, Nokia, Baicells

(Replaces R3-253079)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255055	(BL CR to 38.300 for Femto) Introduction of NR Femto Architecture and Protocol Aspects

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm

(Replaces R3-254017)

Abstract:

BL CR

Decision: 		The document was revised to R3-256013.

R3-256013	(BL CR to 38.300 for Femto) Introduction of NR Femto Architecture and Protocol Aspects

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm

(Replaces R3-255055)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255056	(BL CR to TS 38.413 for Femto) Support of NR Femto architecture with NR Femto Gateway

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1232  rev 5 Cat: B (Rel-19)

					Source: Nokia, Huawei

(Replaces R3-254018)

Abstract:

BL CR

Decision: 		The document was revised to R3-256015.

R3-256015	(BL CR to TS 38.413 for Femto) Support of NR Femto architecture with NR Femto Gateway

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1232  rev 6 Cat: B (Rel-19)

					Source: Nokia, Huawei

(Replaces R3-255056)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255776	Summary of Offline Discussion on additional topological enhancement

Type: discussion		For: Decision
					Source: NTTDOCOMO, INC.

Decision: 		The document was revised to R3-255778.

R3-255778	Summary of Offline Discussion on additional topological enhancement

Type: discussion		For: Decision
					Source: NTTDOCOMO, INC.

(Replaces R3-255776)

Decision: 		The document was noted.

### 12.2	Wireless Access Backhaul (WAB)

R3-255156	Functional Aspects of WAB-Nodes

Type: discussion		For: Agreement
					Source: Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255157	(TP for WAB BL CR for TS 38.401) Functional Aspects of WAB-Nodes

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Jio Platforms

Decision: 		The document was revised to R3-255789.

R3-255789	(TP for WAB BL CR for TS 38.401) Functional Aspects of WAB-Nodes

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Jio Platforms

(Replaces R3-255157)

Discussion:

-	Add Huawei, China Telecom, NTT DOCOMO, Airbus, NEC, CATT, ZTE, Samsung, Nokia as co-source

Decision: 		The document was revised to R3-255939.

R3-255939	(TP for WAB BL CR for TS 38.401) Functional Aspects of WAB-Nodes

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson, Jio Platforms, Huawei, China Telecom, NTT DoCoMo, Airbus, NEC, CATT, ZTE, Samsung, Nokia, Nokia Shanghai Bell

(Replaces R3-255789)

Decision: 		The document was agreed.

R3-255169	Remaining aspects for the support of WAB

Type: discussion		For: (not specified)
					Source: CANON Research Centre France

Decision: 		The document was not treated.

R3-255202	(TP to BL CR of 38.423 on WAB) Discussion on access and reliability for WAB

Type: other		For: Agreement
					38.423 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255224	(TP to 38.423, 38.473) Supporting resource coordination in WAB

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255225	(TP to 38.401, 38.413) Remaining issues for WAB

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255243	(TP for to BLCR for TSto 38.401) On remaining issues of WAB

Type: other		For: Agreement
					38.401 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255244	On resource coordination for WAB

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255252	Remaining issues of WAB

Type: discussion		For: Discussion
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R3-255253	(TP to TS 38.401) WAB resource coordination

Type: other		For: Agreement
					38.401 v..
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R3-255254	BL draft CR to TS 38.300 on Support of WAB

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell

Discussion:

-	For the definition of WAB-gNB and WAB-MT in BLCR 38.300 use “as defined in TS 38.401 [4]”

Decision: 		The document was revised to R3-255784.

R3-255784	BL draft CR to TS 38.300 on Support of WAB

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell

(Replaces R3-255254)

Decision: 		The document was revised to R3-255931.

R3-255931	BL draft CR to TS 38.300 on Support of WAB

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell

(Replaces R3-255784)

Discussion:

-	revise last sentence to “The WAB-node may be stationary or mobile”

-	Add Huawei, China Telecom, NTT DOCOMO, Airbus, NEC as co-source

Decision: 		The document was revised to R3-255937.

R3-255937	Support for Wireless Access Backhaul

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell, NTTDOCOMO, Samsung, LG Electronics, Huawei, China Telecom, NTT DOCOMO, Airbus, NEC

(Replaces R3-255931)

Decision: 		The document was endorsed.

R3-255290	(TP for TS 38.401) Discussion on NG management and Xn management for WAB

Type: other		For: Agreement
					38.401 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-255291	(TP to BL CR for TS 38.413 and TS 38.423) Enhancement for WAB

Type: other		For: Agreement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-255400	(TP to BLCR 38.423) Xn management for WAB-node

Type: other		For: Agreement
					38.423 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255401	(TP to BLCR 38.413) Discussion on remaining issues for WAB-node

Type: other		For: Agreement
					38.413 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255411	(TPs for WAB BL CRs) Architecture, Access Control and Additional ULI for WAB

Type: other		For: Agreement
					Source: Huawei

Discussion:

-	Focus on the change in 8.2.1.3 of XnAP

-	Take into account the agreement on the MT ID

Decision: 		The document was revised to R3-255785.

R3-255785	(TPs for WAB BL CRs) Architecture, Access Control and Additional ULI for WAB

Type: other		For: Agreement
					Source: Huawei

(Replaces R3-255411)

Discussion:

-	9.2.2.Y: remove “, which includes the NR CGI of the WAB-MT’s serving cell and the C-RNTI allocated to the WAB-MT” from the IE description

-	Add Nokia, Ericsson, Qualcomm, ZTE, Samsung, NTT DOCOMO, NEC, Airbus as co-source

Decision: 		The document was revised to R3-255938.

R3-255938	(TP for WAB BL CR for TS 38.423) WAB-MT ID and Multi-hop prevention

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, CATT, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, ZTE, Samsung, NTT DOCOMO, NEC, Airbus

(Replaces R3-255785)

Decision: 		The document was agreed.

R3-255412	(TP for WAB BL CRs) Radio Resource Multiplexing Coordination for WAB-node

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-255523	Way Forward On Multi-hop Prevention for WAB

Type: discussion		For: (not specified)
					Source: China Telecom, CATT, Huawei, DoCoMo, Lenovo, Samsung, NEC

Decision: 		The document was not treated.

R3-255591	Discussion on remaining issues for WAB

Type: discussion		For: (not specified)
					Source: LG Electronics

Decision: 		The document was not treated.

R3-255599	(TP to TS 38.401, 38.413 and 38.423) TP for WAB support

Type: other		For: Agreement
					Source: LG Electronics

Decision: 		The document was not treated.

R3-255609	Discussion on Wireless Access Backhaul

Type: discussion		For: (not specified)
					Source: NTT DOCOMO INC.

Decision: 		The document was not treated.

R3-255631	(draft LS) Discussion on the left issues for WAB

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255632	(TP to BLCR for TS 38.423) Enhancements for WAB

Type: other		For: Agreement
					38.423 v..
					Source: Samsung

Decision: 		The document was not treated.

R3-255673	[Draft] LS on Multi-hop Topology Avoidance for WAB

Type: LS out		For: Approval
					to RAN2, cc SA2
					Source: CATT, China Telecom, Huawei, NTT Docomo, Lenovo, Samsung, NEC

(Replaces R3-253301)

Decision: 		The document was not treated.

[Agreement]

Endorse BLCR 38.300 based on R3-255254.

Keep the Additional ULI IE in NGAP UE-associated messages.

Add Note to BLCR 38.401: using SeGW for authorization is out of scope of this specification.

No need to introduce a new “WAB-gNB” indication in the NG SETUP REQUEST message.

Single-gNB solution for WAB-gNB mobility with change of UE’s AMF is not supported in Rel-19.

ng-eNB is not supported for BH-RAN, change “BH-RAN node” to “BH-gNB” everywhere.

This release does not specify any signalling for Resource coordination between WAG nodes access and BH link. Any previous agreements on resource coordination are obsolete. Update the stage2 spec accordingly. Rapporteur will update the WID.

Keep the WAB-MT ID in the Xn BL CR. The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT.

Xn Connection Management:

Option A

BH-gNB can provide to the WAB-gNB the TNL information of other gNBs

CATT: This is an enhancement, does not seem necessary

Nokia: Needed, useful since WAB node is mobile

Lenovo: We agree its useful

SAMSUNG: Although it may be considered an enhancement, its for a critical issue so is necessary

ZTE: Support

HUAWEI: Support

Canon: Support

QUALCOMM: Xn is not critical (particularly since there is no resource coordination)

Option B

The source BH-gNB can provide to the WAB-gNB the TNL information of the target BH-gNB before the WAB-MT HO.

The source BH-gNB can provide to the WAB-gNB the TNL information of its neighbour gNBs.

No consensus

Notification of UE’s CN of NTN backhauling

Include an indication of wireless backhaul type (terrestrial or non-terrestrial) in:

•	NGAP interface management signalling, between a WAB-gNB and the UE’s 5GC.

•	XnAP interface management signalling, between a WAB-gNB and the BH-gNB serving the WAB-MT co-located with the WAB-gNB.

HUAWEI, Canon: Useful for UE’s CN to select proper QoS parameters, also enables AMF to report satellite BH to SMF

QUALCOMM: Ack the issue, but should be handled generally and by SA2

No consensus

How to select the proper BH-gNB considering the inband/outband mode for WAB-node?

WAB-MT (re-)selects a proper BH-gNB based on the configuration from OAM and the knowledge about BH-gNB.

ERICSSON: Can be solved by deployment, configuration parameters will be location based

QUALCOMM: Similar view as ERICSSON, nothing needed by RAN3

Nokia: no issue in realistic deployments

CB # 19_AddTop_WAB

- Capture all agreements in TPs

- BL CR cleanup and corrections, e.g. 38.401 in 5789

- 38.300 draft BL CR finalization

- Finalize XnAP TP in 5785

(Docomo)

### 12.3	5G Femto

R3-255162	[TP for BL CR NR Femto TS 38.413] Completion of open points of NR Femto

Type: other		For: Agreement
					38.413 v..
					Source: Nokia

Decision: 		The document was noted.

R3-255163	[TP for BL CR NR Femto TS 38.413] Paging issue with NR Femtos

Type: other		For: Agreement
					38.413 v..
					Source: Nokia

Decision: 		The document was noted.

R3-255226	Discussion on IP version selection at Femto GW

Type: discussion		For: Discussion
					Source: ZTE Corporation, China Telecom, Samsung, Baicells, Pengcheng Laboratory

Decision: 		The document was noted.

R3-255227	(TP to TS 38.300) on support of Femto

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation

Discussion:

ERICSSON: TP seems agreeable

-	Further cleanup and corrections to reflect existing agreements

Decision: 		The document was revised to R3-255799.

R3-255799	(TP to TS 38.300) Clean-up for Stage 2

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation, Nokia, Ericsson, Huawei, LG Electronics

(Replaces R3-255227)

Decision: 		The document was agreed.

R3-255246	(TP to BLCR for TS 38.300) Miscellaneous corrections for NR Femto

Type: other		For: Agreement
					38.300 v..
					Source: CATT

Decision: 		The document was noted.

R3-255245	Discussion on remain issue of NR Femto

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was noted.

R3-255413	(TP for Femto BL CR for TS 38.300/38.413) Discussion on NG mobility impact for NR Femto

Type: other		For: Agreement
					Source: Huawei, Ericsson, Qualcomm

Discussion:

-	Change criticality of new IE, check other NGAP details

Decision: 		The document was revised to R3-255805.

R3-255805	(TP for Femto BL CRs for TS 38.413) NG mobility for NR Femto

Type: other		For: Agreement
					38.413 v..
					Source: Huawei, Ericsson, Qualcomm, Nokia, LG Electronics

(Replaces R3-255413)

Decision: 		The document was agreed.

R3-255380	On remaining issues for NR Femto

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255402	Discussion on remaining issues for NR Femto

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-255414	(TP for Femto BL CR for TS38.300) Discussion on NR Femto awareness at CN

Type: other		For: Agreement
					38.300 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255415	(TP for Femto BL CRs for TS 38.413) NR Femto awareness at CN

Type: other		For: Agreement
					38.413 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255540	NR Femto Indication in NGAP

Type: discussion		For: Discussion
					Source: Ericsson, Qualcomm Incorporated

Decision: 		The document was noted.

R3-255612	Discussion on 5G Femto

Type: discussion		For: (not specified)
					Source: NTT DOCOMO INC.

Decision: 		The document was not treated.

R3-255633	Discussion on the left issues for NR Femto

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255634	(TP to BLCR for TS 38.300) Security verification for NR Femto

Type: other		For: Agreement
					38.300 v..
					Source: Samsung

Decision: 		The document was not treated.

R3-255744	(TP to TS 38.413) on support of Femto

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

Paging femtos with limited Xn connectivity

Observation 1: upon paging failure to last served NR Femto the paging escalation to TA or RA could lead to paging tens to hundreds of NR Femtos in the area whereas the end user is likely near the last serving NR cell.

Option 1: introduce a second list of recommended gNbs for paging for second paging

This second list of recommended gNBs for paging can be sent by the last serving NR Femto in the NGAP UE Context Release Complete message together with current list of recommended cells and gNBs. This list contains the list of NR Femtos neighboring the last serving femto as configured in the last serving femto even if no Xn interface exists between them. The AMF may use this second list after failing the first paging to last serving NR femto in the second paging (first paging repetition).

Option 2: introduce a new NR Femto area ID

ERICSSON: Nothing broken, signaling optimization

QUALCOMM: Same view as ERICSSON, nothing needs to be solved

ZTE/Samsung/HUAWEI: Optimization

No consensus

IP version selection at Femto GW

When NR Femto GW is deployed, the Femto GW may perform IP version selection for NG-U transport by implementation. No stage 3 impact.

Agree the Stage 2 TP that the Femto GW may perform IP version selection by implementation.

Nokia: Already agreed that IP version selection can occur at Femto, don’t need a 2nd option

QUALCOMM: Legacy specifications are sufficient, this is not needed and potential dangerous if Femto GW makes incorrect decision

ERICSSON: Agree with Nokia and Qualcomm

No consensus

RAN3 should discuss the NG HO routing issue and clarify how, for NG HO to a target NR Femto under an NR Femto GW, the target NR Femto GW can determine the target NR Femto.

Nokia: No issue, can be managed by implementation. Perhaps no harm to add the IE to avoid any coordination burden

CATT: There are multiple implementation and deployment options to solve the issue

QUALCOMM: Existing specifications require NR Femto to sniff into a container, proposal gives a cleaner solution

ERICSSON: Implementation can work, signaling solution is worth discussing

HUAWEI: Implementation based solution requires special coordination

[Agreement] For the routing of HANDOVER REQUEST message from target NR Femto GW to correct target Femto node in case of NG-based HO, the AMF includes the target global gNB ID in the HANDOVER REQUEST message before sending it to the target NR Femto GW.

CB # 20_ AddTop_Femto

- Implement above agreements in TPs

- BL CR cleanup and corrections

(Docomo)

R3-255834	(TP for Femto BL CR for TS 38.300) Support of NG mobility for NR Femto

Type: other		For: discussion
					38.300 v..
					Source: Huawei, Ericsson, Qualcomm

Discussion:

-	Replace text with “In case of NG handover towards an NR Femto node connected to an NR Femto GW, including the global RAN Node ID of the target NR Femto node in the HANDOVER REQUEST message.”

Decision: 		The document was revised to R3-255940.

R3-255940	(TP for Femto BL CR for TS 38.300) Support of NG mobility for NR Femto

Type: other		For: discussion
					38.300 v..
					Source: Huawei, Ericsson, Qualcomm, Nokia

(Replaces R3-255834)

Decision: 		The document was agreed.

## 13	NR Mobility Enhancements WI [NR_Mob_Ph4-Core]: RP-250339 (target: RAN #109)

### 13.1	General

R3-255057	(BL CR to 37.483) Introducing Rel-19 Mobility enhancement

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0130  rev 9 Cat: B (Rel-19)

					Source: LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC, Ofinno, Jio Platforms (JPL)

(Replaces R3-253085)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255058	(BL CR to 38.420) Support for Inter-CU LTM

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0046  rev 7 Cat: B (Rel-19)

					Source: ZTE Corporation, China Telecom, Samsung, Nokia, CATT, NEC, LG Electronics, Ericsson, Huawei, Lenovo

(Replaces R3-253088)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255059	(BL CR to 38.470) Support for Inter-CU LTM procedure

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0164  rev 3 Cat: B (Rel-19)

					Source: Samsung, Huawei, LG Electronics, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

(Replaces R3-253091)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255060	(BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

(Replaces R3-254006)

Abstract:

BL CR

Decision: 		The document was revised to R3-256003.

R3-256003	(BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

(Replaces R3-255060)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255061	(BL CR to 38.300) Support for Inter-CU LTM

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

(Replaces R3-254007)

Abstract:

BL CR

Decision: 		The document was revised to R3-256004.

R3-256004	(BL CR to 38.300) Support for Inter-CU LTM

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

(Replaces R3-255061)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255062	(BL CR to 38.401) on conditional intra-CU LTM and intra-CU LTM

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0462  rev 5 Cat: B (Rel-19)

					Source: China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm, Jio Platforms

(Replaces R3-254008)

Abstract:

BL CR

Decision: 		The document was revised to R3-256005.

R3-256005	(BL CR to 38.401) on conditional intra-CU LTM and intra-CU LTM

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0462  rev 6 Cat: B (Rel-19)

					Source: China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm, Jio Platforms

(Replaces R3-255062)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255064	(BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1488  rev 3 Cat: B (Rel-19)

					Source: Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

(Replaces R3-254010)

Abstract:

BL CR

Decision: 		The document was revised to R3-256023.

R3-256023	(BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1488  rev 4 Cat: B (Rel-19)

					Source: Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

(Replaces R3-255064)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255065	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1547  rev 6 Cat: B (Rel-19)

					Source: Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated, Jio Platforms

(Replaces R3-254011)

Abstract:

BL CR

Decision: 		The document was revised to R3-256007.

R3-256007	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1547  rev 7 Cat: B (Rel-19)

					Source: Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated, Jio Platforms

(Replaces R3-255065)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255715	(BL CR to 38.423) Xn support for inter-CU LTM

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1414  rev 11 Cat: B (Rel-19)

					Source: Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

(Replaces R3-255063)

Abstract:

BL CR

Decision: 		The document was revised to R3-256006.

R3-256006	(BL CR to 38.423) Xn support for inter-CU LTM

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1414  rev 12 Cat: B (Rel-19)

					Source: Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

(Replaces R3-255715)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255063	(BL CR to 38.423) Xn support for inter-CU LTM

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1414  rev 10 Cat: B (Rel-19)

					Source: Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

(Replaces R3-254009)

Abstract:

BL CR

Decision: 		The document was withdrawn.

### 13.2	Support for inter-CU LTM

R3-255011	LS on RAN2 agreements for SP CSI-RS activation/deactivation

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504727, to RAN3, cc -
					Source: RAN2(CATT)

Decision: 		The document was noted.

R3-255027	Reply LS on security handling for inter-CU LTM in non-DC cases

Type: LS in		For: Discussion
					Original outgoing LS: S3-252398, to RAN3, cc RAN2
					Source: SA3(Huawei)

Abstract:

Att: S3-252423

Decision: 		The document was noted.

R3-255426	[DRAFT] Reply LS on security handling for inter-CU LTM in non-DC cases

Type: LS out		For: Approval
					to SA3, cc RAN2
					Source: Huawei

Decision: 		The document was not treated.

R3-255625	Fetching reference configuration from candidate gNB in inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

Decision: 		The document was not treated.

R3-255724	Support for Semi-persistent CSI-RS transmission

Type: discussion		For: (not specified)
					Source: Ericsson, Jio Platforms, Verizon Wireless, ZTE, Ofinno

Decision: 		The document was not treated.

R3-255532	PRACH resources for RACH-less LTM

Type: discussion		For: (not specified)
					Source: Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

Decision: 		The document was not treated.

R3-255138	TP (BL CR TS 38.300, TS 38.473, TS 38.423) Remaining issues on Inter-CU LTM procedure

Type: other		For: Agreement
					Source: Nokia

Decision: 		The document was revised to R3-255857.

R3-255857	TP (BL CR TS 38.300) Remaining issues on Inter-CU LTM procedure

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, China Telecom, NEC, LG Electronics, CATT, Samsung, Huawei, Google, Ericsson, ZTE, Ofinno

(Replaces R3-255138)

Decision: 		The document was agreed.

R3-255374	(TP for TS38.401) On support of inter-CU LTM

Type: other		For: Agreement
					38.401 v..
					Source: China Telecom

Decision: 		The document was revised to R3-255871.

R3-255871	(TP for TS38.401) On support of inter-CU LTM

Type: other		For: Agreement
					38.401 v..
					Source: China Telecom, Ericsson, Nokia, Google, LG Electronics, Samsung, CATT, ZTE, NEC, Qualcomm

(Replaces R3-255374)

Decision: 		The document was agreed.

R3-255197	(TP to BL CR for TS 38.423 and 38.473 on Inter-CU LTM) Remaining Rel-19 inter-CU LTM issues

Type: other		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-255268	Completion of Inter-CU LTM

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-255424	(TP for LTM BLCR for TS38.300): Inter-CU LTM

Type: other		For: Agreement
					38.300 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255149	(TP to 38.423, 38.473) Inter-CU LTM

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255659	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255301	Signalling enhancements for Inter-CU LTM handover

Type: discussion		For: Agreement
					Source: Qualcomm India Pvt Ltd

Decision: 		The document was not treated.

R3-255421	Discussion on inter-CU LTM

Type: discussion		For: (not specified)
					Source: NTT DOCOMO INC..

Decision: 		The document was not treated.

R3-255604	Discussion for general issues in Inter-CU LTM

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255403	[TP to BLCR for TS 38.401] Inter-CU LTM

Type: other		For: Agreement
					38.401 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255614	Discussions on finalizing the essential aspects of Inter-CU LTM

Type: discussion		For: Decision
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-255550	gNB-DU initiated LTM resource reconfiguration

Type: discussion		For: Agreement
					Source: Rakuten Mobile Inc, Qualcomm Inc, NTT DOCOMO INC

Decision: 		The document was not treated.

R3-255150	(TP to BL CR for TS 38.300, 38.473, 38.423) Rel-19 Set ID assignment

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255375	Discussion on inter-CU LTM in DC scenario

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255139	Discussion on Inter-CU LTM with Dual Connectivity

Type: discussion		For: Decision
					Source: Nokia

Decision: 		The document was not treated.

R3-255660	Additional Discussion on inter-gNB-CU LTM

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255198	(TP to BL CR for TS 38.423 and 37.340 on Inter-CU LTM with DC) Remaining issues of Rel-19 inter-CU LTM in DC scenario

Type: other		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-255404	[TP to BLCR for TS 38.423] Inter-CU LTM in DC

Type: other		For: Agreement
					38.423 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255418	(TP to BL CR 38.423) Clarification on inter-CU LTM and LTM with SCG in NR-DC

Type: other		For: Agreement
					38.423 v..
					Source: Google

Decision: 		The document was not treated.

R3-255419	(TP to BL CR 37.340 and 38.473) Clarification on inter-CU LTM and LTM with SCG in NR-DCs

Type: other		For: Agreement
					38.473 v..
					Source: Google

Decision: 		The document was not treated.

R3-255283	Open Issues on Access Success for Inter-SN SCG LTM

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255281	Remaining Issues on Data Forwarding for SN initiated Inter-SN LTM

Type: discussion		For: Decision
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255282	(TP for TS 38.423) Cell Switch Notification for LTM DC Scenario

Type: other		For: Agreement
					38.423 v..
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255269	(TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 38.401) – Support for inter-CU LTM

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was revised to R3-255842.

R3-255842	(TP for LTM BL CR for TS 38.423) - Support for inter-CU LTM

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson

(Replaces R3-255269)

Discussion:

-	add magic sentences in ASN.1 where needed

Decision: 		The document was revised to R3-255961.

R3-255961	(TP for LTM BL CR for TS 38.423) - Support for inter-CU LTM

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, NEC, Nokia, ZTE, Lenovo, Samsung, Qualcomm, Verizon Wireless, Huawei, China Telecom, Ofinno, CATT, Google, LG Electronics

(Replaces R3-255842)

Decision: 		The document was agreed.

R3-255405	[TP to BLCR for TS 38.423] Inter-CU LTM in DC - text update

Type: other		For: Agreement
					38.423 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255425	(TP for LTM BLCR for TS38.473):Inter-CU LTM

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

Decision: 		The document was revised to R3-255835.

R3-255835	(TP for LTM BLCR for TS38.473):Inter-CU LTM

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

(Replaces R3-255425)

Discussion:

-	Add LGE in co-sources

Decision: 		The document was revised to R3-255962.

R3-255962	(TP for LTM BLCR for TS 38.473):Inter-CU LTM

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, China Telecom, Google, Nokia, NEC, Ericsson, CATT, Samsung, ZTE, LG Electronics

(Replaces R3-255835)

Decision: 		The document was agreed.

R3-255440	(TP for LTM CR for TS38.423): LTM Resource Lifecycle Management in inter-CU LTM

Type: other		For: (not specified)
					38.423 v..
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255533	(TP to BL CR for TS 38.423) – PRACH Resources for RACH-less LTM

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

Decision: 		The document was not treated.

R3-255534	(TP to BL CR for TS 38.473) – PRACH Resources for RACH-less LTM

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

Decision: 		The document was not treated.

R3-255601	Inter-CU LTM Robustness Enhancements

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255605	(TP to BL CR for TS37.340) Discussion for Inter-CU LTM in DC

Type: other		For: Decision
					37.340 v..
					Source: CATT

Decision: 		The document was revised to R3-255904.

R3-255904	(TP for 37.340) Discussion for Inter-CU LTM in DC

Type: other		For: Decision
					37.340 v..
					Source: CATT, Google, China Telecom, Samsung, Nokia, NEC, ZTE, Ofinno, Ericsson, LG Electronics, Huawei

(Replaces R3-255605)

Discussion:

This text is to be further checked in Q4

Decision: 		The document was agreed.

R3-255615	(TP for NR_Mob_Ph4 TS 38.423) Inter-CU LTM

Type: other		For: Agreement
					38.423 v..
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-255626	(TP for LTM BLCR for TS38.423): Fetching reference configuration from candidate gNB in inter-CU LTM

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

Decision: 		The document was not treated.

R3-255627		(TP for LTM BLCR for TS38.300): Fetching reference configuration from candidate gNB in inter-CU LTM

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

Decision: 		The document was not treated.

R3-255628	Clarification on the single UE XnAP association in inter-CU LTM

Type: discussion		For: (not specified)
					Source: Huawei, NEC, LG Electronics

Decision: 		The document was not treated.

R3-255629	(TP for LTM BLCR for TS38.300): Clarification on the single Xn UE association in inter-CU LTM

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, NEC, LG Electronics

Decision: 		The document was not treated.

R3-255630	(TP for LTM BLCR for TS38.423): Clarification on the single Xn UE association in inter-CU LTM

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, NEC, LG Electronics

Decision: 		The document was not treated.

R3-255725	(TP to BL CR for TS 38.423) Support for Semi-persistent CSI-RS transmission (Option 1)

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Verizon Wireless, ZTE

Decision: 		The document was not treated.

R3-255726	(TP to BL CR for TS 38.473) Support for Semi-persistent CSI-RS transmission (Option 1)

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Jio Platforms, Verizon Wireless, ZTE

Decision: 		The document was not treated.

R3-255727	(TP to BL CR for TS 38.423) Support for Semi-persistent CSI-RS transmission (Option 2)

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Jio Platforms, Verizon Wireless, ZTE

Decision: 		The document was not treated.

R3-255728	(TP to BL CR for TS 38.473) Support for Semi-persistent CSI-RS transmission (Option 2)

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Jio Platforms, Verizon Wireless, ZTE

Decision: 		The document was not treated.

R3-255772	Summary of offline discussion on inter-CU LTM

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was noted.

[Agreement] Confirm to have the explicit Request for CSI-RS Resource Configuration indicator in the Handover Request message.

FFS candidate gNB provides the CSI resource config in XnAP.

[Agreement] The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) during the preparation phase.

FFS on the detail of the messages, e.g. Xn HANDOVER REQUEST ACKNOWLEDGEMENT, F1 UE CONTEXT SETUP RESPONSE messages.

Further check the LTM Configuration Update Acknowledge message and UE Context Modification Response message.

RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

CATT: request and response messages include different IDs, e.g. CSI-RS Resource ID and CSI-RS Resource config ID.

ERICSSON: CSI-RS Resource config ID is clean solution.

[Agreement] For deactivation of SP CSI-RS of candidate cell(s) after the UE’s successful cell switch, the CSI-RS Coordination procedure is re-used, triggered by the previous serving gNB-CU after successful cell switch is confirmed, and toward the relevant candidate gNB-DU(s) and gNB-CU(s).

FFS source CU sends the Rel-19 Set ID per candidate cell or Rel-19 Set ID range/list of Rel-19 Set ID per-node to candidate CUs. Down-selection of two candidate solutions.

[Agreement] Agree to reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate CUs.

Once the UE XnAP association is setup, the source gNB includes the target NG-RAN node UE XnAP ID in the handover request message for any follow-up preparation.

To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

[Agreement]

The old source gNB can deliver the old target UE XnAP ID(s) to the new serving gNB via Cell Switch Notification and LTM Configuration Update message.

The source gNB sends the Data Forwarding Information as per-PDU session level to the candidate gNBs in the LTM CONFIGURATION UPDATE message.

Include the Tag ID Pointer and RACH resource request ID in TA Information Transfer in XnAP.

Include the TA values in Cell Switch Notification message.

RAN3 discuss on the potential enhancement on that the source gNB can request a candidate gNB to provide a reference configuration in Rel-19. But there is no consensus.

R3-255781	CB:#MobilityEnh_LTM

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was noted.

[Agreement]

For Rel-19 Set ID assignment, source gNB-CU sends the Rel-19 Set ID list per-node to candidate gNB-CUs in Handover Request message.

To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

For CSI-RS configuration in F1AP and XnAP, including the CSI-RS resource configuration and the CSI-RS resource type in the UE Context Setup Response, UE Context Modification Response and Handover Request Acknowledge message.

For TCI State List configuration over the XnAP, adopt the similar solution as TEI18.

Include the explicit request for CSI-RS Resource Configuration for CSI acquisition indicator in the Handover Request message.

The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) in the LTM Configuration Update Acknowledge and the UE Context Modification Response message.

For CSI-RS coordination procedure, include the mandatory CSI Resource Config ID (i.e. LTM-CSI-ResourceConfigId) in the both request and response message in F1AP and XnAP.

The CSI-RS-based beam measurement results should be sent from the CU to the DU in the F1AP CU-DU Mobility Initiation Request message.

Turn the WA into agreement: Different candidate PSCells in the same SN can have different Rel-19 set IDs.

Do not support LTM modification on the already prepared LTM candidate PScells in DC scenario in Rel-19.

Support both Source SN-initiated and Candidate SN-initiated LTM Cancel, FFS reuse the following messages to cancel the prepared candidate PSCells resource in the candidate SN:

- SN Modification Request/ SN Modification Request Acknowledge

- SN Modification Required/ SN Modification Confirm

R3-255782	CB:#MobilityEnh_LTMNR-DC

Type: discussion		For: Decision
					Source: Lenovo

Decision: 		The document was noted.

[Agreement]

For security update for inter-CU SCG LTM, the list of {KSN, sk-counter} is provided on the granularity of Rel-19 set ID.

The MN provides a list of R19 set IDs to the candidate SN via the SN Addition Request message. MN sends R19 set ID for the candidate cell along with a list of {KSN, sk-counter} in the SN Modification Request message.

The MN sends the R19 set ID for each candidate PSCell to the source SN and other candidate SNs via the SN Modification Request message.

The candidate PSCells to be cancelled is included in the SN Modification Required message.

The MN may inform the source SN about the cancellation of the prepared candidate PSCells at a candidate SN using SN modification request message.

To support inter- or intra-CU MCG LTM with SCG, introduce a new XnAP IE to indicate to the candidate SN that the SN Addition/Modification preparation procedure is triggered as part of a MCG LTM procedure.

To support inter- or intra-CU MCG LTM with SCG, introduce a new F1AP IE to indicate to the DU of a candidate SN that the UE Context Setup/Modification procedure is triggered as part of a MCG LTM procedure.

Remove the FFS: FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

SN Change Confirm message is used to notify the list of data forwarding addresses to the source SN.

The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Addition Request Acknowledge message from the candidate SN to the MN, no RAN3 impact is foreseen.

R3-255903	(TP to BLCR for TS 38.423) Inter-CU LTM in DC

Type: other		For: discussion
					38.423 v..
					Source: LG Electronics Inc., Nokia, NEC, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, Ofinno, Jio Platforms (JPL)

Decision: 		The document was agreed.

### 13.3	Support for Conditional LTM

R3-255270	(TP to CLTM BL CR for TS 38.473, TS 38.401) – Completion of Intra-CU Conditional LTM

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was not treated.

R3-255427	(TP for LTM BLCR for TS38.473, TS38.401): Intra-CU conditional LTM

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was revised to R3-255836.

R3-255836	TP for LTM BLCR for TS38.473): Intra-CU conditional LTM

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

(Replaces R3-255427)

Discussion:

-	add missing criticalities

-	add LG Electronics, Qualcomm in co-sources

Decision: 		The document was revised to R3-255963.

R3-255963	(TP for LTM BLCR for TS38.473): Intra-CU conditional LTM

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, China Telecom, Nokia, CATT, Samsung, Ericsson, ZTE, LG Electronics, Qualcomm

(Replaces R3-255836)

Decision: 		The document was agreed.

R3-255140	Discussion on Conditional LTM in split architecture

Type: discussion		For: Decision
					Source: Nokia

Decision: 		The document was not treated.

R3-255151	(TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255839.

R3-255839	(TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM

Type: other		For: Agreement
					Source: ZTE Corporation

(Replaces R3-255151)

Decision: 		The document was revised to R3-255936.

R3-255936	(TP to BL CR for TS 38.401) Conditional LTM

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation, Nokia, China Telecom, LG Electronics, NEC, Ericsson, CATT, Huawei, Samsung, Google, Ofinno, Qualcomm

(Replaces R3-255839)

Decision: 		The document was agreed.

R3-255199	(TP to BL CR for 38.473 on conditional intra-CU LTM) Remaining issues of Rel-19 intra-CU Conditional LTM

Type: other		For: Agreement
					38.473 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255685	Remaining issues on Intra-CU Conditional LTM

Type: discussion		For: (not specified)
					Source: Samsung

Decision: 		The document was not treated.

R3-255606	(TP to 38.473) Discussion for C-LTM

Type: other		For: Decision
					38.473 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255302	Signalling enhancements for Intra-CU Conditional LTM

Type: discussion		For: Agreement
					Source: Qualcomm India Pvt Ltd

Decision: 		The document was not treated.

R3-255616	(TPs for NR_Mob_Ph4 TS 38.473 and TS 38.401) Discussions on the remaining aspects of Conditional Intra-CU LTM

Type: other		For: Agreement
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R3-255376	(TP to TS38.401) On support of intra-CU Conditional LTM

Type: other		For: Agreement
					38.401 v..
					Source: China Telecom

Decision: 		The document was not treated.

[Agreement]

To convert the working assumptions into agreement:

- To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

- To introduce a new IE with a list of candidate cells for L1 execution condition.

In case of L3 measurement report-triggered early RACH, to reuse the F1AP CU-DU Mobility Initiation procedure to notify the source DU to initiate early RACH procedure to the candidate cells.

To remove the TAT value IE in the UE CONTEXT SETUP RESPONSE message in the BLCR.

R3-255783	CB:#MobilityEnh_CLTM

Type: discussion		For: Decision
					Source: Nokia

Decision: 		The document was noted.

[Agreement]

The source DU sends the TA values and the remaining time of the TA timers, and TAG ID of candidate cells and target cell to the CU by the UE CONTEXT MODIFICATION RESPONSE message after receiving the successful cell change notification from the CU.

The CU reuses the UE Context Modification procedure to transfer the TA values and the remaining time of the TATs, and TAG ID to the target DU.

## 14	NR NTN Enhancements WI [NR_NTN_Ph3-Core]: RP-243300 (target: RAN #109)

### 14.1	General

R3-255066	(BL CR to 38.410) Introduce NG Removal procedure

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0051  rev 6 Cat: B (Rel-19)

					Source: CMCC, Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, Qualcomm, Xiaomi, LG Electronics, China Telecom, Samsung, ZTE, NEC, ETRI

(Replaces R3-253094)

Abstract:

BL CR

Decision: 		The document was revised to R3-256009.

R3-256009	(BL CR to 38.410) Introduce NG Removal procedure

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0051  rev 7 Cat: B (Rel-19)

					Source: CMCC, Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, Qualcomm, Xiaomi, LG Electronics, China Telecom, Samsung, ZTE, NEC, ETRI

(Replaces R3-255066)

Abstract:

Post Meeting: Merging TPs

- Decision: 		The document was endorsed. (*3 CRs are endorsed, to be sent to RAN#109 for approval pending WID revision to include the relevant specifications.)

R3-255067	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI, Jio Platforms

(Replaces R3-254012)

Abstract:

BL CR

Decision: 		The document was revised to R3-256008.

R3-256008	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI, Jio Platforms

(Replaces R3-255067)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255068	Support for Regenerative Payload and MBS broadcast in NR NTN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1212  rev 9 Cat: B (Rel-19)

					Source: CATT, Thales, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Qualcomm, Samsung, Xiaomi, CMCC, China Telecom, Jio Platforms, LG Electronics,  NEC, ETRI, SES, ESA

(Replaces R3-254013)

Abstract:

BL CR

Decision: 		The document was revised to R3-256010.

R3-256010	Support for Regenerative Payload and MBS broadcast in NR NTN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1212  rev 10 Cat: B (Rel-19)

					Source: CATT, Thales, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Qualcomm, Samsung, Xiaomi, CMCC, China Telecom, Jio Platforms, LG Electronics,  NEC, ETRI, SES, ESA

(Replaces R3-255068)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

### 14.2	Support MBS broadcast service

### 14.3	Support of Regenerative payload

R3-255024	Reply LS on OAM requirements to support regenerative payload transport links

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505961, to RAN3, SA5, cc TSG SA, TSG RAN
					Source: SA2(CATT)

Decision: 		The document was noted.

R3-255180	(TP to BL CR for TS38.300) OAM configuration for supported TAI list

Type: other		For: Agreement
					38.300 v..
					Source: CATT, Nokia, Nokia Shanghai Bell, ZTE, Samsung, China Telecom, CSCN, CMCC, LG Electronics, NEC, Xiaomi

Discussion:

QUALCOMM: We have concern for this enhancement without SA2 support

HUAWEI: It is an OAM feature which does not require any signaling support

OAM-based solution is not precluded

Decision: 		The document was noted.

R3-255510	(TP to BLCR for TS 38.300) Further discussion on TAI configuration

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Jio Platforms, Ericsson

Decision: 		The document was noted.

R3-255545	TAI Coordination and OAM

Type: discussion		For: Discussion
					Source: Ericsson, Huawei

Decision: 		The document was noted.

R3-255262	Remaing issues for NR NTN

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255264	(TP to BL CR for 38.413) Support of MBS Broadcast

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255292	(TP to BL CR for TS 38.300) Clarification on the OAM requirements

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to R3-255889.

R3-255889	(TP to BL CR for TS 38.300) Clarification on the OAM requirements

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Nokia Shanghai Bell, Ericsson, Xiaomi, ZTE, Huawei, CATT, Thales, LG Electronics Inc., Airbus

(Replaces R3-255292)

Discussion:

-	Change “hosting NTN payload of the serving gNB” to “NTN payload hosting the serving gNB”

-	Add Jio Platforms, Samsung as co-source

Decision: 		The document was revised to R3-255947.

R3-255947	(TP to BL CR for TS 38.300) Clarification on the OAM requirements

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Nokia Shanghai Bell, Ericsson, Xiaomi, ZTE, Huawei, CATT, Thales, LG Electronics Inc., Airbus, Jio Platforms, Samsung

(Replaces R3-255889)

Decision: 		The document was agreed.

R3-255344	(TP for TS 38.410) NG Removal completion

Type: other		For: Agreement
					38.410 v..
					Source: Samsung, Huawei, Nokia, Nokia Shanghai Bell, Ericsson

Decision: 		The document was agreed.

R3-255506	(TP for TS 38.413) NG Removal completion

Type: other		For: Agreement
					38.413 v..
					Source: Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, Ericsson, Thales, Jio Platforms, CATT, Qualcomm Incorporated, Deutsche Telekom,Samsung

Decision: 		The document was agreed.

R3-255617	(TP for NR_NTN_Ph3 TS 38.300 BL CR) Stage-2 capturing logical connection between NG Removal/Setup and ephemeris info

Type: other		For: Agreement
					38.300 v..
					Source: LG Electronics Inc., Ericsson, Nokia, Nokia Shanghai Bell, Huawei

Decision: 		The document was revised to R3-255890.

R3-255890	(TP for NR_NTN_Ph3 TS 38.300 BL CR) Stage-2 capturing logical connection between NG Removal/Setup and ephemeris info

Type: other		For: Agreement
					38.300 v..
					Source: LG Electronics Inc., Ericsson, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT

(Replaces R3-255617)

Discussion:

-	Add Jio Platforms, Airbus as co-sources

-	Remove discussion and Annex

Decision: 		The document was revised to R3-255948.

R3-255948	(TP for NR_NTN_Ph3 TS 38.300 BL CR) Stage-2 capturing logical connection between NG Removal/Setup and ephemeris info

Type: other		For: Agreement
					38.300 v..
					Source: LG Electronics Inc., Ericsson, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT, Jio Platforms, Airbus

(Replaces R3-255890)

Decision: 		The document was agreed.

R3-255508	(TP to BL CR for TS 38.300) Hard FLSO and AMF management

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

Decision: 		The document was revised to R3-255891.

R3-255891	(TP to BL CR for TS 38.300) Hard FLSO and AMF management

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

(Replaces R3-255508)

Discussion:

-	Add Jio Platforms, Airbus as co-sources

Decision: 		The document was revised to R3-255949.

R3-255949	(TP to BL CR for TS 38.300) Hard FLSO and AMF management

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom, Airbus, LG Electronics Inc., CATT

(Replaces R3-255891)

Decision: 		The document was agreed.

R3-255442	(TP for TS 38.413) Introduce gNB-initiated AMF switch signaling to support UE context transfer for regenerative NTN gNB mobility

Type: other		For: Agreement
					38.413 v..
					Source: Jio Platforms

Decision: 		The document was withdrawn.

R3-255671	Discussion on impacts on Xn interface with SMTC enhancment

Type: discussion		For: Discussion
					Source: CSCN

Decision: 		The document was not treated.

R3-255672	(TP to BL CR for TS 38.300 on NR_NTN_Ph3) Stage 2 Updates for Regenerative Payload

Type: other		For: Agreement
					38.300 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255310	nodelevel indication of temporary NTN feeder-link outage via RAN CONFIGURATION UPDATE

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1289  Cat: B (Rel-19)

					Source: Jio Platforms

Abstract:

NTN regenerative payloads see short, predictable feeder-link outages; without a node-level “temporary outage” signal, AMFs may drop UE context and disrupt service. This CR adds optional RAN CONFIGURATION UPDATE (gNB?AMF) IEs so the gNB can signal suspend/

Decision: 		The document was withdrawn.

[NG suspend/resume]

R3-255179	(TP to BL CR for TS38.300) Support of NG Suspend Resume

Type: other		For: Agreement
					38.300 v..
					Source: CATT, ZTE, Nokia, Nokia Shanghai Bell, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Discussion:

ERICSSON: Principles in this paper are incorrect, see 5546

Decision: 		The document was noted.

R3-255263	(TP to BL CR for 38.413) Support of NG Suspend and Resume Indication

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation, CATT, Nokia, Nokia Shanghai Bell, Qualcomm,  CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Decision: 		The document was not treated.

R3-255546	Downlink NG Transmission Suspend/Resume

Type: discussion		For: Discussion
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

Decision: 		The document was noted.

R3-255547	Downlink NG Transmission Suspend/Resume – Stage 2 TP

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

Decision: 		The document was agreed.

R3-255548	Downlink NG Transmission Suspend/Resume – NGAP TP

Type: other		For: Agreement
					38.413 v..
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

Decision: 		The document was agreed.

R3-255212	(TP to BL CR for TS 38.300 on NR_NTN_Ph3) Discussion on NTN leftover issue

Type: other		For: Agreement
					38.300 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255287	Remaining Issues on Support of NTN Regenerative Architecture

Type: discussion		For: Discussion
					Source: TCL

Decision: 		The document was not treated.

R3-255334	(TP to BL CR TS 38.300) Support of NG suspend/resume

Type: other		For: Agreement
					38.300 v..
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255335	(Draft LS out) Support of NG suspend resume

Type: LS out		For: Approval
					to SA2, cc CT4
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255343	Remaining issues on support of regenerative payload for NR NTN

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255382	NG transmission suspend resume procedure

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255602	Reconsideration on NG Suspend/Resume Signaling for FLSO

Type: discussion		For: (not specified)
					Source: Jio Platforms

Decision: 		The document was not treated.

R3-255640	Optional NGAP IEs for node-level indication of temporary NTN feeder-link outage via RAN CONFIGURATION UPDATE (gNBAMF)

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1305  Cat: B (Rel-19)

					Source: Jio Platforms Limited

Abstract:

NTN regenerative payloads see short, predictable feeder-link outages; without a node-level “temporary outage” signal, AMFs may drop UE context and disrupt service. This CR adds optional RAN CONFIGURATION UPDATE (gNB?AMF) IEs so the gNB can signal suspend/

Decision: 		The document was not treated.

R3-255702	(TP to BLCR for TS 38.300) Indicator for NG Suspend/Resume

Type: other		For: Agreement
					38.300 v..
					Source: CMCC

Decision: 		The document was not treated.

R3-255780	UE Retention at SCTP Path Failure

Type: discussion		For: discussion
					Source: Ericsson, Huawei, Thales, Airbus, Jio Platforms, Deutsche Telekom, ESA

Decision: 		The document was not treated.

NG suspend/resume

Nokia: Path failures take time to recover

HUAWEI: Don’t acknowledge the scenario of AMF “not knowing time of Hard FLSO”, also existing UE retention feature can already handle this

[Agreement] Remove text from BL CRs related to suspend/resume

short NG/S1 interruption, causing e.g. packet loss, that may be predictable or unpredictable, e.g. hard FLSO to be further discussed in Q4

R3-255888	Summary of offline discussion for CB: # 26_NTN

Type: discussion		For: Decision
					Source: CATT

Discussion:- Check remaining issues for NR-DC including UE performance granularity

Decision: 		The document was not treated.

## 15	IoT NTN Enhancements WI [IoT_NTN_Ph3-Core]: RP-250472 (target: RAN #109)

### 15.1	General

R3-255069	(BL CR to 36.410) Support of Full eNB as Regenerative payload

Type: CR		For: Agreement
					36.410 v18.0.0	  CR-0031  rev 2 Cat: B (Rel-19)

					Source: Samsung, Ericsson, Sateliot, Thales, ESA, Deutsche Telekom, Jio Platforms Limited, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Huawei

(Replaces R3-253134)

Abstract:

BL CR

Decision: 		The document was endorsed. (*3 CRs are endorsed, to be sent to RAN#109 for approval pending WID revision to include the relevant specifications.)

R3-255070	(BL CR to 36.413) Support of Full eNB as Regenerative Payload

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1963  rev 4 Cat: B (Rel-19)

					Source: Huawei, Ericsson, Thales, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE, Xaomi, CATT, Samsung, Jio Platforms

(Replaces R3-253135)

Abstract:

BL CR

Decision: 		The document was revised to R3-255968.

R3-255968	(BL CR to 36.413) Support of Full eNB as Regenerative Payload

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1963  rev 5 Cat: B (Rel-19)

					Source: Huawei, Ericsson, Thales, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE, Xaomi, CATT, Samsung, Jio Platforms

(Replaces R3-255070)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255071	(BL CR to 36.300) Support of full eNB as regnerative payload and store&forward operation

Type: draftCR		For: Endorsement
					36.300 v18.5.0
					Source: ZTE Corporation, Samsung, CMCC, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Xiaomi, Qualcomm, Jio Platforms

(Replaces R3-253973)

Abstract:

BL CR

Decision: 		The document was revised to R3-255967.

R3-255967	(BL CR to 36.300) Support of full eNB as regnerative payload and store&forward operation

Type: draftCR		For: Endorsement
					36.300 v18.5.0
					Source: ZTE Corporation, Samsung, CMCC, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Xiaomi, Qualcomm, Jio Platforms

(Replaces R3-255071)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255265	Updated Work Plan for IoT NTN

Type: Work Plan		For: Information
					Source: ZTE Corporation, CATT, MediaTek.Inc

Decision: 		The document was noted.

### 15.2	Support Full eNB as Regenerative Payload

R3-255266	Remaing issues for IoT NTN

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255294	(TP to BL CR for TS 36.300) Correction on S1 Removal

Type: other		For: Agreement
					36.300 v..
					Source: Nokia, Nokia Shanghai Bell, Huawei, Ericsson, LG Electronics, Samsung

Decision: 		The document was agreed.

R3-255295	(TP to BL CR for TS 36.413) Support of regenerative payload

Type: other		For: Agreement
					36.413 v..
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Decision: 		The document was not treated.

R3-255311	(TP for TS 36.300) Support for Regenerative Payload for IoT NTN

Type: other		For: Agreement
					36.300 v..
					Source: Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT, China Telecom, CMCC, LG Electronics, Xiaomi

Decision: 		The document was not treated.

R3-255312	Discussion on Support for IoT NTN for Regenerative Payload

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255345	Remaining issue on support of regenerative payload for IoT NTN

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255381	Discussion on remaining issues for IoT NTN

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255443	(TP for TS 36.300) IoT-NTN Switchover (FLSO) & Store-and-Forward Principles

Type: other		For: Agreement
					36.300 v..
					Source: Jio Platforms

Abstract:

Introduce LTE/E-UTRAN anchors for S&F and FLSO with full eNB as regenerative payload; reuse S1/X2 procedures; minimise UE impact.

Decision: 		The document was not treated.

R3-255507	(TP for TS 36.413) S1 Removal completion

Type: other		For: Agreement
					36.413 v..
					Source: Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, Ericsson, Thales, Jio Platforms, CATT, Qualcomm Incorporated, Deutsche Telekom,Samsung

Decision: 		The document was agreed.

R3-255509	(TP to BL CR for TS 36.300) Hard FLSO and MME management

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

Decision: 		The document was revised to R3-255897.

R3-255897	(TP to BL CR for TS 36.300) Hard FLSO and MME management

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

(Replaces R3-255509)

Discussion:

-	Add LG Electronics, ZTE to co-sources

Decision: 		The document was revised to R3-255950.

R3-255950	(TP to BL CR for TS 36.300) Hard FLSO and MME management

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom, LG Electronics Inc., ZTE

(Replaces R3-255897)

Decision: 		The document was agreed.

R3-255512	(TP for TS 36.300) Full eNB as Regenerative Payload - TNL aspetcs

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Jio Platforms

Decision: 		The document was not treated.

R3-255513	(TP for TS 36.300) Discussion on S&F mode transition

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Jio Platforms, Ericsson

Decision: 		The document was not treated.

R3-255747	Response to R3-255513

Type: discussion		For: (not specified)
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, Qualcomm, CATT, China Telecom, CMCC, Samsung

Decision: 		The document was not treated.

R3-255514	(TP for TS 36.300) Provision of S&F Mode Indication of neighbour cell list

Type: other		For: Agreement
					36.300 v..
					Source: Huawei, Ericsson, Jio Platforms

Decision: 		The document was not treated.

R3-255748	Response to R3-255514

Type: response		For: Discussion
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm, CATT, China Telecom, CMCC, Samsung

Decision: 		The document was not treated.

R3-255515	Discussion on Reply LS to SA2 on support of Full eNB as Regenerative Payload

Type: other		For: Agreement
					Source: Huawei, Jio Platforms, Ericsson

Decision: 		The document was not treated.

R3-255516	[DRAFT] Reply LS on Support of Regenerative-based Satellite Access

Type: LS out		For: Approval
					to SA2, cc RAN2, RAN
					Source: Huawei, Jio Platforms, Ericsson

Decision: 		The document was not treated.

R3-255549	Downlink Transmission Suspend/Resume – Stage 2 TP

Type: other		For: Agreement
					36.300 v..
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

Decision: 		The document was revised to R3-255750.

R3-255750	Downlink Transmission Suspend/Resume – Stage 2 TP

Type: other		For: Agreement
					36.300 v..
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

(Replaces R3-255549)

Decision: 		The document was revised to R3-255892.

R3-255892	Removal of Downlink Transmission Suspend/Resume – Stage 2 TP

Type: other		For: Agreement
					36.300 v..
					Source: Ericsson, Thales, Huawei, Jio Platforms, Airbus, ESA, Sateliot, Deutsche Telekom

(Replaces R3-255750)

Decision: 		The document was agreed.

R3-255607	(TP to BLCR for TS 36.300) Left issues in IoT NTN

Type: other		For: Decision
					36.300 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255293	Discussion on the support of S&F

Type: discussion		For: (not specified)
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was withdrawn.

R3-255511	(TP to BL CR for TS 36.300) Removal of the Suspend/Resume part of TS 36.300

Type: other		For: Agreement
					36.300 v..
					Source: Huawei

Decision: 		The document was withdrawn.

R3-255874	SoD for CB: # 27_IoTNTN

Type: discussion		For: Decision
					Source: ZTE Coroporation

Discussion:

- align with NTN

- discuss store&forward and any other open topics

Decision: 		The document was noted.

## 16	Ambient IoT WI [Ambient_IoT_solutions-Core]: RP-250796 (target: RAN #109)

### 16.1	General

R3-255072	(BL CR to 38.300) Introduction of Ambient IoT

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CMCC, Huawei, Nokia

(Replaces R3-253968)

Abstract:

BL CR

Decision: 		The document was revised to R3-255964.

R3-255964	(BL CR to 38.300) Introduction of Ambient IoT

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CMCC, Huawei, Nokia

(Replaces R3-255072)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255073	(BL CR to 38.401) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0469  rev 3 Cat: B (Rel-19)

					Source: Ericsson, Huawei, Nokia

(Replaces R3-253969)

Abstract:

BL CR

Decision: 		The document was revised to R3-255965.

R3-255965	(BL CR to 38.401) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0469  rev 4 Cat: B (Rel-19)

					Source: Ericsson, Huawei, Nokia

(Replaces R3-255073)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255074	(BL CR to 38.410) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0053  rev 5 Cat: B (Rel-19)

					Source: ZTE Corporation, China Telecom, Huawei, Samsung, CMCC, Nokia, Xiaomi

(Replaces R3-253970)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255075	(BL CR to 38.412) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.412 v18.1.0	  CR-0025  rev 3 Cat: B (Rel-19)

					Source: Xiaomi, Huawei, Nokia

(Replaces R3-253971)

Abstract:

BL CR

Decision: 		The document was endorsed. (*3 CRs are endorsed, to be sent to RAN#109 for approval pending WID revision to include the relevant specifications.)

R3-255076	(BL CR to 38.413) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1260  rev 6 Cat: B (Rel-19)

					Source: Huawei, Nokia

(Replaces R3-253972)

Abstract:

BL CR

Decision: 		The document was revised to R3-255966.

R3-255966	(BL CR to 38.413) Introduction of Ambient IoT

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1260  rev 7 Cat: B (Rel-19)

					Source: Huawei, Nokia

(Replaces R3-255076)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

### 16.2	RAN Architecture and Procedures

R3-255015	LS on parallel service request

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504935, to RAN WG3, cc SA WG2
					Source: RAN2(Xiaomi)

Decision: 		The document was noted.

R3-255021	Reply LS on D2R message size for inventory response

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505849, to RAN2, RAN3, cc -
					Source: SA2(Interdigital)

Abstract:

Att: S2-2505848

Decision: 		The document was noted.

R3-255005	Reply LS on AIoT device identifier length

Type: LS in		For: Discussion
					Original outgoing LS: C4-252465, to SA2, cc CT1, RAN2, RAN3, SA3, CT3
					Source: CT4(Huawei)

Abstract:

Att: C4-252464

Decision: 		The document was noted.

R3-255006	Reply LS on paging ID length

Type: LS in		For: Discussion
					Original outgoing LS: C4-252466, to RAN2, cc SA2, SA3, RAN3
					Source: CT4(CICT Mobile)

Abstract:

Att: C4-252464

Decision: 		The document was noted.

R3-255020	Reply LS on paging ID length

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505793, to RAN2, CT4, SA3, cc RAN3
					Source: SA2(CATT)

Abstract:

Att: S2-2505850

Decision: 		The document was noted.

[Session Release and Inventory Complete Indication]

R3-255154	(TP for 38.300) Session Release procedure

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation, China Telecom

Decision: 		The document was noted.

R3-255155	(TP for 38.413) Session Release procedure

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation, China Telecom

Decision: 		The document was noted.

R3-255218	[TP for A-IoT BL CR TS 38.300] AIoT Release procedures for Inventory and Command

Type: other		For: Agreement
					38.300 v..
					Source: Nokia

Decision: 		The document was noted.

R3-255219	[TP for AIoT BL CR TS 38.413] AIoT Release procedures for Inventory and Command

Type: other		For: Agreement
					38.413 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255328	[TPs to TS 38.300, 38.413 BL CRs] A-IoT Session Release and Inventory Complete Indication

Type: other		For: Agreement
					Source: Huawei, China Telecom, China Unicom, Lenovo

Decision: 		The document was noted.

R3-255347	(TP to BL CR for 38.300 and 38.413) Discussion on inventory complete and session release

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-255710	Discussion on Session Release Aspects

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was noted.

FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

[Agreement]

A-IoT Session release should not only rely on the class 2 A-IoT Session Release Request procedure, the Class 1 CN triggered A-IoT Session Release procedure should be performed in all scenarios.

In the case of Inventory with command scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session.

Inventory Complete indication shall be sent via the container within the Inventory Report message, other than a new procedure.

The Reader Report List IE contained in the Inventory Report Transfer IE included in Inventory Report message shall be optional instead of mandatory.

ERICSSON: gNB should not be required to send the Inventory Complete Indication

[Interface Management and Reader Selection]

R3-255313	Interface management procedures and Reader selection

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-255544	[TP for BL CR 38.413]  Reconsideingr WA on A-IoT support indication

Type: other		For: Agreement
					38.413 v..
					Source: Ericsson

Decision: 		The document was noted.

R3-255329	[TPs to TS 38.300 38.413 BL CRs] Leftovers for Interface Management and Reader Selection

Type: other		For: Agreement
					Source: Huawei, China Telecom, China Unicom, Lenovo

Decision: 		The document was noted.

R3-255708	Discussion on RAN Architecture aspects for Ambient IoT

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

[Agreement]

Turn the previous WA: Define the A-IoT Support IE in NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu,..) into agreement

If the “A-IoT support” IE is included in NG Setup Request and is set to “A-IoT only”, the AMF (in case of indirect connectivity) can ignore those IEs that are not applicable to A-IoT (e.g., Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT and NR Uu”, the AMF (in case of indirect connectivity) will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

In NG Setup Response, add clarification in the Semantics description of the not applicable IEs that: “This IE is ignored if the gNB directly connects to the AIOTF”.

Reader Selection:

Upon receiving only the reader list in Inventory Request, the gNB shall use all the readers indicated by the reader list to perform inventory; or

Upon receiving the reader list in Inventory Request, no matter whether AIoT service area is included, the gNB shall take it into account when performing inventory

To be continued...

[IE Encoding, Time Interval, Command Type]

R3-255346	(TP to BL CR for 38.413 and 38.401) Discussion on general issues for AIoT

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was noted.

R3-255327	[TPs to TS 38.300 38.413 BL CRs] Leftovers for Inventory and Command procedures

Type: other		For: Agreement
					Source: Huawei, China Telecom, China Unicom, Lenovo

Decision: 		The document was not treated.

R3-255181	Discussion on A-IoT leftover issues

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255182	(TP to BL CR for TS38.300) Support of A-IoT

Type: other		For: Agreement
					38.300 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255183	(TP to BL CR for TS38.413) on A-IoT leftover issues

Type: other		For: Agreement
					38.413 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255196	Open issues related to Rel 19 AIoT Procedures

Type: discussion		For: Decision
					Source: Tejas Network Limited

Decision: 		The document was not treated.

[Agreement] Confirm that the Estimate of Expected D2R Message Size IE is mandatory provided in INVENTORY REQUEST.

Introduce Command Type (write, read, disable, …) in Command Request Transfer?

[Agreement] Introduce Time Interval as Inventory Assistance Information.

[Miscellaneous]

R3-255152	(TP for 38.300) Leftover issues on AIoT

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation, China Telecom

Decision: 		The document was not treated.

R3-255153	(TP for 38.413) Leftover issues on AIoT

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation, China Telecom

Decision: 		The document was not treated.

R3-255213	Discussion on A-IoT leftover issues

Type: discussion		For: Agreement
					Source: NEC

Decision: 		The document was not treated.

R3-255216	[TP for AIoT BL CR 38.300] Resolution of open points and signalling for AIoT

Type: other		For: Agreement
					38.300 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255217	[TP for AIoT BL CR TS 38.413]  Resolution of open points and signalling for AIoT

Type: other		For: Agreement
					38.413 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255314	Open issues on Inventory and Command procedures

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-255326	[TPs to TS 38.300 38.413 BL CRs] A-IoT Security Aspects

Type: other		For: Agreement
					Source: Huawei, China Telecom, China Unicom

Decision: 		The document was noted.

R3-255330	Discussion on open issues of A-IoT

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255331	(TP to BL CR TS 38.300) Support of A-IoT

Type: other		For: Agreement
					38.300 v..
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255332	(TP to BL CR TS 38.413) Support of A-IoT

Type: other		For: Agreement
					38.413 v..
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255333	(Draft reply LS) LS on parallel service request

Type: LS out		For: Agreement
					to RAN2
					Source: Xiaomi

Decision: 		The document was noted.

R3-255383	Remaining issues for AIoT

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255482	Discussion on remaining open issues in A-IoT

Type: discussion		For: (not specified)
					Source: LG Electronics

Decision: 		The document was not treated.

R3-255496	(TP to TS 38.300 and 38.413) TP for A-IoT support

Type: other		For: Agreement
					38.413 v..
					Source: LG Electronics

Decision: 		The document was not treated.

R3-255541	Open topics for Rel-19 work on A-IoT

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255542	[TP for BL CR 38.300 and 38.413] Correcting references to AS related protocol elements

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255543	[TP for BL CR 38.300 and 38.413] Applicability of A-IoT procedures for multiple devices

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255709	Leftover issues on Inventory and Command Procedure

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-255711	(TP to BLCR for TS 38.300) Introduction of Ambient IoT

Type: other		For: Agreement
					38.300 v..
					Source: CMCC

Discussion:

-	Capture only the change for “A gNB is aware of the mapping relationship between A-IoT Areas and readers by means of OAM configuration.”

Decision: 		The document was revised to R3-255771.

R3-255771	(TP to BLCR for TS 38.300) Introduction of Ambient IoT

Type: other		For: Agreement
					38.300 v..
					Source: CMCC

(Replaces R3-255711)

Decision: 		The document was agreed.

R3-255608	Define AIoT-FunctionIdentifier and AIoT-CorrelationIdentifier IEs

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1303  Cat: C (Rel-19)

					Source: Jio Platforms Limited

Abstract:

The Ambient-IoT work item introduces new NGAP procedures for inventory and command operations between the NG-RAN and the Ambient-IoT Core Network (AIOTF). Stage-2 (TS 38.305/23.502) defines two identifiers used in these procedures:

•	AIoT Function Identif

Decision: 		The document was not treated.

R3-255619	Inventory completion indication (via Inventory Report Transfer)

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1304  Cat: C (Rel-19)

					Source: Jio Platforms Limited

Decision: 		The document was withdrawn.

R3-255444	Inventory completion indication (via Inventory Report Transfer)

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1294  Cat: C (Rel-19)

					Source: Jio Platforms

Decision: 		The document was withdrawn.

R3-255445	Define AIoT-FunctionIdentifier and AIoT-CorrelationIdentifier IEs (encoding closure)

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1295  Cat: C (Rel-19)

					Source: Jio Platforms

Abstract:

Close FFS on identifier encodings for A-IoT messaging so nodes can correlate sessions and route via AMF toward AIOTF per Stage-2. (A-IoT BL TPs reference these IEs repeatedly.)

Decision: 		The document was withdrawn.

R3-255774	Summary of offline discussion on Ambient IoT

Type: discussion		For: Decision
					Source: Huawei(moderator)

Decision: 		The document was noted.

[Agreement]

gNB is aware of direct connectivity (directly connecting with AIOTF) via OAM.

The “receiving both the area and the reader list in Inventory Request” case is allowed.

R3-255792	[TP for AIoT BL CR TS 38.300] AIoT Release procedures for Inventory and Command

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Huawei, China Telecom, ZTE, Xiaomi, CATT, LG Electronics, CMCC

Decision: 		The document was agreed.

R3-255793	(TP to TS 38.413 BL CR) Session Release and Inventory Complete Indication

Type: other		For: Agreement
					38.413 v..
					Source: Xiaomi, Huawei, China Telecom, Nokia, ZTE, CATT, LG Electronics

Decision: 		The document was agreed.

R3-255794	(TP to TS 38.413 BL CR) Interface Management

Type: other		For: Agreement
					38.413 v..
					Source: CATT, Huawei, China Telecom, Nokia, Xiaomi, LG Electronics

Decision: 		The document was agreed.

R3-255795	(TP to TS 38.300 BL CR) Reader Selection

Type: other		For: Agreement
					38.300 v..
					Source: ZTE Corporation, China Telecom, Huawei, Xiaomi, CATT, LG Electronics, CMCC, Samsung

Decision: 		The document was agreed.

R3-255796	(TP to TS 38.300 BL CR) Detailed IE encoding and miscellaneous

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Huawei, China Telecom, Nokia, ZTE, Xiaomi, LG Electronics, CMCC

Decision: 		The document was agreed.

R3-255797	(TP to TS 38.413 BL CR) Detailed IE encoding and miscellaneous

Type: other		For: Agreement
					38.413 v..
					Source: Huawei,China Telecom, ZTE, CATT, LG Electronics

Decision: 		The document was agreed.

R3-255798	(TP to BL CR for 38.401) Miscellaneous

Type: other		For: Agreement
					38.401 v..
					Source: Samsung, Nokia, ZTE, Huawei, CATT, Xiaomi

Decision: 		The document was agreed.

### 16.3	Location Report

## 17	Network ES Enhancements WI [Netw_Energy_NR_enh-Core]: R3-251678 (target: RAN #109)

### 17.1	General

R3-255077	(BL CR to 38.300) Introduction of Network Energy Saving Enhancement

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia

(Replaces R3-253101)

Abstract:

BL CR

Decision: 		The document was revised to R3-255969.

R3-255969	(BL CR to 38.300) Introduction of Network Energy Saving Enhancement

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia

(Replaces R3-255077)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255078	(BL CR to 38.470) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0161  rev 4 Cat: B (Rel-19)

					Source: Samsung, Huawei, NEC, CATT, ZTE, Ericsson

(Replaces R3-253103)

Abstract:

BL CR

Decision: 		The document was revised to R3-255973.

R3-255973	(BL CR to 38.470) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0161  rev 5 Cat: B (Rel-19)

					Source: Samsung, Huawei, NEC, CATT, ZTE, Ericsson

(Replaces R3-255078)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255079	(BL CR to 38.420) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0049  rev 2 Cat: B (Rel-19)

					Source: ZTE Corporation, Samsung, Lenovo, Huawei, Qualcomm, Ericsson, CATT, Nokia, Rakuten, NEC

(Replaces R3-253917)

Abstract:

BL CR

Decision: 		The document was revised to R3-255971.

R3-255971	(BL CR to 38.420) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0049  rev 3 Cat: B (Rel-19)

					Source: ZTE Corporation, Samsung, Lenovo, Huawei, Qualcomm, Ericsson, CATT, Nokia, Rakuten, NEC

(Replaces R3-255079)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255080	Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1436  rev 9 Cat: B (Rel-19)

					Source: Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC, CATT,  China Unicom, Jio Platforms, Rakuten

(Replaces R3-253974)

Abstract:

BL CR

Decision: 		The document was revised to R3-255972.

R3-255972	Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1436  rev 10 Cat: B (Rel-19)

					Source: Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC, CATT,  China Unicom, Jio Platforms, Rakuten

(Replaces R3-255080)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255081	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1531  rev 7 Cat: B (Rel-19)

					Source: Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio Platforms, CATT, Qualcomm

(Replaces R3-253975)

Abstract:

BL CR

Decision: 		The document was revised to R3-255974.

R3-255974	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1531  rev 8 Cat: B (Rel-19)

					Source: Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio Platforms, CATT, Qualcomm

(Replaces R3-255081)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255576	WI Work plan for R19 Network Energy Savings

Type: Work Plan		For: Information
					Source: Ericsson

Decision: 		The document was noted.

R3-255970	(BL CR to 38.401) Support for On-demand SIB1

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0480  Cat: B (Rel-19)

					Source: Qualcomm Incorporated, Huawei, Ericsson, Deutsche Telekom

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

### 17.2	Support on-demand SSB SCell operation

R3-255209	(TP to BLCR TS38.473) Support on-demand SSB

Type: other		For: Agreement
					38.473 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255239	(TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation

Type: other		For: Agreement
					Source: Huawei

(Replaces R3-253258)

Decision: 		The document was not treated.

R3-255247	(TP to BLCRs for TS 38.473) On remaining issues of OD-SSB

Type: other		For: Agreement
					38.473 v..
					Source: CATT

Decision: 		The document was treated.

R3-255305	(TPs to BL CR for 38.470 and 38.473) Support of on-demand SSB Scell operation

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255379	(TP to 38.473) On support on-demand SSB SCell operation

Type: other		For: Decision
					38.473 v..
					Source: China Telecom

Decision: 		The document was not treated.

R3-255435	(TP for BLCR to TS 38.473) On assistance information for OD-SSB over F1

Type: other		For: Agreement
					38.473 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255577	On-demand SSB SCell operation

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

R3-255635	Discussion on on-demand SSB SCell operation in low-carbon green network

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255636	(TP to BLCR for TS 38.470) On-demand SSB SCell operation

Type: other		For: Agreement
					38.470 v..
					Source: Samsung

Decision: 		The document was revised to R3-255826.

R3-255826	(TP to BLCR for TS 38.470) Support of on-demand SIB1

Type: other		For: Agreement
					38.470 v..
					Source: Samsung

(Replaces R3-255636)

Discussion:

-	Add Deutsche Telekom, ZTE, Rakuten, Verizon, Samsung, CATT, Verizon, NEC, Ericsson, Nokia to co-sources

Decision: 		The document was revised to R3-255955.

R3-255955	(TP to BLCR for TS 38.470) Support of on-demand SIB1

Type: other		For: Agreement
					38.470 v..
					Source: Samsung, Huawei, ZTE, Qualcomm, Deutsche Telekom, Rakuten, Verizon, CATT, NEC, Ericsson, Nokia

(Replaces R3-255826)

Decision: 		The document was agreed.

R3-255770	Summary of offline discussions: Rel-19 Network Energy Saving

Type: discussion		For: Decision
					Source: Ericsson

Decision: 		The document was not treated.

Agreements: it is agreed that the below proposals are not needed:

•	Over Xn, exchange OD-SSB state and OD-SSB request

•	The DU should inform the CU when the OD-SSB is being broadcast

•	CU may request DU to send OD-SSB to obtain L3 measurements?

Conclusion: there is no consensus for the below items.

•	Related to if to reuse Rel-18 Cells Allowed … List IE or a create new List for CU to indicate DU a list of cells for OD-SIB1 or do nothing:

[Agreement] New servingCellMO IE for OD-SSB should be introduced to the SCell To Be Setup List IE

HUAWEI, ZTE, Nokia: support the proposal, aligning with RAN2 agreements

Seems agreeable pending further checking of RAN2 agreements

### 17.3	Support on-demand SIB1 for UEs

R3-255210	(TP to BLCR TS38.473) Support on-demand SIB1

Type: other		For: Agreement
					38.473 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255211	TP to BLCR TS38.423 on support on-demand SIB1

Type: other		For: Agreement
					38.423 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255240	(TP to BLCR for TS 38.473, TS 38.423 and TS 38.420) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Type: other		For: Agreement
					Source: Huawei

(Replaces R3-253259)

Decision: 		The document was revised to R3-255808.

R3-255808	(TP to BLCR for TS 38.473) Discussion on on-demand SIB1 for Ues in idle or inactive mode

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

(Replaces R3-255240)

Discussion:

-	Add Rakuten, Verizon, Qualcomm as co-sources

-	8.2.5.2: Change text color from red to black

Decision: 		The document was revised to R3-255952.

R3-255952	(TP to BLCR for TS 38.473) Discussion on on-demand SIB1 for Ues in idle or inactive mode

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Nokia, ZTE, Ericsson, CATT, NEC, Qualcomm, Deutsche Telekom, Rakuten, Verizon Wireless

(Replaces R3-255808)

Decision: 		The document was agreed.

R3-255248	(TP to BLCR for TS 38.300) On remaining issues of OD-SIB1

Type: other		For: Agreement
					38.300 v..
					Source: CATT

Decision: 		The document was revised to R3-255809.

R3-255809	(TP to BLCR for TS 38.300) On remaining issues of OD-SIB1

Type: other		For: Agreement
					38.300 v..
					Source: CATT

(Replaces R3-255248)

Discussion:

-	Change “can inform” to “informs”

-	Add Deutsche Telekom, ZTE, Rakuten, Verizon, Samsung to co-sources

Decision: 		The document was revised to R3-255953.

R3-255953	(TP to BLCR for TS 38.300) for OD-SIB1

Type: other		For: Agreement
					38.300 v..
					Source: CATT, Qualcomm, Huawei, NEC, Ericsson, Deutsche Telekom, ZTE, Rakuten, Verizon, Samsung

(Replaces R3-255809)

Decision: 		The document was agreed.

R3-255255	(TP to BL CR 38.401) Stage-2 procedures for OD SIB1

Type: other		For: Agreement
					38.401 v..
					Source: Qualcomm Inc.

Decision: 		The document was revised to R3-255807.

R3-255807	(TPs to BL CR 38.401) Support for On-demand SIB1

Type: other		For: Agreement
					38.401 v..
					Source: Qualcomm Incorporated, Huawei, Ericsson, Deutsche Telekom

(Replaces R3-255255)

Decision: 		The document was agreed.

R3-255306	(TPs to BL CR for TS38.420, TS38.470 and TS38.473) Support of on-demand  SIB1

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255810.

R3-255810	(TP to BL CR for TS38.420) Support of on-demand  SIB1

Type: other		For: Agreement
					38.420 v..
					Source: ZTE Corporation

(Replaces R3-255306)

Discussion:

-	Add Ericsson, Rakuten, Deutsche Telekom, NEC, Nokia, Verizon, Vodafone to co-sources

Decision: 		The document was revised to R3-255954.

R3-255954	(TP to BL CR for 38.420) Support of on-demand SIB1

Type: other		For: Agreement
					38.420 v..
					Source: ZTE Corporation, Huawei, Samsung, Qualcomm, CATT, Ericsson, Rakuten, Deutsche Telekom, NEC, Nokia, Verizon

(Replaces R3-255810)

Decision: 		The document was agreed.

R3-255436	(TP for BLCR to TS 38.423, TS 38.473) Stage 3 Proposals for On-Demand SIB1

Type: other		For: Agreement
					Source: Nokia

Decision: 		The document was not treated.

R3-255578	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

Decision: 		The document was revised to R3-255806.

R3-255806	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

(Replaces R3-255578)

Discussion:

-	Add Rakuten, Verizon, Qualcomm as co-sources

Decision: 		The document was revised to R3-255951.

R3-255951	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, Huawei, Nokia, NEC, CATT, Qualcomm, ZTE, Rakuten, Verizon

(Replaces R3-255806)

Decision: 		The document was agreed.

R3-255579	F1 impact of On-demand SIB1 for UEs in idle/inactive mode

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

Decision: 		The document was not treated.

R3-255580	Stage 2 Specifications TPs for On-demand SIB1 for UEs in idle/inactive mode

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was merged.

R3-255637	Discussion on on-demand SIB1 in low-carbon green network

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255716	On-Demand SIB1 Availability for Energy Saving in the Handover Scenario

Type: discussion		For: Discussion
					Source: ISSDU, NTU

Abstract:

This document explores the applicability of on-demand SIB1 availability during HO, aiming to reduce energy consumption while maintaining HO performance.

Decision: 		The document was not treated.

R3-255743	Signalling to support OD-SIB1

Type: discussion		For: (not specified)
					Source: Rakuten Mobile, Inc

Decision: 		The document was not treated.

For the class 1 procedures:

[Agreement]

Change the procedure and message name from “UL WUS Configuration Provision” to “OD-SIB1 Configuration Provision”

Change from NR CGI at NG-RAN node1 (Naming FFS) to NES Cell ID

Change from NR CGI at NG-RAN node2 (Naming FFS) to Cell A ID

Please all TP using the name above for the procedure, messages

For the class 2 procedures:

[Agreement] UL WUS CONFIGURATION PROVISION STATUS UPDATE => OD-SIB1 Configuration Provision Status Update

Please all TP using the name above for the procedure, messages

The structure of the class 2 message

[Agreement]

In the class 2 message “OD-SIB1 Configuration Provision Status Update”, it is agreed to include “NES Cell ID” as Mandatory presented.

Cell A ID is optionally presented.

To capture in the class 1 procedure that:

[Agreement] If Cell A ID IE is included in the provision request, then Cell A gNB shall, if support, broadcast the OD-SIB1 configuration in the indicated Cell A.

To capture in the Class 2 procedure text:

[Agreement]

If Cell A ID IE is present, this message indicates that the indicated Cell A has stopped the broadcasting.

If Cell A ID IE is not present, then all the Cell A(s) have stopped the broadcasting for the given NES Cell.

The below proposals have no consensus, and we have agreed not to proceed.

•	the “Update option/code point” in the class 1 message OD-SIB1 Configuration Provision Request.

•	Include multiple OD-SIB1 configurations in a REQUEST message (and partial success in RESPONSE message)

•	NES gNB-CU sends a list of the Cell List OD-SIB1 status to NES gNB-DU over F1.

•	Cell A to NES Cell to have a list of Cell status in the class 2 update over Xn

•	Exchange OD-SIB1 provision capabilities

[Agreement] If Cell A ID is included in the OD-SIB1 Configuration Provision Request message, then it shall be included in the OD-SIB1 Configuration Provision Response message. Capture this in specifications.

### 17.4	Others

R3-255241	(TP to BLCR for TS 38.473) Finalizing common signal adaptation

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

(Replaces R3-253407)

Decision: 		The document was merged.

R3-255249	(TP to BLCR for TS 38.473) Adaptation of paging

Type: other		For: Agreement
					38.473 v..
					Source: CATT, ZTE, China Telecom, Samsung

Decision: 		The document was merged.

R3-255307	(TP to BL CR for 38.470) Support of Paging Adaptation

Type: other		For: Agreement
					38.470 v..
					Source: ZTE Corporation

Decision: 		The document was merged.

R3-255437	Discussion on adaptation of common signals/channels

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-255581	Adaptation of common signal/channel transmissions

Type: discussion		For: (not specified)
					Source: Ericsson

Decision: 		The document was not treated.

Agreements: it is agreed that the below proposals are not needed:

•	For PRACH adaptation: impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

•	For paging adaptation: the DU decides “paging energy saving mode” and informs the CU. gNB-DU informs the gNB-CU about not be transmitted Paging message over the air due to shortage of paging resources.

[Agreement] Introduce new per-cell PEI adaptation indication over F1 and stage 2 description

CATT, ZTE, Nokia: Agree with the proposal, aligns with RAN2 agreement

CB # 28_NetworkES

- Capture above agreements in TPs

- BL CR cleanup and corrections

- 38.401 BL CR

- Conclude on open issues captured above

(Ericsson)

## 18	NR Low Power WUS/WUR WI [NR_LPWUS-Core]: RP-251200 (target: RAN #109)

### 18.1	General

R3-255082	(BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei

(Replaces R3-253105)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255083	(BL CR to 38.470) Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0157  rev 4 Cat: B (Rel-19)

					Source: Nokia, Samsung, ZTE, Ericsson, Huawei, CATT

(Replaces R3-253108)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255084	(BL CR to 38.410) Introduction of LP-WUS

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0056  rev 1 Cat: B (Rel-19)

					Source: CATT, Nokia, Ericsson, Huawei, Vivo, ZTE, Qualcomm, NTT Docomo

(Replaces R3-254001)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255085	(BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1261  rev 4 Cat: B (Rel-19)

					Source: ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT DOCOMO

(Replaces R3-254002)

Abstract:

BL CR

Decision: 		The document was revised to R3-256000.

R3-256000	(BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1261  rev 5 Cat: B (Rel-19)

					Source: ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT DOCOMO

(Replaces R3-255085)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255086	(BL CR to 38.420) Introduction of LP-WUS

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0051  rev 1 Cat: B (Rel-19)

					Source: Qualcomm, ZTE Corporation, Nokia, Ericsson, CATT, Huawei, Samsung

(Replaces R3-254003)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255087	Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1341  rev 9 Cat: B (Rel-19)

					Source: Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT

(Replaces R3-254004)

Abstract:

BL CR

Decision: 		The document was revised to R3-256001.

R3-256001	Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1341  rev 10 Cat: B (Rel-19)

					Source: Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT

(Replaces R3-255087)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255088	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1443  rev 9 Cat: B (Rel-19)

					Source: Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.

(Replaces R3-254005)

Abstract:

BL CR

Decision: 		The document was revised to R3-256002.

R3-256002	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1443  rev 10 Cat: B (Rel-19)

					Source: Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.

(Replaces R3-255088)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

### 18.2	Support LP-WUS Indicating Paging Monitoring

R3-255026	Response to Reply LS on paging capability loss issue

Type: LS in		For: Discussion
					Original outgoing LS: S2-2506087, to RAN2, RAN3, cc TSG RAN, CT1
					Source: SA2(Vodafone)

Decision: 		The document was not treated.

R3-255528	[Draft] Reply LS on paging capability loss issue

Type: LS out		For: Approval
					to SA2, cc TSG RAN, RAN2, CT1
					Source: VODAFONE Group Plc

Decision: 		The document was not treated.

R3-255420	Further discussion on LP-WUS

Type: discussion		For: (not specified)
					Source: NTT DOCOMO INC..

Decision: 		The document was not treated.

R3-255237	(TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was not treated.

R3-255220	[TP for LP-WUS BL CR TS 38.423] Conclusions for LP-WUS open points

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

Decision: 		The document was revised to R3-255877.

R3-255877	[TP for LP-WUS BL CR TS 38.423] Conclusions for LP-WUS open points

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

(Replaces R3-255220)

Discussion:

-	Add Ericsson, CATT, vivo, Vodafone, NTT DOCOMO, Qualcomm as co-sources

Decision: 		The document was revised to R3-255944.

R3-255944	[TP for LP-WUS BL CR TS 38.423] Conclusions for LP-WUS open points

Type: other		For: Agreement
					38.423 v..
					Source: Nokia, Huawei, ZTE Corporation, Ericsson, CATT, Vivo, Vodafone, NTT DOCOMO, Qualcomm

(Replaces R3-255877)

Decision: 		The document was agreed.

R3-255256	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE

Type: discussion		For: Discussion
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R3-255567	Discussion on forwarding of paging capabilities

Type: other		For: Agreement
					Source: Ericsson

Decision: 		The document was not treated.

R3-255527	Missing UE radio paging capabilities

Type: discussion		For: Decision
					Source: VODAFONE Group Plc

Decision: 		The document was not treated.

R3-255230	Remaining issues on LP-WUS

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255250	(TP to BLCR of TS 38.300) On remaining issues of LP-WUS

Type: other		For: Agreement
					38.300 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255706	Discussion on paging capability loss issue

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R3-255652	Further discussion on paging capability loss issue

Type: discussion		For: (not specified)
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255221	[TP for LP-WUS BL CR TS 38.300] Conclusion of Paging Loss

Type: other		For: Agreement
					38.300 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255231	(TP to TS 38.413) on support of LP-WUS

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255879.

R3-255879	(TP to TS 38.413) on support of LP-WUS

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation

(Replaces R3-255231)

Discussion:

-	Add Ericsson, CATT, vivo, Vodafone, NTT DOCOMO as co-sources

Decision: 		The document was revised to R3-255942.

R3-255942	(TP to 38.413) on support of LP-WUS

Type: other		For: Agreement
					38.413 v..
					Source: ZTE Corporation, Huawei, Nokia, Qualcomm, Ericsson, CATT, vivo, Vodafone, NTT DOCOMO

(Replaces R3-255879)

Decision: 		The document was agreed.

R3-255238	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

Decision: 		The document was revised to R3-255878.

R3-255878	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

(Replaces R3-255238)

Discussion:

-	Add CATT, vivo, Vodafone, NTT DOCOMO, Qualcomm as co-sources

Decision: 		The document was revised to R3-255943.

R3-255943	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR

Type: other		For: Agreement
					38.473 v..
					Source: Huawei, Nokia, ZTE Corporation, Ericsson, CATT, Vivo, Vodafone, NTT Docomo, Qualcomm

(Replaces R3-255878)

Decision: 		The document was agreed.

R3-255251	(TP to BLCR for TS 38.413, 38.423, 38.473) On CN Subgroup ID range for LP-WUS

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255529	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1300  Cat: F (Rel-17)

					Source: VODAFONE Group Plc

Decision: 		The document was not treated.

R3-255530	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1301  Cat: A (Rel-18)

					Source: VODAFONE Group Plc

Decision: 		The document was not treated.

R3-255531	Paging capability loss in RAN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1302  Cat: A (Rel-19)

					Source: VODAFONE Group Plc

Decision: 		The document was not treated.

R3-255568	(TP for LP-WUS BL CR for TS38.423) Removal of FFS

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255653	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v16.16.0	  CR-1306  Cat: F (Rel-16)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255654	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v17.12.0	  CR-1307  Cat: A (Rel-17)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255655	Avoiding paging capability loss for CN paging

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1308  Cat: A (Rel-18)

					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255656	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v16.20.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255657	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v17.13.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

R3-255658	Avoiding paging capability loss for CN paging

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Huawei, China Unicom, China Telecom

Decision: 		The document was not treated.

paging capability loss issue:

HUAWEI: whether the NG-RAN node can check with any UE Radio Capability paging feature is missing in the current work.

Nokia: similar views with Huawei. Check with SA2 solution first.

CMCC, ERICSSON: CMCC CR provides one solution.

HUAWEI: CMCC CR only refers to SA2 spec.

whether the NG-RAN node can check with any UE Radio Capability paging feature is missing in the current work.

Discuss VDF optimization solutions.

Draft the LS to SA2 on the above issue.

CATT: For some case, SA2 potential solution cannot solve the issue totally.

ZTE: SA2 potential solution cannot apply for RRC_IDLE mode UE.

Vodafone: cannot agree with ZTE.

[Agreement] The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30.

NTT: RAN3 understand that AMF consider no to use the common codepoint for different PO per LO as defined in 38.213 while assign the CN based subgroup ID.

Further Extended UE Identity Index Value IE is used in case that LP-WUS is used regardless of the use of eDRX/PEI.

Further Extended UE Identity Index Value IE is sent with Extended UE Identity Index Value IE if sending node does not know receiving node’s capability of R19 IEs.

R3-255828	CB:#LPWUS

Type: discussion		For: Decision
					Source: NTT DoCoMo

Decision: 		The document was noted.

[Paging capability loss issue]

[Agreement] The solution for missing UE capability for paging features will be introduced from rel 17 for initial context setup.

The following may be captured in the reply LS sent in the next meeting: RAN3 note that the possibility of paging loss due to paging capability mismatch may still exist in case that paging message is sent to the gNB before paging capability mismatch resolution.

- RAN3 will agree corresponding CRs and reply LS in the next meeting based on the above, considering following aspects:

- Whether the extension of the solution for Path Switch Request procedure and Handover Resource Allocation procedure is needed or not.

- Whether the optimization for the solution proposed by Vodafone in R3-255529/R3-255530/R3-255531 is supported or not.

[Last Used Cell]

[Agreement] The anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

R3-255880	LS on allocation of CN assigned subgroup ID for LP-WUS

Type: LS out		For: Approval
					to SA2, cc RAN1, RAN2
					Source: RAN3(NTT DoCoMo)

Discussion:

-	Replace “CN based” with “CN assigned”

-	Remove “up to implementation,”

-	Replace “according to the TS 38.213, for CN based subgroup ID” with “according to the TS 38.213, for a CN based subgroup ID range”

Decision: 		The document was revised to R3-255941.

R3-255941	LS on allocation of CN assigned subgroup ID for LP-WUS

Type: LS out		For: Approval
					to SA2, cc RAN1, RAN2
					Source: RAN3(NTT DoCoMo)

(Replaces R3-255880)

Decision: 		The document was approved.

## 19	Evolution of Duplex Operation WI [NR_duplex_evo-Core]: RP-251874 (target: RAN #109)

### 19.1	General

R3-255089	(BL CR to 38.300) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Nokia

(Replaces R3-253989)

Abstract:

BL CR

Decision: 		The document was revised to R3-255986.

R3-255986	(BL CR to 38.300) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Ericsson, Nokia

(Replaces R3-255089)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255090	(BL CR to 38.401) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0470  rev 3 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, Samsung, Qualcomm, Huawei, CATT,  Nokia, CMCC, LGE

(Replaces R3-253990)

Abstract:

BL CR

Decision: 		The document was revised to R3-255987.

R3-255987	(BL CR to 38.401) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0470  rev 4 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, Samsung, Qualcomm, Huawei, CATT,  Nokia, CMCC, LGE

(Replaces R3-255090)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255091	(BL CR to 38.420) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0048  rev 3 Cat: B (Rel-19)

					Source: CMCC, Ericsson, Huawei, Samsung, ZTE, Nokia, Jio Platforms

(Replaces R3-253991)

Abstract:

BL CR

Decision: 		The document was revised to R3-255988.

R3-255988	(BL CR to 38.420) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.420 v18.1.0	  CR-0048  rev 4 Cat: B (Rel-19)

					Source: CMCC, Ericsson, Huawei, Samsung, ZTE, Nokia, Jio Platforms

(Replaces R3-255091)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255092	(BL CR to 38.423) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1486  rev 3 Cat: B (Rel-19)

					Source: Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT

(Replaces R3-253992)

Abstract:

BL CR

Decision: 		The document was revised to R3-255989.

R3-255989	(BL CR to 38.423) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1486  rev 4 Cat: B (Rel-19)

					Source: Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT

(Replaces R3-255092)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255093	(BL CR to 38.470) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0163  rev 3 Cat: B (Rel-19)

					Source: CATT, Ericsson, CMCC, Nokia, ZTE, Qualcomm, Samsung, Jio Platforms

(Replaces R3-253993)

Abstract:

BL CR

Decision: 		The document was revised to R3-255990.

R3-255990	(BL CR to 38.470) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0163  rev 4 Cat: B (Rel-19)

					Source: CATT, Ericsson, CMCC, Nokia, ZTE, Qualcomm, Samsung, Jio Platforms

(Replaces R3-255093)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255094	(BL CR to 38.473) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1566  rev 3 Cat: B (Rel-19)

					Source: Samsung, Huawei, Ericsson, ZTE, LG Electronics Inc., Nokia, Qualcomm, Jio Platforms

(Replaces R3-253994)

Abstract:

BL CR

Decision: 		The document was revised to R3-255991.

R3-255991	(BL CR to 38.473) introduction of Evolution of NR duplex operation

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1566  rev 4 Cat: B (Rel-19)

					Source: Samsung, Huawei, Ericsson, ZTE, LG Electronics Inc., Nokia, Qualcomm, Jio Platforms

(Replaces R3-255094)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255684	Updated workplan for Evolution of NR Duplex Operation

Type: Work Plan		For: Information
					Source: Huawei, Sumsang

Decision: 		The document was noted.

### 19.2	Support CLI Handling

R3-255008	Reply LS on simultaneous configuration of SBFD and DC

Type: LS in		For: Discussion
					Original outgoing LS: R1-2504858, to RAN2, cc RAN3, RAN4
					Source: RAN1(Xiaomi)

Decision: 		The document was noted.

R3-255010	LS on TP for TS38.300 on Rel-19 SBFD

Type: LS in		For: Discussion
					Original outgoing LS: R1-2505081, to RAN2, cc RAN3
					Source: RAN1(Huawei)

Abstract:

Att: R1-2505079

Decision: 		The document was noted.

R3-255679	Further discussion on remaining open issues

Type: discussion		For: Discussion
					Source: Huawei, China Telecom, China Unicom

Decision: 		The document was noted.

R3-255681	(TP to BL CR for 38.423) miscellaneous clean-ups for Evolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, China Telecom, China Unicom

Decision: 		The document was revised to R3-255846.

R3-255846	(TP to BL CR for 38.423) miscellaneous clean-ups for Evolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, China Telecom, China Unicom

(Replaces R3-255681)

Discussion:

-	Add ZTE, Nokia, CMCC, CATT co-sources

Decision: 		The document was revised to R3-255921.

R3-255921	(TP to BL CR for TS 38.423) Miscellaneous clean-ups for Evolution of SBFD

Type: other		For: Agreement
					38.423 v..
					Source: Huawei, China Telecom, China Unicom, Samsung, Ericsson, LG Electronics, CMCC, CATT, Nokia, ZTE

(Replaces R3-255846)

Decision: 		The document was agreed.

R3-255267	UE-to-UE CLI mitigation support

Type: other		For: Agreement
					Source: Charter Communications, Inc

Decision: 		The document was noted.

R3-255276	[TP for BL CR TS 38.300] Introduction of SRS resource configurations for Evolution of NR duplex operation Sub-Band Full Duplex

Type: other		For: Agreement
					38.300 v..
					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R3-255277	[TP for BL CR 38.423] Introduction of UE-to-UE CLI mitigation support signaling

Type: other		For: Agreement
					38.423 v..
					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R3-255278	[TP for BL CR 38.473] Introduction of UE-to-UE CLI mitigation support signaling

Type: other		For: Agreement
					38.473 v..
					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R3-255324	Signaling design to enable NR SBFD operation

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

(Replaces R3-253563)

Decision: 		The document was noted.

R3-255439	(TP for BLCRs to TS 38.423, 38.473) Finalizing stage 3 for Sub-band Full Duplex operation

Type: other		For: Agreement
					38.423 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255480	(TP to BLCR to TS38.423 for SBFD) Discussion on SBFD WI completion

Type: other		For: Agreement
					38.423 v..
					Source: Ericsson

Decision: 		The document was noted.

R3-255645	Discussion on Evolution of Duplex Operation

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255646	(TP for 38.423 and 38.473) Support on Evolution of Duplex Operation

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255667	Further discussion on information exchange for CLI handling

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255668	Text proposals to BLCRs of TS38.473 on CLI handling

Type: other		For: Agreement
					38.473 v..
					Source: Samsung

Decision: 		The document was revised to R3-255847.

R3-255847	(TP to BLCRs of TS38.473) on CLI handling

Type: other		For: Agreement
					38.473 v..
					Source: Samsung

(Replaces R3-255668)

Discussion:

-	Add Nokia, CATT as co-sources

Decision: 		The document was revised to R3-255922.

R3-255922	(TP to BL CR for 38.473) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: Agreement
					38.473 v..
					Source: Samsung, Ericsson, LG Electronics, ZTE, Huawei, Nokia, CATT

(Replaces R3-255847)

Decision: 		The document was agreed.

R3-255680	Discussion on the exchange of SRS resource configurations

Type: discussion		For: Discussion
					Source: Huawei, China Telecom, China Unicom

Decision: 		The document was noted.

R3-255682	(TP to BL CR for 38.423) introduction of SRS resource configurations for Evolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: Agreement
					38.423 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255683	(TP to BL CR for 38.473) introduction of  SRS resource configurations forEvolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255691	(TP to BLCR/draft CR for 38.300, 38.401, 38.423 and 38.473) SBFD

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255848.

R3-255848	(TP to TS 38.401 BL CR) Introduction of SBFD

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation

(Replaces R3-255691)

Discussion:

-	Add description of CU-DU interactions for transfer of SRS configuration

-	Add Nokia cosign

Decision: 		The document was revised to R3-255925.

R3-255925	(TP to TS 38.401 BL CR) Introduction of SBFD

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation, Nokia

(Replaces R3-255848)

Discussion:

-	Add Qualcomm, CMCC, Samsung, LG Electronics to co-sources, fix typo in co-source

-	Fix meeting number inheader

Decision: 		The document was revised to R3-255956.

R3-255956	(TP to TS 38.401 BL CR) Introduction of SBFD

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation, Ericsson, China Telecom, Huawei, Nokia, Qualcomm, CMCC, Samsung, LG Electronics

(Replaces R3-255925)

Decision: 		The document was agreed.

R3-255692	Discussion on information exchange for SBFD

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255703	(TPs to BLCR for TS 38.423 and TS 38.473) Discussion on supporting CLI handling in SBFD

Type: other		For: Agreement
					Source: CMCC

Decision: 		The document was not treated.

R3-255438	(TP for BLCRs to TS 38.420, 38.470) Finalizing stage 2 for Sub-band Full Duplex operation

Type: other		For: Agreement
					38.420 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255481	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson

Decision: 		The document was revised to R3-255845.

R3-255845	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson

(Replaces R3-255481)

Discussion:

-	Add Nokia cosign

Decision: 		The document was revised to R3-255920.

R3-255920	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD

Type: other		For: Agreement
					38.300 v..
					Source: Ericsson, Huawei, ZTE, Samsung, Nokia

(Replaces R3-255845)

Decision: 		The document was agreed.

[Agreement]

Remove the Editor’s Note in section 8.4.y CLI Indication procedure about the procedure text, and update the message name in figure to CLI INDICATION.

Agree that the semantics description of the SBFD Frequency Configuration IE added to the Served Cell Information NR IE in the BLCR to TS38.423 reference the SBFD-Subband-Allocation-r19 IE defined in TS38.331.

Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is a relative index of the CSI-RS resources within the set of resources signalled.

UE-to-UE CLI:

gNB2 serving victim UEs will send request to gNB1 serving aggressor UEs requesting a list of SRS-Resource configuration and it is up to gNB2 implementation when to request

[Agreement] gNB1 serving aggressor UEs will provide SRS-Resource configuration information to gNB2 serving victim UEs either based on gNB2 request or without request. It is up to gNB1 implementation which UEs SRS-Resource configuration will be provided to gNB2.

FFS whether to use Served Cell Information NR IE or CLI INDICATION message

RAN3 agrees to introduce option 2 SBFD RACH configuration (which will have separate configuration for legacy ROs and SBFD specific ROs) related IEs as part of Served Cell Information IE and Neighbour NR Cells for SON List IEs in both F1 and Xn signalling messages.

HUAWEI, ERICSSON: Was previously discussed, and thought it was concluded that it is out of scope of Rel-19 WID

ZTE, CATT: Support the proposal.

Nokia: OK to handle the proposal.

RAN3 agree that CU makes the decision of whether to configure SBFD UEs with either L1 or L3 based measurements. If CU decides to configure L1 based Inter-UE CLI measurement configuration for UE, then CU has to request DU to generate SBFD Inter-UE L1 CLI measurement based configuration for UE.

HUAWEI, ZTE: No need to deal with this case, both configurations can be provided to the UE simultaneously.

QUALCOMM: Both cannot be configured simultaneously.

R3-255851	SoD for CB: # 9_DuplexOperation

Type: discussion		For: Decision
					Source: Huawei

Discussion:

- Reflect above agreements in TPs

- Discuss exchange of SBFD RACH configuration and agree (if possible) on solution

- Whether and how to clarify in Stage 2 to avoid simultaneous configuration of L1 and L3 based inter-UE CLI measurements.

- Cleanup BL CRs as needed

Decision: 		The document was noted.

R3-255849	(TP to TS 38.420 BL CR) Introduction of SBFD

Type: other		For: discussion
					38.420 v..
					Source: CMCC

Discussion:

-	Add Nokia to co-sources

Decision: 		The document was revised to R3-255924.

R3-255924	(TP to BL CR for 38.420) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)

Type: other		For: discussion
					38.420 v..
					Source: CMCC, Ericsson, Samsung, ZTE, Huawei, Nokia

(Replaces R3-255849)

Decision: 		The document was agreed.

R3-255850	(TP to TS 38.470 BL CR) Introduction of SBFD

Type: other		For: discussion
					38.470 v..
					Source: CATT

Discussion:

-	Add Nokia to co-sources

Decision: 		The document was revised to R3-255923.

R3-255923	(TP to TS 38.470 BL CR) Introduction of SBFD

Type: other		For: discussion
					38.470 v..
					Source: CATT, Ericsson, ZTE, Huawei, China Telecom, Nokia

(Replaces R3-255850)

Decision: 		The document was agreed.

R3-255854	Summary of Discussion in CB: # 10_ DuplexUEtoUECLI

Type: discussion		For: Decision
					Source: Samsung

Discussion: - Discuss and agree (if possible) on solution for UE-to-UE CLI based on the above framework

Decision: 		The document was noted.

[Agreement]

Add a new indication in CLI INDICATION procedure for the gNB serving victim UEs to request for SRS resource configurations from the receiving gNBs.

Use the following IEs for gNB1 serving aggressor UEs to provide SRS-Resource configuration to gNB2 serving victim UEs:

- XnAP: Served Cell Information NR IE

- F1AP: Served Cell Information IE (DU to CU), Neighbour Cell Information List IE in GNB-CU CONFIGURATION UPDATE message (CU to DU).

R3-255855	(TP to BLCR to TS38.423 for SBFD) exchange of SRS resource configurations

Type: other		For: Agreement
					38.423 v..
					Source: ZTE

Discussion:

-	Add Charter Communications, Samsung, CATT, Nokia, Qualcomm, Verizon to co-sources

Decision: 		The document was revised to R3-255926.

R3-255926	(TP to BLCR 38.423) SBFD on UE to UE CLI

Type: other		For: Agreement
					38.423 v..
					Source: ZTE Corporation, Ericsson, China Telecom, Huawei, Charter Communications, Samsung, CATT, Nokia, Qualcomm, Verizon

(Replaces R3-255855)

Decision: 		The document was agreed.

R3-255856	(TP to BL CR for 38.473) exchange of SRS resource configuartions

Type: other		For: Agreement
					Source: CATT

Discussion:

-	Add Samsung, Ericsson, Nokia, ZTE, Charter Communications, Qualcomm, Verizon to co-sources

Decision: 		The document was revised to R3-255927.

R3-255927	(TP to BL CR for 38.473) Exchange of SRS resource configurations

Type: other		For: Agreement
					38.473 v..
					Source: CATT, China Telecom, Huawei, Samsung, Ericsson, Nokia, ZTE, Charter Communications, Qualcomm, Verizon

(Replaces R3-255856)

Decision: 		The document was agreed.

## 20	AI/ML for Air Interface WI [NR_AIML_air-Core]: RP-251186 (target: RAN #109)

### 20.1	General

R3-255095	(BL CR to 38.305) Introduction of AI/ML air

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT, Jio Platforms

(Replaces R3-253977)

Abstract:

BL CR

Decision: 		The document was revised to R3-255976.

R3-255976	(BL CR to 38.305) Introduction of AI/ML air

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT, Jio Platforms

(Replaces R3-255095)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255096	(BL CR to 38.401) Support of AI/ML assisted positioning

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0477  rev 1 Cat: B (Rel-19)

					Source: ZTE Corporation, Xiaomi, CATT, Ericsson, Huawei, NEC, Nokia, Jio Platforms

(Replaces R3-253978)

Abstract:

BL CR

Decision: 		The document was revised to R3-255977.

R3-255977	(BL CR to 38.401) Support of AI/ML assisted positioning

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0477  rev 2 Cat: B (Rel-19)

					Source: ZTE Corporation, Xiaomi, CATT, Ericsson, Huawei, NEC, Nokia, Jio Platforms

(Replaces R3-255096)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255097	(BL CR to 38.413) Support of Case 3a

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1285  rev 1 Cat: B (Rel-19)

					Source: Huawei, ZTE, Lenovo, Nokia, Ericsson, Jio Platforms

(Replaces R3-253979)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255098	(BL CR to 38.455) Introduction of AI/ML air

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0190  rev 3 Cat: B (Rel-19)

					Source: Ericsson, Nokia

(Replaces R3-253980)

Abstract:

BL CR

Decision: 		The document was revised to R3-255978.

R3-255978	(BL CR to 38.455) Introduction of AI/ML air

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0190  rev 4 Cat: B (Rel-19)

					Source: Ericsson, Nokia

(Replaces R3-255098)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255099	(BL CR to 38.473) Support of Sample-based measurement (case 3b)

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1575  rev 1 Cat: B (Rel-19)

					Source: Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia, Jio Platforms

(Replaces R3-253981)

Abstract:

BL CR

Decision: 		The document was revised to R3-255979.

R3-255979	(BL CR to 38.473) Support of Sample-based measurement (case 3b)

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1575  rev 2 Cat: B (Rel-19)

					Source: Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia, Jio Platforms

(Replaces R3-255099)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255100	(BL CR to 38.300) Stage 2 for case 3a

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, CATT, Xiaomi, Ericsson, CMCC, Huawei, ZTE, Lenovo, Jio Platforms

(Replaces R3-254027)

Abstract:

BL CR (need to update spec ver.)

Decision: 		The document was revised to R3-255975.

R3-255975	(BL CR to 38.300) Stage 2 for case 3a

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, CATT, Xiaomi, Ericsson, CMCC, Huawei, ZTE, Lenovo, Jio Platforms

(Replaces R3-255100)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255569	Work Plan for Rel-19 on AI/ML for PHY (Positioning)

Type: Work Plan		For: Information
					Source: Ericsson (Rapporteur)

Decision: 		The document was noted.

### 20.2	Support Positioning Accuracy Enhancements

R3-255009	LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121

Type: LS in		For: Discussion
					Original outgoing LS: R1-2505073, to RAN WG2, RAN WG3, cc RAN WG4
					Source: RAN1(Ericsson)

Abstract:

Att: R1-2505074

Decision: 		The document was noted.

R3-255016	Logged Data Handling During Handover

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504950, to RAN3, cc SA5
					Source: RAN2(Nokia)

Decision: 		The document was noted.

R3-255023	Reply LS on AI/ML Positioning Case 3a

Type: LS in		For: Discussion
					Original outgoing LS: S2-2505943, to RAN3, cc -
					Source: SA2(Nokia)

Discussion:

Solution 2 is not pursued in Rel-19

Decision: 		The document was noted.

R3-255029	Reply LS on OAM-centric solution for NW-side data collection

Type: LS in		For: Discussion
					Original outgoing LS: S5-252842, to RAN2, cc RAN3
					Source: SA5(Huawei)

Decision: 		The document was noted.

R3-255572	(TP to NRPPa BL CR): addition of UL SRS time domain channel measurement

Type: other		For: Agreement
					Source: Ericsson, Xiaomi, CATT, CEWIT, Nokia, Samsung

Discussion:

HUAWEI: Power Information needs further checking offline

Decision: 		The document was revised to R3-255823.

R3-255823	(TP to NRPPa BL CR): addition of UL SRS time domain channel measurement

Type: other		For: Agreement
					38.455 v..
					Source: Ericsson, Xiaomi, CATT, CEWIT, Nokia, Samsung, Huawei, Qualcomm Inc., ZTE

(Replaces R3-255572)

Decision: 		The document was agreed.

R3-255337	(TP to BL CR TS 38.473) addition of UL SRS time domain channel measurement

Type: other		For: Agreement
					38.473 v..
					Source: Xiaomi, Ericsson, CATT, Nokia, Samsung

Discussion:

F1AP mirror of 5572, should be aligned with NRPPa

Decision: 		The document was revised to R3-255813.

R3-255813	(TP to BL CR TS 38.473) addition of UL SRS time domain channel measurement

Type: other		For: Agreement
					38.473 v..
					Source: Xiaomi, Ericsson, CATT, Nokia, Samsung, ZTE, CEWIT, Huawei, Qualcomm Incorporated, NEC

(Replaces R3-255337)

Decision: 		The document was agreed.

R3-255160	Views on the Open Issues for AI/ML Positioning Accuracy Enhancements

Type: discussion		For: Decision
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

R3-255161	TP to BL CR for TS 38.305

Type: other		For: Agreement
					38.305 v..
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R3-255184	(TP to BL CR for TS38.305) Support of AI Positioning

Type: other		For: Agreement
					38.305 v..
					Source: CATT

Discussion:

-	update taking into account other 38.305 TPs

-	Figure should be updated to reflect up to N number of gNGs

Decision: 		The document was revised to R3-255775.

R3-255775	(TP to BL CR for TS38.305) Support of AI Positioning

Type: other		For: Agreement
					38.305 v..
					Source: CATT

(Replaces R3-255184)

Discussion:

-	Add CEWiT, ZTE, Samsung as co-source

-	Remove the MS Word comment

Decision: 		The document was revised to R3-255934.

R3-255934	(TP to BL CR for TS38.305) Support of AI Positioning

Type: other		For: Agreement
					38.305 v..
					Source: CATT, Ericsson, Huawei, Qualcomm, Nokia, Xiaomi, ZTE, CEWiT, Samsung

(Replaces R3-255775)

Decision: 		The document was agreed.

R3-255185	(TP to BL CR for TS38.455) Support of AI Positioning for Case 3a

Type: other		For: Agreement
					38.455 v..
					Source: CATT

Decision: 		The document was not treated.

R3-255200	[TP for BL CR to TS 38.305 & TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3a

Type: other		For: Agreement
					38.305 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255201	[TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3b

Type: other		For: Agreement
					38.455 v..
					Source: NEC

Decision: 		The document was not treated.

R3-255288	Remaining Issues for AI/ML based positioning case 3a and TP to NRPPa

Type: other		For: Decision
					38.455 v..
					Source: CEWiT, IITM, Tejas Networks

Decision: 		The document was not treated.

R3-255336	(TPs for TS 38.305 and TS 38.455) Support of gNB-side model (case 3a)

Type: other		For: Agreement
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255372	(TPs for AI/ML BLCR to TS 38.300/305/401/455/473) Remaining open issues for AI/ML-based positioning Case 3a and Case 3b

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was revised to R3-255873.

R3-255873	(TP to NRPPa BL CR): Support of Case 3a remaining issue

Type: other		For: Agreement
					38.455 v..
					Source: Huawei, Ericsson, Xiaomi, Nokia, ZTE

(Replaces R3-255372)

Decision: 		The document was agreed.

R3-255377	Discussion on support of AI/ML assisted positioning (Case 3a)

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255378	(TP to TS38.305) On support of AIML assisted positioning

Type: other		For: Agreement
					38.305 v..
					Source: China Telecom

Decision: 		The document was not treated.

R3-255407	(TP to BLCR 38.455) Provision of associated ID in Case 1

Type: other		For: Agreement
					38.455 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255456	(TP to TS 38.300) AI/ML Training in OAM for case 3a

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Ericsson, Xiaomi, Samsung, CATT, ZTE

Decision: 		The document was agreed.

R3-255457	(TP to TS 38.305) Addressing remaining aspects in case 3a and case 3b

Type: other		For: Agreement
					38.305 v..
					Source: Nokia

Decision: 		The document was not treated.

R3-255503	[TP to BLCR to 38.401] Support of AI/ML assisted positioning

Type: other		For: Agreement
					38.401 v..
					Source: ZTE Corporation, Ericsson, Xiaomi, Nokia, Samsung, CATT

Decision: 		The document was agreed.

R3-255504	(TP to BL CR to 38.455) Support of AI/ML assisted Positioning (case 3a)

Type: other		For: Agreement
					38.455 v..
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255570	(TP to NRPPa/TS 38.305 BL CR): Addressing remaining issues of NW-assisted AI/ML positioning with gNB-sided model

Type: other		For: Agreement
					38.305 v..
					Source: Ericsson, Nokia

Decision: 		The document was not treated.

R3-255571	Way forward for Proactive (NGAP-based solution) for AI/ML NG-RAN assisted positioning

Type: discussion		For: Approval
					Source: Ericsson

Decision: 		The document was not treated.

R3-255583	[Draft] Reply LS on AI/ML Positioning Case 3a

Type: LS out		For: Approval
					to SA2
					Source: Nokia

Decision: 		The document was revised to R3-255825.

R3-255825	Reply LS on AI/ML Positioning Case 3a

Type: LS out		For: Approval
					to SA2
					Source: RAN3(Nokia)

(Replaces R3-255583)

Discussion:

-	include description of solution 1

Decision: 		The document was revised to R3-255935.

R3-255935	Reply LS on AI/ML Positioning Case 3a

Type: LS out		For: Approval
					to SA2
					Source: RAN3(Nokia)

(Replaces R3-255825)

Decision: 		The document was approved.

R3-255669	Discussion on AI for positioning case 3a and case 3b

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R3-255758	Summary of AIML for PHY (Positioning) offline discussion before online session

Type: discussion		For: Decision
					Source: Ericsson

Decision: 		The document was not treated.

Inferred measurement results and request

[Agreement] The "LoS/NLoS indicator" can also be a gNB inferred measurement.

Nokia, HUAWEI: Can be inferred but no need for gNB to indicate that it has been inferred

[Agreement]

In a POSITIONING DATA COLLECTION REPORT message, a LMF can provide the LoS/NLoS label information independently of any timing information label. The current semantics description in IE Positioning Data Information (9.2.Z2 [1]) is removed.

It should be possible for an LMF to specifically request for AI/ML inferred measurements from TRPs.

POSITIONING DATA COLLECTION REPORT: Indication of "Positioning Data Unavailable"

[Agreement] Add the following error causes to the Positioning Data Unavailable IE:

- Positioning Data Information Not Supported

- Positioning Data Information Supported But Currently Not Available

CB # 17_AIPHY

- Further check 5572 and 5337

- TPs to reflect the above agreements

- BL CR cleanup and completion

- Reply LS to SA2 based on 5583

(Ericsson)

[Logged data handling during handover]

R3-255338	(Draft LS out) Logged data handling during handover

Type: LS out		For: Approval
					to RAN2
					Source: Xiaomi

Decision: 		The document was not treated.

R3-255373	(Draft Reply LS to RAN2) Discussion on R3-255016 (R2-2504950) LS Logged Data Handling During Handover

Type: LS out		For: Agreement
					to RAN2, cc SA5
					Source: Huawei

Decision: 		The document was not treated.

R3-255406	Discussion on NW side data collection

Type: discussion		For: (not specified)
					Source: Lenovo

Decision: 		The document was not treated.

R3-255458	Logged Data Handling after a Handover Event

Type: discussion		For: (not specified)
					Source: Nokia

Decision: 		The document was not treated.

R3-255459	[Draft] Reply LS on Logged Data Handling During Handover

Type: LS out		For: Approval
					to RAN2, cc SA5
					Source: Nokia

Decision: 		The document was revised to R3-255824.

R3-255824	Reply LS on Logged Data Handling During Handover

Type: LS out		For: Approval
					to RAN2, cc SA5
					Source: RAN3(Nokia)

(Replaces R3-255459)

Decision: 		The document was approved.

R3-255505	[Draft LS] Reply LS on logged Data Handling During Handover

Type: LS out		For: Agreement
					to RAN2, cc SA5
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255670	Discussion on Logged Data Handling During Handover

Type: other		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255551	(TP for TS 38.455) Enhancements for AI/ML-assisted Positioning – Feedback, Model Identification, and Training-Inference Consistency

Type: other		For: Discussion
					38.455 v..
					Source: Jio Platforms

Abstract:

This document proposes enhancements to TS 38.455 to support improved lifecycle management and transparency of AI/ML-assisted positioning. Specifically, the enhancements target:

- Explicit feedback from LMF to gNB regarding inference quality

- Identificati

Decision: 		The document was withdrawn.

## 21	NR XR Enhancements WI [NR_XR_Ph3-Core]: RP-250107 (target: RAN #109)

### 21.1	General

R3-255101	(BL CR to 37.340) Support of XR enhancements

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom, Ofinno

(Replaces R3-254019)

Abstract:

BL CR

Decision: 		The document was revised to R3-256017.

R3-256017	(BL CR to 37.340) Support of XR enhancements

Type: draftCR		For: Endorsement
					37.340 v18.6.0
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom, Ofinno

(Replaces R3-255101)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255102	(BL CR to 37.483) Support of XR enhancements

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0171  rev 3 Cat: B (Rel-19)

					Source: CMCC, Nokia, Nokia Shanghai Bell, Huawei, CATT, Xiaomi, Ofinno, China Telecom, ZTE, Ericsson, Qualcomm, Samsung, NEC

(Replaces R3-254020)

Abstract:

BL CR

Decision: 		The document was revised to R3-256018.

R3-256018	(BL CR to 37.483) Support of XR enhancements

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0171  rev 4 Cat: B (Rel-19)

					Source: CMCC, Nokia, Nokia Shanghai Bell, Huawei, CATT, Xiaomi, Ofinno, China Telecom, ZTE, Ericsson, Qualcomm, Samsung, NEC

(Replaces R3-255102)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255103	(BL CR to 38.300) Support of XR enhancements

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, Nokia Shanghai Bell, Lenovo, Qualcomm, Xiaomi, CATT, Ericsson, CMCC, Samsung, Huawei, China Telecom

(Replaces R3-254021)

Abstract:

BL CR

Decision: 		The document was revised to R3-256019.

R3-256019	(BL CR to 38.300) Support of XR enhancements

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: Nokia, Nokia Shanghai Bell, Lenovo, Qualcomm, Xiaomi, CATT, Ericsson, CMCC, Samsung, Huawei, China Telecom

(Replaces R3-255103)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was endorsed.

R3-255104	(BL CR to 38.413) Support of XR enhancements

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1282  rev 3 Cat: B (Rel-19)

					Source: Lenovo, Nokia, Nokia Shanghai Bell, CMCC, China Telecom, Huawei, ZTE, Ericsson, Samsung, Qualcomm, Ofinno

(Replaces R3-254022)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255105	(BL CR to 38.415) Support of XR enhancements

Type: CR		For: Agreement
					38.415 v18.2.0	  CR-0056  rev 3 Cat: B (Rel-19)

					Source: China Telecom, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CMCC, Ericsson, Samsung, Lenovo

(Replaces R3-254023)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255106	(BL CR to 38.423) Support of XR enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1269  rev 14 Cat: B (Rel-19)

					Source: CATT, Samsung, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, ZTE, Qualcomm, Huawei, CMCC, NEC, Ofinno

(Replaces R3-254024)

Abstract:

BL CR

Decision: 		The document was revised to R3-256020.

R3-256020	(BL CR to 38.423) Support of XR enhancements

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1269  rev 15 Cat: B (Rel-19)

					Source: CATT, Samsung, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, ZTE, Qualcomm, Huawei, CMCC, NEC, Ofinno

(Replaces R3-255106)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255107	(BL CR to 38.425) Support of XR enhancements

Type: CR		For: Agreement
					38.425 v18.1.0	  CR-0156  rev 10 Cat: B (Rel-19)

					Source: Ericsson, AT&T, BT, T-Mobile USA, Charter Communications, Qualcomm, Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC, ZTE, CATT, Ofinno

(Replaces R3-254025)

Abstract:

BL CR

Decision: 		The document was revised to R3-256021.

R3-256021	(BL CR to 38.425) Support of XR enhancements

Type: CR		For: Agreement
					38.425 v18.1.0	  CR-0156  rev 11 Cat: B (Rel-19)

					Source: Ericsson, AT&T, BT, T-Mobile USA, Charter Communications, Qualcomm, Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC, ZTE, CATT, Ofinno

(Replaces R3-255107)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255108	(BL CR to 38.473) Support of XR enhancements

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1485  rev 7 Cat: B (Rel-19)

					Source: Huawei, Ericsson, Lenovo, CMCC, Qualcomm, Xiaomi, Nokia, Nokia Shanghai Bell, China Telecom, Ofinno

(Replaces R3-254026)

Abstract:

BL CR

Decision: 		The document was revised to R3-256022.

R3-256022	(BL CR to 38.473) Support of XR enhancements

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1485  rev 8 Cat: B (Rel-19)

					Source: Huawei, Ericsson, Lenovo, CMCC, Qualcomm, Xiaomi, Nokia, Nokia Shanghai Bell, China Telecom, Ofinno

(Replaces R3-255108)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255296	Rapporteur Inputs on workplan

Type: Work Plan		For: Information
					Source: Nokia, Qualcomm (Rapporteurs)

Decision: 		The document was noted.

### 21.2	Support XR in DC

### 21.3	Others

R3-255013	Reply to LS on Indicating Time to the Next Data Burst (TTNB)

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504812, to SA WG4, cc SA WG2, RAN WG3
					Source: RAN2(Qualcomm)

Decision: 		The document was not treated.

R3-255014	Reply LS to SA2 on XR rate control

Type: LS in		For: Discussion
					Original outgoing LS: R2-2504815, to SA2, cc RAN3
					Source: RAN2(vivo)

Decision: 		The document was not treated.

R3-255297	(TP to BL CR for TS 37.340) Discussion on the remaining issues for Rel-19 XR

Type: other		For: Agreement
					37.340 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-255215	Discussion on RLC enhancement

Type: discussion		For: Decision
					Source: NEC

Decision: 		The document was not treated.

R3-255279	Remaining issues on XR uplink rate control

Type: discussion		For: (not specified)
					Source: Ofinno, LLC

Decision: 		The document was not treated.

R3-255280	(TPs for TS 38.423, TS 37.340) Support of UL rate control

Type: other		For: Agreement
					Source: Ofinno, LLC

Decision: 		The document was revised to R3-255869.

R3-255869	(TP to BL CR for TS 38.423) Support of UL rate control

Type: other		For: Agreement
					38.423 v..
					Source: Ofinno, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Huawei, Samsung, CATT, Qualcomm, Meta

(Replaces R3-255280)

Decision: 		The document was agreed.

R3-255298	(TP for BL CR for TS 38.300) enhancements to Stage-2

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, Ericsson

Decision: 		The document was revised to R3-255864.

R3-255864	(TP for BL CR for TS 38.300) enhancements to Stage-2

Type: other		For: Agreement
					38.300 v..
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, Ericsson, Huawei, CATT, Qualcomm, Meta, Samsung

(Replaces R3-255298)

Decision: 		The document was agreed.

R3-255323	R19 XR Signaling Enhancements

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

(Replaces R3-253564)

Decision: 		The document was not treated.

R3-255389	Discussion on XR rate control

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R3-255408	(TP to XR BLCR for 38.473) On Timely RLC Retransmission

Type: other		For: Agreement
					38.473 v..
					Source: Lenovo

Decision: 		The document was not treated.

R3-255409	(TP to XR BLCR for 38.425) On Unnecessary RLC Retransmission Avoidance

Type: other		For: Agreement
					38.425 v..
					Source: Lenovo

Decision: 		The document was revised to R3-255861.

R3-255861	(TP to XR BL CR for 38.425) On Unnecessary RLC Retransmission Avoidance

Type: other		For: Agreement
					38.425 v..
					Source: Lenovo, Huawei, Nokia, Nokia Shanghai Bell, ZTE, CATT, Qualcomm, Meta, Samsung

(Replaces R3-255409)

Decision: 		The document was agreed.

R3-255416	Discussion on UL rate control

Type: discussion		For: (not specified)
					Source: Huawei

Decision: 		The document was not treated.

R3-255417	(TP for XR BL CRs) Discussion on RLC enhancements for XR

Type: other		For: Agreement
					Source: Huawei

Decision: 		The document was revised to R3-255863.

R3-255863	(TP to BL CR for TS 38.425) enhancements to support timely RLC retransmissions

Type: other		For: Agreement
					38.425 v..
					Source: Huawei, Nokia, Nokia Shanghai Bell, CATT, ZTE, Qualcomm, Meta, Samsung

(Replaces R3-255417)

Decision: 		The document was agreed.

R3-255422	(TP for BL CR for TS 38.473) enhancements to support timely RLC retransmissions

Type: other		For: Agreement
					38.473 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-255573	[TP for BL CR TS 37.483] Support of DL PDU Set Information Marking Support Indication

Type: other		For: Agreement
					37.483 v..
					Source: Ericsson, ZTE Corporation, Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to R3-255865.

R3-255865	(TP to BL CR for TS 37.483) Support of DL PDU Set Information Marking Support Indication

Type: other		For: Agreement
					37.483 v..
					Source: Ericsson, ZTE Corporation, Nokia, Nokia Shanghai Bell

(Replaces R3-255573)

Discussion:

-	Remove “ZTE” at end of co-source list

Decision: 		The document was revised to R3-255945.

R3-255945	(TP to BL CR for TS 37.483) Support of DL PDU Set Information Marking Support Indication

Type: other		For: Agreement
					37.483 v..
					Source: Ericsson, ZTE Corporation, Nokia, Nokia Shanghai Bell, Huawei, CATT, Qualcomm Inc., Meta, Samsung

(Replaces R3-255865)

Decision: 		The document was agreed.

R3-255574	[TP to XR BL CR for 38.413] RAN status indication of Available data rate reporting

Type: other		For: Agreement
					38.413 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255610	(TP for XR BL CR for TS38.300) Clean-up for QNC

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, CMCC, China Telecom

Decision: 		The document was revised to R3-255867.

R3-255867	(TP to BL CR for TS 38.300) Clean-up for QNC

Type: other		For: Agreement
					38.300 v..
					Source: Huawei, CMCC, China Telecom, ZTE, Nokia, Nokia Shanghai Bell, Samsung, Ofinno, CATT, Qualcomm, Meta

(Replaces R3-255610)

Decision: 		The document was agreed.

R3-255611	(TP for XR BL CR for TS37.340) Clean-up for QNC

Type: other		For: Agreement
					37.340 v..
					Source: Huawei, CMCC, China Telecom

Decision: 		The document was revised to R3-255868.

R3-255868	(TP to BL CR for TS 37.340) Clean-up for QNC

Type: other		For: Agreement
					37.340 v..
					Source: Huawei, CMCC, China Telecom, ZTE, Samsung, Ofinno, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, Meta

(Replaces R3-255611)

Decision: 		The document was agreed.

R3-255638	(draft LS to SA2) Discussion on other aspects for NR XR enhancements

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255639	(TP to BLCR for TS 38.473 and TS 38.413) NR XR enhancements

Type: other		For: Agreement
					38.473 v..
					Source: Samsung

Decision: 		The document was not treated.

R3-255647	Disccsion on NR XR Enhancements for others

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255648	(TP for 38.473 and 38.425) NR XR enhancement for others

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255717	Discussion on XR RAN Awareness and UL Rate Control

Type: discussion		For: (not specified)
					Source: Meta

Decision: 		The document was not treated.

R3-255731	[TP to BLCR 38413, 38473, 37483, 38425, 38300] TPs for XR remaining issues on UL Bit Rate Control and Timely RLC retransmission

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255862.

R3-255862	(TP to BL CR for TS 37.483) Enhancement to support timely RLC retransmissions

Type: other		For: Agreement
					37.483 v..
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Huawei, CATT, Qualcomm, Meta, Samsung

(Replaces R3-255731)

Decision: 		The document was agreed.

R3-255732	Discussion on remaing issues in XR with draft reply LS to SA2

Type: discussion		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255733	[TP for BL CR TS 38.473] Support of DL PDU Set Information Marking Indication

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson

Decision: 		The document was revised to R3-255866.

R3-255866	(TP to BL CR for TS 38.473) Support of DL PDU Set Information Marking Support Indication

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, CATT, Qualcomm, Meta, Samsung

(Replaces R3-255733)

Decision: 		The document was agreed.

R3-255759	Summary of offline discussion before online session

Type: discussion		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

For Unnecessary RLC retransmission avoidance, gNB-CU provide signalling to the gNB-DU to enable or disable unnecessary RLC retransmission avoidance via F1AP.

HUAWEI: RAN2 agreement is used by UE for UL transmission, this proposal is used by DU for DL transmission

ZTE: RAN2 agreed RRC parameters makes this proposal unnecessary

Further offline checking

[Agreement]

Add the text in F1-U spec, e.g. gNB-DU performs the required behavior as defined in TS 38.322.

For UL Timely RLC retransmission, no enhancement is needed.

For DL Timely RLC retransmission, adopt Option 1 “CU based solution” that CP inform UP for the thresholds and CU inform DU for retransmission/poll

Indication is needed in F1AP and E1AP for support of DL PDU Set Information Marking without PSQP, RAN3 to develop TPs based on Contribution 5573, 5733 and 5298.

For UL Rate Control / Recommended bit rate,

•	Whether F1AP enhancement is needed.

-	Option 1: gNB-CU provides Recommended UL bit rate info per QoS flow to gNB-DU.

-	Option 2: No additional information

[Agreement] For support of exposure of available bitrate, decision to be made based on the reply LS from SA2.

For QNC: RAN3 discuss whether use general term (e.g. GBR QoS) rather the specific QoS parameter (e.g. GFBR, PDB, etc) to describe the QNC notification.

For MMSID: RAN3 discuss whether using the STRING format defined in CT3 specification also for RAN3 specifications.

For UL Rate Control / Recommended bit rate, whether need coordination between MN and SN.

R3-255860	Summary of offline discussion

Type: discussion		For: Agreement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was noted.

For Unnecessary RLC retransmission avoidance, no consensus on whether needs new F1AP IE for gNB-CU to provide signalling to the gNB-DU to enable or disable unnecessary RLC retransmission avoidance.

[Agreement]

For Clean-up for QNC, agree to use general terminology (e.g. GBR QoS) rather the specific QoS parameter (e.g. GFBR, PDB, etc) to describe the QNC notification, and add the reference to TS 23.501.

For MMSID, agree to not change the MMSID format in RAN3 BL CRs.

For UL Rate Control,

•	agree no coordination between MN and SN.

•	agree no need for gNB to inform SMF for the rate control information

•	agree to develop XnAP TP based on R3-255280, to add the missing behaviour text for Indication of Bitrate Adaptation IE.

•	no consensus on gNB-CU provides Recommended UL bit rate info per QoS flow to gNB-DU

For Support of exposure of available bitrate, agree to wait for reply LS from SA2.

## 22	NR Sidelink Multi-hop Relay WI [NR_SL_relay_multihop-Core]: RP-250188 (target: RAN #109)

### 22.1	General

R3-255109	(BL CR to 38.413) Support of Multi-hop relay

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1262  rev 3 Cat: B (Rel-19)

					Source: Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST

(Replaces R3-253127)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255110	(BL CR to 38.423) Support of Multi-hop relay

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1461  rev 3 Cat: B (Rel-19)

					Source: Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST

(Replaces R3-253128)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255111	(BL CR to 38.401) Support of Multi-hop relay

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0463  rev 5 Cat: B (Rel-19)

					Source: LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei

(Replaces R3-254014)

Abstract:

BL CR

Decision: 		The document was revised to R3-256011.

R3-256011	(BL CR to 38.401) Support of Multi-hop relay

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0463  rev 6 Cat: B (Rel-19)

					Source: LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei

(Replaces R3-255111)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

R3-255112	(BL CR to 38.470) Support of Multi-hop relay

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0165  rev 3 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell

(Replaces R3-254015)

Abstract:

BL CR

Decision: 		The document was endorsed.

R3-255113	(BL CR to 38.473) Support of Multi-hop relay

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1548  rev 5 Cat: B (Rel-19)

					Source: Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet

(Replaces R3-254016)

Abstract:

BL CR

Decision: 		The document was revised to R3-256012.

R3-256012	(BL CR to 38.473) Support of Multi-hop relay

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1548  rev 6 Cat: B (Rel-19)

					Source: Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet

(Replaces R3-255113)

Abstract:

Post Meeting: Merging TPs

Decision: 		The document was agreed.

### 22.2	Support multi-hop Layer-2 UE-to-Network relay

R3-255158	(TP for TS 38.401 and 38.473) Support of Multi-hop relay

Type: other		For: Agreement
					Source: LG Electronics, FirstNet

Decision: 		The document was not treated.

R3-255159	Discussion on remaining issues in service continuity

Type: discussion		For: (not specified)
					Source: LG Electronics, FirstNet

Decision: 		The document was not treated.

R3-255257	(TP to BL CR for TS 38.401) Stage-2 Updates for SL Multi-hop

Type: other		For: Agreement
					38.401 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255493	(TP for 38.401) Multi-hop relay

Type: other		For: Agreement
					38.401 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255494	(TP for 38.473) Multi-hop relay

Type: other		For: Agreement
					38.473 v..
					Source: Huawei

Decision: 		The document was not treated.

R3-255228	Remaining issues on multi-hop relay

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R3-255299	(TP to BL CR for TS 38.473) Discussion on the support of multi-hop Layer-2 UE-to-Network relay

Type: other		For: Agreement
					38.473 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R3-255649	Discussion on Multi-hop U2N relay

Type: discussion		For: Decision
					Source: CATT

Decision: 		The document was not treated.

R3-255348	Remaining issues on multi-hop for SL relay

Type: discussion		For: Agreement
					Source: Samsung

Decision: 		The document was not treated.

R3-255384	Further discussion on multi-hop relay

Type: discussion		For: Decision
					Source: China Telecom

Decision: 		The document was not treated.

R3-255258	(TP to BL CR for TS 38.473) Support of SL Multi-hop in F1AP

Type: other		For: Agreement
					38.473 v..
					Source: Ericsson

Decision: 		The document was not treated.

R3-255300	(TP to BL CR for TS 38.401) update for service continuity

Type: other		For: Agreement
					38.401 v..
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was revised to R3-255763.

R3-255763	(TP to BL CR for TS 38.401) Support of service continuity in Multi-hop U2N relay

Type: other		For: Agreement
					38.401 v..
					Source: Nokia, Nokia Shanghai Bell, ZTE Corporation, LG Electronics, Samsung, Ericsson

(Replaces R3-255300)

Decision: 		The document was agreed.

R3-255349	(TP to BL CR for 38.473) Support of multi-hop relay

Type: other		For: Agreement
					38.473 v..
					Source: Samsung

Decision: 		The document was revised to R3-255762.

R3-255762	(TP to BL CR for 38.473) Support of multi-hop relay

Type: other		For: Agreement
					38.473 v..
					Source: Samsung, Nokia, Nokia Shanghai Bell, LG Electronics, ZTE Corporation, Ericsson

(Replaces R3-255349)

Decision: 		The document was agreed.

R3-255229	(TP to TS 38.401, 38.473) on support of multi-hop relay

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was revised to R3-255764.

R3-255764	(TP to BL CR for TS 38.473) Support of service continuity in Multi-hop U2N relay

Type: other		For: Agreement
					38.473 v..
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, LG Electronics, Samsung, Ericsson

(Replaces R3-255229)

Decision: 		The document was agreed.

R3-255650	(TP for 38.473 and 38.401) Support on Multi-hop relay

Type: other		For: Agreement
					Source: CATT

Decision: 		The document was not treated.

R3-255761	Summary on NR Sidelink Multi-hop Relay

Type: discussion		For: discussion
					Source: LGE

Decision: 		The document was noted.

[Agreement]

RAN3 supports that the Remote UE local ID uniquely identifies a multi-hop Remote UE within the Last Relay UE.

The same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

The Peer UE ID IE is introduced for other PC5 channel related lists in F1AP UE CONTEXT MODIFICATION REQUEST/RESPONSE/REQUIRED/CONFIRM message.

Remove the Peer UE ID IE in the UE CONTEXT SETUP REQUEST message.

For Scenario C, both intra-DU and inter-DU cases are supported by existing procedures.

The existing Path Switch Configuration IE is reused for Scenario B, C and D.

For Scenario D, both intra-DU and inter-DU cases are supported by existing procedures.

From RAN3, this work item is complete.

## 23	LTE-based 5G Broadcast WI [LTE_terr_bcast_Ph2-Core]: R3-250794 (target: RAN #109)

### 23.1	General

### 23.2	Signalling Support

R3-255674	Consideration on LTE-based 5G Broadcast Phase 2

Type: discussion		For: Approval
					Source: Huawei, Qualcomm Incorporated, EBU

Decision: 		The document was noted.

R3-255675	Introduction of LTE-based 5G Broadcast Phase 2

Type: CR		For: Agreement
					36.443 v18.0.0	  CR-0133  Cat: B (Rel-19)

					Source: Huawei, Qualcomm Incorporated, EBU

Discussion:

CB # 18_LTE5Gbroadcast

- add ZTE as cosource

- check alignment with RAN2 CR

(Huawei)

Decision: 		The document was revised to R3-255811.

R3-255811	Introduction of LTE-based 5G Broadcast Phase 2

Type: CR		For: Agreement
					36.443 v18.0.0	  CR-0133  rev 1 Cat: B (Rel-19)

					Source: Huawei, Qualcomm Incorporated, EBU, ZTE Corporation

(Replaces R3-255675)

Decision: 		The document was agreed.

R3-255739	Discussion on LTE-based 5G Broadcast

Type: discussion		For: (not specified)
					Source: ZTE Corporation

Decision: 		The document was noted.

R3-255740	Correction for LTE-based 5G Broadcast

Type: other		For: Agreement
					Source: ZTE Corporation

Decision: 		The document was noted.

## 31	Corrections and Enhancements to Rel-19

### 31.1	Corrections

R3-255289	Correction on the description of UE Context Information - Retrieve UE Context Response

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1504  Cat: F (Rel-19)

					Source: ZTE Corporation

Discussion:

ERICSSON: No chance to confuse the IE, it is included in only one place

Nokia: We should have procedural text for optional IE

Decision: 		The document was noted.

R3-255390	Correction of system information management function

Type: CR		For: Agreement
					38.470 v18.5.0	  CR-0166  Cat: F (Rel-19)

					Source: Huawei

Decision: 		The document was agreed.

R3-255518	PNI NPN with limited CAG validity

Type: discussion		For: Decision
					Source: Nokia

Discussion:

[ERICSSON] Should be discussed in SA2 for end-to-end behaviour

[Huawei] CAG validity should not have RAN3 impact

Decision: 		The document was noted.

R3-255694	Correction on Location Reporting Procedure

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1312  Cat: F (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was noted.

### 31.2	Enhancements

R3-255519	Enhancement of Split Bearers

Type: discussion		For: Decision
					Source: Nokia

Discussion:

[Huawei] Does this create potential security issue?

[ZTE] Can already be achieved by omitting an optional IE

Decision: 		The document was noted.

R3-255539	Partial Data Collection Reporting

Type: discussion		For: Decision
					38.423 v..
					Source: Ericsson, Jio Platforms, BT, LG Electronics

Discussion:

[ZTE] Not applicable to one-time reporting. For periodic, does not seem useful

[Nokia] Does not seem to solve the problem since it doesn’t consider the number of samples

Decision: 		The document was noted.

R3-255575	Support of UE RRC state indication for faster UE context setup

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1585  Cat: B (Rel-19)

					Source: Ericsson

Discussion:

[ZTE] Not convinced about benefit

[Nokia] Stage 2 aspects are unclear

[Huawei] Need to see the full picture, including Stage 2

Decision: 		The document was noted.

### 31.3	Endorsed TEI19 CRs Review

R3-255114	Correction of message level criticality of E1AP procedures

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0144  rev 2 Cat: F (Rel-19)

					Source: Huawei

(Replaces R3-245712)

Abstract:

Technically Endorsed at #125bis meeting

Decision: 		The document was agreed.

R3-255115	Time Reference Distribution Information in S1AP

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1956  rev 2 Cat: F (Rel-19)

					Source: Qualcomm, Vodafone Group Plc, Vodafone Telekomnikasyon A.S.

(Replaces R3-245771)

Abstract:

Technically Endorsed at #125bis meeting

Discussion:

-	Cat.B

-	correct ASN.1

Decision: 		The document was revised to R3-255909.

R3-255909	Time Reference Distribution Information in S1AP

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1956  rev 3 Cat: B (Rel-19)

					Source: Qualcomm, Vodafone Group Plc, Vodafone Telekomnikasyon A.S.

(Replaces R3-255115)

Decision: 		The document was agreed.

R3-255116	Support of Multicast MBS session restoration procedure for N3mb path failure

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1220  rev 4 Cat: B (Rel-19)

					Source: Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated, Nokia

(Replaces R3-247876)

Abstract:

Technically Endorsed at #126 meeting

Discussion:

-	Cat.F

-	8.18.5.2, add “the” before the word “via”

Decision: 		The document was revised to R3-255910.

R3-255910	Support of Multicast MBS session restoration procedure for N3mb path failure

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1220  rev 5 Cat: F (Rel-19)

					Source: Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated, Nokia

(Replaces R3-255116)

Decision: 		The document was agreed.

R3-255117	Recovery of N3mb path failure for unicast transport of multicast session

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0146  rev 3 Cat: F (Rel-19)

					Source: Nokia, Ericsson, Huawei

(Replaces R3-247877)

Abstract:

Technically Endorsed at #126 meeting

Discussion:

-	remove impact statement

Decision: 		The document was revised to R3-255912.

R3-255912	Recovery of N3mb path failure for unicast transport of multicast session

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0146  rev 4 Cat: F (Rel-19)

					Source: Nokia, Ericsson, Huawei

(Replaces R3-255117)

Decision: 		The document was agreed.

R3-255118	Recovery of N3mb path failure for unicast transport of multicast session

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: ZTE Corporation, Nokia, Huawei, CATT

(Replaces R3-247878)

Abstract:

Technically Endorsed at #126 meeting

Discussion:

-	Cat.F

-	Remove impact statement

Decision: 		The document was revised to R3-255911.

R3-255911	Recovery of N3mb path failure for unicast transport of multicast session

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: ZTE Corporation, Nokia, Huawei, CATT

(Replaces R3-255118)

Decision: 		The document was endorsed.

R3-255119	Inactivity Timer for Fixed Wireless Access [Inactivity_Timer_FWA]

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0143  rev 4 Cat: B (Rel-19)

					Source: Nokia, Ericsson, Huawei, ZTE

(Replaces R3-250782)

Abstract:

Technically Endorsed at #127 meeting

Decision: 		The document was agreed.

R3-255120	Inactivity Timer for Fixed Wireless Access [Inactivity_Timer_FWA]

Type: CR		For: Agreement
					38.401 v18.6.0	  CR-0455  rev 2 Cat: B (Rel-19)

					Source: Huawei, Nokia, ZTE, Ericsson

(Replaces R3-250783)

Abstract:

Technically Endorsed at #127 meeting

Decision: 		The document was agreed.

R3-255121	Correction on UE Radio Capability Retrieval

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1264  rev 2 Cat: F (Rel-19)

					Source: ZTE Corporation, CMCC, Nokia, Huawei, Ericsson, CATT

(Replaces R3-252278)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255122	Correction on UE Radio Capability Retrieval

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1964  rev 2 Cat: F (Rel-19)

					Source: Huawei, Ericsson, Nokia, ZTE, CATT, CMCC

(Replaces R3-252279)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255123	E-CID measurement enhancement [ECID_enh1]

Type: CR		For: Agreement
					38.455 v18.6.0	  CR-0186  rev 3 Cat: B (Rel-19)

					Source: Ericsson, NTT Docomo, Nokia, Polaris Wireless, ZTE, CATT, Huawei, Samsung, China Telecom

(Replaces R3-252283)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255124	E-CID measurement enhancement [ECID_enh1]

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1543  rev 3 Cat: B (Rel-19)

					Source: Ericsson, NTT Docomo, Nokia, Polaris Wireless, ZTE, CATT, Huawei, Samsung, China Telecom

(Replaces R3-252284)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255125	Trace Depth for Vendor Specific Trace Record

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1266  rev 2 Cat: B (Rel-19)

					Source: Nokia, Ericsson, Huawei, ZTE Corporation

(Replaces R3-252393)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255126	Trace Depth for Vendor Specific Trace Record

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1485  rev 2 Cat: B (Rel-19)

					Source: Ericsson, Huawei, ZTE Corporation, Nokia

(Replaces R3-252394)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255127	Trace Depth for Vendor Specific Trace Record

Type: CR		For: Agreement
					37.483 v18.5.0	  CR-0169  rev 1 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, Nokia, Huawei

(Replaces R3-252396)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255128	Trace Depth for Vendor Specific Trace Record

Type: CR		For: Agreement
					38.473 v18.6.0	  CR-1564  rev 3 Cat: B (Rel-19)

					Source: Huawei, Ericsson, ZTE Corporation, Nokia

(Replaces R3-252403)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255129	Correction of the interaction with other procedures related to the User Plane Failure

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1270  rev 3 Cat: F (Rel-19)

					Source: Ericsson, Nokia, Huawei

(Replaces R3-252425)

Abstract:

Endorsed at #127bis meeting

Discussion:

-	Correct the meeting # and location

Decision: 		The document was revised to R3-255913.

R3-255913	Correction of the interaction with other procedures related to the User Plane Failure

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1270  rev 4 Cat: F (Rel-19)

					Source: Ericsson, Nokia, Huawei

(Replaces R3-255129)

Decision: 		The document was agreed.

R3-255130	X2 TNL address exchange

Type: CR		For: Agreement
					36.413 v18.3.0	  CR-1962  rev 4 Cat: F (Rel-19)

					Source: Vodafone, Ericsson, Huawei, Nokia, ZTE Corporation

(Replaces R3-252429)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255131	Xn TNL address exchange

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1235  rev 4 Cat: F (Rel-19)

					Source: Vodafone, Ericsson, Huawei, ZTE Corporation, Nokia, Jio Platforms

(Replaces R3-252460)

Abstract:

Endorsed at #127bis meeting

Decision: 		The document was agreed.

R3-255132	E-CID measurement enhancement [ECID_enh1]

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: Ericsson, ZTE

(Replaces R3-253872)

Abstract:

Endorsed at #128 meeting

Discussion:

-	Spec version should be 18.6.0

-	Check Core Network box

Decision: 		The document was revised to R3-255914.

R3-255914	E-CID measurement enhancement [ECID_enh1]

Type: draftCR		For: Endorsement
					38.305 v18.6.0
					Source: Ericsson, ZTE

(Replaces R3-255132)

Decision: 		The document was endorsed.

R3-255133	Support Aerial UE Flight Information Reporting

Type: draftCR		For: Endorsement
					38.300 v18.6.0
					Source: CATT, ZTE, Ericsson, CMCC, Nokia, China Telecom, Huawei

(Replaces R3-253900)

Abstract:

Endorsed at #128 meeting

Decision: 		The document was noted.

R3-255134	Support Aerial UE Flight Information Reporting [UAS_Ph3]

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1460  rev 4 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, CATT, CMCC, Nokia, China Telecom, Huawei

(Replaces R3-253901)

Abstract:

Endorsed at #128 meeting

Discussion:

-	WI code changed to UAS_Ph3, TEI code removed

Decision: 		The document was revised to R3-255786.

R3-255786	Support Aerial UE Flight Information Reporting

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1460  rev 5 Cat: B (Rel-19)

					Source: ZTE Corporation, Ericsson, CATT, CMCC, Nokia, China Telecom, Huawei

(Replaces R3-255134)

Decision: 		The document was agreed.

R3-255135	Support Aerial UE Flight Information Reporting to CN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1259  rev 6 Cat: B (Rel-19)

					Source: Ericsson, CMCC, ZTE, CATT, Nokia, Huawei

(Replaces R3-253902)

Abstract:

Endorsed at #128 meeting

Discussion:

-	Failure related text removed

Decision: 		The document was revised to R3-255757.

R3-255757	Support Aerial UE Flight Information Reporting to CN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1259  rev 7 Cat: B (Rel-19)

					Source: Ericsson, CMCC, ZTE, CATT, Nokia, Huawei

(Replaces R3-255135)

Discussion:

-	Fix RAN3 meeting date and location

Decision: 		The document was revised to R3-255900.

R3-255900	Support Aerial UE Flight Information Reporting to CN

Type: CR		For: Agreement
					38.413 v18.6.0	  CR-1259  rev 8 Cat: B (Rel-19)

					Source: Ericsson, CMCC, ZTE, CATT, Nokia, Huawei

(Replaces R3-255757)

Decision: 		The document was agreed.

R3-255136	Support Aerial UE Flight Information Reporting

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0054  rev 3 Cat: B (Rel-19)

					Source: Huawei, Nokia, ZTE, CATT, Ericsson

(Replaces R3-253903)

Abstract:

Endorsed at #128 meeting

Discussion:

-	tick Core Network box on cover page

Decision: 		The document was revised to R3-255901.

R3-255901	Support Aerial UE Flight Information Reporting

Type: CR		For: Agreement
					38.410 v18.3.0	  CR-0054  rev 4 Cat: B (Rel-19)

					Source: Huawei, Nokia, ZTE, CATT, Ericsson

(Replaces R3-255136)

Decision: 		The document was agreed.

R3-255137	Correction for PDCP SN gap report during UE's handover - Alt 1

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1491  rev 3 Cat: F (Rel-19)

					Source: Huawei, LG Electronics, Qualcomm, China Telecom, Xiaomi, CMCC, Samsung

(Replaces R3-253905)

Abstract:

Endorsed at #128 meeting

Discussion:

-	Replace NR_newRAT-Core with NR_XR_enh-Core

-	Remove “- Alt 1” from tdoc title

Decision: 		The document was revised to R3-255915.

R3-255915	Correction for PDCP SN gap report during UE's handover

Type: CR		For: Agreement
					38.423 v18.6.0	  CR-1491  rev 4 Cat: F (Rel-19)

					Source: Huawei, LG Electronics, Qualcomm, China Telecom, Xiaomi, CMCC, Samsung

(Replaces R3-255137)

Decision: 		The document was agreed.

## 32	Any other business

R3-255517	Draft skeleton of the RAN3 Technical Report (TR) for 6G

Type: discussion		For: Information
					Source: VODAFONE Group Plc

Decision: 		The document was noted.

R3-255872	Early preview of agenda for RAN3#129bis

Type: other		For: discussion
					Source: RAN3 Chair

Decision: 		The document was noted.

Email# 1 Review R19 BL CRs

- BL CR review for all completed R19 work items

- Remove all evidence of “BL CR”, linked specs need only include RAN3 specs

Deadline 2025-09-05 2100 UTC

(RAN3 Vice Chair - Lixiang)

## 33	Closing of the meeting

3GPP TSG RAN WG3 Chair Mr. Sean Kelley (Nokia) closed the f2f meeting RAN WG3 #129 on Friday August 29, 2025 at 16:20 thanking the participants for the contributing to this meeting and the host IF for organizing the meeting.

- The reasons for Tdoc numbers allocated after the meeting are in 'Abstract'

| TDoc | revision of | Title | Source | Contact | Type | TDoc Status | Abstract |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R3-255964 | R3-255072 | Introduction of Ambient IoT | CMCC, Huawei, Nokia, ZTE, China Telecom, CATT, Xiaomi | Miaoqi Zhang | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255965 | R3-255073 | Introduction of Ambient IoT | Ericsson, Huawei, Nokia, ZTE, CATT | Alexander Vesely | CR | agreed | Post Meeting: Merging TPs |
| R3-255966 | R3-255076 | Introduction of Ambient IoT | Huawei, Nokia, ZTE, CATT, CMCC, China Telecom, Xiaomi | Yan Wang | CR | agreed | Post Meeting: Merging TPs |
| R3-255967 | R3-255071 | Support of full eNB as regnerative payload and store&forward operation | ZTE Corporation, Samsung, CMCC, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Xiaomi, Qualcomm, Jio Platforms | Jiren Han | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255968 | R3-255070 | Support of Full eNB as Regenerative Payload | Huawei, Ericsson, Thales, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE, Xaomi, CATT, Samsung, Jio Platforms | Philippe Reininger | CR | agreed | Post Meeting: Merging TPs |
| R3-255969 | R3-255077 | Network support for on-demand SIB1 | CATT, Ericsson, Huawei, Samsung, China Telecom, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia, Qualcomm, NEC, Verizon | Yang Lu | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255970 |  | Support for On-demand SIB1 | Qualcomm Incorporated, Huawei, Ericsson, Deutsche Telekom, ZTE, Nokia, CATT | Georg Hampel | CR | agreed | Post Meeting: Merging TPs |
| R3-255971 | R3-255079 | Introduction of Network Energy Saving Enhancement | ZTE Corporation, Samsung, Lenovo, Huawei, Qualcomm, Ericsson, CATT, Nokia, Rakuten, NEC | Jiren Han | CR | agreed | Post Meeting: Merging TPs |
| R3-255972 | R3-255080 | Introduction of Network Energy Saving Enhancement | Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC, CATT, China Unicom, Jio Platforms, Rakuten | Nianshan Shi | CR | agreed | Post Meeting: Merging TPs |
| R3-255973 | R3-255078 | Introduction of Network Energy Saving Enhancement | Samsung, Huawei, NEC, CATT, ZTE, Ericsson, Nokia | Lixiang Xu | CR | agreed | Post Meeting: Merging TPs |
| R3-255974 | R3-255081 | Introduction of Network Energy Saving Enhancement | Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio Platforms, CATT, Qualcomm | Feng Han | CR | agreed | Post Meeting: Merging TPs |
| R3-255975 | R3-255100 | AI/ML-based positioning with gNB-side model (case 3a) | Nokia, CATT, Xiaomi, Ericsson, CMCC, Huawei, ZTE, Lenovo, Jio Platforms, Samsung | Anna Pantelidou | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255976 | R3-255095 | Introduction of AI/ML for PHY (positioning use cases) | CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT, Jio Platforms, Qualcomm | Jiancheng SUN | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255977 | R3-255096 | Support of AI/ML assisted positioning | ZTE Corporation, Xiaomi, CATT, Ericsson, Huawei, NEC, Nokia, Jio Platforms | Jiajun Chen | CR | agreed | Post Meeting: Merging TPs |
| R3-255978 | R3-255098 | Introduction of AI/ML for PHY positioning functions | Ericsson, Nokia, Xiaomi, ZTE, CATT, Samsung, Jio Platforms | Yazid Lyazidi | CR | agreed | Post Meeting: Merging TPs |
| R3-255979 | R3-255099 | Support of Sample-based measurement (case 3b) | Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia, Jio Platforms | Lisi Li | CR | agreed | Post Meeting: Merging TPs |
| R3-255980 | R3-255042 | Support of enhancements on AI/ML for NG-RAN | Samsung, ZTE Corporation, Nokia, NEC, CATT, Huawei, Ericsson, LG Electronics, InterDigital, Lenovo | Yanru Wang | CR | agreed | Post Meeting: Merging TPs |
| R3-255981 | R3-255048 | Support of enhancements on AI/ML for NG-RAN | NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia, Jio Platforms | Hui Ma | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255982 | R3-255043 | Enhancement of AI/ML for NGRAN split architecture | CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, BT, Ofinno, Qualcomm | SU GU | CR | agreed | Post Meeting: Merging TPs |
| R3-255983 | R3-255044 | Support of enhancements on AI/ML for NG-RAN | ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno | Jiajun Chen | CR | agreed | Post Meeting: Merging TPs |
| R3-255984 | R3-255045 | UE performance enhancement for split architecture | Nokia, Samsung, Ericsson, Huawei, ZTE, Deutsche Telekom, FiberCop, Jio Platforms, CATT, NEC, Lenovo | Anna Pantelidou | CR | agreed | Post Meeting: Merging TPs |
| R3-255985 | R3-255046 | Support of enhancements on AI/ML for NG-RAN | Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno, Samsung | Angelo Centonza | CR | agreed | Post Meeting: Merging TPs |
| R3-255986 | R3-255089 | Evolution of NR duplex operation: Sub-band full duplex (SBFD) | Ericsson, Nokia, China Telecom | Angelo Centonza | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255987 | R3-255090 | Introduction of Evolution of NR duplex operation | ZTE Corporation, Ericsson, Samsung, Qualcomm, Huawei, CATT, Nokia, CMCC, LGE | Yansheng Liu | CR | agreed | Post Meeting: Merging TPs |
| R3-255988 | R3-255091 | Introduction of Evolution of NR duplex operation | CMCC, Ericsson, Huawei, Samsung, ZTE, Nokia, Jio Platforms | Miaoqi Zhang | CR | agreed | Post Meeting: Merging TPs |
| R3-255989 | R3-255092 | Introduction of Evolution of NR duplex operation | Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT | Xudong Yang | CR | agreed | Post Meeting: Merging TPs |
| R3-255990 | R3-255093 | Introduction of Evolution of NR duplex operation | CATT, Ericsson, CMCC, Nokia, ZTE, Qualcomm, Samsung, Jio Platforms | ziqiao liu | CR | agreed | Post Meeting: Merging TPs |
| R3-255991 | R3-255094 | Introduction of Evolution of NR duplex operation | Samsung, Huawei, Ericsson, ZTE, LG Electronics Inc., Nokia, Qualcomm, Jio Platforms, CATT, China Telecom, Charter Communications, Verizon | Man Zhang | CR | agreed | Post Meeting: Merging TPs |
| R3-255992 |  | MDT enhancement for NTN | CMCC, ZTE, Nokia, Ericsson | Miaoqi Zhang | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255993 | R3-255035 | Addition of SON enhancements | CMCC, ZTE, Nokia, Ericsson | Miaoqi Zhang | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255994 | R3-255036 | Addition of SON enhancements | China Unicom, Nokia, ZTE, Ericsson | Xufei Dong | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-255995 | R3-255037 | Addition of SON enhancements | ZTE Corporation, Nokia, Huawei, Lenovo, CATT, Ericsson, Samsung, Jio Platforms | Dapeng Li | CR | agreed | Post Meeting: Merging TPs |
| R3-255996 | R3-255031 | Addition of MDT enhancements | Nokia, ZTE, Ericsson | Krzysztof Kordybach | CR | agreed | Post Meeting: Merging TPs |
| R3-255997 | R3-255032 | Addition of MDT enhancements | CATT, ZTE, Ericsson | ziqiao liu | CR | agreed | Post Meeting: Merging TPs |
| R3-255998 | R3-255039 | Addition of SON enhancements | ZTE Corporation, Samsung, Nokia, Lenovo, Jio, Ericsson | Jiren Han | CR | agreed | Post Meeting: Merging TPs |
| R3-255999 | R3-255040 | Addition of SON enhancements | Huawei, Nokia, Jio, Ericsson | Henrik Olofsson | CR | agreed | Post Meeting: Merging TPs |
| R3-256000 | R3-255085 | Introduction of low-power wake-up signal and receiver for NR | ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT DOCOMO, vivo, Vodafone, China Telecom | Mengzhen Wang | CR | agreed | Post Meeting: Merging TPs |
| R3-256001 | R3-255087 | Introduction of low-power wake-up signal and receiver for NR | Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT, China Telecom | Yazid Lyazidi | CR | agreed | Post Meeting: Merging TPs |
| R3-256002 | R3-255088 | Introduction of low-power wake-up signal and receiver for NR | Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC., China Telecom | Feng Han | CR | agreed | Post Meeting: Merging TPs |
| R3-256003 | R3-255060 | Support inter-CU LTM in NR-DC | CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm | Jiaying Sun | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256004 | R3-255061 | Support for Inter-gNB LTM | Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno, Qualcomm | Andres Arjona | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256005 | R3-255062 | Support for conditional intra-CU LTM and intra-CU LTM | China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm, Jio Platforms | Shengnan Liu | CR | agreed | Post Meeting: Merging TPs |
| R3-256006 | R3-255715 | Support of inter-CU LTM | Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm, China Telecom, Verizon Wireless | Liwei Qiu | CR | agreed | Post Meeting: Merging TPs |
| R3-256007 | R3-255065 | Support for Inter-CU LTM and intra-CU conditional LTM | Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated, Jio Platforms | Hongzhuo Zhang | CR | agreed | Post Meeting: Merging TPs |
| R3-256008 | R3-255067 | Support for Regenerative Payload and MBS broadcast in NR NTN | Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI, Jio Platforms | Gino Masini | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256009 | R3-255066 | Introduce NG Removal procedure | CMCC, Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, Qualcomm, Xiaomi, LG Electronics, China Telecom, Samsung, ZTE, NEC, ETRI | SU GU | CR | endorsed | Post Meeting: Merging TPs |
| R3-256010 | R3-255068 | Support for Regenerative Payload and MBS broadcast in NR NTN | CATT, Thales, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Qualcomm, Samsung, Xiaomi, CMCC, China Telecom, Jio Platforms, LG Electronics, NEC, ETRI, SES, ESA | Jiancheng SUN | CR | agreed | Post Meeting: Merging TPs |
| R3-256011 | R3-255111 | Support of Multi-hop relay | LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei | Seokjung Kim | CR | agreed | Post Meeting: Merging TPs |
| R3-256012 | R3-255113 | Support of Multi-hop relay | Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet | Henrik Olofsson | CR | agreed | Post Meeting: Merging TPs |
| R3-256013 | R3-255055 | Introduction of NR Femto Architecture and Protocol Aspects | Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm | Gino Masini | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256014 | R3-255050 | Support for Wireless Access Backhaul | Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms, Airbus, NEC, China Telecom, NTT DoCoMo | Filip Barac | CR | agreed | Post Meeting: Merging TPs |
| R3-256015 | R3-255056 | Support of NR Femto architecture with NR Femto Gateway | Nokia, Huawei | Philippe Godin | CR | agreed | Post Meeting: Merging TPs |
| R3-256016 | R3-255053 | Support for Wireless Access Backhaul | Nokia, Nokia Shanghai Bell, Ericsson, ZTE, LG Electronics, Qualcomm, Huawei, China Telecom, Samsung, Lenovo | Steven XU | CR | agreed | Post Meeting: Merging TPs |
| R3-256017 | R3-255101 | Support of XR enhancements | ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom, Ofinno | Yansheng Liu | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256018 | R3-255102 | Support of XR enhancements | CMCC, Nokia, Nokia Shanghai Bell, Huawei, CATT, Xiaomi, Ofinno, China Telecom, ZTE, Ericsson, Qualcomm, Samsung, NEC | Miaoqi Zhang | CR | agreed | Post Meeting: Merging TPs |
| R3-256019 | R3-255103 | Support of XR enhancements | Nokia, Nokia Shanghai Bell, Lenovo, Qualcomm, Xiaomi, CATT, Ericsson, CMCC, Samsung, Huawei, China Telecom | Steven XU | draftCR | endorsed | Post Meeting: Merging TPs |
| R3-256020 | R3-255106 | Support of XR enhancements | CATT, Samsung, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, ZTE, Qualcomm, Huawei, CMCC, NEC, Ofinno, China Telecom | ziqiao liu | CR | agreed | Post Meeting: Merging TPs |
| R3-256021 | R3-255107 | Support of XR enhancements | Ericsson, AT&T, BT, T-Mobile USA, Charter Communications, Qualcomm, Huawei, Nokia, Nokia Shanghai Bell, China Telecom, CMCC, ZTE, CATT, Ofinno | Yazid Lyazidi | CR | agreed | Post Meeting: Merging TPs |
| R3-256022 | R3-255108 | Support of XR enhancements | Huawei, Ericsson, Lenovo, CMCC, Qualcomm, Xiaomi, Nokia, Nokia Shanghai Bell, China Telecom, Ofinno, Samsung | Yuanping Zhu | CR | agreed | Post Meeting: Merging TPs |
| R3-256023 | R3-255064 | Xn support for inter-CU LTM in DC | Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LG Electronics, China Telecom, Google, ZTE, Samsung | Mingzeng Dai | CR | agreed | Post Meeting: Merging TPs |
| R3-256024 | R3-255074 | Introduction of Ambient IoT | ZTE Corporation, China Telecom, Huawei, Samsung, CMCC, Nokia, Xiaomi | ZIJIANG MA | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256025 | R3-255075 | Introduction of Ambient IoT | Xiaomi, Huawei, Nokia | Lisi Li | CR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256026 | R3-255069 | Support of Full eNB as Regenerative payload | Samsung, Ericsson, Sateliot, Thales, ESA, Deutsche Telekom, Jio Platforms Limited, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Huawei, ZTE | Xingyu HAN | CR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256027 | R3-255097 | Support of Case 3a | Huawei, ZTE, Lenovo, Nokia, Ericsson, Jio Platforms | Damiano Rapone | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256028 | R3-255047 | AIML for NR-DC | Huawei, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Huawei, Ofinno, FiberCop, LG Electronics, Jio Platforms | Damiano Rapone | draftCR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256029 | R3-255041 | Support of enhancements on AI/ML for NG-RAN | CATT, ZTE Corporation, Nokia | Aijuan Liu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256030 | R3-255030 | Addition of SON enhancements | Lenovo, Ericsson | Le Yan | draftCR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256031 | R3-255034 | Addition of SON enhancements | CATT, Samsung, Lenovo, Huawei, Ericsson, ZTE, CMCC, Nokia, Jio Platforms | ziqiao liu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256032 | R3-255038 | Addition of SON enhancements | Ericsson, Jio Platforms, ZTE | Julien Muller | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256033 | R3-255033 | Addition of SON enhancements | Samsung, ZTE, CATT, Lenovo, Cybecore, Ericsson | Xingyu HAN | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256034 | R3-255082 | Introduction of low-power wake-up signal and receiver for NR | vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei | LI CHEN | draftCR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256035 | R3-255084 | Introduction of LP-WUS | CATT, Nokia, Ericsson, Huawei, Vivo, ZTE, Qualcomm, NTT Docomo | Yang Lu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256036 | R3-255086 | Introduction of LP-WUS | Qualcomm, ZTE Corporation, Nokia, Ericsson, CATT, Huawei, Samsung, vivo | Georg Hampel | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256037 | R3-255083 | Introduction of low-power wake-up signal and receiver for NR | Nokia, Samsung, ZTE, Ericsson, Huawei, CATT, Vivo, China Telecom | Philippe Godin | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256038 | R3-255057 | Introducing Rel-19 Mobility enhancement | LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC, Ofinno, Jio Platforms (JPL) | Jaemin Han | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256039 | R3-255058 | Support for Inter-CU LTM | ZTE Corporation, China Telecom, Samsung, Nokia, CATT, NEC, LG Electronics, Ericsson, Huawei, Lenovo | ZIJIANG MA | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256040 | R3-255059 | Support for Inter-CU LTM procedure | Samsung, Huawei, LG Electronics, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT | Jinwoo Ock | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256041 | R3-255109 | Support of Multi-hop relay | Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST | Steven XU | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256042 | R3-255110 | Support of Multi-hop relay | Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST | Liwei Qiu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256043 | R3-255112 | Support of Multi-hop relay | ZTE Corporation, Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell | Mengzhen Wang | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256044 | R3-255049 | Support of Location Service Involving WAB-Nodes | ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, Lenovo, CATT, Samsung, Huawei, China Telecom | Mengzhen Wang | draftCR | endorsed | Post Meeting: allocated for finalize BL CRs |
| R3-256045 | R3-255051 | Introduction of NR Femto in NGAP list of functions | ZTE Corporation, Nokia, Baicells | Mengzhen Wang | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256046 | R3-255052 | Support for Wireless Access Backhaul | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo | Yuanping Zhu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256047 | R3-255054 | Support of Location Services Involving WAB-Nodes | Lenovo, ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, CATT, Samsung, Huawei, China Telecom, Jio Platforms (JPL) | Yibin Zhuo | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256048 | R3-255104 | Support of XR enhancements | Lenovo, Nokia, Nokia Shanghai Bell, CMCC, China Telecom, Huawei, ZTE, Ericsson, Samsung, Qualcomm, Ofinno | Mingzeng Dai | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256049 | R3-255105 | Support of XR enhancements | China Telecom, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CMCC, Ericsson, Samsung, Lenovo | sen xu | CR | agreed | Post Meeting: allocated for finalize BL CRs |
| R3-256050 | R3-255937 | Support for Wireless Access Backhaul | Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell, NTTDOCOMO, Samsung, LG Electronics, Huawei, China Telecom, NTT DOCOMO, Airbus, NEC | Georg Hampel | draftCR | endorsed | Post Meeting: Finalize CR |
| R3-256051 | R3-255811 | Introduction of LTE-based 5G Broadcast Phase 2 | Huawei, Qualcomm Incorporated, EBU, ZTE | Yan Wang | CR | agreed | Post Meeting: For fixing 9.4.7 to 9.3.7(M2AP) |