3GPP TSG-RAN3 Meeting #130																R3-258109

Dallas, USA, 17-21 November, 2025

Agenda Item:	12.2.2

Source:	ZTE Corporation

Title:	(TP to TR 38.745) Consideration on AI/ML assisted intra-CU LTM

Document for:	pCR

- Introduction

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

The Rel-20 study will focus on evaluating AI/ML-based mobility use cases, including multi-hop UE trajectory across gNBs, lower-layer triggered mobility (LTM), and other handover enhancements. The objective is to improve handover reliability, reduce latency, and optimize mobility-related resource allocation through predictive and data-driven approaches.

Following agreement is achieved during the last meeting:

Prioritize normal intra-CU LTM scenarios.

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

In this contribution, we focus on the topic of AI/ML-assisted intra-CU LTM and futher provide our proposed solutions for enabling AI/ML-assisted intra-CU LTM.

- Discussion

LTM is a procedure in which a gNB receives L1 or L3 measurement reports from a UE, and based on these reports the gNB may change the UE’s serving cell by sending a cell switch command via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB has previously prepared and provided to the UE through RRC signalling. The UE then switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce mobility latency.

For intra-CU LTM, AI/ML can be applied to assist in optimizing both intra-gNB-DU LTM and inter-gNB-DU LTM, enabling more accurate candidate selection, improved prediction of UE mobility patterns, optimized UL synchronization and better balancing between latency reduction and service continuity.

- For intra-CU LTM, AI/ML can be applied to assist in optimizing both intra-gNB-DU LTM and inter-gNB-DU LTM, enabling more accurate candidate selection, improved prediction of UE mobility patterns, optimized UL synchronization.

2.1 Finer granularity of UE trajectory

In the conventional intra-CU LTM, the gNB-CU determines whether to trigger an LTM configuration based on the L3 measurement results and selects candidate cells for LTM. The gNB-CU then sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell. This message contains the candidate cell ID and the CSI resource configuration for the subsequent LTM. If the gNB-DU accepts the LTM configuration request, it responds with a UE CONTEXT MODIFICATION RESPONSE message, which includes the generated lower-layer RRC configurations and the TCI state configuration for the accepted candidate cell.

With AI/ML enhancement, the candidate cell selection can be significantly improved. Instead of relying solely on L3 measurement thresholds, the gNB-CU can leverage AI/ML predictions to pre-select an optimized candidate cell set. The candidate set can be dynamically adapted according to the predicted UE trajectory. To further improve the accuracy of candidate cell set selection, predicted L3 measurement results and finer-granularity predicted UE trajectory information can be used as a kind of information.

- Finer granularity UE trajectory prediction and predicted L3 measurement results can be used as a kind of assistance information for candidate cell selection.

Candidate cell selection is primarily performed at the gNB-CU. In this case, the CU can rely on its internal AI/ML models to predict the UE trajectory or generate predicted L3 measurements, and use this information internally to determine the candidate cell set.

- The gNB-CU can internally use the predicted information (e.g., cell/beam-level UE trajectory prediction or predicted L3 measurements) to assist with candidate cell selection.

However, since the gNB-DU may also play a role in deciding whether to accept or reject a candidate cell, it is beneficial for the CU to forward these assistance information (e.g., UE trajectory prediction or predicted L3 measurements) to the DU. By providing such information, the DU can use it as auxiliary input to assist its own decision-making on whether to accept the candidate cell selection. This enables a more collaborative CU–DU process for AI/ML-assisted LTM, potentially improving the accuracy of candidate cell selection.

- The assistance information (e.g., cell/beam-level UE trajectory prediction) can be provided by the gNB-CU to the gNB-DU, as it is helpful for the DU to support its own decision-making on whether to accept the candidate cell selection.

  - Optimized UL synchronization

2.2.1 TA Value prediction

In the legacy LTM, when configured by the network, it is possible to initiate an early TA procedure for one or multiple cells that are different from the current serving cells. As shown in Figure below, the early TA acquisition procedure is triggered by a PDCCH order sent by the source DU. The gNB-DU to which the candidate cell belongs calculates the TA value and sends it to the gNB-DU of the serving cell via the gNB-CU.

Figure 1. Early TA procedure

With the AI/ML enhancement, the TA calculation can be made more accurate. In addition to the legacy approach where the candidate DU performs the TA calculation, AI/ML can enable alternative options. For example, the source DU itself may generate the TA value based on predictive AI/ML models trained on historical mobility and timing data. Alternatively, the candidate DU may compute the TA value and forward it to the source DU, where AI/ML can refine or validate the result before it is applied.

- TA value calculation can be performed more accurate with AI/ML techniques.

Following is the option for predicted TA value can be further discussed in RAN3:

- Option 1: Candidate DU performs TA value prediction

This follows the legacy approach, where the candidate DU calculates the TA value, but enhanced with AI/ML prediction models.

- Option 2: Source DU performs TA value prediction

The source DU applies AI/ML models trained on historical timing advance and mobility data to predict the TA value directly. It can eliminate reliance on the candidate DU, enabling earlier availability of TA information.

- Option 3: Source CU performs TA value prediction

The CU aggregates broader measurement and mobility information (e.g., cell/beam-level UE trajectory, predicted L3 measurement) and uses AI/ML models to predict TA values.

- RAN3 is kindly to discuss the solutions above implement TA value prediction.

Option 1& Option 2 require that DU must perform AI/ML inference locally. Option 1 (candidate DU performs TA prediction) and Option 2 (source DU performs TA prediction) both require that the DU must perform AI/ML inference locally. However, during Rel-18 and Rel-19, all AI/ML-assisted use cases only assumed that the CU has the capability to host and execute model inference. DU-side inference capability was not considered in those phases.

- RAN3 is kindly to discuss whether DU can deploy model inference to support TA value prediction.

2.2.2 Validity time of measured TA Value

In addition to predicted TA value using AI/ML function, the validity time of measured TA value can also be predicted by AI/ML techniques. The concept of predicted validity time for the measured TA value addresses this challenge by estimating the duration during which the TA remains sufficiently precise for practical use in LTM execution. This prediction incorporates AI/ML models based on UE velocity, historical TA trends, cell geometry, and radio quality metrics, allowing the network to proactively configure validity timers or trigger re-measurements. By optimizing this validity time, intra-CU LTM can achieve lower interruption times while maintaining robust link quality and reducing signalling overhead.

- Predicted validity time of measured TA value (the duration during which the TA remains sufficiently precise) is beneficial of intra-CU LTM execution.

Above all, following is the potential impacts to specification identified:

- Predicted cell-level UE trajectory, or Predicted beam-level UE trajectory from CU to DU

- Predicted L3 measurement report from CU to DU

- Coordination between CU and DU for future TA value for each candidate cell

- Coordination between CU and DU for predicted validity time of measured TA value.

- Agree the TP below to TR38.745, including the use case description, solutions and standard impacts.

- Conclusion

Following is the observations and proposals:

- For intra-CU LTM, AI/ML can be applied to assist in optimizing both intra-gNB-DU LTM and inter-gNB-DU LTM, enabling more accurate candidate selection, improved prediction of UE mobility patterns, optimized UL synchronization.

Finer granularity of UE trajectory

- Finer granularity UE trajectory prediction and predicted L3 measurement results can be used as a kind of assistance information for candidate cell selection.

- The gNB-CU can internally use the predicted information (e.g., cell/beam-level UE trajectory prediction or predicted L3 measurements) to assist with candidate cell selection.

- The assistance information (e.g., cell/beam-level UE trajectory prediction) can be provided by the gNB-CU to the gNB-DU, as it is helpful for the DU to support its own decision-making on whether to accept the candidate cell selection.

Optimized UL synchronization

- TA value calculation can be performed more accurate with AI/ML techniques.

- RAN3 is kindly to discuss the solutions above implement TA value prediction.

- RAN3 is kindly to discuss whether DU can deploy model inference to support TA value prediction.

- Predicted validity time of measured TA value (the duration during which the TA remains sufficiently precise) is beneficial of intra-CU LTM execution.

- Agree the TP below to TR38.745, including the use case description, solutions and standard impacts.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

4.2	AI/ML assisted Intra-CU LTM

4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-assisted intra-CU LTM:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2		Input data of AI/ML-assisted intra-CU LTM

For optimized LTM operation, the following information can be considered as input data for AI/ML-assisted intra-CU LTM:

- Measured/Predicted Radio Resource Status

- Measured/Predicted cell-level UE trajectory

- Measured/Predicted beam-level UE trajectory

- Measured/Predicted L3 measurement report

#### 4.2.2.3		Input data of AI/ML-assisted intra-CU LTM

For optimized LTM operation, the following information can be considered as output for AI/ML-assisted intra-CU LTM:

- Predicted Radio Resource Status

- Predicted cell-level UE trajectory

- Predicted beam-level UE trajectory

- Predicted L3 measurement report

- Future TA value for each candidate cell

- Candidate cell set for LTM  (used by gNB-CU internally)

#### 4.2.2.4	Feedback of AI/ML-assisted intra-CU LTM

For optimized LTM operation, the following information can be considered as feedback for AI/ML-assisted intra-CU LTM:

- Measured beam-level UE trajectory after LTM execution

- Measured L3 measurement report after LTM execution

- Actual TA value for the target cell.

- Legacy UE performance feedback for the UE handed over from the source cell.

#### 4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work:

- F1 interface:

  - Predicted cell-level UE trajectory, or Predicted beam-level UE trajectory from CU to DU

  - Predicted L3 measurement report from CU to DU

  - Coordination between CU and DU for future TA value for each candidate cell

  - Coordination between CU and DU for predicted validity time of measured TA value.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>