3GPP TSG-RAN WG3 Meeting #130	R3-258120

Dallas, US, 17th ~21th November 2025

Title:	Consideration on RAN-CN interface options

Agenda Item:	10.3.2

Source:	Huawei

Document for:	Agreement

# Introduction

In the 6G SID [1], the following bullet can be found:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

And it is required for RAN3 to provide interim study results to allow TSGs to make a decision on the RAN-CN interface: P2P vs SBI by June 2026.

In this contribution, we discuss the RAN CN interface options.

# Discussion

## RAN-CN Control Plane Interface

In 4G and 5G, both the S1 interface and NG interface are designed as point-to-point interface, and the point-to-point logical interface is feasible even in the absence of a physical direct connection.

The 4G/5G RAN-CN control plane interface seperates the Radio Network Layer and the Transport Network Layer. S1AP and NGAP were introduce as the Radio Network Layer for the respective generation. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP.

Figure 1: Control plane of NG interface

On the other hand, the service-based Control Plane interfaces are used between most of NFs within the 5G Core Network, the service-based interface represents how a set of services is provided/exposed by a given NF, and the protocol stack for the service-based interfaces is shown as below. HTTP/2 protocol with JSON is used as the application layer serialization protocol, TCP is used as the transport protocol. OpenAPI Specification is used as Interface Definition Language (IDL) of SBI.

Figure 2: SBI Protocol Stack

For 5GC Control Plane network functions, the interactions between the them are also described by point-to-point reference points as follows:

| Non-Roaming 5G System Architecture (from TS 23.501) |  |
| --- | --- |
| Service-based interfaces are used within the Control Plane of 5GC | how various network functions interact with each other using the reference point representation. |

There are two options (P2P vs SBI) for 6G RAN-CN control plane interface:

Option 1: RAN-CN interface: P2P

In this option, from a logical standpoint, the 6G RAN-CN interface is a point-to-point interface between an RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the RAN node and CN node. In the transport, IP layer point-to-point transmission is used to deliver the signalling PDUs. The application layer signalling protocol is referred to as NGAP*.

Option 2: RAN-CN interface: SBI

In this option, the 6G RAN-CN interface is a service-based interface which represents how a service is provided/exposed.

NG interface is a P2P interface between the gNB and AMF, and NG flex is supported enabling a gNB to connect with multiple AMFs within the same region. With P2P interface design, a clear RAN CN function split is achieved, for example, for the signalling processing of a UE, the gNB only needs to route the relevant signalling to the UE’s serving AMF, which, when necessary, communicates with the SMF/LMF and other CN functions. The gNB does not need to understand the processing sequence and logic among CN functions.

Observation 1: P2P-based interface, ensures clear RAN CN function-split, with the clear boundary the RAN node does not need to understand the processing sequence and logic among CN functions.

In the existing design, the RAN is always connected to AMF as the anchor for core network functions. For some features, there is a potential need to allow RAN to direct connect to other core network functions. For instance, lightweighted deployment is beneficial for some features, e.g., the Ambient IoT of Rel-19 supports using the NG interface between gNB and AIOTF. Therefore, if necessary, a direct connection between the 6G RAN and new core network functions can be achieved through the P2P RAN-CN interface. Some companies proposed using service-based interface for the same purpose, i.e., to allow RAN to directly connect to core network functions. However, compared with the P2P interface between RAN and core NF, no clear gains are seen (as illustrated below).

Observation 2: P2P-based interface can support flexible and lightweighted deployments of core network by allowing direct connectivity between the 6G RAN and 6G new CN function, if needed.

It was agreed that “the 6G RAN-CN interface is designed with a clear functional split between RAN and CN”. The service-based interface solution will increase the RAN-CN interactions, communicating with various kinds of CN functions will lead to unclear boundaries between RAN and CN, the RAN node may need to determine when to perform parallel operations and when to perform sequential operations with those CN functions, these will significantly increase the RAN processing complexity. The complexity and tightly coupled functionality of RAN will pose challenges to registration, discovery, and API definition, especially when taking the large scale of the RAN nodes in to account.

Take mobility as an example, in Xn based handover, once the UE accesses to the target gNB, the target gNB only needs to send a Path Switch Request to the UE’s serving AMF, then the AMF will contact with the SMF(s), the SMF(s) will further contact with UPF(s), but in service-based interface solution, the gNB may need to understand which CN functions need to contact to and in which sequence, adjust the logic when some of the peer CN functions fail the request while some others not, etc.

Observation 3: If service-based interface is used, due to the large scale of RAN nodes, the RAN-CN interactions will be largely increased and will lead to signaling performance deterioration, while unclear boundaries between RAN and CN will lead to extensive cross-domain data exchange within APIs and significantly increase the RAN processing complexity.

Proposal 1: RAN3 to list the RAN-CN interface options and identify aspects to be analyzed for different options.

## RAN-CN user plane interface

GTP-U tunnel over UDP/IP is used as the user plane interface solution for many interfaces, e.g., between gNB and UPF, between gNB and MB-UPF, between eNB and MBMS GW, between gNBs, between eNBs, between gNB-CU and gNB-DU, etc. It provides non guaranteed delivery of PDU Session user plane PDUs, and supports large volume of user data transmission.

Figure 1: User plane of legacy RAN-CN user plane interface

From our understanding, when discussing the RAN-CN user plane interface for 6G, the GTP-U tunnel over UDP/IP should adopted by default.

Proposal 2: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP is used.

## RAN-CN Data transfer design

The following objective can also be found in the SID:

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Data transfer and management are hot topics in both RAN2 and SA2, for new services data for which some companies considered to be not suitable to be transmitted on legacy control plane or user plane due to some claimed reasons (e.g., large-volume, MNO visibility/ controllability, QoS differentiation between different types of data…), a new data framework is proposed by some companies.

In 5G, there are multiple mechanisms for collecting data for different purposes. However, these different types of data are collected and utilized independently using different mechanisms, and cannot be well integrated or combined. There might be overlapping, duplicated, mixed classification of data, which is not efficient to be utilized by the network, and also difficult to be provided as a service friendly to the data consumers. Additionally, for data with large volume, such as AI models, the collection mechanisms may not flexible enough. Looking ahead to future 6G systems, for the collection of different types of data from various sources for different services and applications, maybe there is an opportunity to establish a data framework, to enable collection, management and distribution of data within 3GPP system for different use cases.

From RAN3 perspective, we can further discuss potential RAN3 impacts over RAN and CN interface to support the new 6G Data Transfer design, after further SA2/RAN2 progress.

Proposal 3: RAN3 further study potential RAN3 impacts over RAN and CN interface to support the 6G Data Transfer design, after further SA2/RAN2 progress.

# Conclusion

This document discusses the RAN-CN interface options, get the following observations and proposals:

RAN-CN control plane interface

There are two options (P2P vs SBI) for 6G RAN-CN control plane interface:

Option 1: RAN-CN interface: P2P

In this option, from a logical standpoint, the 6G RAN-CN interface is a point-to-point interface between an RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the RAN node and CN node. In the transport, IP layer point-to-point transmission is used to deliver the signalling PDUs. The application layer signalling protocol is referred to as NGAP*.

Option 2: RAN-CN interface: SBI

In this option, the 6G RAN-CN interface is a service-based interface which represents how a service is provided/exposed.

Observation 1: P2P-based interface, ensures clear RAN CN function-split, with the clear boundary the RAN node does not need to understand the processing sequence and logic among CN functions.

Observation 2: P2P-based interface can support flexible and lightweighted deployments of core network by allowing direct connectivity between the 6G RAN and 6G new CN function, if needed.

Observation 3: If service-based interface is used, due to the large scale of RAN nodes, the RAN-CN interactions will be largely increased and will lead to signaling performance deterioration, while unclear boundaries between RAN and CN will lead to extensive cross-domain data exchange within APIs and significantly increase the RAN processing complexity.

Proposal 1: RAN3 to list the RAN-CN interface options and identify aspects to be analyzed for different options.

RAN-CN user plane interface

Proposal 2: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP is used.

RAN-CN Data Transfer design

Proposal 3: RAN3 further study potential RAN3 impacts over RAN and CN interface to support the 6G Data Transfer design, after SA2/RAN2 progress.

# Reference

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] RP-2529093 Summary of Discussion on 6G Requirements for architecture and migration, NTT DOCOMO, INC. (Rapporteur)

# Text Proposal for 6G TR (based on R3-255517)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

# 6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

## 6.1	General Principles and Requirements

## 6.2 		RAN-CN interface

### 6.2.1	RAN-CN interface options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services and include control plane and user plane aspects considering interim milestone.

#### 6.2.1.x		RAN-CN Control Plane interface

The following options are to be studied to represent RAN-CN control plane interface:

Option 1: RAN-CN interface: P2P

In this option, from a logical standpoint, the 6G RAN-CN interface is a point-to-point interface between an RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the RAN node and CN node.

In the transport, IP layer point-to-point transmission is used to deliver the signalling PDU s.

The application layer signalling protocol is referred to as NGAP*.

NOTE:	The name of “NGAP*” is FFS.

Option 2: RAN-CN interface: SBI

In this option, the 6G RAN-CN interface is a service-based interface which represents how a service is provided/exposed.

#### 6.2.1.y		RAN-CN User Plane Interface

The RAN-CN user plane interface protocol stack is shown in Figure 6.2.1.y-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs between the RAN node and the CN node.

Figure 6.2.1.y-1: RAN-CN Interface User Plane protocol structure

### 6.2.2	Evaluation of RAN-CN interface options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 5.2.1

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>