3GPP TSG-RAN WG3 Meeting #130	R3-258173

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.2.2

Source:	Nokia, Qualcomm

Title:	Discussion on Continuous Management Based MDT

Document for:	Decision

# 1	Introduction

We here follow up on discussion on continuous MDT from last meeting.

# 2	Discussion

While the Rel-19 Continuous MDT (C-MDT) solution is intended to essentially rely on management-based legacy logged and immediate MDT, it is common understanding that some stage 3 impact is required related to network signalling for Xn Handover Preparation and UE Context Retrieval. For this purpose, an XnAP CR [1] was proposed at the last meeting, but it could not be agreed.

The XnAP CR introduces a new optional IE including the TR and TRSR assigned to the UE for an ongoing C-MDT session in the XnAP HANDOVER REQUEST and in the RETRIEVE UE CONTEXT RESPONSE messages. There was no corresponding procedural text for the HANDOVER REQUEST message in [1]. For the RETRIEVE UE CONTEXT RESPONSE message, the proposed procedural text was the following in [1]:

If the Continuous MDT IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, take it into account to configure the UE with Continuous Management Based MDT.

The current principles for C-MDT are the following:

- the OAM sends C-MDT configuration to all participating gNBs involved in the same C-MDT job (same TR/TRSR for logged and immediate MDT)

- the gNB selects suitable UEs and configures those with corresponding immediate MDT and logged MDT

- after reconnection from idle mode, the gNB shall upload the logs, including TR/TRSR from any UE that has logs available (this aspect requires correction in SA5 specification and we have submitted a corresponding CR to SA5)

- if the TR/TRSR reported by the UE corresponds to C-MDT, the gNB shall reconfigure the UE with immediate MDT and a logged MDT

In line with RAN3’s request in an earlier LS, SA5 did not describe in their specifications the handling of C-MDT in case of handover. Such description therefore needs to be introduced in RAN3 specification. However, no agreements cover this aspect to this day which would be needed to complete the C-MDT feature.

In order to solve this, a first aspect that needs clarification is for how long continuous MDT is required to continue. In fact, this aspect is not addressed in SA5 specifications. In the last RAN3 meeting, RAN3 also captured a related open point: How can a UE be deconfigured from Logged Continuous MDT.

In our view, there are two possible options:

Solution 1 – “endless” continuity: According to SA5, when the network selects a UE for continuous MDT it configures it with both Logged and Immediate MDT, and the UE is also reconfigured when it reconnects from idle mode with both Logged and Immediate MDT configurations. If a new TRSR is used for each reconfiguration, the continuous MDT job will gradually stop by itself when the gNBs run out of unused TRSRs for the given TR (reuse of an already used TRSR would create a correlation issue in the TCE).

The only way to stop C-MDT in a controlled way is that the OAM cancels / stops the entire C-MDT Trace job in all the involved gNBs in the configured area. However, also this aspect is not fully clarified in SA5’s specification and needs further correction in SA5. In case C-MDT Trace job is de-configured, no gNBs will select UEs for this C-MDT job anymore. This means that the logged MDT configuration will continue in the already configured UEs for 2 hours, and if/when those UEs reconnect from idle mode the gNB will retrieve the available logs for reporting to the TCE (but no additional reconfiguration to the UE will be done).

However, the most important aspect is a security issue that arises from the repeated “reconfiguration” of the Logged MDT configuration. SA5 has introduced the following text in TS 32.422 to address this point as follows:

##### 4.1.1.9.4	NG-RAN activation mechanisms for C-MDT data collections for non-split RAN architecture

For non-split RAN architecture, the C-MDT data collection procedure shall be same as specified in subclause 4.1.1.9.2 with following additions.

1)	The management system shall either send two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with same TR to the gNB, or send one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT) to the gNB. In additional of the trace control and configuration parameters specified in subclause 4.1.1.9.2 step 1, the Trace Session activation request(s) shall contain a TRSR Prefix Configuration parameter, which reserves a TRSR range(s) for the gNB during TRSR assignment. This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request. Furthermore, the management system shall ensure that Trace Session activation requests with the same TR are sent to all participating NR-RAN nodes involved in the same C-MDT job.

2)	Upon receiving the Trace Session activation request(s), the gNB shall identify the Trace Session activation request(s) as a C-MDT job, start a C-MDT session, and store the associated configuration parameters.

3)	The gNB and UE shall follow the steps specified in subclause 4.1.1.9.2, step 3 to 8, with following additions:

-	The gNB shall configure the selected UE with an Immediate MDT session and a Logged MDT session using the configurations from the Trace Session activation request(s). A unique TRSR is assigned to the UE for Immediate MDT and Logged MDT. The TRSR is allocated based on the received TRSR Prefix Configuration parameter.

-	Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and a Logged MDT.

-	In any case, if a new TRSR is assigned, the correlation between the old TRSR and the new TRSR shall be reported by gNB to the TCE.

The issue is highlighted in the above in that the rationale is to provide an optional mechanism to update (renew) the TRSR if required for security reasons. However, with this approach the 3GPP standard becomes incomplete because there has been no evaluation whether TRSR renewal is required or not, and therefore there is no mandate or recommendation in the standard relative to the TRSR renewal. However, security is a critical aspect, and it is therefore  essential to have such mandate or recommendation if TRSR renewal is needed for security reasons in Solution 1. Otherwise, it could imply that the same identifier “TR/TRSR” is sent over the air, both in the UL (when UE uploads the logged MDT reports to the network) and in the DL (when the UE is reconfigured with logged MDT). As a general principle, once a permanent identifier is transmitted over the air it cannot be reused in order to avoid security risks to the UE. Therefore, SA3 analysis is needed on this point if Solution 1 is maintained.

Introducing C-MDT feature according to Solution 1 in RAN3 specifications would mean that over a handover as well as during transition from RRC_Inactive to RRC_Connected a UE requires C-MDT reconfiguration. For Solution 1 this would imply that not only Immediate, but also Logged MDT configuration is reconfigured to the UE.

Observation 1: For Solution 1, SA3 would need to check security aspect of a C-MDT job of indefinite duration, and whether repeated reconfiguration of logged MDT (including information that can identify the C-MDT session in a permanent way) represents a security issue. SA3 should also provide guidance on whether the same TR/TRSR can be maintained throughout the C-MDT session, or whether renewal of the TRSR represents a security benefit.

Solution 2 – two-hour continuity: A solution closer to the legacy behaviour limits continuity to the duration of the legacy logged MDT session which is 2 hours. The gNB configured with C-MDT would identify UEs to configure with C-MDT. Those UEs will be configured with both Logged and Immediate C-MDT configuration. When the UE transitions from RRC_Idle to RRC_Connected, the network does not need to send the TR/TRSR to the UE (since the Logged MDT configuration is already stored in the UE according to legacy methods), but it should reconfigure the UE with immediate MDT configuration. Since, in immediate MDT TR/TRSR is not sent to the UE there won’t be a security breach. In this case, continuity is provided over the 2h period which is the maximum logging duration in the UE. UE could go to idle mode multiple times (within the 2h duration) and get back to connected where the network can reconfigure the UE with Immediate MDT measurements according to the receiving configuration from OAM. The key difference in this solution is that TR/TRSR identifier is not resent to the UE (which is key to meet the security requirements).

So, differently from Solution 1, in Solution 2 only Immediate MDT is reconfigured in the UE in case of handover. This will require some simple functional modification of the SA5 specification.

Observation 2: Solution 2 does not pose a security threat for the UE since only immediate MDT (and not Logged MDT) is reconfigured in the UE in case of handover or in case of transition from RRC_Inactive to RRC_Connected, but requires functional modification of the SA5 specification.

It can be noticed that neither of these solutions enables a “clean stop” of the C-MDT session.

For solution 1, UEs that are in idle mode when the OAM system stops C-MDT will still indicate report availability when they transition to connected mode. The gNBs will retrieve the logs by the UEs and forward them to the TCE. However, there will be no immediate MDT configuration available correlated to these logs. The network will therefore collect discontinuous logged MDT (i.e. without corresponding immediate MDT in connected mode) during 2 hours.

For solution 2, if the logged MDT duration expires when the UE is in idle mode, the gNB will not be aware of the expiry of the C-MDT session when the UE reconnects. So, the gNB will still reconfigure immediate MDT, and also transfer the Continuous MDT Indicator to the target node in case of Xn Handover or during transition from RRC_Inactive to RRC_Connected. Solution 2 therefore implies that the immediate MDT measurements collected during the last connection will not represent continuity with logged MDT measurements.

As can be seen from the above, solution 1 and solution 2 result in different XnAP CRs and stage 2 CRs for TS 37.320. As leading WG, we believe RAN3 therefore needs to first choose between these two solutions. If solution 1 is chosen, SA3’s confirmation would be required as discussed above. However, because RAN3 is going to agree stage 2 and stage 3 CRs at this meeting, it is possible to choose solution 2, which avoids some of the issues described for solution 1 and which would not require SA3’s confirmation in our view.

Proposal: RAN3 to choose solution 2 (two-hour continuity) for C-MDT.

We have submitted corresponding stage 2 CR (TS 37.320) and stage 3 CR (TS 38.423) to this meeting in [2] and [3].

# 3	Conclusion

In this paper we make the following observations and proposal:

Observation 1: For Solution 1, SA3 would need to check security aspect of a C-MDT job of indefinite duration, and whether repeated reconfiguration of logged MDT (including information that can identify the C-MDT session in a permanent way) represents a security issue. SA3 should also provide guidance on whether the same TR/TRSR can be maintained throughout the C-MDT session, or whether renewal of the TRSR represents a security benefit.

Observation 2: Solution 2 does not pose a security threat for the UE since only immediate MDT (and not Logged MDT) is reconfigured in the UE in case of handover or in case of transition from RRC_Inactive to RRC_Connected, but requires functional modification of the SA5 specification.

Proposal: RAN3 to choose solution 2 (two-hour continuity) for C-MDT.

# References

[1]		R3-257309, Continuous MDT support, Ericsson

[2]		R3-258417, Introduction of support for Continuous Management Based MDT in stage 2, Nokia

[3]		R3-258416, Support of Continuous Management Based MDT, Nokia