3GPP TSG-RAN3 Meeting #130	R3-258193

Dallas, US, Nov 17 - 21, 2025

Agenda Item:	12.2.2

Source: 	Qualcomm Incorporated

Title: 	(pCR To TR 38.745) Discussion on Intra-CU LTM

Document for:	Discussion/Decision

# Introduction

In the last RAN3#129bis few basic agreements were made and some open topics for further discussion was identified as below

RAN3#129bis Agreements Prioritize normal intra-CU LTM scenarios. For CU-DU split architecture, the following two scenarios are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. AI/ML assisted candidate cell selection for LTM Handover Preparation Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation. Open Issues For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM. Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

In this paper, we discuss on the open issues for AI/ML based Intra-CU LTM optimization.

# Discussion

## L1 and L3 Measurements based LTM

LTM is primarily a lower layer mobility function as it name states. L1 measurements-based LTM was specified in Rel18 and then L3 measurements-based LTM was added as an alternative due to some infra vendors and some UEs may not be able to support L1 measurements in initial product development phase.. Based on L1 MR, gNB-DU makes decisions for early sync and cell switch. For any AI/ML based optimization for LTM, optimizations will not be useful without considering L1 measurements for input. Whether there is an interface impact or not due to inputs can be analysed based on the use case agreed to be studied.

LTM is a lower layer mobility function and L1 measurement reports are key input information for any AI/ML based optimization of the Intra-CU LTM.

Study on AI/ML based Intra-CU LTM should consider both L1 and L3 Measurement Reports as input data. Interface impact due to L1/L3 MR can be analysed during normative phase based on the use cases/features agreed to be studied.

Similar to Release 19 AI/ML use cases, the Release 20 Intra CU LTM should also consider both split and non-split architecture for study. Based on the use case and the feature under discussion, the AI/ML inference should be flexibly located in gNB-CU or gNB-DU.

Based on the AI/ML based Intra CU LTM use case, inference should be flexibly located in gNB-CU or gNB-DU.

## Target and Candidate Cell/Beam Prediction

In the last meeting it was agreed to study AI/ML assisted candidate cell and beam selection for LTM Handover Preparation. Selection of potential candidate cells is required during Handover preparation. Among the prepared candidate cells, subset of them can be chosen for early synchronization considering them as potential target cells. For handover execution, target cell needs to be selected among the prepared candidate cells. AI/ML can be used for selection of potential candidate cells in all the above-mentioned steps.

RAN3 to study the following as a part of LTM HO preparation and execution–

  - AI/ML assisted potential candidate cell selection among the prepared candidate cells to trigger Early Synchronization

  - AI/ML assisted Target cell selection among the prepared candidate cells for LTM cell switch.

- For initial candidate cell selection, AI/ML can be used in gNB-CU. Whether the UE Context Setup Request for LTM candidate cells is based on AI/ML, is upto gNB-CU. In LTM HO, the candidate cells are prepared in advance. Hence providing predicted candidate cells in advance will not be any use. Input and feedback needed to assist AI/ML based selection of initial candidate cell in gNB-DU can be studied. gNB-CU can provide assistance information to gNB-DU on predicted candidate cell and beam for early sync and predicted target cell for cell switch.

In LTM, candidate cells are selected and prepared in advance by gNB-CU. It is not useful to specify whether the selected candidate cells are predicted or not.

Candidate cells selected by gNB-CU can be based on AI/ML technique. No standards impact for the output in this case. Input and feedback needed to assist AI/ML based selection of initial candidate cell in gNB-CU can be studied.

Candidate cells for early sync and target cell for cell switch can be selected by source gNB-DU using AI/ML. No Standards impact in this case.

Study gNB-CU providing assistance information to gNB-DU, on predicted candidate cells and beams for early sync and predicted target cell and beam for cell switch. Agree to TP in the Annex

In the last meeting it was agreed to study AI/ML assisted beam selection for LTM HO preparation. Measured UE Trajectory and UE Trajectory prediction is one of the key input information for candidate cell selection at gNB-CU. Hence it is beneficial to study addition of predicted beam in the UE Trajectory prediction. This can be used as input for beam prediction for candidate cells in the gNB-CU.

UE Trajectory Prediction carried in the LTM Handover Request should include beam prediction to assist subsequent LTM cell switch and can also be used as input for beam prediction in the candidate cells.

## TA Value Prediction

In the last meeting companies were not clear on TA prediction scenarios and how it can benefit LTM HO procedures. Prediction of TA value in the source gNB can assist LTM HO in two different scenarios as described below.

Spatial Domain TA Prediction

Source gNB-DU can request the UE to perform early sync on few candidate cells. Source gNB-DU can use the measured TA values as input and predict the TA values for the same UE for remaining prepared candidate cells, without UE performing early sync on those candidate cells. This will reduce the early UL sync procedures for  few candidate cells. TA value can also be predicted for the UE, if the previous TA value expires. In this case, UE does not need to perform early UL sync again due to TA value expiry.

Inputs that can be used by source gNB-CU or gNB-DU to Predict TA value are –

- L1/L3 Measurement Reports

- Beam information

- Invalid TA report

- Measured TA values by UE for few candidate cells

- UE Location information (if available)

Output: Predicted TA value. If gNB-CU performs TA value prediction, it can pass it as assistance information to gNB-DU. If gNB-DU performs TA value prediction, it can use it internally and no standards impact foreseen with prediction.

Benefits: Reduction in early sync procedure for few candidate cells.

Temporal Domain TA Prediction

Source gNB-DU can use the historical TA values of UE in a cell in time domain as input, and predict TA value for the UE in a cell. UE can be provided with predicted TA value during the Cell Switch without UE performing early sync. This will reduce early sync procedures. TA value can also be predicted for the UE, if the previous TA value expires. In this case, UE does not need to perform early UL sync again due to TA value expiry.

Inputs that can be used by source gNB-CU or gNB-DU to Predict TA value are –

- L1/L3 Measurement Reports

- Beam Information

- Historical TA values

- Invalid TA report

- UE Location information (if available)

Output: Predicted TA value. If gNB-CU performs TA value prediction, it can pass it as assistance information to gNB-DU. If gNB-DU performs TA value prediction, it can use it internally and no standards impact foreseen.

Benefits: Reduction in early sync procedure for candidate cells.

RAN3 to study TA prediction to optimize early UL sync procedure. Agree to TP in Annex

## Measured TA Value Validity Prediction

Most of the companies agreed to study prediction of Measured TA value validity prediction. Currently the TA value validity is determined internally in the gNB-DU. Based on historical TA values, TA expiry instances and TA failures, source gNB-CU or gNB-DU can predict the validity of the measured TA value. If gNB-DU predicts the validity of the measured TA value, there is no standards impact with the prediction. Input and feedback information needed for TA value validity prediction can be studied. If gNB-CU predicts the TA value validity, then gNB-CU sends this as assistance information to gNB-DU.

Currently TA value validity time is internally decided by source gNB-DU and a timer is maintained for the same.

RAN3 to study TA value validity prediction for measured TA value to reduce LTM cell switch failures due to TA value expiry.

## LTM Resource Optimization

Though LTM enables faster HO execution for the UE, the trade off is resource reservation in multiple candidate cells. Especially reserving lower layer resources like RACH and CG resources, when UE may or may not be handed over to the candidate cells will impact the resource availability in the candidate cells. LTM also has conditional LTM and subsequent LTM flavours. In case of conditional LTM, the candidate cells are completely unaware if UE will be handed over and its left to UE’s decision to perform RACH to the candidate cell when the conditions are met. Subsequent LTM configuration also involves keeping lower layer resources reserved. AI/ML can be applied to optimally reserve resources in the candidate cells.  The lower layer resources cannot be reserved for a longer duration as these are physical layer resources and be reused among other UEs in the cell. Hence LTM configuration can be valid for a certain period of time to optimize resource reservation at the target and the resources can be released or re-negotiated upon expiry.

In Intra-CU LTM and subsequent LTM radio resources like CG, RACH are reserved in multiple candidate cells for a longer duration without knowing if UE will be handed over.

RAN3 to study and discuss resource reservation optimization in candidate cells using AI/ML for Intra-CU LTM and subsequent Intra-CU LTM

Study (predicted) LTM configuration validity for LTM configuration (CG, RACH resources) provided from candidate cell to source cell as assistance information.

## CSI-RS Activation Deactivation Prediction

CSI-RS is needed for granular/ narrow beam measurements at the target/candidate by the UE for LTM HO execution. Based on the SSBs measured by the UE, Semi Persistent CSI-RS can be activated or deactivated in the Target/Candidate cells. Early CSI acquisition is also needed to reduce the scheduling delay at the target after the LTM HO. Based on the UE measurements, the SP CSI-RS can be activated or deactivated in the target. AI/ML can be used to optimize the SP-CSI-RS resource activation and deactivation at the target.

SP CSI-RS is required for Measurement Reporting and early CSI acquisition for reduce scheduling delay.

RAN3 to study prediction of SP-CSI-RS resource activation and deactivation for candidate cells.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	LTM is a lower layer mobility function and L1 measurement reports are key input information for any AI/ML based optimization of the Intra-CU LTM.

Observation 2.	In LTM, candidate cells are selected and prepared in advance by gNB-CU. It is not useful to specify whether the selected candidate cells are predicted or not.

Observation 3.	Currently TA value validity time is internally decided by source gNB-DU and a timer is maintained for the same.

Observation 4.	In Intra-CU LTM and subsequent LTM radio resources like CG, RACH are reserved in multiple candidate cells for a longer duration without knowing if UE will be handed over.

Observation 5.	SP CSI-RS is required for Measurement Reporting and early CSI acquisition for reduce scheduling delay.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Study on AI/ML based Intra-CU LTM should consider both L1 and L3 Measurement Reports as input data. Interface impact due to L1/L3 MR can be analysed during normative phase based on the use cases/features agreed to be studied.

Proposal 2:	Based on the AI/ML based Intra CU LTM use case, inference should be flexibly located in gNB-CU or gNB-DU.

Proposal 3:	RAN3 to study the following as a part of LTM HO preparation and execution–

a.	AI/ML assisted potential candidate cell selection among the prepared candidate cells to trigger Early Synchronization

b.	AI/ML assisted Target cell selection among the prepared candidate cells for LTM cell switch.

For initial candidate cell selection, AI/ML can be used in gNB-CU. Whether the UE Context Setup Request for LTM candidate cells is based on AI/ML, is upto gNB-CU. In LTM HO, the candidate cells are prepared in advance. Hence providing predicted candidate cells in advance will not be any use. Input and feedback needed to assist AI/ML based selection of initial candidate cell in gNB-DU can be studied. gNB-CU can provide assistance information to gNB-DU on predicted candidate cell and beam for early sync and predicted target cell for cell switch.

Proposal 4:	Candidate cells selected by gNB-CU can be based on AI/ML technique. No standards impact for the output in this case. Input and feedback needed to assist AI/ML based selection of initial candidate cell in gNB-CU can be studied.

Proposal 5:	Candidate cells for early sync and target cell for cell switch can be selected by source gNB-DU using AI/ML. No Standards impact in this case.

Proposal 6:	Study gNB-CU providing assistance information to gNB-DU, on predicted candidate cells and beams for early sync and predicted target cell and beam for cell switch. Agree to TP in the Annex

Proposal 7:	UE Trajectory Prediction carried in the LTM Handover Request should include beam prediction to assist subsequent LTM cell switch and can also be used as input for beam prediction in the candidate cells.

Proposal 8:	RAN3 to study TA prediction to optimize early UL sync procedure. Agree to TP in Annex

Proposal 9:	RAN3 to study TA value validity prediction for measured TA value to reduce LTM cell switch failures due to TA value expiry.

Proposal 10:	RAN3 to study and discuss resource reservation optimization in candidate cells using AI/ML for Intra-CU LTM and subsequent Intra-CU LTM

Proposal 11:	Study (predicted) LTM configuration validity for LTM configuration (CG, RACH resources) provided from candidate cell to source cell as assistance information.

Proposal 12:	RAN3 to study prediction of SP-CSI-RS resource activation and deactivation for candidate cells.

4 pCR To TR 38.745

# 4	Use cases and Solutions

## 4.1	AI/ML based Intra-CU LTM

### 4.1.1	Use case description

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [x].

Intra-CU LTM is specified in TS38.401[x].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

### 4.1.2	Solutions and Standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as input data:

- L1 Measurement Reports

- Measured/Predicted L3 Measurement Reports

- Resource Status Report

- Measured/Predicted UE Trajectory information

- UE History Information

- Historical TA values

- Measured TA value by UE for few candidate cells

- UE Location (if available)

- SON Reports

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation (including early sync)

- Target cell and beam for LTM cell switch

- TA value

- Validity of measured TA value

- LTM Configuration validity

- CSI-RS Resource (de)activation

#### 4.1.3.4	Feedback of AI/ML-assisted Intra-CU LTM

For optimized Intra-CU LTM operation, the following information can be considered as feedback data:

- Legacy UE Performance

- Measured UE Trajectory

- Measured Resource Status

- SON reports

#### 4.1.3.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work:

- F1 interface

5 References

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3