3GPP TSG-RAN WG3 Meeting #130	R3-258207

# Dallas, US, 17 – 21 November 2025

Agenda item:	13.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on architecture for ISAC

Document for:	Discussion and Decision

# 1	Introduction

The ISAC SID ([1]) agrees following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution discusses the architecture for ISAC.

# 2	Overall ISAC architecture

## 2.1	Interface between SF and SE

SA2 is discussing the ISAC architecture, and will have the conclusion before the RAN3 complete the analysis. The RAN3 discussion shall align with SA2 architecture decision.

One of the architecture issues is whether the gNB directly connect with the SF, or indirectly connect with the SF via the AMF. The main impact is the AMF, which is in the scope of SA2. So RAN3 should wait for the SA2 conclusion for the architecture issues.

In last meeting, RAN3 agreed the logical architecture for ISAC:

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

While RAN3 is awaiting for the conclusion from SA2, RAN3 can discuss the detail of the Nx interface.

Current gNB uses NGAP/SCTP for communication with CN CP nodes. No matter whether the direct or indirect interface is used between SF and SE, the NGAP/SCTP shall be reused for the control plane signalling to/from the gNB.

Proposal 1: Nx interface should be based on NGAP/SCTP.

## 2.2	How to report sensing data to SF

Since the sensing data is transparent to the AMF, and AMF does not perform any processing for the sensing data, it makes sense that a direct interface is used for gNB to report the sensing data to the SF.

The next question is whether the sensing data is sent via signaling interface or via a data tunnel. This is highly related to the actual data or data volume of the reported sensing data. RAN1 is discussing the various levels of measurements ([3]).

[FL3][H] Proposal 7.1-1-rev2 (all records from offline) - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - E.g., data size: Msubcarrier*Ncpi*Mport*Nport*Nquantization, e.g., 3276*(80*4)*(4*8*2)*32 = 2.1Gbits - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle, which is the first-stage processing products from sensing raw data. The amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle, which is the first-stage processing products from sensing raw data. Only amplitudes/phases of partial profile within reduced range of delay/angle, e.g., <= CP length is reported for overhead reduction. - Other examples are not precluded - E.g., data size: Mcp*Ncpi*Mport*Nport* Nquantization, e.g., 288*(80*4)*(4*8*2)*32 = 188.7Mbits - Level 3 per path (subset of Level 2): QC - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Supported by: QC - Level 4 per point: LG, OPPO, Xiaomi, - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - vivo, CATT, Xiaomi, OPPO, Sony, LGE, HW, QC, IDCC - Supported by: - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - ZTE, Nokia, - Supported by: - Note: A detected point is expressed by a 4-dimention information of {Range, velocity, 3D angle} or {3D Position, velocity}. One or multiple detected points for a target/object can be derived as the outputs of the sensing algorithm. - E.g., data size: Npoint*5* Nquantization - ‘5’ means 5 parameters, i.e., delay, Doppler, horizontal angle, vertical angle, power - E.g., 100*5*32 = 16Kbits - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - E.g., data size: (Npoint=1)*5* Nquantization - ‘5’ means 5 parameters, i.e., 3D location, velocity, power - E.g., 15*5*32 = 2400bits - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

At current stage, RAN3 can continue the analysis on both options

* Option 1: the sensing data is sent to SF via signalling

* Option 2: the sensing data is sent to SF via a data tunnel.

The signaling interface is not supposed to send large volume of data, so Option 1 is restrictive to only support small volume of sensing data. Using the data tunnel (Option 2) is flexible to support both small volume of sensing data and potential large volume of sensing data. In case the data tunnel is used, GTP-U tunnel can be an option.

Proposal 2: using a direct data tunnel, e.g. GTP-U, to report the sensing result to SF.

## 2.3	gNB capability awareness in the SF

The SF need to be aware of the sensing capability (including the serving area) of the gNB. There are two options that SF can be aware of the sensing capability:

- Via signaling

This has been used in positioning. The TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node.

- Via OAM

This has been used in the recent Ambient IoT, where the AIOTF is aware of the serving area of the gNB, the reader’s location, the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration.

Both options are feasible. The final solution may depend on the detailed capability information of the gNB that needs to be known in the SF. In case the required information is limited and static, the OAM based solution can be used. Otherwise, the signaling-based solution may be better. So it may be better that RAN3 first discuss the detailed capability information of the gNB that needs to be known in the SF.

Observation: both signaling-based solution and OAM-based solution are feasible. The final solution depends on the list of gNB capability information that needs to be known by the SF.

Proposal 3: RAN3 need to first discuss the list of gNB capability information that needs to be known by the SF.

# 3	Conclusion

In this contribution, we discussed the high-level issues for ISAC. Our proposals are:

Proposal 1: Nx interface should be based on NGAP/SCTP.

Proposal 2: using a direct data tunnel, e.g. GTP-U, to report the sensing result to SF.

Proposal 3: RAN3 need to first discuss the list of gNB capability information that needs to be known by the SF.

# References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 23.700-14

- R1-2507427, Summary #5 on evaluations for NR ISAC

- R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR