3GPP TSG-RAN WG3 Meeting #130						R3-258244

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		13.3

Title:		(TP to BL pCR of TR38.765) The discussion on ISAC RAN-CN procedure and signaling

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the ISAC SID [1], the following items should be studied in RAN that:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Additionally, the following procedure was agreed upon during the last meeting:

Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

This contribution will continue to address the procedures and signaling related to ISAC.

# 2. Discussion

## 2.1 gNB sensing information reporting

Below is the progress from the latest SA2 meeting regarding the Sensing Entity, as well as Discovery and (Re-)Selection that [2]:

- Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

As the statement here, the Sensing Function (SF) selects Sensing Entities (gNBs) based on information about the Sensing Entities. Consequently, the SF needs to obtain gNB sensing information to perform Sensing RAN node discovery and selection. Ambient IoT has similar requirements for gNBs, and an OAM solution is adopted for this purpose—specifically, OAM transmits gNB AIoT information to the AIoTF. Likewise, the OAM solution is applicable for ISAC to transmit gNB sensing information to the SF.

As mentioned in [1], the gNB sensing information at least includes the supported sensing area. Further details on whether additional information should be included in the gNB sensing information will depend on SA2's ongoing progress and requirements.

Proposal 1: The SF may acquire gNB sensing information via OAM, which includes at least the supported sensing area.

## 2.2 RAN-SF Interface establishment procedure for direct connection

For direct connections between the gNB and SF (either via a newly defined interface or NG-AP reuse), the corresponding interface must be established before any signaling is transmitted between the RAN and SF..

Upon receiving a sensing service request, the SF selects the appropriate sensing RAN node based on available RAN node information and sensing service requirements. Subsequently, the SF sends an Interface Setup Request message to the selected sensing RAN node. If the interface setup is successful, the sensing RAN node responds with an Interface Setup Response message. This signaling procedure is illustrated in Figure 1.

Figure 1: Interface establishment procedure for direct connection and indirect connection.

For indirect connections between the RAN and SF via the AMF, the existing NG-AP interface is reused to carry control signaling. In this scenario, no separate interface establishment procedure is needed.

Proposal 2: A Class 1 interface establishment procedure is required for direct connections.

## 2.3 Sensing service configuration

The content of Sensing Request Message

| IEs | Reason |
| --- | --- |
| Sensing session ID | A gNB may support parallel sensing services from an SF, so a sensing session ID shall be used to distinguish these parallel services over the Nx interface. |
| Intended sensing area | This parameter indicates the intended service area for the sensing service, which includes at minimum a gNB ID list and a cell list. |
| Sensing parameters configuration | Per SA2 TR [2]: “The Sensing Entity can provide one-time or periodic sensing data to the Sensing Function.” Accordingly, the sensing parameters relate to one-time or periodic sensing configuration and are used for resource allocation at the sensing gNB. |
| Sensing Function ID | For indirect connections, the SF ID is required to enable the AMF to transmit Sensing Response messages and Sensing Report messages to the corresponding SF. |

The content of Sensing Response message

| IEs | Reason |
| --- | --- |
| Sensing session ID | It indicates the request sensing session is initiated successfully. |
| Sensing Function ID | For indirect connections, this parameter specifies the target SF ID to which the AMF should forward the Sensing Response message. |

The content of Sensing Report message

| IEs | Reason |
| --- | --- |
| Sensing session ID | This parameter helps the SF correlate different sensing reports for the same sensing session. |
| Sensing Data | This includes sensing measurement data received from the lower layer. Its specific content is to be further studied (FFS), pending RAN1’s progress. |
| Sensing Function ID | For indirect connections, this parameter specifies the target SF ID to which the AMF should forward the Sensing Data. |

Proposal 3: A gNB should be able to support parallel sensing services.

Proposal 4: RAN3 should consider the following information for sensing procedures and TPs in the Annex:

- Sensing Request Message

  - Sensing session ID

  - Intended sensing area

  - Sensing parameters configuration

  - Sensing Function ID

- Sensing Response message

  - Sensing session ID

  - Sensing Function ID

- Sensing Report message

  - Sensing session ID

  - Sensing Data

  - Sensing Function ID

## 2.4 Sensing service revocation

Sensing service revocation is a key topic in TR 23.700-14 [2], which documents several solutions (e.g., Solutions #8, #9, #11, #12) and includes the following statement regarding service revocation:

c) The Sensing Authorization information is pre-configured or configured by OAM, and stored in Sensing authorization functionality for authorisation of the sensing service request. d) Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. e) The AF can cancel the on-going Sensing Service Request when needed. f) There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

The AF may decide to revoke an ongoing sensing service and send a sensing revocation request to the SF. The SF then initiates a Sensing Revocation Request to the sensing RAN node, and this request shall include the corresponding sensing session ID. Upon receiving the request, the sensing RAN node shall release the ongoing sensing session context and associated sensing resources. The sensing RAN node subsequently sends a Sensing Revocation Response message to the SF to confirm successful revocation. This signaling procedure is illustrated in Figure 2.

Figure 2: Sensing Revocation procedure for procedure for direct connection and indirect connection

Proposal 5: A Class 1 sensing service revocation procedure shall be triggered by the SF. The sensing session ID shall be included in both the Sensing Revocation Request and Sensing Response messages.

Proposal 6: Upon receiving a Sensing Revocation Request, the sensing RAN node shall release the ongoing sensing session context and allocated sensing resources.

# 3. Conclusion

Proposal 1: The SF may acquire gNB sensing information via OAM, which includes at least the supported sensing area.

Proposal 2: A Class 1 interface establishment procedure is required for direct connections.

Proposal 3: A gNB should be able to support parallel sensing services.

Proposal 4: RAN3 should consider the following information for sensing procedures and TPs in the Annex:

- Sensing Request Message

  - Sensing session ID

  - Intended sensing area

  - Sensing parameters configuration

  - Sensing Function ID

- Sensing Response message

  - Sensing session ID

  - Sensing Function ID

- Sensing Report message

  - Sensing session ID

  - Sensing Data

  - Sensing Function ID

Proposal 5: A Class 1 sensing service revocation procedure shall be triggered by the SF. The sensing session ID shall be included in both the Sensing Revocation Request and Sensing Response messages.

Proposal 6: Upon receiving a Sensing Revocation Request, the sensing RAN node shall release the ongoing sensing session context and allocated sensing resources.

# 4. References

[1] RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR

[2] TR 23-700-14 V1.1.0, Study on Integrated Sensing and Communication Stage 2

# 5.	Annex (TP for TR 38.765)

Start of Change

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

A gNB should be able to support parallel sensing services from an SF.

The SF can acquire gNB sensing information via OAM, with the information including the supported sensing area.

Next Change

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

- The SF sends sensing request to the gNB. It may include the Sensing session ID, intended sensing area, sensing parameters configuration and SF ID for indirect connection.

2.	The gNB sends sensing response to the SF. It may include Sensing session ID and SF ID for indirect connection.

3.	If requested in Step 1, the gNB sends sensing report to the SF. It may include Sensing session ID, Sensing Data, SF ID for indirect connection.

End Change