3GPP TSG-RAN WG3 Meeting #130	R3-258265

Dallas, TX, USA, November 17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0508 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 Correction of WAB Xn Connection Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
| ss | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is not clear that the normative text in Clause 12.8 text refers to Xn establishment between a gNB or a WAB-gNB, and a WAB-gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that the normative text on Xn connection management refers to Xn establishment between a gNB or a WAB-gNB, and a WAB-gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading specification text in TS 38.401. Impact assessment towards the previous version of the specification (same release): Isolated impact. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

## 12.8	Xn connection management

A WAB-gNB can establish an Xn connection with the BH-gNB serving the WAB-MT co-located with the WAB-gNB, and with the neighbouring gNBs. During the Xn setup or NG-RAN node configuration update, the WAB-gNB can include an ID of the co-located WAB-MT, to indicate that it is a WAB-gNB. In case the peer gNB is the WAB-MT’s BH-gNB, the WAB-MT ID makes the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB, and the cell ID of BH-gNB´s cell serving the WAB-MT.

Establishment of Xn connection between a gNB or a WAB-gNB and a WAB-gNB can be avoided, in which case the gNB or the WAB-gNB rejects the Xn setup initiated by a WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.

A gNB or a WAB-gNB can be configurable with respect to whether it should accept or reject Xn setup requests received from WAB-gNBs.

-------------------------------------------End of changes-------------------------------------------