3GPP TSG-RAN WG3 Meeting #130	R3-258320

Dallas, US, 17-21 Nov, 2025

Title:	Discussion on Reply LS on geographical area scope MDT

Agenda Item:	9.2.1

Source:	Huawei

Document for:	Discussion

1	Introduction

There is an incoming LS in [1] R3-258005/ S5-254790 from SA5, which is the reply LS on geographical area scope MDT.

This contribution discusses one issue existing in the reply LS from SA5.

2	Discussion

In the reply LS from SA5 [1], it says:

SA5 thanks RAN3 for the incoming LS on geographical area scope MDT (R3-255960). SA5 has been requested to update SA5 specifications based on the RAN3 agreed geographical area scope for MDT and optionally a PLMN ID list. SA5 has discussed and agreed on the proposed changes related to Geographical Area. - Geographical Area is agreed and added as a new choice for area scope for MDT. - Reuse PLMN ID list in configuration parameters which are specific for MDT. Agreed contributions are attached for reference.

It can be concluded that SA5 implemented the geographical area for NTN mistakenly as it states to reuse the PLMN ID list in the configuration parameters for MDT. However, the existing PLMN ID  list in MDT configuration in SA5 specifications should refer to the Management Based MDT PLMN List IE or the Signalling Based MDT PLMN List IE in NGAP which is used to indicate the user consent of the UE. This can be confirmed by the specification text in section 5.10.24 MDT PLMN List in TS 32.422.

5.10.24 MDT PLMN List This is an optional parameter indicating the PLMNs where measurement collection, status indication and log reporting is allowed. E.g. the UE performs these actions for Logged MDT when the RPLMN is part of this set of PLMNs. Maximum of 16 PLMNs can be defined. To the UE it is communicated as the plmn-IdentityList. Between the NEs it is communicated either as the Management Based MDT PLMN List or as the Signalling Based MDT PLMN List, depending on how the MDT was activated. For further details see also TS 37.320 [30], TS 36.331 [32], TS 38.331 [43], TS 36.413 [36] and TS 38.413 [49].

Technically, the Signalling Based MDT PLMN List IE cannot be reused for the optional MDT PLMN list which was newly introduced in the geographical area IE in NGAP. The implementation in NGAP is quoted below.

9.3.3.68 Geographical Area This IE is used to indicate the geographical area scope for NTN MDT. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NTN Geographical Area; 1..<maxnoofAreaNTN>. >CHOICE Area Type; M. >>Circle. >>>Reference Location; M; OCTET STRING; ReferenceLocation-r17 as defined in TS 38.331[18]. >>>Distance Radius; M; INTEGER(1..65535); Each step represents 50m distance.. >>Polygon. >>>Polygon; M; OCTET STRING; Defined in TS 37.355 [59]. The first/leftmost bit of the first octet contains the most significant bit.. MDT PLMN List; O; 9.3.1.168.

Therefore, a reply LS to SA5 is necessary to inform them to correct this error implementation.

- To reply SA5 that the existing PLMN ID list in the MDT configuration cannot be reused for the geographical area scope. A new optional MDT PLMN list parameter shall be introduced into the geographical area scope.

3	Conclusion

This document proposes:

Proposal 1:	To reply SA5 that the existing PLMN ID list in the MDT configuration cannot be reused for the geographical area scope. A new optional MDT PLMN list parameter shall be introduced into the geographical area scope.

The reply LS is provided in [2].

Reference

[1] R3-258005 Reply LS on geographical area scope MDT

[2] R3-258321 [Draft]Reply LS on geographical area scope MDT