3GPP TSG-RAN WG3 Meeting #130	R3-258399

Dallas, USA, 17th – 21st November 2025

Agenda Item:	10.3.1

Source: 				LG Electronics Inc.

Title:			General principles and requirements for 6G RAN-CN interface

Document for:		Discussion and Decision

1.	Introduction

In RAN3#129bis meeting, it was agreed in [1] to capture the following general principles and functions for RAN-CN interface in 6G into TR 38.760-3 [2]:

Clauses 6.1.1 and 6.1.2 of TR 38.760-3 [2]: … ### 6.1.1 General Principles Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface. The general principles for the specification of the 6G RAN-CN interface are as follows: - the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN; - the 6G RAN-CN interface supports control plane and user plane separation; - the 6G RAN-CN interface supports future enhancements; - the 6G RAN-CN interface supports all possible RAN deployment scenarios; - the 6G RAN-CN interface supports RAN sharing between multiple operators; - the 6G RAN-CN interface supports the operation of network slicing; - the 6G RAN-CN interface supports enhanced service awareness in RAN; - the 6G RAN-CN control plane interface supports reliable signalling transmission; - the 6G RAN-CN interface is designed with a clear functional split between RAN and CN. ### 6.1.2 RAN-CN Interface Functions RAN-CN control plane interface supports following functions: - UE context management : The functionality to manage UE context between the RAN and CN; - Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress; - PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress. …

In this contribution, we will discuss additional general principles and functions on the RAN-CN interface, and provide our view on it.

2.	Discussion

In RAN3#129bis meeting, it was agreed that the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN. It is a reasonable requirement for the RAN-CN interface. However, there is one open issue to be discussed.

In 5G, the point-to-point (P2P) interface (i.e., NG interface) is supported between the NG-RAN and the AMF in 5GC. However, the SCTP used for the NG control plane interface was not designed to perform the routing of AP messages in cloud system. In this option, the functionalities are supported by the procedures defined as the part of NG interface between NG-RAN and the AMF. Therefore, it seems to be far from the cloud-friendly protocol.

As mentioned before, we think that the 6G RAN should advance toward a fully virtualized and modular design. It simplifies the introduction of a new services without impacting other services. In other words, this would allow RAN functions to be dynamically deployed at optimal locations based on network needs and real-time conditions. Therefore, in order to support dynamic deployment of RAN functions in RAN-CN interface, the following principle was agreed in last meeting [1].

- 6G RAN-CN interface supports all possible RAN deployment scenarios.

This means that in 6G, since it is important to adopt flexible scaling, improved resource utilization, and the virtualization of RAN NFs, cloud-friendly protocol should be considered even if the P2P interface is adopted for 6G.

Another possible option in 6G RAN-CN interface is to introduce the service-based interface (SBI) between 6G RAN and each NF in 6G CN. The service-based interface has a cloud-friendly design. The SBI enables scalable and flexible network deployments to meet various requirements for new services. In 5G, the SBI is already adopted in the core network. All NFs in 5GC are inter-connected via this SBI. Each NF exposes its NF services to other NFs. Then, this NF service can be requested by another NF to provide a certain NF service. It is also possible that the NF subscribes to NF service offered by another NF. Therefore, the exchange of signalling information between the 6G RAN and 6G CN can be supported by SBI. Anyhow, the details on RAN-CN interface options are further discussed in [3].

Observation 1: The RAN-CN interface should support the cloud-friendly design.

Also, we think that for new services, the RAN information needs to be exposed directly to the entity (e.g., 6G AF) in 6G CN, e.g., bypassing the other entity (e.g., 6G AMF) in 6G CN. In 5GC, it is difficult for the NG-RAN to expose the RAN information to the AF directly. Based on the request from 5GC, the NG-RAN just reports the RAN information to the entity (e.g., AMF/SMF/PCF/NEF/UPF) in 5GC. Then, this CN entity exposes this information to AF, thus resulting in additional latency. In 6G, the RAN should be able to report the network information directly to the consumer based on the instructions provided by the entity in 6G CN. It is also needed to support provision of information which can be used for new services in 6G RAN by external third party. For example, in Sensing service, the sensing function in 6G CN can request some sensing operations (e.g., object detection, object tracking, environment sensing) together with the sensing assistance information to the sensing entity (e.g., in 6G RAN) over the SBI. After performing the sensing operations requested from the sensing function, the sensing entity (e.g., in 6G RAN) can provide the measurement (or sensing) results to directly the sensing function without the help of the 6G AMF or to the external third party via the 6G NEF/AF bypassing the sensing entity. Another example is that in 5G, the energy related information in NG-RAN is not exposed via the NG interface to the entity in 5GC. In 6G, if the RAN-CN interface supports exposure of RAN information for new services, we think that the energy related information in NG-RAN can be provided to the 6G CN.

Observation 2: The RAN-CN interface should support exposure of RAN information for new services.

Considering the limited initial coverage and the need to minimize impact on user experience (e.g., QoS, QoE) during early 6G deployments, it is deemed essential to support interworking with 5GS from the beginning. Seamless service continuity requires interworking between 6G and the existing 5G system. This means that the RAN-CN interface needs to support inter-system mobility between existing 5G system and new 6G system irrespective of the option selected for the RAN-CN interface. We think that if the P2P interface is adopted for 6G, the existing inter-system mobility procedure between 4GS and 5GS can be used as a baseline for 6G. However, for now, it is unclear how to support the inter-system mobility between 5G system and 6G system if the SBI interface is considered between 6G RAN and 6G CN.

Observation 3: The RAN-CN interface should support inter-system mobility between 5G system and 6G system.

In order to capture the above observations, the following proposal is suggested to RAN3:

Proposal 1: It is proposed to capture the following principles into TR 38.760-3 [2]:

- The 6G RAN-CN interface supports the cloud-friendly design.

- The 6G RAN-CN interface supports exposure of RAN information for new services.

- The 6G RAN-CN interface supports inter-system mobility between 5G system and 6G system.

According to the pCR in [1], it was agreed that the 6G RAN-CN interface supports the UE context management, the transport of NAS messages, and the PDU Session management functions. However, in 5G study, the following functions were also considered and captured in TR 38.801 [4]. We think that these functions can be considered as a baseline for 6G RAN-CN interface.

Clause 7.3.3 of TR 38.801 [4]: … ### 7.3.3 NG Interface Functions NG-C interface supports following functions: - Interface management: The functionality to manage the NG-C interface; - UE context management : The functionality to manage the UE context between the New RAN and CN; NOTE 1: The UE context information may include roaming and access restriction and security information. NOTE 2: The UE context information may include the information related with network slicing. - UE mobility management: The functionality to manage the UE mobility for connected mode between the New RAN and CN; - Transport of NAS messages: procedures to transfer NAS messages between the CN and UE; - Paging: The functionality to enable the CN to generate Paging messages sent to the New RAN and to allow the New RAN to page the UE in RRC_IDLE state; - PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective New RAN resources that are made of data flows carrying UP traffic. - Configuration Transfer: the functionality to transfer the New RAN configuration information (e.g. transport layer addresses for establishment of Xn interface) between two New RAN nodes via the NGC. NOTE 3: It can be discussed on whether congestion and overload control function is needed or not in normative phase. …

First, it is reasonable that the interface management function is supported if the P2P interface option is selected for the 6G RAN-CN interface. For SBI between 6G RAN and 6G CN, our understanding is that if HTTP/2 or HTTP/3 protocol is used, the network function (NF) discovery and registration function needs to be considered instead of the interface management function for the 6G RAN-CN interface. As a result, whether the interface management function or NF discovery and registration function is newly defined is pending to the discussion on RAN-CN interface options.

Observation 4: Whether the interface management function or NF discovery and registration function is newly defined is pending to the discussion on RAN-CN interface options.

Next, the UE mobility management function should be supported to ensure the service continuity for the UE, thus providing better user experience. We think that even if the SBI is considered, the UE context needs to be managed by a single entity (e.g., 6G AMF) in 6G CN. That is, the UE-associated signaling which can be protected by the NAS security should be exchanged via the 6G AMF. Therefore, during the UE mobility, the mobility related signalings are exchanged between 6G RAN and 6G AMF even for the SBI option. That is, the UE mobility management function should be supported irrespective of the RAN-CN interface options.

Observation 5: The UE mobility management function needs to be supported irrespective of the RAN-CN interface options.

In 5G, there are two types of paging in NG-RAN: RAN paging, and CN paging. From the NG interface point of view, the 5G AMF sends the CN paging to the NG-RAN to trigger the state transition of the UE from the RRC_IDLE to the RRC_CONNECTED. Then, the NG-RAN sends the Uu Paging message towards the UE. For the UE in RRC_INACTIVE, the NG-RAN also sends the RAN paging to the UE upon the reception of the DL data from 5GC. Based on the observations from the 5GC, we think that the paging function are also supported in 6G as well. As mentioned above, the UE-associated signaling should be exchanged via the 6G AMF. Therefore, it is natural for the 6G AMF to send the paging message to the 6G RAN as in 5G.

Observation 6: The paging function needs to be supported in the RAN-CN interface.

Regarding the configuration transfer function, if the SBI option is adopted, the 6G RANs belonging to the same 6G CN can be connected via the service-based interface. In this case, there is no need to establish additional interface (e.g., Xn interface in 5G) between two 6G RANs. However, it is possible that two neighbour 6G RANs belong to different 6G CNs. In this scenario, the Xn-like interface may need to be established between these two 6G RANs. Therefore, the 6G RAN-CN interface needs to support the configuration transfer function for this scenario.

Observation 7: The RAN-CN interface needs to support the configuration transfer function to establish the Xn-like interface between two 6G RANs.

In order to capture the above observations, the following proposal is suggested to RAN3:

Proposal 2: It is proposed to capture the following functions into TR 38.760-3 [2]:

- UE mobility management: The functionality to manage the UE mobility for connected mode between the 6G RAN and 6G CN.

- Paging: The functionality to enable the 6G CN to generate Paging messages sent to the 6G RAN and to allow the 6G RAN to page the UE in RRC_IDLE state.

- Configuration Transfer: The functionality to transfer the 6G RAN configuration information (e.g. transport layer addresses for establishment of  Xn-like interface) between two 6G RAN nodes via 6G CN.

3.	Conclusion

In this contribution, we focused on the general principle and requirements on the RAN-CN interface, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to capture the following principles into TR 38.760-3 [2]:

- The 6G RAN-CN interface supports the cloud-friendly design.

- The 6G RAN-CN interface supports exposure of RAN information for new services.

- The 6G RAN-CN interface supports inter-system mobility between 5G system and 6G system.

Proposal 2: It is proposed to capture the following functions into TR 38.760-3 [2]:

- UE mobility management: The functionality to manage the UE mobility for connected mode between the 6G RAN and 6G CN.

- Paging: The functionality to enable the 6G CN to generate Paging messages sent to the 6G RAN and to allow the 6G RAN to page the UE in RRC_IDLE state.

- Configuration Transfer: The functionality to transfer the 6G RAN configuration information (e.g. transport layer addresses for establishment of  Xn-like interface) between two 6G RAN nodes via 6G CN.

4.	Reference

- R3-257324, “RAN-CN interface principles and functions”, Huawei, October, 2025.

- TR 38.760-3, “Study on 6G Radio RAN3 aspects”, v0.1.0.

- R3-258400, “Consideration on RAN-CN interface options”, LG Electronics, November, 2025.

- TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”, v14.0.0.

5.	Appendix: Text proposal to TR 38.760-3 [2]

This appendix provides the text proposal to TR 38.760-3 [2] based on the proposals of this contribution.

----------------Start of the First Change---------------

# 6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interfaces between RAN and CN.

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN;

-	the 6G RAN-CN interface supports the cloud-friendly design;

-	the 6G RAN-CN interface supports exposure of RAN information for new services;

-	the 6G RAN-CN interface supports inter-system mobility between 5G system and 6G system.

### 6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the 6G RAN and 6G CN;

-	Paging: The functionality to enable the 6G CN to generate Paging messages sent to the 6G RAN and to allow the 6G RAN to page the UE in RRC_IDLE state;

-	Configuration Transfer: The functionality to transfer the 6G RAN configuration information (e.g. transport layer addresses for establishment of  Xn-like interface) between two 6G RAN nodes via 6G CN.

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

-----------------End of the First Change---------------