3GPP TSG-RAN WG3 Meeting #130	R3-258400

Dallas, USA, 17th – 21st November 2025

Agenda Item:	10.3.2

Source: 				LG Electronics Inc.

Title:			Consideration on RAN-CN interface options

Document for:		Discussion and Decision

1.	Introduction

New 6G WG SID on Study on 6G Radio was agreed at RAN #108 [1]. This SID was updated in RAN #109 to reflect the agreement on the additional migration in [2]. One of the objectives in this SID is to study the 6G Radio Access Network architecture, interface protocols and procedures as Reader as follows:

… - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary … #### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. …

In this contribution, we will discuss the possible options for the RAN-CN interface, and provide our view on it.

2.	Discussion

As described in the 6G WG SID [2], two following options can be considered for the RAN-CN interface:

- Option 1: Point-to-point (P2P) interface,

- Option 2: Service-based interface (SBI).

The overall system architectures considering these two possible options are illustrated in Figure 1.

Figure 1: Possible options for RAN-CN interface in 6G

Option 1 has a similar concept with the NG interface in 5G. The 6G RAN may be directly connected to the 6G AMF in the 6G CN, thus resulting in the signaling processing at the 6G AMF. Therefore, if the P2P interface is adopted in 6G, additional signaling latency is foreseen because of redundant signalling processing at 6G AMF. This may be so critical issue for new service such as XR, AI/ML and sensing.

As described in TR 22.870 [3], compared with the previous generations (mainly providing connectivity services), 6G network is expected to be more versatile to provide more advanced services (e.g., Sensing, AI/ML, and etc). As mentioned in [4], therefore, the 6G RAN should advance toward a fully virtualized and modular design. It simplifies the introduction of a new services without impacting other services. In other words, this would allow RAN functions to be dynamically deployed at optimal locations based on network needs and real-time conditions. This means that the RAN-CN interface in 6G needs to be cloud-friendly to support the dynamic deployments of the RAN NFs within 6G RAN. However, the SCTP which is used for the current NG interface in 5G is far from cloud-native design. In 6G, since it is important to adopt flexible scaling, improved resource utilization, and the virtualization of RAN NFs, cloud-friendly protocol (e.g., QUIC) should be considered in the transport layer even if the P2P interface is adopted for 6G.

Regarding Option 1, however, we also assume that the 6G CN is evolved from 5GC considering the CAPEX of the operators. It is a critical burden to deploy a new 6G CN for the operators already having the 5GC. Therefore, it is reasonable and beneficial to evolve the existing 5GC to support the new services (e.g., Sensing, AI/ML, and etc) and to enable improved scalability and flexibility. Hence, it seems to be natural that the 6G AMF evolved from the 5G AMF is connected to the 6G RAN via the P2P interface. In addition, the legacy functionalities served to provide connectivity service in 5G should be supported in 6G as well. For example, we think that in 6G, the UE context is still stored at the single entity (i.e., 6G AMF) in 6G CN. Therefore, all NAS signaling can be integrity protected between UE and a single reference point (i.e., 6G AMF) in core network. From that point of view, it is reasonable to support the NG-like (i.e., P2P) interface in 6G.

According to 6G WG SID [2], considering the limited initial coverage and the need to minimize impact on user experience (e.g., QoS, QoE) during early 6G deployments, the interworking between 5G and 6G should be supported. If the P2P interface is selected for 6G, the interworking can be easily supported because a similar mechanism is already considered for the interworking between 4G and 5G. However, if the SBI is adopted for the RAN-CN interface, additional work may be required to convert the signaling for the P2P interface in 5G into the SBI signaling in 6G, and vice versa. As a result, it seems to be better to keep the P2P interface for the support of the connectivity services which are provided by 4G and 5G.

Observation 1: The P2P interface can be considered in 6G for the connectivity services supported by previous generations (e.g., 4G and 5G).

Another possible option is to introduce the service-based interface (SBI) between 6G RAN and each NF in 6G CN. The service-based interface has a cloud native design. The SBI enables scalable and flexible network deployments to meet various requirements for new services. In 5G, the SBI is already adopted in the core network. All NFs in 5GC are inter-connected via this SBI. Each NF exposes its NF services to other NFs. Then, this NF service can be requested by another NF to provide a certain NF service. It is also possible that the NF subscribes to NF service offered by another NF. Therefore, the signalling information between the NF in 6G RAN and the NF in 6G CN can be exchanged by SBI. As discussed above, the new services (e.g., AI/ML, Sensing) are considered in 6G. We think that if the service-based interface between 6G RAN and 6G CN is considered for these new services (i.e., beyond-connectivity services), it can take full advantage of cloud-based services. Also, as mentioned in [4], we think that for new services, the SBI option can easily support exposure of RAN information directly to the entity (e.g., 6G AF) in 6G CN, e.g., bypassing the other entity (e.g., 6G AMF) in 6G CN.

However, we have concern on this SBI option. For example, whenever new NF and NF service are defined to support new 6G service, the overall system operation or deployment complexity may be also increased. According to 5G core network, NRF and each NF in Core Network are tightly coupled due to service registration and discovery. We think that this problem can be occurred in 6G as well. Also, in P2P interface, we can predict specification impact based on lesson from 5G. However, in SBI option, critical specification efforts are required to define service-based interface between 6G RAN and each NF in 6G CN. Also, since RAN function has direct connection with each NF in 6G core network, we think that additional interoperability issue may be occurred, for example, to handle failure case for service registration and discovery.

Observation 2: The service-based interface can be considered in 6G for the beyond-connectivity services (e.g., Sensing, AI/ML) newly supported by 6G network.

As a result, we do not consider full service-based interface between RAN and CN. At least, the NG-like (i.e., P2P) interface needs to be supported for legacy functionalities served in 5G to provide the connectivity services. For new service such as AI/ML, sensing, our view is that service-based interface can be considered as a candidate option.

Proposal 1: It is proposed to support the P2P interface for the connectivity services which are provided by the previous generation (e.g., 4G, 5G).

Proposal 2: It is proposed to support the service-based interface for the beyond-connectivity services (e.g., Sensing, AI/ML) which are newly provided by 6G.

In order to capture the above observations and proposals, the following proposal is also suggested to RAN3:

Proposal 3: It is proposed to agree the corresponding TP for TR 38.8xx [5] in Appendix.

3.	Conclusion

In this contribution, we discussed the possible options for the RAN-CN interface, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to support the P2P interface for the connectivity services which are provided by the previous generation (e.g., 4G, 5G).

Proposal 2: It is proposed to support the service-based interface for the beyond-connectivity services (e.g., Sensing, AI/ML) which are newly provided by 6G.

Proposal 3: It is proposed to agree the corresponding TP for TR 38.8xx [5] in Appendix.

4.	Reference

- RP-251881, “SID: Study on 6G Radio”, RAN (NTT DOCOMO, CMCC, AT&T, Vodafone), June, 2025.

- RP-252912, “Revised SID: Study on 6G Radio”, RAN (NTT DOCOMO, CMCC, AT&T, Vodafone), September, 2025.

- TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”, v0.3.1.

- R3-258399, “General principles and requirements for 6G RAN-CN interface”, LG Electronics, November, 2025.

- TR 38.760-3, “Study on 6G Radio RAN3 aspects”, v0.1.0.

5.	Appendix: Text proposal to TR 38.760-3 [5]

This appendix provides the text proposal to TR 38.760-3 [5] based on the proposals of this contribution.

----------------Start of the First Change---------------

# 3	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP RP-252912: “New SID: Study on 6G Radio”

[3]	3GPP TR 38.914: “Study on 6G Scenarios and Requirements”

[x1]	3GPP	TR 22.870: “Study on 6G Use Cases and Service Requirements; Stage 1”

<<<<<< NEXT CHANGE >>>>>>

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

Two following options can be considered for the RAN-CN interface:

- Option 1: Point-to-point (P2P) interface,

- Option 2: Service-based interface (SBI).

The overall system architectures considering these two possible options are illustrated in Figure 6.1.3-1.

Figure 6.1.3-1: Possible options for RAN-CN interface in 6G

Option 1 has a similar concept with the NG interface in 5G. The 6G RAN may be directly connected to the 6G AMF in the 6G CN, thus resulting in the signaling processing at the 6G AMF. Therefore, if the P2P interface is adopted in 6G, additional signaling latency is foreseen because of redundant signalling processing at 6G AMF. This may be so critical issue for new service such as XR, AI/ML and sensing.

As described in TR 22.870 [x1], compared with the previous generations (mainly providing connectivity services), 6G network is expected to be more versatile to provide more advanced services (e.g., Sensing, AI/ML, and etc). Therefore, the 6G RAN should advance toward a fully virtualized and modular design. It simplifies the introduction of a new services without impacting other services. In other words, this would allow RAN functions to be dynamically deployed at optimal locations based on network needs and real-time conditions. This means that the RAN-CN interface in 6G needs to be cloud-friendly to support the dynamic deployments of the RAN NFs within 6G RAN. However, the SCTP which is used for the current NG interface in 5G is far from cloud-native design. In 6G, since it is important to adopt flexible scaling, improved resource utilization, and the virtualization of RAN NFs, cloud-friendly protocol (e.g., QUIC) should be considered in the transport layer even if the P2P interface is adopted for 6G.

Regarding Option 1, however, it is also assumed that the 6G CN is evolved from 5GC considering the CAPEX of the operators. It is a critical burden to deploy a new 6G CN for the operators already having the 5GC. Therefore, it is reasonable and beneficial to evolve the existing 5GC to support the new services (e.g., Sensing, AI/ML, and etc) and to enable improved scalability and flexibility. Hence, it seems to be natural that the 6G AMF evolved from the 5G AMF is connected to the 6G RAN via the P2P interface. In addition, the legacy functionalities served to provide connectivity service in 5G should be supported in 6G as well. For example, all NAS signaling can be integrity protected between UE and a single reference point in core network. From that point of view, it is reasonable to support the NG-like (i.e., P2P) interface in 6G.

According to 6G WG SID [2], considering the limited initial coverage and the need to minimize impact on user experience (e.g., QoS, QoE) during early 6G deployments, the interworking between 5G and 6G should be supported. If the P2P interface is selected for 6G, the interworking can be easily supported because a similar mechanism is already considered for the interworking between 4G and 5G. However, if the SBI is adopted for the RAN-CN interface, additional work may be required to convert the signaling for the P2P interface in 5G into the SBI signaling in 6G, and vice versa. As a result, it seems to be better to keep the P2P interface for the support of the connectivity services which are provided by 4G and 5G.

Another possible option is to introduce the service-based interface (SBI) between 6G RAN and each NF in 6G CN. The service-based interface has a cloud native design. The SBI enables scalable and flexible network deployments to meet various requirements for new services. In 5G, the SBI is already adopted in the core network. All NFs in 5GC are inter-connected via this SBI. Each NF exposes its NF services to other NFs. Then, this NF service can be requested by another NF to provide a certain NF service. It is also possible that the NF subscribes to NF service offered by another NF. Therefore, the signalling information between the NF in 6G RAN and the NF in 6G CN can be exchanged by SBI. Note that the new services (e.g., AI/ML, Sensing) are considered in 6G. If the service-based interface between 6G RAN and 6G CN is considered for these new services (i.e., beyond-connectivity services), it can take full advantage of cloud-based services. Also, for new services, the SBI option can easily support exposure of RAN information directly to the entity (e.g., 6G AF) in 6G CN, e.g., bypassing the other entity (e.g., 6G AMF) in 6G CN.

However, there are some concerns on this SBI option. For example, whenever new NF and NF service are defined to support new 6G service, the overall system operation or deployment complexity may be also increased. According to 5G core network, NRF and each NF in Core Network are tightly coupled due to service registration and discovery. This problem can be occurred in 6G as well. Also, in P2P interface, the specification impact is predictable based on lesson from 5G. However, in SBI option, critical specification efforts are required to define service-based interface between 6G RAN and each NF in 6G CN. Also, since RAN function has direct connection with each NF in 6G core network, additional interoperability issue may be occurred, for example, to handle failure case for service registration and discovery.

### 6.1.4	Evaluation of RAN-CN Interface Options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.

-----------------End of the First Change---------------