3GPP TSG-RAN WG3 #130																	R3-258547

Dallas, U.S.A., 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1663 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for gNB-DU Configuration Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, InterDigital, Fibercop |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A first issue is that the procedure text for gNB-DU Configuration Update procedure is not accurate. It does not explicitly mention that when a predicted CCO issue is cancelled, the future coverage modification should be cancelled for all the relevant cells and beams. Instead, the specification refers to a single instance of the Future Coverage Modification Cause IE which is associated to a single cell. A second issue is that the gNB-CU behaviour is unclear if the gNB-CU receives from the gNB-DU a Future Coverage Modification Notification IE with some instances of the Future Coverage Modification Cause IE set to “cancel” (for some cells) and some other instances of the Future Coverage Modification Cause IE not set to “cancel” (for some other cells). |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the first issue, clarify that gNB-CU shall, if supported, consider that the gNB-DU has cancelled the future coverage modifications indicated for the cells and beams listed in the Future Coverage Modification Notification IE, only if all the instances of the Future Coverage Modification Cause IE are set to “cancel”. For the second issue, add as abnormal condition the case when some of the instances of the Future Coverage Modification Cause IE are set to “cancel”, while some other instances are set to values different from "cancel". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification text in case of cancelling a predicted CCO issue is inaccurate. Also, the CU behavior is unclear if the CU receives from the DU a Future Coverage Modification Notification IE with some instances of the Future Coverage Modification Cause IE set to “cancel” and some other instances of the Future Coverage Modification Cause IE not set to “cancel”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

** skip unchanged **

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message and if all the instances of the Future Coverage Modification Cause IE are set to “cancel”, the gNB-CU shall, if supported, consider it as a notification that the gNB-DU has cancelled the future coverage modifications indicated for the cell(s) and optionally beam(s) listed in the Future Coverage Modification Notification IE, as specified in TS38.401[4].

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message and if some of the instances of the Future Coverage Modification Cause IE are set to “cancel”, while some other instances are set to values different from "cancel", the gNB-CU shall ignore the Future Coverage Modification Notification IE and fail the gNB-DU Configuration Update procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>