3GPP TSG RAN WG3 Meeting #130	             						R3-258564

Dallas, USA, Nov. 17th –21st, 2025

Agenda Item:	10.2.2

Source:	AT&T

Title:	Data Framework Considerations for 6G

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-252912 [1]. The following objectives are of relevance to the AI/ML framework and corresponding data collection in 6G [1]:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we discuss the requirements and the main design principles for enabling an extensible data management framework in 6G.

# Requirements for an integrated data framework for 6G

6G is expected to natively enable a broad range of use cases and requirements, including new services that require support for various types of data, including AI/ML data collection and management. One of the cornerstones to achieve the 6G design goals is to establish an integrated framework for data collection to enable MNOs to effectively manage network capacity, foster innovation, and unlock new value opportunities. The following core principles should be followed for such an integrated framework:

- A unified data collection framework across domains to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 1: An integrated data framework that includes AI/ML data collection and data management, with MNO control and visibility, is supported in 6G.

To ensure scalability and availability of data when and where it is needed, a unified data collection framework especially entails unification of data collection and management framework across the network domains, including RAN, CN and OAM. To that end, it is important, and well captured by the RAN SI, to have close collaboration between all relevant 3GPP working groups, including RAN2 and RAN3 with the relevant SA working groups, to avoid any fragmentation that would hinder extensibility and future integration.

Proposal 2: Study and specify an integrated 6G data management and collection framework in close collaboration between RAN2, RAN3, SA2 and SA5.

In line with the requirements set by the RAN plenary, and proposal 2, RAN2#131bis started discussing requirements for a data collection framework in 6G. The following agreement was made in RAN2#131bis [2]

Agreements

1. 	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

- These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.    Study model transfer/delivery requirements/functions

It is timely for RAN3 to start discussing a unified framework for data collection and management for 6G, in tandem with RAN2. Such a framework can focus on the AI/ML data collection framework as a starting point, and other data types such as for sensing, etc. can be included as the requirements on the data and data types for these new services further crystallize.

Furthermore, as we identify AI/ML use cases for 6G, in RAN3 as well as RAN1 and RAN2 and as we reflect on the 5G-A specifications for the different use cases, we believe it is crucial to study and specify a data collection and data management framework that avoids fragmentation and per-use case specification.  This framework should also ensure scalability to new use cases and new services as needed. RAN3 should thus start studying a unified data framework for 6G in parallel with the AI/ML use cases and corresponding requirements.

The data framework allows for an efficient reuse of the collected data across use cases, thus improving network efficiency and reducing overhead. It further enables better observability into the network and unlocks better opportunities for network automation.

Observation 1: Unlike 5G-A, a unified data framework improves network efficiency, reduces overhead of data collection across use cases, and unlocks better opportunities for network automation.

RAN2#131bis also agreed to study termination points for the data collected, with better understanding on the consumer(s) and producer(s) of the data.

We expect AI/ML models to be embedded in different layers in the network L1/L2/L3, in different domains RAN/UE/CN/OAM, and for these use cases to operate efficiency in the network, with MNO visibility and control over the data collection, it is important to support multiple termination points for the data within the MNO network, as part of the data framework. Data collected can be made available at the OAM, RAN and/or CN, to be used when and where needed.

Proposal 3: The integrated 6G data framework supports multiple termination points for data collection, e.g. in the RAN, OAM and CN.

Data collection and management, for AI/ML, encompasses data collection for training, performance monitoring as well as model transfer, both at the UE side and the network side. This will allow all AI/ML related aspects to be managed from within a new management plane, or data plane, which enables us to onboard use cases quickly and efficiently in 6G.

Proposal 4: The integrated 6G data framework encompasses data collection and management for AI/ML models across different domains (UE, RAN, etc.), including UE-sided data collection, network-sided data collection, performance monitoring, and model transfer.

The data collection framework should be designed to cater to the properties of the data being collected and used. This data is different from the user plane or the control plane traffic and should be specified as such.

Proposal 5: The integrated 6G data collection and management framework is designed to differentiate the data plane traffic from the user plane and control plane traffic.

The new data collection framework or newly designed data plane allows for much better data management as the network can collect data for multiple use cases simultaneously improving efficiency. Furthermore, the network can add relevant additional information and labels to the data. The network can also ensure that the privacy is preserved, and proprietary and security aspects of the data are properly addressed before the data is shared with the network or UE vendors. Additionally, it gives network control over when to collect data or transfer models so that there is minimal impact on system performance. Finally, the integrated data collection framework is extensible to be used to quickly develop and deploy AI/ML use cases based on the network requirements, reducing the standardization effort to onboard new use cases.

As we develop the new data plane framework, and we allow for multiple termination points for the data collection in the MNO network across multiple domains, a service-based design can be considered whereas data producers and data consumers are created to allow for the data to be available and consumed when and where it is needed, in a flexible and scalable manner.  This can naturally yield itself to a service-based design of the interface between the RAN and the CN, especially when considering the RAN for AI and sensing use cases within the data collection framework. The pTp NG interface can potentially also be enhanced to transport the data plane traffic as needed from the RAN to the CN.

Proposal 6: For the integrated 6G data framework, enhancements to the RAN-CN interface should be studied.

# Conclusion

In this contribution, we proposed a data management framework for 6G, and the corresponding requirements. We made the following observation and proposals.

Observation 1: Unlike 5G-A, a unified data framework improves network efficiency, reduces overhead of data collection across use cases, and unlocks better opportunities for network automation.

Proposal 1: An integrated data management framework that includes AI/ML data collection and data management, with MNO control and visibility, is supported in 6G.

Proposal 2: Study and specify an integrated 6G data framework in close collaboration between RAN2, RAN3, SA2 and SA5.

Proposal 3: The integrated 6G data framework supports multiple termination points for data collection, e.g. in the RAN, OAM and CN.

Proposal 4: The integrated 6G data framework encompasses data collection and management for AI/ML models across different domains (UE, RAN, etc.), including UE-sided data collection, network-sided data collection, performance monitoring, and model transfer.

Proposal 5: The integrated 6G data collection and management framework is designed to differentiate the data plane traffic from the user plane and control plane traffic.

Proposal 6: For the integrated 6G data framework, enhancements to the RAN-CN interface should be studied.

# References

[1] RP-252912, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic

[2] RAN2 131bis Chair notes, Prague, Czech Republic