3GPP TSG-RAN WG3 Meeting #130	R3-258567

Dallas, TX, USA, 17th – 21st November 2025

Title: 	General Principles and Requirements for the 6G RAN architecture

Source: 	Ericsson, Jio Platforms

Agenda item:	10.2.1

Document Type:	Discussion and agreement

# 1.	Introduction

During RAN3#129-bis RAN3 has discussed general principles and requirements for the 6G RAN.

The discussion has been driven by parsing through an initial set of requirements and principles reflecting some of the views exposed in the submitted papers. From these discussions the following requirements and principles were agreed to be captured in TR 38.760-3 (see [3]):

# 5 Objectives and requirements Editor’s note: The detailed objectives of the study are: Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] ## 5.1 General Principles All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles. The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS] ## 5.2 Deployment Scenarios Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914. The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914. - FFS on the implications of this requirement on 6G RAN architecture. - FFS whether all deployment scenarios of TR 38.914 can be supported.

At the same time the following topics were agreed as “to be continued”:

Study whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G.

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

In this paper we bring to the discussions the requirements that are in our opinion important for the correct development of the 6G RAN and we follow up on the requirements agreed during the last RAN3 meeting.

We then touch upon the open points captured during the last RAN3 meeting, providing our initial opinions about them.

# 2.	Discussion

## 2.1	Basic Requirements and Principles for the 6G RAN

Below are the principles and requirements we consider fundamental for the correct standardisation of the 6G RAN.

The 6G RAN architecture shall enable efficient support of RAN-UE communication as well as flexible and effective communication within the RAN and between RAN entities.

One of the fundamental principles established both by the SID in [1] and by the requirements agreed in [2] is that the 6G RAN shall be based on a “stand-alone” architecture. “Stand-alone” is a term derived during the course of 5G specifications and that, in the context of 5G, indicates a network that does not interwork with any other RAT.

In the context of 6G, “stand-alone” shall be interpreted in a similar way as for 5G, with the addition of important design principles that would enable to greatly enhance the 6G RAN performance.

One of the main design blocks we think a standalone 6G RAN should support is the ability to maintain functionalities for the configuration and handling of a UE in a single logical entity. We have seen times and times again in 5G how complexity increases when the configuration of a UE has to be negotiated between multiple logical nodes. Each logical node would select configuration options suited to its own capabilities and to the capabilities available at the UE. The result is often a sub-optimal configuration, which impacts the overall performance with which the UE can be served.

Additionally, coordination of UE configuration and handling procedures between different logical entities implies delays in exchanging information, processing, and converging to a viable configuration option. Such delays greatly impact performance, as they slow down the activation of optimised data transfer options. In a communication world where data is generated in high bursts for very short time durations this is a very limiting factor.

We would therefore like to propose the following principle:

Proposal 1: Capture in the “General Principles” section of TR 38.760-3 that “handling of a UE context is under the responsibility of a single RAN logical entity”.

And as a consequence:

Proposal 2: Capture in the “General Principles” section of TR 38.760-3 that “handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity”.

Another important aspect we would like to focus on is legacy features already supported in 5G and to be supported in 6G and how these would be supported between the 6G RAN and the Core Network for the 6G system. 
We assume that such legacy features to be supported in 6G will be derived from features supported in 5G. Undoubtedly, enhancements to such features would be appropriate, but at the same time the strength of legacy 5G design should be exploited and reused. This would also ensure prompt feature parity between 5G and 6G. For this reason, we believe that the well-established feature framework available for legacy 5G features should be maintained.

For this reason, we would like to propose the following principle:

Proposal 3: Capture in the “General Principles” section of TR38.760-3 that “for 5G legacy features already supported in 5G and to be supported in 6G, the feature design is based on the RAN-CN functional split and interactions established for the 5G RAN”.

Continuing on principles that would guide the design of the overall 6G RAN, it would be useful for RAN3 to agree on basic guidelines on how the RAN interfaces will be designed. There are several aspects of the RAN interface design that could be considered, but one fundamental point is how such interface application protocols will be encoded.

An obvious choice would be to select ASN.1 as the way to encode AP messages over the RAN interfaces. This would provide continuity with respect to how interfaces were defined in previous RAN generations.

It is worth noting that RAN2 has had a similar discussion and they have taken the following agreement:

Agreements - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

Indeed, ASN.1 has proven to be one of the most (if not the most) compact encoding method available for network interface procedures. Choosing the ASN.1 encoding for RAN interfaces would ensure alignment with the RRC encoding. This could provide advantages when RRC IEs are exchanged between RAN nodes as part of an AP message. Indeed, if the content of the RRC IE needs to be understood at the function that terminates the AP message, the advantage would be that such function could use an ASN.1 decoder to decode both the AP message and the RRC IE. On the contrary, if the AP message is encoded in a way different than the ASN.1, the function terminating the AP message would need to support two decoders (ASN.1 and AP message decoder).

Yet another reason to choose the ASN.1 would be that, if we need communication between the 6G system and the 5G system, we would have to comply with the fact that the 5G system supports ASN.1 encoding. Therefore, if messages are ASN.1 encoded from 6G towards 5G, then 5G would be able to decode them. Likewise, any message encoded by 5G towards 6G would be decodable at the 6G RAN node. However, if 6G uses a different encoding mechanism than 5G, a message translation function would be needed when messages are exchanged between 5G and 6G. As an example, any CN based handover messages would need to be “translated” between 5G and 6G.

Proposal 4: Capture in the “General Principles” section of TR38.760-3 that “RAN terminated interfaces support application protocols that are encoded with ASN.1”.

## 2.2	6G RAN requirements on virtualisation and cloud-based support

The following principle was agreed to be captured in TR38.760-3:

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

Virtualised RAN and cloud-based RAN are deployment options. We note that these deployment options are possible already today in 5G, where products are offered by most vendors on the market to support such frameworks.

Observation 1: Virtualised RAN and Cloud Based RAN are already possible deployment options in 5G, hence they can be achieved already today by means of the 5G standard framework

While the 5G design offers a number of benefits that we should reuse in 6G, we do acknowledge that improvements could be made to how the RAN has been specified in 5G to achieve optimal support of these deployment options.

In that respect, we believe that a modular design of RAN functions, protocols and interfaces is key to achieve virtualisation. Virtualization is an increasingly used tool to create additional instances of a function (or module) in a data centre to distribute load across additional CPUs or servers.

The possibility to re-start individual processes or to execute different software versions in parallel is vital to minimize the impact on end users and to avoid that, e.g. failures affect all cells in a geographic area.

To support a virtualised or a cloud-based RAN environment, a logical entity (e.g. AMF, neighbour 6G RAN node) that communicates with a 6G RAN node should not be required to know the internal structure of the 6G RAN node. Instead, to support such deployment options, the logical entity should be able to signal an application protocol message to the 6G RAN node at its known IP address(s) and port number(s) and it should have a guarantee that the AP message is successfully delivered at the instance of the function where its content is meant to be used.

Therefore, to enhance support of virtualised and cloud-based RAN, the standard should be based on a design of functions/protocols and interfaces that is modular, so that such “modules” can be virtualised, instantiated and distributed. Such design does not necessarily imply a departure from the general architecture framework followed in 5G. Indeed, the interface design in 5G has a number of positive aspects that should be reused in 6G, but it could be enhanced to lead to an optimised support of virtualisation and cloud-based deployments.
Maintaining the 5G design as baseline could also facilitate the reuse of large parts of the application protocol logic developed and deployed during 5G, while applying enhancements that would address the requirements captured by RAN3.

For this reason, we would like to propose the following principle, to be captured in TR38.760-3:

Proposal 5: Capture in the “General Principles” section of TR38.760-3 that the 6G RAN design should be based on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes while reusing as much as possible the advantages of the application protocol logic developed and deployed for 5G.

## 2.3	Open points raised so far

In this section we provide an initial evaluation of the following points:

Study whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G.

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

Concerning the first point on how to evolve the protocol stack of RAN defined interfaces, we believe this is an interesting study area that opens a discussion on how to replace protocols at any layer of the interface stack according to identified needs.

While the purpose of this proposal may lead to benefits, we note that there could be two cases of replacing a protocol in an interface protocol stack:

- The new protocol has the same functionalities as the old protocol

- The new protocol has different functionalities as the old protocol

In option 1) swapping the old protocol for the new protocol should be possible at standardisation level without minimum or no impact. 
This is because the assumption is that the interface application protocols defined by RAN3 would not be affected. This is due to a one to one mapping of functionalities between the old protocol and the new protocol, leading to a “transparent swap” of protocols from a standard point of view.

In option 2) the new protocol has different functionalities with respect to the old protocol. In this case replacing the old protocol with the new will have to be carefully analysed and impacts on the interface application protocol may be incurred. The reason is that a change of functionalities in the protocol stack may derive in new demands on the functionalities supported at interface application protocol level.

Therefore, we conclude that the protocol stack for the interfaces defined by RAN3 could evolve in time, but changes to the stack will have to be evaluated case by case to determine the impact of such changes and whether they are feasible/viable.

Conclusion 1: The protocol stack for the interfaces defined by RAN3 could evolve in time, but changes to the stack will have to be evaluated case-by-case to determine the impact of such changes and whether they are feasible/viable.

Concerning the open point on service awareness, we believe that “service awareness” should first be clearly defined. It is currently not clear what is meant for “service”. For example, if “service” was intended as an application level service, it could be argued that there are very large numbers of such services (enabled by, e.g. mobile applications). It would therefore be questionable whether making the RAN aware of these types of services is scalable at all.

Conclusion 2: Before the impact of enhanced RAN-based service awareness on 6G RAN architecture can be evaluated, there should be a clear definition of what is intended for “services” and for “service awareness”.

# 3.	Conclusions

In this paper we have discussed the new principles introduced at the last RAN3 meeting for the 6G RAN, as well as overall principles for the design and standardisation work needed to derive a sound 6G RAN specification.

Additionally, we have provided an initial analysis of the open point captured during RAN3-129bis discussions.

The following proposals and conclusions have been derived and are reported below:

Proposal 1: Capture in the “General Principles” section of the RAN3 TR for 6G that “handling of a UE context is under the responsibility of a single logical entity”.

Proposal 2: Capture in the “General Principles” section of the RAN3 TR for 6G that “handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity”.

Proposal 3: Capture in the “General Principles” section of the RAN3 TR for 6G that “for 5G legacy features already supported in 5G and to be supported in 6G, the feature design is based on the RAN-CN functional split and interactions established for the 5G RAN”.

Proposal 4: Capture in the “General Principles” section of TR38.760-3 that “RAN terminated interfaces support application protocols that are encoded with ASN.1”.

Observation 1: Virtualised RAN and Cloud Based RAN are already possible deployment options in 5G, hence they can be achieved already today by means of the 5G standard framework.

Proposal 5: Capture in the “General Principles” section of TR38.760-3 that the 6G RAN design should be based on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes while reusing as much as possible the advantages of the application protocol logic developed and deployed for 5G

Conclusion 1: The protocol stack for the interfaces defined by RAN3 could evolve in time, but changes to the stack will have to be evaluated case-by-case to determine the impact of such changes and whether they are feasible/viable.

Conclusion 2: Before the impact of enhanced RAN-based service awareness on 6G RAN architecture can be evaluated, there should be a clear definition of what is intended for “services” and for “service awareness”.

It is proposed to agree to the principles above and to capture them in the RAN3 6G TR 38.760-3, as proposed in the Annex.

# 4.	References

[1] RP-252912, New SID: Study on 6G Radio

[2] RP-252870, pCR for 38.914 on Requirements for architecture and migration for 6G, NTT DOCOMO (moderator)

[3] R3-257323, [TP for draft TR 38.760-3] General principles and requirements, Qualcomm

# 5.	Annex: pCR for TR 38.760-3

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

The following principles should be fulfilled to achieve standardisation of the 6G RAN:

-	Handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity.

-	Handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity

-	For 5G legacy features already supported in 5G and to be supported in 6G, base the feature design on the RAN-CN functional split and interactions established for the 5G RAN

-	The 6G RAN design is based on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes while reusing as much as possible the advantages of the application protocol logic developed and deployed for 5G.