3GPP TSG-RAN WG3 Meeting #130	R3-258683

Dallas, US, 17 – 21 November 2025

Agenda Item:	13.3

Source: 	CEWiT

Title: 	Discussion paper on ISAC procedures and signaling

Document for:	Discussion and Decision

# Introduction

RAN#108 agreed study on ISAC which is updated in the RAN#109 [1]. The study is divided between the evaluation of the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case and study the procedures, signalling between RAN and CN to support ISAC along with network architecture for gNB-based mono-static sensing for UAV sensing target use cases. The latter objective is relevant from RAN 3 perspectives based on the RAN 1 input for the prior objective. This relevant objective regarding for ISAC from RAN 3 perspective is as below,

Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Further a stage 1 work on ISAC has been completed by SA1 including the use cases and requirement aspects for ISAC. TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401). In Release 19, a channel modelling framework for ISAC to enable evaluation of sensing techniques for such use cases was added to the existing communication models in TR38.901.

In the last meeting of RAN 3 a pCR to 38.765 [2] is agreed TP providing the call flow for sensing reporting for sensing. The pCR text is the following.

## 8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

Further the agreed guidance for this meeting is to “To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.”

Considering these aspects and the defined objective, this contribution highlights necessary aspects of the procedure and signalling for ISAC considering mono static sensing as baseline but can be extended to bistatic sensing too.

# ISAC Procedures and signaling

ISAC or sensing is new service expected to be supported to detect a passive object(s) in the area of interest. SA2 is working on the different key issues to define the sensing architecture, sensing function, sensing entity, discovery of sensing functions & sensing entity, sensing data collection, sensing parameters configuration provisioning and sensing result exposure. Whereas RAN 1 is working on the physical layer parameter evaluation for sensing purpose. Further the scope of the study limits to mono static sensing for detection and/or tracking of Uncrewed Aerial Vehicles (UAV). It is agreed to extend the same framework to bistatic sensing too without specific enhancement related to it.

SA2 is discussing their conclusion under key issue 4 “Sensing Data and the Associated Information Collection and Transport” sensing data collection mechanism and involved sensing nodes/functions. Interim agreements are captured in TR 23.700-14 section 7.1.4.

TR 23.700-14 7.1.4 The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one-time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

These initial agreements indicate that to enable sensing in gNB (NG-RAN) (being sensing entity) it is necessary to define the procedure for sensing control information sharing, sensing configuration sharing and sensing data reporting.

Proposal 1: RAN3 should study the sensing procedure with respect to sensing control information sharing, sensing assistance sharing and sensing data reporting.

Initial agreement in the last meeting agreed for basic control signaling viz. sensing request and sensing response. Before the sensing request SF may need some sensing assistance information including sensing configuration information like reference singal configuration, timing synchronization information etc and TRP information. This configuration information exchange should be added before sensing request.

Figure 1: Message flow for sensing reporting

Proposal 2: Sensing signalling should include sensing assistance information, including sensing configuration information and TRP information before sensing request message.

Further the sensing processing functionality will include acquiring the sensing data at the TRP(s) and processing the sensing data to obtain the sensing measurements and/or sensing results which will be reported to sensing function (SF). The sensing data related study is going on in the RAN 1. RAN 3 should be aware of the sensing metrics, measurements, and relevant measurement quantization which RAN 1 studying to clearly define the sensing functionality and procedures. Particularly, it is necessary for RAN 3 to know whether gNB or SF is generating final results and what type of sensing measurements are expected to be generated in the gNB. To clarify this, RAN 3 can send LS to RAN asking the particular clarifications on what level of processing on sensing data is expected at gNB.

Proposal 3: RAN 3 should study the sensing result generation capability of gNB in terms of intermediate sensing results or final sensing results that gNB can generate. RAN 3 should send LS to RAN 1 asking clarification on this.

# Conclusion

This contribution provides the network architecture based on SA2 architecture related discussions. The contribution provides following proposals for the discussion.

Proposal 1: RAN3 should study the sensing procedure with respect to sensing control information sharing, sensing assistance sharing and sensing data reporting.

Proposal2: Sensing signalling should include sensing assistance information, including sensing configuration information and TRP information before sensing request message.

Proposal 3: RAN 3 should study the sensing result generation capability of gNB in terms of intermediate sensing results or final sensing results that gNB can generate. RAN 3 should send LS to RAN 1asking clarification on this.

# References

[1] RP-252819: Revised SID: Study on Integrated Sensing and Communication (ISAC) for NR

[2] R3-257326 (TP to BL pCR to 38.765) Basic sensing call flow

[3]TR 22.837 : Feasibility Study on Integrated Sensing and Communication (Stage 1)

[4] TS 22.137 : Service requirements for Integrated Sensing and Communication.

Stage 1 (Release 19)

[5] TR 38.901 : Study on channel model for frequencies from 0.5 to 100 GHz  (Release 19)

[6] TS 23.502 : Procedures for the 5G System (5GS)

# Annex – TP for TR 38.765

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1. The SF request for the sensing assistance information from the gNB which include TRPs information of gNB and sensing configuration information.

2.	The SF sends sensing request to the gNB.

3.	The gNB sends sensing response to the SF.

4.	If requested in Step 1, the gNB sends sensing report to the SF.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>