3GPP TSG-RAN WG3 Meeting #130	R3-258829

Dallas, US, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0517 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of BH-gNB and WAB-MT for WAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, CATT, Nokia, Nokia Shanghai Bell, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
| ss | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the initial access procedure, the WAB-MT and its co-located WAB-gNB may connect to different SNPN or same SNPN as specified in TS23.501. Additionally, to align with clause12.1 and TS38.423, the WAB-gNB can establish an Xn connection with neighbouring “NG-RAN node(s)” rather than “gNBs”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To support the WAB-MT and its co-located WAB-gNB may connect to different SNPN or same SNPN - To use “NG-RAN node(s)” instead of “gNBs” in clause 12.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support the deployment using SNPN, and Xn between WAB-gNB and ng-eNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.7, 12.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: remove the first and second change in R3-258571 Rev2: add back the accepted removal of ”gNB” |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

6.1.7	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-gNB. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN/SNPN or to different PLMNs/SNPNs.

Figure 6.1.7-1 shows the WAB architecture for 5GS.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

12.8	Xn connection management

A WAB-gNB can establish an Xn connection with the BH-gNB serving the co-located WAB-MT, and with the neighbouring NG-RAN node(s). During the setup or update of its Xn connections, the WAB-gNB can include an ID of the WAB-MT, to indicate that it is a WAB-gNB. In case the peer gNB is the WAB-MT’s BH-gNB, the WAB-MT ID makes the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB, and the cell ID of BH-gNB´s cell serving the WAB-MT.

Establishment of Xn connections between two WAB-gNBs can be avoided. To achieve this, the WAB-gNB may reject the Xn setup initiated by another WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.

A WAB-gNB should be configurable with respect to whether it should accept or reject Xn setup requests received from WAB-gNBs.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////