3GPP TSG-RAN WG3 Meeting #130	R3-258763

Dallas, TX, USA, November 17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of WAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, Qualcomm, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
| ss | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | -The current version of the specification states that a WAB-MT is served by a stationary gNB, which is not always the case, for example when the WAB-MT is served by a non-terrestrial link where the BH-gNB is onboard a non-geostationary satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Removed the adjective “stationary” from the WAB clause 4.11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect statement in the Wireless Access Backhaul specification. Impact assessment towards the previous version of the specification (same release): Isolated impact. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------First change-------------------------------------------

## 4.11	Wireless Access Backhaul

Wireless Access Backhaul (WAB) is supported by means of the WAB-node, which provides NR access links to UEs via a gNB functionality, referred to as the WAB-gNB. The WAB-node further supports a WAB-MT function, which terminates an NR Uu interface to a gNB that is used for the backhauling of the WAB-gNB’s traffic. The WAB-node may be stationary or mobile.

Further details of WAB are defined in TS 23.501 [3] and TS 38.401 [4].

-------------------------------------------End of changes-------------------------------------------