3GPP TSG-RAN WG3 Meeting #73bis	 R2-112644

# Zhuhai, China, 10-14 October 2011

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | CRNum | rev | - | Current version: | 10.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSFB procedure for UEs with LIPA PDN connections only in 36.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Research In Motion UK Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEL10¸LTE-L23 |  |  |  |  |  |  | Date: |  | 10/08/2011 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | In release 10 EPC networks, LIPA PDN connections for a UE shall be deactivated if the UE moves out of the original HeNB coverage. If a UE is connected to a HeNB, CSFB procedures for MO/MT call establishment cause a UE to move out of the HeNB cell, thus upon MO/MT call establishment any LIPA PDN connections belonging to the UE and established while connected to the HeNB need to be deactivated. It is proposed that when a HeNB detects the UE has correlation Ids associated with each of its contexts, the HeNB performs CSFB options “RRC Connection Release” or “cell change”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If HeNB determines that the UE has only LIPA PDN connection(s) before triggering the PS handover, then the HeNB shall avoid performing the PS HO for CSFB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE with only LIPA PDN connection(s) is detached for PS services and CSFB will fail, thus leading the UE to not being able to establish the CS service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.6.5	Support of LIPA with HeNB

Figure 4.6.5-1 shows the logical architecture for the HeNB when it supports the LIPA function.

Figure 4.6.5-1: E-UTRAN - HeNB operating in LIPA mode - Logical Architecture

For a LIPA PDN connection, the HeNB sets up and maintains an S5 connection to the EPC.

The S5 interface does not go via the HeNB GW, even when present.

The HeNB may reuse the IP address used for S1 interface for this S5 interface in order to reuse the S1 IPSEC tunnel or it may also use another IP address which would result in the establishment of another IPSEC tunnel.

The mobility of the LIPA PDN connection is not supported in this release of the specification. The LIPA connection is always released at outgoing handover as described in TS23.401 [17]. The L-GW function in the HeNB triggers this release over the S5 interface. In addition, in case of CS fallback, if all connections of a UE are LIPA connections, the HeNB:

-	shall use the cell change order CS fallback option or the RRC Connection Release with Redirection CS fallback option (see subclause 10.2.5) for CS fallback to UMTS or CS fallback to GSM,

-	for CS fallback to 1xRTT (see subclause 10.3.2.3.3), CS fallback to 1xRTT shall be executed according to Table 10.3.2.3.3-1 without concurrent HRPD handover option.

In case of LIPA support, the HeNB supports the following additional functions, regardless of the presence of a HeNB GW:

-	transfer of the collocated L-GW IP address of the HeNB over S1-MME to the EPC at every idle-active transition,

-	transfer of the collocated L-GW IP address of the HeNB over S1-MME to the EPC within every Uplink NAS Transport procedure,

-	support of basic P-GW functions in the collocated L-GW function such as support of the SGi interface corresponding to LIPA,

-	additional support of first packet sending, buffering of subsequent packets, internal direct L-GW - HeNB user path management and in sequence packet delivery to the UE,

-	support of the necessary restricted set of S5 procedures corresponding to the strict support of LIPA function as specified in TS 23.401 [17],

-	notification to the EPC of the collocated L-GW function uplink TEIDs for the LIPA bearers over S5 interface within the restricted set of procedures to be forwarded over S1-MME and further used by the HeNB as "correlation id" for correlation purposes between the collocated L-GW function and the HeNB,

-	in case of outgoing handover triggering the L-GW function to release the LIPA PDN connection and only handing over the non-LIPA E-RABs.

In case of LIPA support, the MME may support the following additional functions:

-	verification of UE authorization to request LIPA activation for the requested APN at this CSG and transfer of the received collocated L-GW IP address,

-	transfer of the "correlation id" i.e. collocated L-GW uplink TEID to the HeNB within the UE context setup procedure and E-RAB setup procedure,

-	verification of whether the LIPA PDN connection has been released during the handover preparation procedure over S1,

-	deactivation of the LIPA PDN connection of an idle-mode UE if it detects that the UE has moved out of the coverage area of the HeNB collocated with L-GW function, as specified in TS 23.401 [17].