3GPP TSG-RAN WG3 #75bis	R3-120583

26-30 March, 2012

San Jose del Cabo, Mexico

Agenda Item:	12.2

Source: 	DAC-UPC (Universitat Politècnica de Catalunya)

Title: 	Solution for the additional carrier selection for pico eNB

Document for:	Discussion and Approval

# Introduction

According the agreement at RAN3#75, the scenario in which a pico cell shares one or more carriers with other pico cells or with a macro cell is considered for the CB-ICIC WI. Further, in the email discussion has been agreed the DAC-UPC proposed a solution for the additional carrier selection by the single carrier pico eNB. In this contribution we provide details of the solution (Solution 5 in ‎[1]) and also the proposed changes to X2 interface protocols.

With this contribution we bring the pricing algorithms presented in [2]…[5] at the level that the needed measurements can be executed and reported by a legacy UE using the CQI and RSSI measurements already defined in 3GPP standards. The CQI degradation caused by a specific interferer can be measured within the frequency domain, at carrier or subband level, or in time domain, using a pattern of the measurement subframes.

Another important aspect resolved by the pricing algorithm applied to the additional or basic carrier selection is a clear criteria for determining which additional carrier should be selected and which is the criteria for switching on or off an additional carrier.

The contribution is organized as follows: In Section 2 we present the general solution, in Section 3 we present the summary of the measurement of the CQI degradation due to a specific eNB, in Section 4 we present the criteria for the additional carrier selection, in Section 5 we give details on the new messages to be used, in Section 6 we provide simulation results. Concluding remarks are presented in Section 7.

# General solution

The steps of the proposed solution are as follows:

- The single carrier pico eNB is assigned initially by the OAM with the initial operational carrier and a set of possible operational carriers to be eventually used.

- If the load on the used carriers requires the addition of a new carrier, the problem to be resolved is to select this carrier such that will create minimum and acceptable interference to neighbour eNBs. The eNB wishing to add a new carrier transmits data on a time/frequency resource to be announced by X2 messages. There is no need to actually serve UEs on these carriers, but is only needed to activate the carriers and transmit information (for example to un-valid UE identifiers).

- One or more UEs served by strongly interfered eNBs evaluate the CQI, while the new candidate carriers are transmitting data.

- The influence of the new activated carrier on CQI (cost) can be evaluated by each UE served by a victim eNB by subtracting the CQI values measured with the new carrier from the CQI values measured before the new carrier were activated.

- The eNB receiving the reports of the served UEs, function of the own available capacity and the actual offered traffic, will decide to transmit or not the measurement result (cost) to the multi-carrier pico eNB. For transmitting the cost to other eNBs it is needed a new X2AP message, also defined for Solution 4. High cost means high interference impact.

- The eNB will evaluate, for each potential operational carrier, the impact of its operation on other eNBs in terms of cumulated interference to other pico eNBs and to the MeNB. The cumulated interference to other pico eNBs and to the MeNB is measured as the cumulated CQI degradation (cost) reported by all the interfered eNBs.

- The eNB will estimate, for each potential operational carrier, the CQI value. For such estimation, the eNB can use the RSSI reporting in different channels (actually interference power), where UE reports the RSSI measured according to 3GPP 36.214. The useful signal power can be derived from the CQI and RSSI measurements inside the operational channel.

Based on 6 and 7, in the first step the eNB will select for operation the carrier with the best the trade-off between maximizing performance and minimizing the cumulated interference to other pico eNBs and to the MeNB. In fact, it is possible to derive other criteria, for example the decision may be based on accumulated CQI degradation only and selecting for operation the carrier which causes minimum accumulated CQI degradation. In such a case, step 7 is not necessary.

- The above selected carrier will be activated for regular operation. If the CQI reported by the UEs operating on this carrier is higher than the cost of using the carrier, the eNB will decide to use the carrier, otherwise will switch it off.

- Based on cost measurement results, the pico eNB may also decide to change the frequency of its basic operation.

- The pico eNB repeats steps 3...8 to add new carriers if needed.

# CQI degradation due to the new activated carriers

The CQI is measured and reported by the UEs affected by the interference created by the new activated carrier. The serving eNBs compute the CQI degradation, as the difference between the previous CQI, when the new carriers were not activated, and the new CQI, for each one of the used carriers. The average CQI degradation in each frequency channel, named interference cost, is transmitted over the X2 interface to the eNB starting the measurement.

# Criteria for the additional carrier selection

The eNB trying to activate a new carrier will have to decide:

Step 1: Which one of the possible additional carriers causes minimum interference to the surrounding network? For doing this, the eNB will sum the costs (CQI degradation) received for each carrier and will select the carrier with the best the trade-off between maximizing performance and minimizing the cumulated interference to other pico eNBs and to the MeNB. Other criteria are possible, such as selecting the additional carrier with the minimum cost or forbade those carriers with a cost greater than zero.

Step 2: The eNB will temporary activate the carrier selected in Step 1 for the real operation. One or more UEs served by it will measure and report the CQI. If the average CQI on the new activated carrier is higher than the cumulated CQI degradation experienced by the eNBs in the area, the eNB will decide to use this carrier as additional carrier, otherwise the carrier will be switched off.

# New messages and their usage

The new messages to be defined on the X2 interface include command and information messages.

These messages have the same IEs as the messages defined for Solution 4‎[5], however they are described here again due to the different functionality.

The start of the CQI measurement is initiated by the eNB evaluating the use of an additional carrier by sending a COST MEASUREMENT REQUEST message to a supposed victim eNB. The IEs of this message are defined in the contribution R3-120582‎[5].

The victim eNB responds with a COST MEASUREMENT INFORMATION message. The IEs of this message are defined in the contribution R3-120582‎[5].

The COST MEASURMENT INFORMATION message may be transmitted also to other eNBs in the area; these eNB may decide to use the opportunity and also assess the effect of this aggressor eNB on their served UEs.

The result of measurements reported by UE is processed or not by the victim eNB, which sends to the initiating eNB an informative message containing the measured cost, representing the CQI degradation caused by the interfering eNB. The IEs of the INTERFER COST message are defined in the contribution R3-120582‎[5].

# Simulation results

We have used a CA (Carrier Aggregation) MeNB, operating over four 5MHz channels. The cost is estimated per individual channel. In addition, we have applied the pricing policy for selecting an operating channel or a bandwidth part. The deployment topography is shown in Figure 4, where the pico eNBs are placed indoors.

Most of the simulation parameters are based on TR 36.814 ‎[4] section A.2.1.1.2. Specific parameters are:

- Pico eNB channel bandwidth: 5MHz;

- Number of PUEs per pico eNB: 1;

- Number of MUEs served by MeNB and placed in the pico eNB area: 1; probability of being indoor: 0.2

- For the MUEs located in the vicinity of the dual strip is used one 5MHz carrier out of four carriers;

- The dual-strip deployment was considered over a number of floors varying between 1 and 6;

- One pico eNB considers itself interfered if the average SNR received from an interfering pico eNB is greater than the SNR of the serving pico eNB minus 15 dB;

- The simulation considers that each pico eNB and the MeNB will start with one carrier and will try to activate a second carrier. The pico eNB also may change the first (basic) carrier, for optimising it.

UE

UE

Pico eNBs

HeNBs

MeNB

MeNB

70

m

10

m

120

m

Figure 4: Deployment topology

## Performance

Figure 5 Left shows the MeNBs throughput in Mb/s (5 MHz), with and without pricing, versus the average number of active pico eNBs in the area, including the case of no active pico eNB. Note that it can be achieved a MeNB capacity gain of aprox. 30% when pricing is exchanged (see Figure 5 left) and also an important increase of pico eNB capacity (see Figure 5 right).

Figure 5: Left: MeNB throughput            Right: Pico eNB throughput

A dramatic change in performance can be seen if we look at the MUE and PUEs which are most affected by interference. In Figure 6 we show the performance improvement for the 10% of users most affected by interference.

In Figure 6 left it can be seen that for a several pico eNBs the worst 10% MUEs performance is improved, while for a number of pico eNBs between 5 to 10, the MUE users which suffered from DENIAL of SERVICE due to the harmful interference, after applying our algorithm are able to operate with reasonable performance.

In Figure 6 right it can be observed that if there are 10- 15 pico eNBs located in the considered area, the pico eNB user throughput 30% higher with the pricing algorithm!

Figure 6: Throughput performance improvement of worst 10% users: Left: MUE, Right: PUEs

Figure 6: Throughput performance improvement of worst 20% users: Left: MUE, Right: PUEs

## Stability

From Figure 7 it is possible to observe the behaviour of the system when the second carrier is activated (the operational carrier can be also changed based on the received reports).

Once the system is stable, further operational channel selection will be needed only if there are significant changes in topology or radio channel behaviour.

Figure 7: Algorithm stability  Left: MeNB   Right: pico eNB

# Conclusions

In this contribution we have presented the details of the additional carrier selection, based on the assessment of CQI degradation caused by each potentially activated carrier.

We have defined clear and measurable criteria for the additional carrier selection and activation.

We have defined a one request message and two information messages, which are identical with the messages defined for Solution 4, but here are used differently:

- COST MEASUREMENT REQUEST, sent by the aggressor eNB to a victim eNB, for starting the power modulation to enable the CQI degradation measurement procedure;

- COST MEASUREMENT INFORMATION, sent by the victim eNB to the aggressor eNB and also to other eNBs for providing the measurement procedure details;

- INTERFER COST message, to be sent by a victim eNB to the aggressor eNB, with the scope to be used in the new operational frequency selection.

We have applied the measurement procedures to eNBs and UEs situated a dual-strip, multi floor deployment area and have simulated the proposed measurement procedures; the results indicate with no doubt the significant improvement of the performance when the pricing algorithm is used.

In Annex A we propose a slight text change to the existing description of Solution 5, for simplifying and improving the clarity of the description.

# References

- R3-120484, “Report email#02bis: operational carrier selection at pico”, NSN (Rapporteur).

- O. Muñoz-Medina, J. Vidal, A. Agustin, A. Pascual-Iserte, S. Barbarossa, “Coordinated MIMO Precoding for Power Minimization in Femtocell Systems”, IEEE 2nd International Workshop on Femtocell Networks (FEMnet), in conjunction with IEEE GLOBECOM 2011

- R3-112752, “Proposal for DL interference coordination in Macro - SC HeNB scenario”, DAC-UPC

- FREEDOM, D3.2 - Interference Coordination Protocols in Femto-based Networks,

http://www.ict-freedom.eu/documents/deliverables/FREEDOM_3D2UPCf.pdf

- R3-120087, “Autonomous operational carrier selection: gain, X2 overhead and other aspects”, DAC-UPC

- R3-120582, “Solution for the autonomous optimization of the operational carrier”, DAC-UPC.

# ANNEX A - Text changes to Solution 5

Solution 5: Optimization of multi-frequency pico eNB

The multi carrier pico eNB is assigned initially by the OAM with the initial operational carrier and a set of possible additional operational carriers to be eventually used, as is also the case in Solution 3.

For measurement purposes, if the load on the used carriers requires the addition of a new carrier, the problem to be resolved is to select this carrier such that will create minimum interference to neighbour eNBs. There is no need to actually serve UEs on these carriers, but is only needed to activate the carriers and transmit information (for example to un-valid UE identifiers).

One or more UEs served by strongly interfered eNB evaluate the CQI in the situation that new candidate carriers are transmitting data (by using regular CQI measurements, as defined in 3GPP 36.213).

The influence of the new activated carrier on CQI (cost) can be evaluated by each eNB

by subtracting the CQI values measured with the new carrier from the CQI values measured before the new carrier were activated

The eNB receiving the reports of the UEs, function of the own available capacity and the actual offered traffic, will decide to transmit or not the measurement result (cost) to the multi-carrier pico eNB. For transmitting the cost to other eNBs it is needed a new X2AP message. High cost means high interference impact.

The eNB will evaluate, for each potential operational carrier, the impact of its operation and will activate for regular operation only those carriers which do not cause significant interference. Based on cost measurement results, the pico eNB may also change the frequency of its basic operation.