3GPP TSG-RAN WG3 Meeting #77bis	R3-122184

# Lecce, Italy 8-12 October 2012

Agenda Item:	14.2.4

Source:	TruePosition

Title:	LMU Management

Document for:	Discussion and decision

# Introduction

RAN1#68bis sent an LS to RAN3 [1] including the list of parameters required for the LMU to be able to obtain a UL RTOA measurement. The LS indicates that RAN1 has not discussed or included any parameters required by the E-SMLC to manage LMUs [1]:

“Note: RAN1 has not discussed other, not related to SRS, parameters that may be needed to manage LMUs. “

The purpose of this document is to clarify the need for including some other parameters that allow E-SMLC to manage LMUs during UTDOA location:

- Manage LMUs antenna ports

- Manage LMU reference time for UL RTOA measurement

# LMU Antenna Port

Similar to the base station, a single LMU can support multiple sectors.  The most typical deployment case is depicted in figure 1 below, where the LMU has three directional sectors. For each sector, the LMU can be equipped to receive on 1, 2 or 4 antennas to achieve diversity gain.  A single LMU could have 3 sectors x 4 rx diversity = 12 antenna ports.

As described in positioning stage 2 [2], the E-SMLC selects a set of LMUs to participate in a positioning. That selection is done based on the LMU antenna’s geometry/direction relative to the serving eCGI. As part of that LMU selection, it is natural that the E-SMLC would select LMUs which allow it to obtain UL RTOA from sectors facing the serving eCGI only. Therefore, E-SMLC needs the ability  to indicate to the LMU which antenna ports to use for obtaining the UL RTOA. In the example illustrated in figure 1, the E-SMLC would request LMU1 (co-located with serving site) to use the ports of the antennas on sector “alpha”, while requesting LMU2 to use theports of the antennas on sector “beta”.

The E-SMLC is aware of the number of antenna ports per LMU sector based on the information received from the SLm Setup procedure. The number of sectors supported/connected to actual RF antennas can be configured in the E-SMLC via OAM.

Proposal 1: E-SMLC can indicate over the SLmAP the antenna ports that the LMU shall use for UL RTOA measurement.

Figure 1: Example of LMU sectors

# UL RTOA Reference Time

RAN1#79 agreed on the UL RTOA definition [3] and requested RAN3 to introduce the UL RTOA reference time in TS36.459. The UL RTOA measurement is relative to “configurable reference time”:

| Definition | The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in LMU j, relative to the configurable reference time [13], [14]. The reference point [14] for the UL relative time of arrival shall be the RX antenna connector of the LMU node when LMU has a separate RX antenna or shares RX antenna with eNB and the eNB antenna connector when LMU is integrated in eNB. |
| --- | --- |

This section includes a proposal on how to address the RAN1 request and allow the E-SMLC to indicate the reference time to the LMUs. Per the RAN1 definition, the UL RTOA is the relative time between:

- Configurable Reference Time

- Beginning of subframe i containing UE SRS Transmissions Time received at LMU j

Regarding (a), we suggest E-SMLC can signal over SLmAP the following IE:

| Measurement Start Time | M |  | BIT STRING (64) | Time to start the UL RTOA measurement Time in seconds relative to 00:00:00 on 1 January 1900 where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| --- | --- | --- | --- | --- |

Regarding (b) -  The LMU has to search for the SRS symbol in 1ms sub-frame time period. The UE will transmit SRS only on the last symbol in a sub-frame (1 out of 14). Hence, if the LMU knows the serving eNB frame boundary start, it can reduce the search window to the last OFDMA symbol in a frame +/- propagation delay  to LMU j. Therefore, we propose that the eNB would signal the SFN timing to the E-SMLC and that would subsequently be signalled to the LMU over the SLmAP:

| SFN | M |  | BIT STRING (SIZE (10)) | SFN at which first SRS for UL positioning occurs |
| --- | --- | --- | --- | --- |
| SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |

Proposal 2: The E-SMLC will signal the “Measurement Start Time” in the SLmAP Information Request message.

Proposal 3: The eNB and subsequently the E-SMLC will signal the “SFN time” to the LMUs participating in the UTDOA positioning.

# Reference

[1]  R1-121921, LS Response on UL Positioning Parameters.

[2]  3GPP TS 36.305, E-UTRAN; Stage 2 functional specification of UE positioning.

[3]  R1-124029, LS on reference time in UL RTOA measurement.