3GPP TSG-RAN3 Meeting #85 	R3-141801

Dresden, Germany, August 18th – 22nd, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR |  | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Counter Check procedure in X2 for DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SC_enh_dualC-core |  |  |  |  |  |  | Date: |  | 2014-08-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The Counter Check procedure is needed for Dual Connectivity in X2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces Counter Check procedure in X2 for Dual Connectivity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not support the Counter Check procedure for Dual Connectivity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7, 8.1, 8.2.x, 9.1.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

- 7  Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the eNB for setup the X2 interface and implicitly perform an X2 Reset.

-	eNB Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the eNB to coordinate adaptation of mobility parameter settings with a peer eNB.

-	Mobility Robustness Optimisation.  This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel x) SeNB Counter Check y) SeNB Counter Check Indication |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |

- 8  X2AP procedures

- 8.1  Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

# Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST | SENB COUNTER CHECK RESPONSE |  |

# Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| SeNB Counter Check Indication | SENB COUNTER CHECK INDICATION |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.x	  SeNB Counter Check (MeNB initiated)

8.2.x.1  General

The purpose of the SeNB Counter Check (MeNB initiated) procedure is to make the SeNB report the current COUNT values associated to each SCG bearer when it is needed by the MeNB.

The procedure uses UE-associated signalling.

8.2.x.2  Successful Operation

Figure 8.2.x.2-1: SeNB Counter Check procedure (MeNB initiated), successful operation.

The MeNB initiates the procedure by sending the SENB COUNTER CHECK REQUEST message to the SeNB e.g. when any of the COUNT values associated to the MCG bearer reaches a specific value.

For the UE whose identity contained in the SENB COUNTER CHECK REQUEST message, the SeNB reports the current COUNT values associated to each SCG bearer by including the ERAB-ID, the count-Uplink and the count-Downlink IEs in the SENB COUNTER CHECK RESPONSE message.

8.2.x.3  Unsuccessful Operation

NOTE: It is FFS for the failure message.

8.2.x.4  Abnormal Conditions

Void.

8.2.x	  SeNB Counter Check Indication (SeNB initiated)

8.2.x.1  General

The SeNB Counter Check Indication (SeNB initiated) procedure is to used by the SeNB to indicate the current COUNT values associated to each SCG bearer e.g. when any of the COUNT values reaches a specific value.

The procedure uses UE-associated signalling.

8.2.x.2  Successful Operation

Figure 8.2.x.2-1: SeNB Counter Check Indication procedure (SeNB initiated), successful operation.

The SeNB initiates the procedure by sending the SENB COUNTER CHECK INDICATION message to the MeNB.

The SeNB reports the current COUNT values associated to each SCG bearer by including the ERAB-ID, the count-Uplink and the count-Downlink IEs in the SENB COUNTER CHECK INDICATION message.

8.2.x.3  Unsuccessful Operation

Void.

8.2.x.4  Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.X  SENB COUNTER CHECK REQUEST

This message is sent by the MeNB to request the SeNB to report the amount of data sent/ received on each SCG bearer about the UE.

Direction: MeNB  SeNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |

9.1.2.X  SENB COUNTER CHECK RESPONSE

This message is sent by the SeNB to report the current amount of data sent/ received on each SCG bearer about the UE which the MeNB requested.

Direction: SeNB  MeNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| E-RABs Report List |  | 1 |  |  | YES | ignore |
| >E-RABs Report Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | - | - |
| >>COUNT Uplink | M | INTEGER (0..232-1) |  | Indicates the value of uplink COUNT associated to this DRB. | - | - |
| >>COUNT Downlink | M | INTEGER (0..232-1) |  | Indicates the value of the downlink COUNT associated to this DRBs. | - | - |

9.1.2.X  SENB COUNTER CHECK INDICATION

This message is sent by the SeNB to indicate the current amount of data sent/ received on each SCG bearer for the UE.

Direction: SeNB  MeNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | reject |
| E-RABs Indicate List |  | 1 |  |  | YES | ignore |
| >E-RABs Indicate Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | - | - |
| >>COUNT Uplink | M | INTEGER (0..232-1) |  | Indicates the value of uplink COUNT associated to this DRB. | - | - |
| >>COUNT Downlink | M | INTEGER (0..232-1) |  | Indicates the value of the downlink COUNT associated to this DRBs. | - | - |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>