3GPP TSG-RAN WG3 #93	Tdoc R3-162032

# Gothenburg, Sweden, 22nd – 26th August 2016	was R3-161893

Agenda Item:	10.3.1

Source:	Ericsson

Title:	Mobility between LTE and NR for inactive UEs

Document for:	Discussion, Decision

# Introduction

This is a text proposal following discussions on R3-161893.

The chair minutes are copied here:

CB: # 71_TP_Iddle_UE

- In may section

- Note: Pending to decision on LC + note on paging

(E///)

Rev in R3-162032

# Text Proposal to TR 38.801

----------------------------------------------Start of Changes----------------------------------------------

### 6.1.1	RAN-CN functional split

#### 6.1.1.1	NR RAN functions

NOTE 1:	The further detail of the functionalities needs further study.

NOTE 2: 	NR RAN refers to a RAN supporting NR. It may only consist of NR (standalone NR operation) or it may consist of NR and LTE (tight interworking) or it may consist of NR, LTE and WLAN (tight interworking).

Functions similar to E-UTRAN as listed in TS 36.401 [3]

-	Transfer of user data

-	Radio channel ciphering and deciphering

-	Integrity protection

-	Header compression

-	Mobility control functions:

-	Handover

-	Inter-cell interference coordination

-	Connection setup and release

-	Load balancing

-	Distribution function for NAS messages

-	NAS node selection function

-	Synchronization

-	Radio access network sharing

-	Paging

-	Positioning

Functions specific for NR RAN:

-	Network Slice support

-	This function provides the capability for NR RAN to support network slicing

-	Tight Interworking with LTE

-	This function enables tight interworking between NR and LTE by means of data flow aggregation. This function includes at least dual connectivity. Interworking with LTE is supported for collocated and non-collocated site deployments.

-	Multi-connectivity

-	This function provides means for connectivity between an NR node and multiple NR nodes by means of data flow aggregation.

-	LTE-NR handover through LTE-NR interface

-	This function provides means for LTE-NR handover via the direct interface between an eLTE eNB and a NR BS.

NR RAN may also include other functions:

-	Contacting UEs in inactive mode (feasibility of UE Inactive mode to be further studied and coordinated with RAN2)

-	This function may be associated with a NR location area management function providing the capability for NR RAN to manage location areas. If it exists, it is FFS whether and how this NR location area management function will be coordinated with LTE and whether and how it will interact with the NG Core location area management.

-	Direct services support (further study related with D2D, coordinate with RAN1, RAN2)

-	This function provides communication whereby UEs can communicate with each other directly

-  Interworking with non-3GPP systems

- This function provides interworking between NR and Non-3GPP RAT (e.g. WLAN).

-	LTE-NR handover via CN

-	This function provides means for LTE-NR handover via CN.

-	evolved E-UTRAN-NR inactive mode mobility using a direct interface between an eLTE eNB and an NR BS.

-	This function provide means for UE mobility in a RAN controlled inactive mode between NR and evolved E-UTRAN using a direct interface between an eLTE eNB and an NR BS for context fetching and paging.

Editor’s Note:	This approach is pending discussions on “light connection” and respective discussions on paging

----------------------------------------------End of Changes----------------------------------------------