3GPP TSG-RAN WG3 #93bis	Tdoc R3-162471

Sophia Antipolis, France, October 10th – 14th, 2016

Agenda Item:	10.6.1.1

Source:	InterDigital Communications

Title:	Split Option 1 benefits

Document for:	Discussion, Decision

# Introduction

Following discussions in past meetings, a number of functional split options between a central unit (CU) and a distributed unit (DU) were captured as baseline input to TR 38.801.

Option 1 has yet to be fully documented in particular its benefits.

# Discussion

Option 1 is an option where the entire user plane is located in the distributed unit. It has two main benefits, first it can streamline pcell and scell operation in the distributed units when doing “1A” like aggregation which has already been recognized.

The second benefit would be for the URLLC use case when the data source has to be close to the radio for example mobile edge computing use cases.

Therefore:

##### 6.1.2.2.1	Option 1 (RRC/PDCP, 1A-like split)

Description:  In this split option, RRC is in the central unit. PDCP, RLC, MAC, physical layer and RF are in the distributed unit, thus the entire user plane is in the distributed unit.

Benefits and Justification: This option will provide some support for traffic aggregation based on standardization of a aggregation like LTE Dual Connectivity, alternative 1A. Additionally, it can handle edge computing or low latency use cases where the user data needs to be located close to the transmission point.

# Conclusion

This paper puts a discussion to resolve an architecture question posed previously.

Proposal: 	Include section 6.1.2.2.1 above in the TR.