3GPP TSG-RAN WG3 Meeting # 94	R3-162678

Reno, USA, Nov. 14th – 18th, 2016

Agenda item:		10.3.3

Source:	LG Electronics Inc.

Title:	Text Proposal for RAN internal interface Xn

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

7.3	RAN internal interface

7.3.1	Xn Interface

The interface allowing to interconnect two gNBs or one gNB and one eLTE eNB with each other is referred to as the Xn interface. The interface Xn is also applicable for the connection between two eLTE eNBs.

7.3.1.1	General principles

The general principles for the specification of the Xn interface are as follows:

-	the Xn interface shall be open;

-	the Xn interface shall support the exchange of signalling information between the endpoints, in addition the interface shall support data forwarding to the respective endpoints;

-	from a logical standpoint, the Xn is a point-to-point interface between the endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the Xn interface shall support control plane and user plane separation;

-	the Xn interface shall separate Radio Network Layer and Transport Network Layer;

-	the Xn interface shall be future proof to fulfil different new requirements, support new services and new functions.

7.3.1.2	Xn Interface Functions

The Xn-C interface supports the following functions:

-	Xn interface management and error handling functions to manage the Xn-C interface;

-	Error indication;

-	Setting up the Xn;

-	Resetting the Xn;

-	Updating the Xn configuration data;

-	Xn removal.

-	UE connected mode mobility management:  functions to manage the UE mobility for connected mode between nodes in the New RAN;

-	Handover preparation;

-	Handover cancellation.

-	Dual connectivity:  functions to enable usage of additional resources in a secondary node in the New RAN.

-	Load Management: This function is used by new RAN to indicate resource status, overload and traffic load to each other.

-	Load indication (FFS);

-	Resource status reporting (FFS);

-  Paging: function to enable the RAN paging for contacting UEs in inactive mode (FFS)

-  Mobility failure event notification and information exchange in support of handover settings negotiation

-	Mobility Parameters Management

-	Mobility Robustness Optimization

-	Energy Saving: This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the Xn interface (FFS)

The Xn-U interface supports the following functions:

-	Data forwarding

-	Flow control

7.3.1.3	Xn Interface Procedures

To support the functions listed in Section 7.3.1.2, the Xn interface should support the following procedures. The procedures are classified in different categories.

Xn Interface Management and Error Handling Procedures:

-	Xn Setup: To establish an Xn interface between the New RANs

-	Xn Reset: To reset the Xn interface

-	Error Indication: To report detected errors in one incoming message

-	New RAN Configuration Update: to update the configuration for the New RANs over Xn interface

-	Xn Removal: to remove the signaling connection between two New RANs in a controlled manner

UE connected mode Mobility Management Procedures:

-	Handover Preparation: to establish necessary resources in a New RAN for an incoming handover

-	Handover Cancellation: to cancel an ongoing handover preparation or an already prepared handover

Dual Connectivity Procedures:

-	Secondary Node Addition Preparation: to request the secondary node to allocate resources for dual connectivity operation for a specific UE

-	Master Node initiated Secondary Node Modification Preparation: to request the secondary node to modify the UE context at the Secondary node

-	Secondary Node initiated Secondary Node Modification: to allow the secondary node to modify the UE context at the Secondary node

-	Master Node initiated Secondary Node Release: to request the secondary node to release the resources for a specific UE

-	Secondary Node initiated Secondary Node Release: to allow the secondary node to initiate the release of the resources for a specific UE

Load Management Procedures:

-	Load Indication: to transfer load and interference co-ordination information between New RANs (FFS)

-	Resource Status Reporting Initiation: used by a New RAN to request the reporting of load measurements to another New RAN (FFS)

-	Resource Status Reporting Update: used by a New RAN to report the load measurements to the corresponding initiated New RAN (FFS)

Paging Procedures (FFS):

- Xn Paging: to provide a paging instruction from a New RAN to another New RAN (FFS)

Mobility Failure Event Notification and Information Exchange Procedures:

-	Mobility Settings Change: to enable a New RAN to negotiate the handover trigger settings with a neighbor new RAN

-	Radio Link Failure Indication: to transfer information regarding radio link re-establishment attempts or received RLF reports between New RANs

Energy Saving Procedures:

- New RAN Configuration Update: to notify neighbor New RAN that the concerned cell is switched off to lower energy consumption (FFS)

- Cell Activation: to request a neighboring New RAN to switch on one or more cells previously reported as inactive due to energy saving reasons (FFS)

----------------End of the First Change---------------