3GPP TSG-RAN WG3 Meeting #95	R3-170404

Athens, Greece, Feb. 13 – 17, 2017

Agenda item:		12.3

Source:	LG Electronics Inc.

Title:	Open Issue on UE-PC5-AMBR

Document for:		Discussion

1. Introduction

In the last meeting, the following open issue was raised for UE-PC5-AMBR [1]:

- UE-PC5-AMBR:

  - Further check on “AMBR valid only for TX, no need to limit RX?”

In this contribution, we focus on this issue and provide our view on it.

2. Discussion

During the SI phase for V2V/V2X, we had discussion on the following issue:

  - On whether the authorization is only related to the transmit UE or not

On this issue, the conclusion was that the authorization is only related to transmit UE for the following reason.

According to TS 36.331 [2], the following text is specified:

- A UE capable of sidelink communication or discovery may initiate the procedure to request assignment of dedicated resources for the concerned sidelink communication transmission or discovery announcements

Therefore, for sidelink communication, the UE requests eNB to allocate the transmission resources. Then the eNB checks whether the UE is authorized for sidelink communication and then assigns dedicated resources. There is nothing related to the assignment of reception resources for sidelink communication.

Secondly, there exists one sentence in the current baseline CRs for UE-PC5-AMBR, which applies to all the messages. One example is given as follows:

#### 9.1.4.8	UE CONTEXT MODIFICATION REQUEST

This message is sent by the MME to provide UE Context information changes to the eNB.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Security Key | O |  | 9.2.1.41 | A fresh KeNB is provided after performing a key-change on the fly procedure in the MME, see TS 33.401 [15]. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| UE Security Capabilities | O |  | 9.2.1.40 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.x | This IE applies only if the UE is authorized for V2X services. | YES | ignore |

Thirdly, from the original concept of UE-AMBR, defined in TS 23.401,

- The UE-AMBR is limited by a subscription parameter stored in the HSS. The MME shall set the UE-AMBR to the sum of the APN-AMBR of all active APNs up to the value of the subscribed UE-AMBR. The UE-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR bearers of a UE (e.g. excess traffic may get discarded by a rate shaping function). Each of those Non-GBR bearers could potentially utilize the entire UE-AMBR, e.g. when the other Non-GBR bearers do not carry any traffic. GBR bearers are outside the scope of UE AMBR. The E-UTRAN enforces the UE-AMBR in uplink and downlink except for PDN connections using the Control Plane CIoT EPS optimization.

From the highlighted text above, it can be understood that the UL UE-AMBR is used to limit a UE’s total uplink transmit resources without over requesting. The DL UE-AMBR is to limit eNB’s total downlink transmit resources for this UE. So they are from the transmitter point of view instead of receiver.

From the analysis above, the following proposal is suggested to RAN3:

Proposal: UE-PC5-AMBR is valid only for transmitter and it is not necessary to limit receiver.

- Conclusion

In this contribution, we focused on the open issue on UE-PC5-AMBR. The following proposal is suggested to RAN3:

Proposal: UE-PC5-AMBR is valid only for transmitter and it is not necessary to limit receiver.

- References

- R3-163186, “Way forward for “LTE-based V2X Services WI” in RAN3”, LG Electronics

- TS 36.331, v13.1.0

- R3-162719, “Introduction of UE PC5 AMBR over S1”, CATT, ZTE, Huawei

- R3-162720, “Introduction of UE PC5 AMBR over X2”, Huawei, CATT