3GPP TSG RAN WG3 Meeting #95                          R3-170591

Athens, Greece 13th - 17th February 2017

Agenda item:		10.7.1

Source:	Altiostar Networks

Title:	Consideration of RAN Function Support in CU-DU Split Architecture

Document for:		Discussion

# 1	Introduction

In 3GPP RAN3 NR#1, TP for TR 38.801 on architecture and functional description of gNB architecture with CU-DU split and C-plane and U-plane interfaces were agreed for high-layer split (HLS) option. Furthermore, definition of CU and DU was agreed [1,2,3]:

Central Unit (CU): a logical node that includes the gNB functions as listed in section 6.2 excepting those functions allocated exclusively to the DU. CU controls the operation of DUs.

Distributed Unit (DU): a logical node that includes, depending on the functional split option, a subset of the gNB functions. The operation of DU is controlled by the CU.

However, how to support RAN functions in the split architecture remains open. In particular, RRM functions supported via the CU-DU interface need to be discussed in detail. In this contribution, we discuss RAN functions in the context of CU-DU split architecture and interface specification approaches we can take for Stage 3 for HLS option.

# 2	Discussion

RAN functions are described in Section 6.2 in TR 38.801. These include U-Plane functions, C-Plane functions, RRM functions and additional NR-specific functions. To further proceed with interface specification, it is worthwhile to discuss mapping of the RAN functions to CU-DU split architecture. Without agreement on the functions supported by CU/DU, it is difficult to proceed with the definition of the procedures and the interfaces in Stage 3 work. We observe that

- Functions similar to E-UTRAN described in Section 6.2 can be mapped to CU/DU in a straight-forward way, depending on the HLS option decision.

- RRM functions and new NR-specific functions needs to be discussed in detail.

RRM functions defined in E-UTRAN can be reused in NR for efficient radio resource usage in multi-cell environment.

Table 1 shows the common RRM functions described in TS 36.300 [4] and how it can be mapped to NR CU-DU split architecture. Table 2 describes the NR-specific functions and its mapping to CU-DU logical nodes. In addition to these functions, new central coordination function is needed for NR for efficient resource usage and coordination to ensure network performance in multi-vendor environment, taking into account new capabilities and features being defined in NR.

It is observed that identification and specification of all functions in CU-DU split architecture may be difficult to achieve in the first phase. Considering large number of functions to be supported, a phased approach may be considered for Stage 3 specification.

Table 1: Mapping of RRM functions similar to E-UTRAN as listed in TS 36.300*

| Function | Description | NR CU/DU |
| --- | --- | --- |
| Radio Bearer Control (RBC) | Establishment, maintenance and release of Radio Bearers depending on radio resources and QoS requirements | CU |
| Radio Admission Control (RAC) | Admission or rejection of the establishment requests for new radio bearers | CU |
| Connection Mobility Control (CMC) | Management of radio resources in connection with idle or connected mode mobility | CU |
| Dynamic Resource Allocation (DRA) – Packet Scheduling (PS) | Allocation and de-allocation of resources to user and control plane packets | DU/MAC |
| Inter-Cell Interference coordination (ICIC) | Management of radio resources such that inter-cell interference is kept under control | CU |
| Load Balancing (LB) | Handling of uneven distribution of the traffic load over multiple cells | CU |
| Inter-RAT RRM | Management of radio resources in connection with inter-RAT mobility | CU |
| Subscriber profile ID for RAT/Frequency Priority | Configuration and mapping of RRM strategy from SPID | CU |
| Inter-eNB CoMP | Coordination of multiple NBs for improved cell edge throughput | CU |
| Cell on/off and cell discovery | Adaptively turn on/off downlink transmission to deactivate SCell | CU |

* Description of the functions is from E-UTRAN description TS 36.300 and needs to be revised depending on NR specification progress.

Table 2: NR-specific functions

| Function | Description | CU/DU |
| --- | --- | --- |
| Network Slice Support | Network slicing capability support | CU |
| Tight-Interworking with E-UTRA | Data flow aggregation for DC/MC | CU |
| E-UTRA-NR Handover through new NR interface | E-UTRA-NR handover via eLTE eNB and gNB I/F | CU |
| Session Management | Creation/modification/release of a context & related resources in new RAN | CU |
| Central Coordination | Coordination of multiple cells, TRPs, beams, TDD configurations dynamically for network performance improvement | CU |

# 3	Conclusion

In this contribution, we discussed RAN function support in CU-DU split architecture. In order to proceed with Stage 3 standardization of RAN function support in CU-DU architecture, we propose to consider the following:

Proposal 1: Define the mapping between the RAN functions and CU-DU logical nodes in Stage 2.

Proposal 2: Introduce central coordination function as a new RRM function in CU-DU split architecture.

# References

- RAN3 Chairman’s Notes, 3GPP TSG RAN WG3-NR#1, 3GPP MCC.

- TR 38.801 V1.0.0, “Study on New Radio Access Technology: Radio Access Architecture and Interfaces,” 3GPP

- R3-170305, “TP of Architecture and functional description,” Nokia, Alcatel-Lucent Shanghai Bell

- TS 36.300 V14.1.0, “E-UTRA and E-UTRAN Overall Description Stage 2 (Release 14),” 3GPP

- R3-170092, “RRM functions pertinent to DU,” IAESI, Thales, Fairspectrum, VTT