3GPP TSG-RAN WG3 Meeting #96	R3-171471

Hangzhou, China, 15 - 19 May 2017

Agenda item:	10.9.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Changes needed to the LTE DC and EN DC procedures to support NGEN-DC

Document for:	Agreement

# 1	Introduction

In another paper at this meeting, we present arguments why procedures for EN-DC handling ought to be defined anew [1]. This is important aspect of the X2AP discussion, because DC procedures already exist there (defined for LTE DC). In Xn, such procedures must be defined, because they do not exist. However, it is still worth taking the EN-DC as the baseline for NGEN-DC, too. In this paper we consider the needed changes as compared to the EN-DC.

# 2	Discussion

NGEN-DC is in principle very similar to EN-DC: they share the same functional principles and features. Therefore, the same split of the responsibility for the RRC signalling is expected, as well as SgNB autonomy or bearer architecture. Therefore, all the procedures defined for EN-DC apply to NGEN-DC.

The difference is in details: besides obvious editorial aspects (Xn instead of X2, different CN nodes and functions), QoS model is different. More on QoS handling is presented in a separate contribution, but the impact on the NGEN-DC procedures is that there is no E-RAB. Instead, splitting should be based on DRBs.

No other changes at this moment seem necessary as compared to EN-DC. This allows for considering a common description of both aspects of MR-DC in the TS 37.340. However, such common description would require keeping X2 and Xn signalling in perfect synchronisation also in future. We find it challenging and therefore, we prefer to describe both separately.

# 3	Summary

In this paper, we have reviewed the differences between EN-DC and NGEN-DC and concluded that they are truly minimal. Therefore, in our opinion, at this stage the procedures for NGEN-DC should be the same as for EN-DC. However, since they are to be implemented on different interfaces (X2 vs. Xn), we believe it is safer to define them separately in the TS 37.340.

A text proposal for the TS 37.340 is proposed in [2].

# References

- R3-171468, RAN3 #96

- R3-171472, RAN3 #96