3GPP TSG-RAN WG3 #96	Tdoc R3-171729

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.11.3

Source:	Ericsson

Title:	TP for Fast centralized retransmission of lost PDUs

Document for:	pCR

# TP for 38.401

- In this contribution we provide a TP for TS 38.401 to capture the procedure described in R3-171728.

Start of Text Proposal for TS 38.401

## F1-C functions

[…]

### 5.2.x Centralized retransmission of lost PDUs

This function allows to perform the retransmission of the PDUs that were not delivered by a DU (DU1), e.g., because the radio link served by DU1 is subject to temporary outage, via another DU (DU2) with a well-functioning radio link toward the UE.

DU1 shall detect the outage (e.g., based on measurement reports, CQI feedback, UL measurements, HARQ ACK/NACK ratios, RLC status reports). DU1 shall inform the CU about the outage and about the PDCP PDU SNs corresponding to RLC SDUs that have been either partially or totally undelivered. The CU shall decide on which other DU (DU2) with well-functioning radio link to forward traffic to the UE. The CU may take this decision based on, e.g., UE measurement reports. The CU shall forward to DU2 the incoming UP traffic for the UE as well as PDCP PDUs with SNs indicated by DU1. The CU shall monitor the radio link served by DU1. If the radio link served by DU1 starts functioning, the CU may decide to start sending again UP traffic through DU1.

End of Text Proposal for TS 38.401