3GPP TSG-RAN WG3 Meeting # 96	R3-171780

Hangzhou, China, May 15 – 19, 2017

Agenda item:		10.2

Source:	Huawei.

Title:					TP for TS 38.413 on Slice exchange during NG Setup

Document for:		pCR

# 1. Introduction

This is a text proposals for TS 38.413 on slice exchange during NG Setup.

START OF CHANGES

### 9.1.8	Interface Management Messages

#### 9.1.8.1	NG SETUP REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the gNB to transfer application layer information for an NG-C interface instance.

Direction: gNB  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| Global gNB ID | M |  | <ref> | eNote: definition of Global gNB ID is FFS. It may relate to identification of cells hosted by the gNB (FFS). | YES | reject |
| gNB Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the gNB. eNote: TAs applicable for 5G as well | GLOBAL | reject |
| >TAC | M |  | <ref> | Broadcast TAC | - |  |
| >S-NSSAI list |  | 1..<maxnoofSNSSAI> |  | Supported list of S-NSSAI | GLOBAL | ignore |
| >>S-NSSAI | M |  | <ref> |  |  |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >>PLMN Identity | M |  | <ref> |  |  |  |
| Default Paging DRX | M |  | <ref> | eNote: assumption that this is common for NR and E-UTRA. (to be verified with RAN2 and is FFS) | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSNSSAI | Maximum no. of S-NSSAI. Value is FFS. |

END OF CHANGES

START OF CHANGES

#### 9.1.8.2	NG SETUP RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <ref> |  | YES | reject |
| AMF Name | O |  | PrintableString (SIZE(1..150,…)) | FFS | YES | ignore |
| AMF pool and AMF code related information | M |  | <ref> | eNote: Will be similar to EPS, but details are pending discussions in SA2 (FFS) | GLOBAL | reject |
| Relative AMF Capacity | M |  | <ref> | eNote: Load balancing concepts are pending discussions in SA2 (FFS) | YES | ignore |
| S-NSSAI list |  | 1..<maxnoofSNSSAI> |  | Supported list of S-NSSAI | GLOBAL | ignore |
| >S-NSSAI | M |  | <ref> |  |  |  |
| Criticality Diagnostics | O |  | <ref> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSNSSAI | Maximum no. of S-NSSAI. Value is FFS. |

END OF CHANGES