3GPP TSG-RAN WG3 Meeting # 96	R3-171782

Hangzhou, China, May 15 – 19, 2017

Agenda item:		10.10.1

Source:	Huawei.

Title:					Capturing agreed procedures into 38.470

Document for:		Discussion

# 1. Introduction

In last RAN3 meeting, it was agreed to capture (with FFS) some management functions for F1AP. This paper propose how to capture these in 38.470.

# Annex – Text proposal 38.470

# A	Procedures of the F1 interface

## A.B	Control plane procedures

### A.B.x	Interface Management procedures

#### A.B.x.y1	Reset procedure

The purpose of the Reset procedure is to re-initialize the peer entity or part of the peer entity after node setup and after a failure event occurred. This procedure is initiated by both the gNB-DU and gNB-CU.

##### A.B.x.y1.1	gNB-DU initiated Reset procedure

Figure 19.2.2.7.1a-1: gNB-DU initiated Reset procedure

-	The gNB-DU triggers the RESET message to indicate that an initialisation in the gNB-CU is required. The gNB-CU releases the corresponding references and resources.

-	Afterwards the gNB-CU sends the RESET ACK message to confirm that the resources and references are cleared.

##### A.B.x.y1.2	gNB-CU initiated Reset procedure

Figure 19.2.2.7.1b-1: gNB-CU initiated Reset procedure

-	The gNB-CU triggers the RESET message to indicate that an initialisation in the gNB-DU is required. The gNB-DU releases the corresponding references and resources.

-	Afterwards the gNB-DU sends the RESET ACK message to confirm that the resources and references are cleared.

#### A.B.x.y2		Error Indication procedure

The Error Indication procedure is initiated by the gNB-DU and the gNB-CU, to report detected errors in one incoming message, if an appropriate failure message cannot be reported to the sending entity.

##### A.B.x.y2.1	gNB-DU initiated error indication

Figure 19.2.2.7.2a-1: gNB-DU initiated Error Indication procedure

The gNB-DU sends the ERROR INDICATION message to report the peer entity which kind of error occurs.

##### A.B.x.y2.2	gNB-CU initiated error indication

Figure 19.2.2.7.2b-1: gNB-CU initiated Error Indication procedure

The gNB-CU sends the ERROR INDICATION message to report the peer entity which kind of error occurs.

#### A.B.x.y3	F1 Setup procedure

The F1 Setup procedure is used to exchange configured data which is required in the gNB-CU and in the gNB-DU respectively to ensure a proper interoperation. The F1 Setup procedure is triggered by the gNB-DU. The F1 Setup procedure is the first F1AP procedure which will be executed.

Figure 19.2.2.8-1: F1 Setup procedure

-	The gNB-DU initiates the F1 Setup procedure by sending the F1 SETUP REQUEST message to the gNB-CU.

-	In the successful case the gNB-CU responds with the F1 SETUP RESPONSE message.

The gNB-CU and the gNB-DU may agree at the F1 Setup procedure that UE-related contexts and related signalling connection that have been existing before the F1 Setup shall not be affected.

-	If the gNB-CU cannot accept the F1 Setup Request the gNB-CU responds with the F1 SETUP FAILURE message indicating the reason of the denial.