3GPP TSG-RAN WG3 #96	Tdoc R3-172029

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.11.3

Source:	Ericsson

Title:	TP for Fast centralized retransmission of lost PDUs

Document for:	pCR

# TP for 38.401

Start of Text Proposal for TS 38.470

### X.Y Mechanism of centralized retransmission of lost PDUs

Editor’s Note: whether this section could be captured in the same paragraph of mobility procedures is FFS.

This mechanism allows to perform the retransmission of the PDUs that were not delivered by a gNB-DU (gNB-DU1), e.g., because the radio link served by gNB-DU1 is subject to temporary outage, via another gNB-DU (gNB-DU2) with a well-functioning radio link toward the UE.

The gNB-DU1 shall detect the outage (e.g., based on measurement reports, CQI feedback, UL measurements, HARQ ACK/NACK ratios, RLC status reports). The gNB-DU1 shall send a notification to the gNB-CU about the outage.

Editor’s Note: how to define radio link outage is FFS.

Editor’s Note: whether the notification is over F1-C or F1-U is FFS.

Editor’s Note: whether new notification mechanism is needed is FFS.

Editor’s Note: whether the notification includes the PDCP PDU SNs corresponding to the RLC SDUs that have been either partially or totally undelivered in DU1 is FFS.

The gNB-CU shall decide on which gNB-DU (e.g. gNB-DU2) with well-functioning radio link to forward traffic to the UE. The gNB-CU may take this decision based on, e.g., previously recorded UE measurement reports. The gNB-CU shall forward to gNB-DU2 the incoming data traffic for the UE as well as PDCP PDUs with SNs potentially indicated by gNB-DU1. The gNB-CU may monitor the radio link served by gNB-DU1. If the radio link served by gNB-DU1 starts functioning, the gNB-CU may decide to start sending again data traffic through gNB-DU1.

Editor’s Note: how this mechanism is applied to dual/multi connectivity or single connectivity is FFS.

End of Text Proposal for TS 38.470