3GPP TSG-RAN3 Meeting #97	R3-173035

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.466 | CR | 0069 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the FDD L-band (Band 74) into TS 25.466 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_FDD_L_Band-Core |  |  |  |  |  | Date: |  |  | 2017-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of requirements for the FDD L-band as Band 74. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are introduced. - Annex B (normative): Assigned fields for additional data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for the FDD L-band are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS25.461 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The content was separated from the draft merged CR (R4-1705934) which was endorsed in RAN4#83 for bands 49, 50, 71, 72, 72, 73, 74 and 75. |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

Annex B (normative):
Assigned fields for additional data

The following standard fields have no operational impact and are used by the procedures SetDeviceData, GetDeviceData, AntennaSetDeviceData and AntennaGetDeviceData. Little endian order is used for storage of multiple-octet numbers. Where ASCII variables are shorter than the assigned field lengths the characters are right aligned and leading blanks are filled with null characters (0x00). Unused or not intialized parameter shall return the value 0x00 for the GetDeviceData, AntennaGetDeviceData and TMAGetDeviceData procedure.

Table B.1: Assigned fields for additional data of RETAP

| Field No. | Length (octets) | Format | Description |
| --- | --- | --- | --- |
| 0x01 | 15 | ASCII | Antenna model number |
| 0x02 | 17 | ASCII | Antenna serial number |
| 0x03 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2 below |
| 0x08 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2-1 below |
| 0x09 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2-2 below |
| 0x0A | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2-3 below |
| 0x04 | 8 | 4 x 16-bit unsigned | Beamwidth for each operating band in band order (deg), beginning with lowest band. The lowest band is transmitted within the first 16-bit value. (example: width for band I, width for band III) |
| 0x05 | 4 | 4 x 8-bit unsigned | Gain [dBi] for each operating band in band order , expressed in gain value times 10, beginning with the lowest band. The lowest band is transmitted within the first 8-bit value (example: gain for band I, gain for band III) |
| 0x06 | 2 | 16-bit signed | Maximum supported electrical tilt [degree], expressed in tilt value times 10, format as in subclause 3.1 |
| 0x07 | 2 | 16-bit signed | Minimum supported electrical tilt [degree], expressed in tilt value times 10,format as in subclause 3.1 |
| 0x21 | 6 | ASCII | Installation date |
| 0x22 | 5 | ASCII | Installer's ID |
| 0x23 | 32 | ASCII | Base station ID |
| 0x24 | 32 | ASCII | Sector ID |
| 0x25 | 2 | 16-bit unsigned | Antenna bearing [degree], in the range of 0 – 359,9 degree, expressed as bearing value times 10 |
| 0x26 | 2 | 16-bit signed | Installed mechanical tilt [degree], expressed in tilt value times 10, format as in subclause 3.1 |

Table B.2: Coding for operating bands in field 0x03

| Bit no | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating band UTRA | Ext. 09 | Ext. 08 | XIV | XIII | XII | XI | X | IX | VIII | VII | I | II | III | IV | V | VI |
| Operating band E-UTRA | Ext. 09 | Ext. 08 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 1 | 2 | 3 | 4 | 5 | 6 |

Bit 14 set to “1” indicates that field 0x08 is defined.

Bit 15 set to “1” indicates that field 0x09 is defined.

Table B.2-1: Coding for operating bands in field 0x08

| Bit no | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating band UTRA | - | - | - | - | XXVI | XXV | - | - | XXII | XXI | XX | XIX | XVIII | XVII | Res. | Res. |
| Operating band E-UTRA | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | Res. | Res. |

Table B.2-2: Coding for operating bands in field 0x09

| Bit no | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating band UTRA | Ext 0A | - | XXXII | - | - | - | - | - | e | f | d | c | b)36 | b)35 | a)34 | a)33 |
| Operating band E-UTRA | Ext 0A | 65 | 32 | 31 | 44 | 43 | 42 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 |

Bit 15 set to “1” indicates that field 0x0A is defined.

Table B.2-3: Coding for operating bands in field 0x0A

| Bit no | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating band UTRA | Res. | Spare | Spare | Spare | - | Spare | Spare | Spare | - | - | - | - | - | - | - | - |
| Operating band E-UTRA | Res. | Spare | Spare | Spare | 74 | Spare | Spare | Spare | 48 | 69 | 70 | 46 | 68 | 45 | 67 | 66 |

The operating bands are defined in subclause 4.3.7 in TS 25.461 [4]. In Table B.2-2, the notation a)33 is equivalent to E-UTRA band 33.

Bits are numbered from 0 to 15, bit no 0 set=1 represents the value 0x0001.

Bit set=1 represents operating band is supported.

Bit set=0 represents operating band is not supported.

Spare bits shall be set=0.

Unused Beamwidth and Gain octets shall be set to 0x0000.

Examples of operating bands: 	0000 0000 0001 0000 = Operating band II

0000 0000 0011 1000 = Operating band I, II and III

Table B.3: Assigned fields for additional data of TMAAP

| Field No. | Length (octets) | Format | Description |
| --- | --- | --- | --- |
| 0x01 | 15 | ASCII | Antenna model number |
| 0x02 | 17 | ASCII | Antenna serial number |
| 0x03 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2 above |
| 0x08 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2-1 above |
| 0x09 | 2 | 16-bit unsigned | Antenna operating band(s): see Table B.2-2 above |
| 0x04 | 8 | 4 x 16-bit unsigned | Beamwidth for each operating band in band order (deg) beginning with lowest band. The lowest band is transmitted within the first 16-bit value. |
| 0x05 | 4 | 4 x 8-bit unsigned | Gain [dBi] for each operating band in band order, expressed in gain value times 10, beginning with the lowest band. The lowest band is transmitted within the first 8-bit value (example: gain for band I, gain for band III) |
| 0x21 | 6 | ASCII | Installation date |
| 0x22 | 5 | ASCII | Installer's ID |
| 0x23 | 32 | ASCII | Base station ID |
| 0x24 | 32 | ASCII | Sector ID |
| 0x25 | 2 | 16-bit unsigned | Antenna bearing |
| 0x26 | 2 | 16-bit signed | Installed mechanical tilt (degrees * 10) |
| 0x13 | 1 | 8-bit unsigned | TMA subunit type (see table B.4) |
| 0x14 | 4 | 2x16-bit unsigned integer | TMA subunit receive frequency band (see Table B.5) |
| 0x15 | 4 | 2x16-bit unsigned integer | TMA subunit transmit frequency band (see Table B.5) |
| 0x16 | 1 | Unsigned integer | Maximum supported gain given as gain figure expressed in dB/4 |
| 0x17 | 1 | Unsigned integer | Minimum supported gain given as gain figure expressed in dB/4 |
| 0x18 | 1 | Unsigned integer | Gain resolution given as the gain resolution figure expressed in dB/4 |

Table B.4: Field 0x13 interpretation

| Bit number | 7… 2 | 1 | 0 |
| --- | --- | --- | --- |
| TMA subunit type | Spare | Reserved for AISG use | Bypass |

Bits are numbered from 0….7, bit numbered 0 set to 1 represents the value 0x01.

Bit value 0 represents TMA subunit type is not supported.

Bit value 1 represents TMA subunit type is supported.

Spare bits and Bit 1 shall be set to 0.

Example: 	0000 0001 = Bypass

Table B.5: Field 0x14 and 0x15 interpretation

| Octet number | Description |
| --- | --- |
| 0 | fmin low octet |
| 1 | fmin high octet |
| 2 | fmax low octet |
| 3 | fmax high octet |

fmin and fmax are expressed in 100kHz steps from 0kHz.

Example:	For 850MHz, low octet = 0x34, high octet = 0x21.

<Unchanged sections omitted>