3GPP TSG-RAN3 Meeting # 97	R3-173101

Berlin, Germany, Aug 21-25, 2017

Agenda item:		10.10.2.1

Source:	Huawei

Title:					TP on F1 interface management procedures to 38.473

Document for:		pCR

Text proposal for 38.473

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-DU Configuration ID | M |  | 9.3.1.x1 |  | YES | reject |

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |

#### 9.2.1.6		F1 SETUP FAILURE

This message is sent by the gNB-CU to indicate F1 Setup failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | FFS |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### <<Unchanged text omitted>>

### 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

### <<Unchanged text omitted>>

#### 9.3.1.x1 gNB-DU Configuration ID

The gNB-DU configuration ID identifies a configuration of the gNB-DU uniquely among configurations for gNB-DU(s) connected to the same gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU Configuration ID | M |  | INTEGER (0 .. 232 -1) |  |