3GPP TSG-RAN WG3 Meeting #97	 R3-173154

Berlin, Germany, 21 – 25 August 2017

Title:            TP: 5GC involved Inter registration area mobility

Source: 	Huawei

Agenda item:	10.2

Document for:	pCR

# 1	Introduction

This paper contains the Text Proposal for the 5GC involved handover messages.

# Text Proposal for TS 38.413

Beginning of Text Proposal to TS 38.413

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

Editor's note:	Further details are FFS.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source gNB initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source gNB sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source gNB shall indicate the appropriate cause value for the handover in the Cause IE.

All PDU sessions with active UP connections shall be included in the PDU Sessions request List IE.

The S-NSSAI of each PDU session shall be included in the HANDOVER REQUIRED message, and AMF shall use this information for removing slices that are not supported by the target gNB.

When the preparation, including the reservation of resources at the target side is ready, the AMF responds with the HANDOVER COMMAND message to the source gNB.

For the admitted PDU sessions, if there are any QoS flows of that could not be admitted in the target, they shall be indicated in QoS Flows to Release List IE.

If there are any PDU sessions that could not be admitted in the target, they shall be indicated in the PDU Sessions to Release List IE.

Editor’s Note:	Further details are FFS.

Interactions with other NGAP procedures:

Editor’s Note:	Further details are FFS.

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target gNB for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target gNB.

After all necessary resources for the admitted PDU Session Resources have been allocated, the target gNB shall generate the HANDOVER REQUEST ACKNOWLEDGE message. The target gNB shall include in the PDU Sessions Admitted List IE the PDU sessions for which resources have been prepared at the target cell. The PDU sessions that have not been admitted in the target cell, if any, shall be included in the PDU Sessions Failed to Setup List IE.

For the admitted PDU sessions, if there are any QoS flows of that has not been admitted in the target cell, they shall be indicated in QoS Flows Failed to Setup List IE.

If the Paging Assistance Information IE is included in the HANDOVER REQUEST message, the target gNB shall, if supported, store this information in the UE context and use it for RAN paging if any for a UE in RRC-INACTIVE state.

Editor’s Note:	Further details are FFS.

### 9.2.3	UE Mobility Management Messages

#### 9.2.3.1	HANDOVER REQUIRED

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

Editor’s Note:  The name PDU Sessions Request List is to be confirmed.

This message is sent by the source gNB to the AMF to request the preparation of resources at the target.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| gNB UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Handover Type | M |  | <ref> |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Target ID | M |  | <ref> |  | YES | reject |
| Direct Forwarding Path Availability | O |  | <ref> |  | YES | ignore |
| PDU Sessions Request List | M | 1..<maxnoofPDUSessions> |  |  |  |  |
| >PDU Sessions Request Item IEs |  |  |  |  |  |  |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | <ref> |  | - |  |
| Source to Target Transparent Container | M |  | <ref> |  | YES | reject |

#### 9.2.3.2	HANDOVER COMMAND

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

Editor’s Note:	Whether the SMF related information should be grouped in a container is FFS.

This message is sent by the AMF to inform the source gNB that resources for the handover have been prepared at the target side.

Direction: AMF gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| gNB UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Handover Type | M |  | <ref> |  | YES | reject |
| PDU Sessions Subject to Forwarding List |  | 0..1 |  |  | YES | ignore |
| >PDU Sessions Subject to Forwarding Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>QoS Flows Subject to Forwarding List |  | 0..1 |  | Whether this IE should be grouped in the SMF related information container is FFS | - |  |
| >>>QoS Flows Subject to Forwarding Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >> QoS Flows to Release List | O |  | QoS Flow List <ref> | Whether this IE should be grouped in the SMF related information container is FFS | - |  |
| PDU Sessions to Release List | O |  | <ref> |  | YES | ignore |
| Target to Source Transparent Container | M |  | <ref> |  | YES | reject |

#### 9.2.3.5	HANDOVER REQUEST ACKNOWLEDGE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

Editor’s Note:	Whether the SMF related information should be grouped in a container is FFS.

This message is sent by the target gNB to inform the AMF about the prepared resources at the target.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| gNB UE NGAP ID | M |  | 9.3.3.2 | Allocated at the target gNB. | YES | ignore |
| PDU Sessions Admitted List |  | 1 |  |  | YES | ignore |
| >PDU Sessions Admitted Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>QoS Flows Admitted List |  | 0..1 |  | Whether this IE should be grouped in the SMF related information container is FFS | - |  |
| >>>QoS Flows Admitted Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >> QoS Flows Failed to Setup List | O |  | QoS Flow List <ref> | Whether this IE should be grouped in the SMF related information container is FFS | - |  |
| >>Transport Layer Address | M |  | <ref> |  | - |  |
| >>GTP-TEID | M |  | <ref> | To deliver DL PDUs. | - |  |
| PDU Sessions Failed to Setup List | O |  | <ref> |  | YES | ignore |
| Target to Source Transparent Container | M |  | <ref> |  | YES | reject |