3GPP TSG-RAN WG3 #97	Tdoc R3-173229

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.10.1.4

Source:	Ericsson

Title:	System information in disaggregated gNB

Document for:	Discussion, Decision

# Introduction

During the last RAN3 meeting (NR AH#2) the control of the system information (SI) in the disaggregated (CU-DU) gNB deployment was discussed. There was some consensus that the SI parameters that are frequently updated should be controlled by the DU. The following working assumption was captured in the Chairman’s notes [1].

Working assumption: - SFN info resides in the DU

Besides the SI parameters that are frequently updated, we believe that the DU should also control the parameters that are related to radio resources. This approach would create a clear separation of responsibilities between CU and DU, and would increase the chances that the F1 becomes a truly interoperable interface. In this contribution, we provide more details about which parameters should be controlled by the DU.

# Discussion

RAN1 and RAN2 have not yet defined the content of the SI for NR radio. However, we assume that the content of the SI in NR will not be significantly different with respect to the content of the SI in LTE. Consequently, we consider the SI parameters in LTE and for each parameter we discuss whether it should be controlled by the CU or by the DU. We focus mainly on the parameters that are contained in the master information block (MIB) and in the system information blocks 1 and 2 (SIB1 and SIB2).

The MIB defines the most essential physical layer information of the cell required to receive further system information. The MIB includes a limited number of essential and frequently transmitted radio parameters. The parameters contained in the MIB are reported in Appendix I, where we use color-coding to indicate whether each parameter should be configured by the CU or by the DU. Since the MIB parameters are related to the cell physical layer configuration, we believe that they should all be under responsibility of the DU.

Proposal 1	The parameters in the MIB should be controlled by the DU.

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of the other system information blocks. The parameters contained in the SIB1 are reported in Appendix I, where we use color-coding to indicate whether each parameter should be configured by the CU or by the DU. We believe that the majority of the parameters in SIB1 fall under the responsibility of the DU and specifically all the parameters related to: frequency bands (e.g., frequency band indicator), SI scheduling (e.g., SI window length and repetition pattern), and radio configuration (e.g., TDD configuration and cell access related info). On the other hand, the parameters related to the core network, cell (re)selection control (e.g. cell selection info) and frequencies priorities should be controlled by the CU.

Proposal 2	The DU should control the parameters in SIB1 related to frequency bands and radio configuration. The CU should control the parameters in SIB1 related to core network, cell (re)selection, and frequency priority.

SIB2 contains common and shared channel information. The parameters contained in the SIB2 are reported in Appendix I, where we use color-coding to indicate whether each parameter should be configured by the CU or by the DU. We believe that the majority of the parameters in SIB2 fall under the responsibility of the DU and specifically all the parameters related to access class barring (ACB) and spectrum emission. It is worth noting that the ACB mechanism will be different in NR w.r.t. LTE. In fact, RAN2 is working on a unified ACB scheme that will be probably based on the LTE ACDC access control mechanism. On the other hand, the parameters related to PLMN (e.g., PLMN identity list) and UE identification (e.g., resume ID) should be configured by the CU.

Proposal 3	The DU should control the parameters in SIB2 related to radio configuration and access barring (when related to radio resources). The CU should control the parameters in SIB2 related to core network, UE identification and access barring (when related to non-radio resources).

SIB3 – SIB8 contain information about neighbour cells and we believe that the corresponding information should be configured by the CU. SIB9 contains home eNB name and the corresponding information is not going to be used in NR. SIB10 – SIB12 contain information related to public warning and we believe that the corresponding information should be under the responsibility of the CU. SIB14 contains information about extended access barring (EAB) for access control that will probably not be used in NR (assuming that RAN2 succeeds in defining a unified ACB scheme based on LTE ACDC). SIB13 and SIB15 contain MBMS related information and, even if the corresponding information may not be defined in release 15, if MBMS will be introduced in future releases, we believe that the corresponding information should be configured by the CU. SIB16 contains information related to GPS time and coordinated universal time (UTC) and the corresponding information could be configured by CU. SIB17 contains information relevant for traffic steering between E-UTRAN and WLAN, and if similar functionality will be defined for NR, it could be configured by CU. The same applies also for SIB18 – SIB19 that contain information related to sidelink communications and SIB20 that contains information related to SC-PTM.

Proposal 4	The parameters contained in the other SIBs should be controlled by the CU.

Another aspect to consider is that the parameters highlighted in the Annex as under DU control are tightly related to MAC and PHY processes as well as related to characteristics of the DU such as capabilities, configuration, and specific performance. Examples of parameters tightly depending on MAC and PHY are: p-Max (dependent on the physical layer needs in terms of max UE Tx power), freqBandIndicator (depending on DU capabilities at a specific point in time), schedulingInfoList (depending on scheduling conditions at MAC level). For this reason, it seems beneficial to allow the DU to setup such parameters in a way that best fits MAC and PHY operation. This would avoid substantial increase in complexity caused by a model where the CU is assumed to be aware at every point in time about the DU capabilities, performance, configuration etc.

Indeed, a model where the CU is in full control of SI configuration may render interoperability between CU and DU more challenging because the CU would be assumed to always know the DU configuration, performance conditions, operational needs at MAC and PHY, as well as it will imply the CU to know the exact implementation of the DU. It is believed that, if the F1 interface is supposed to follow a clear and easy to interoperate design, such detailed knowledge at the CU would be challenging to achieve.

Observation 1	It is a natural choice that the DU controls the SI parameters that are tightly coupled to the MAC and PHY processes. This would indeed facilitate the definition of an interoperable F1 interface.

Proposal 5	RAN3 is kindly asked to agree with the TP in Annex II for TS 38.401.

# Conclusion

In this paper, we discussed which part of the SI should be configured by the CU and by the DU. We conclude by highlighting the following observation and proposals.

Observation 1	It is a natural choice that the DU controls the SI parameters that are tightly coupled to the MAC and PHY processes. This would indeed facilitate the definition of an interoperable F1 interface.

Proposal 1	The parameters in the MIB should be controlled by the DU.

Proposal 2	The DU should control the parameters in SIB1 related to frequency bands and radio configuration. The CU should control the parameters in SIB1 related to core network, cell (re)selection, and frequency priority.

Proposal 3	The DU should control the parameters in SIB2 related to radio configuration and access barring (when related to radio resources). The CU should control the parameters in SIB2 related to core network, UE identification and access barring (when related to non-radio resources).

Proposal 4	The parameters contained in the other SIBs should be controlled by the CU.

Proposal 5	RAN3 is kindly asked to agree with the TP in Annex II for TS 38.401.

# References

Chairman’s notes, RAN3 AH #2, Qingdao, China, June 2017.

# Annex I

In this annex, we report the parameters contained in MIB, SIB1 and SIB2. We use colour coding to indicate whether each parameter should be controlled by the DU or CU. Note that the actual configuration of the parameters is performed using a separate O&M system (at today in LTE).

- Yellow: controlled by DU (configuration via separate O&M)

- Green: controlled by CU (configuration via separate O&M)

## Master information block (MIB)

-- ASN1START

MasterInformationBlock ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100},

phich-Config						PHICH-Config,

systemFrameNumber					BIT STRING (SIZE (8)),

schedulingInfoSIB1-BR-r13			INTEGER (0..31),

spare								BIT STRING (SIZE (5))

}

-- ASN1STOP

## System information block 1 (SIB1)

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Need OR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL, 	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10)) 		OPTIONAL, 	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

-- ASN1STOP

## System information block 2 (SIB2)

SystemInformationBlockType2 ::=		SEQUENCE {

ac-BarringInfo						SEQUENCE {

ac-BarringForEmergency				BOOLEAN,

ac-BarringForMO-Signalling			AC-BarringConfig				OPTIONAL,	-- Need OP

ac-BarringForMO-Data				AC-BarringConfig				OPTIONAL	-- Need OP

}																		OPTIONAL,	-- Need OP

radioResourceConfigCommon			RadioResourceConfigCommonSIB,

ue-TimersAndConstants				UE-TimersAndConstants,

freqInfo							SEQUENCE {

ul-CarrierFreq						ARFCN-ValueEUTRA				OPTIONAL,	-- Need OP

ul-Bandwidth						ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

},

mbsfn-SubframeConfigList			MBSFN-SubframeConfigList			OPTIONAL, 	-- Need OR

timeAlignmentTimerCommon			TimeAlignmentTimer,

...,

lateNonCriticalExtension		OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs)						OPTIONAL,

[[	ssac-BarringForMMTEL-Voice-r9		AC-BarringConfig				OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r9		AC-BarringConfig				OPTIONAL	-- Need OP

]],

[[	ac-BarringForCSFB-r10				AC-BarringConfig			OPTIONAL	-- Need OP

]],

[[	ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringPerPLMN-List-r12			AC-BarringPerPLMN-List-r12	OPTIONAL	-- Need OP

]],

[[	voiceServiceCauseIndication-r12		ENUMERATED {true}			OPTIONAL	-- Need OP

]],

[[	acdc-BarringForCommon-r13			ACDC-BarringForCommon-r13		OPTIONAL,	-- Need OP

acdc-BarringPerPLMN-List-r13		ACDC-BarringPerPLMN-List-r13	OPTIONAL	-- Need OP

]],

[[

udt-RestrictingForCommon-r13			UDT-Restricting-r13				OPTIONAL,	-- Need OR

udt-RestrictingPerPLMN-List-r13		UDT-RestrictingPerPLMN-List-r13	OPTIONAL,	-- Need OR

cIoT-EPS-OptimisationInfo-r13		CIOT-EPS-OptimisationInfo-r13 	OPTIONAL,	-- Need OP

useFullResumeID-r13					ENUMERATED {true}				OPTIONAL	-- Need OP

]]

}

SystemInformationBlockType2-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList				SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission	OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType2-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType2-v9e0-IEs ::= SEQUENCE {

ul-CarrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0		OPTIONAL,	-- Cond ul-FreqMax

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

AC-BarringConfig ::=				SEQUENCE {

ac-BarringFactor					ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

ac-BarringTime						ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

ac-BarringForSpecialAC				BIT STRING (SIZE(5))

}

MBSFN-SubframeConfigList ::= 		SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig

AC-BarringPerPLMN-List-r12 ::= 		SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12

AC-BarringPerPLMN-r12 ::=			SEQUENCE {

plmn-IdentityIndex-r12					INTEGER (1..maxPLMN-r11),

ac-BarringInfo-r12						SEQUENCE {

ac-BarringForEmergency-r12			BOOLEAN,

ac-BarringForMO-Signalling-r12		AC-BarringConfig	OPTIONAL,	-- Need OP

ac-BarringForMO-Data-r12			AC-BarringConfig	OPTIONAL	-- Need OP

}															OPTIONAL,	-- Need OP

ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}		OPTIONAL,	-- Need OP

ac-BarringForCSFB-r12				AC-BarringConfig		OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Voice-r12		AC-BarringConfig		OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r12		AC-BarringConfig		OPTIONAL	-- Need OP

}

ACDC-BarringForCommon-r13 ::=			SEQUENCE {

acdc-HPLMNonly-r13					BOOLEAN,

barringPerACDC-CategoryList-r13				BarringPerACDC-CategoryList-r13

}

ACDC-BarringPerPLMN-List-r13 ::= 		SEQUENCE (SIZE (1.. maxPLMN-r11)) OF ACDC-BarringPerPLMN-r13

ACDC-BarringPerPLMN-r13 ::=			SEQUENCE {

plmn-IdentityIndex-r13				INTEGER (1..maxPLMN-r11),

acdc-OnlyForHPLMN-r13					BOOLEAN,

barringPerACDC-CategoryList-r13				BarringPerACDC-CategoryList-r13

}

BarringPerACDC-CategoryList-r13 ::= SEQUENCE (SIZE (1..maxACDC-Cat-r13)) OF BarringPerACDC-Category-r13

BarringPerACDC-Category-r13 ::= SEQUENCE {

acdc-Category-r13				INTEGER (1..maxACDC-Cat-r13),

acdc-BarringConfig-r13			SEQUENCE {

ac-BarringFactor-r13			ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

ac-BarringTime-r13				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512}

}										OPTIONAL	-- Need OP

}

UDT-Restricting-r13	::= SEQUENCE {

udt-Restricting-r13					ENUMERATED {true}			OPTIONAL, --Need OR

udt-RestrictingTime-r13				ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512} OPTIONAL --Need OR

}

UDT-RestrictingPerPLMN-List-r13 ::= 	SEQUENCE (SIZE (1..maxPLMN-r11)) OF UDT-RestrictingPerPLMN-r13

UDT-RestrictingPerPLMN-r13 ::= SEQUENCE {

plmn-IdentityIndex-r13					INTEGER (1..maxPLMN-r11),

udt-Restricting-r13						UDT-Restricting-r13		OPTIONAL	--Need OR

}

CIOT-EPS-OptimisationInfo-r13 ::=	SEQUENCE (SIZE (1.. maxPLMN-r11)) OF CIOT-OptimisationPLMN-r13

CIOT-OptimisationPLMN-r13::= SEQUENCE {

up-CIoT-EPS-Optimisation-r13		ENUMERATED {true}			OPTIONAL,	-- Need OP

cp-CIoT-EPS-Optimisation-r13		ENUMERATED {true}			OPTIONAL,	-- Need OP

attachWithoutPDN-Connectivity-r13	ENUMERATED {true}			OPTIONAL	-- Need OP

}

-- ASN1STOP

# Annex II TP for 38.401

Start of Text Proposal for TS 38.401

### 10.3.2.2.4	System Information management function

The system information parameters are partly controlled by the CU and partly controlled by the DU.

NOTE: With reference to the system information parameters in LTE:

- The parameters in the MIB are controlled by the DU;

- The DU controls the parameters in SIB1 related to frequency bands and radio configuration. The CU controls the parameters in SIB1 related to core network, cell (re)selection, and frequency priority;

- The DU controls the parameters in SIB2 related to radio configuration and access barring (when related to radio resources). The CU controls the parameters in SIB2 related to core network, UE identification and access barring (when related to non-radio resources);

- The parameters contained in the other SIBs are controlled by the CU

End of Text Proposal for TS 38.401