3GPP TSG-RAN Meeting #97-bis 	R3-173495

# Prague, Czechia, EU, 9-13 Oct 2017

Agenda item:	10.8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Completion of the RRC tunnelling in MR-DC

Document for:	Decission

# 1	Introduction

At RAN3 meeting #97, a mechanism to enable DL RRC tunneling for MCG split SRBs has been added to the baseline solutions for MR-DC. As explained in [1], this mechanism is incomplete. This TP provides changes to X2AP corresponding to the proposals in [1].

# References

- R3-173494, RAN3 #97-bis

# Text proposal for baseline CR for TS 36.423

First change, omitted text not changed

### 8.X.Y	RRC Transfer

#### 8.X.Y.1	General

The purpose of the RRC Transfer procedure is to deliver an LTE RRC message encapsulated in a PDCP-C PDU to the SgNB so that it may then be forwarded to the UE, or from the SgNB, if it was received from the UE.

The procedure is also to enable transfer of the NR RRC message container from the SgNB to the MeNB so that it may then be forwarded to the UE encapsulated in the LTE RRC, or from the MeNB, if it was received from the UE

The procedure uses UE-associated signalling.

#### 8.X.Y.2	Successful Operation

Figure 8.X.Y.2-1: RRC Transfer procedure, successful operation.

The MeNB initiates the procedure by sending the RRC TRANSFER message to the SgNB or the SgNB initiates the procedure by sending the RRC TRANSFER message to the MeNB.

#### 8.X.Y.3	Abnormal Conditions

In case of the MCG split SRBs, the receiving node may ignore the message, if the MeNB has not indicated possibility of RRC transfer at the bearer setup.

Next change, omitted text not changed

#### 9.1.Z.1	RRC TRANSFER

This message is sent by the MeNB to the SgNB to transfer an RRC message.

Direction: MeNB  SgNB or SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB | YES | reject |
| RRC Container | M |  | OCTET STRING | RRC message | YES | reject |
| SRB Type | M |  | ENUMERATED (srb1, srb2, srb3, ...) | The SRB type | YES | reject |

Next change, omitted text not changed

SRBType ::= ENUMERATED {srb1, srb2, srb3, ...}

Remaining text not changed