3GPP TSG-RAN WG3 Meeting #97bis	R3-173941

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.5.6

Source:	Ericsson

Title:	PDU session vs PDU session resource

Document for:	pCR

# 1	Background

At the last RAN3#96 meeting, we have discussed if to use PDU session or PDU session resource in PDU session management procedure without reaching any conclusion. The TS 38.413 at the moment states this matter is subject for further study:

## 8.2	PDU Session Management Procedures

Editor’s Note:	It is FFS whether to name the PDU session/QoS flow as PDU session resource/QoS flow resource or other names in the descriptions and IE definitions.

Editor’s Note:	Definition of PDU Session Resource is FFS.

# 2	Discussion

TS 23.501 has defined that:

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service. The type of association can be IP, Ethernet or unstructured.

Further Table 5.6.1-1 defines the Attributes of a PDU session

| PDU session attribute | May be modified later during the lifetime of the PDU session | Notes |
| --- | --- | --- |
| Slicing information | No | (Note 1)(Note 2) |
| DNN (Data Network Name) | No | (Note 1)(Note 2) |
| PDU session Type | No | (Note 1) |
| SSC mode | No | (Note 1) The semantics of Service and Session Continuity mode is defined in clause 5.6.9.2 |
| PDU session Id | No |  |
| NOTE 1: If it is not provided by the UE, the network determines the parameter based on default information received in user subscription. Subscription to different DNN(s) may correspond to different default SSC modes and different default PDU session Types NOTE 2: Slicing information and DNN are used by AMF to select a SMF to handle a new session. Refer to clause 5.2. |  |  |

Obviously PDU session is a logic connection between UE and the Data Network and it is in the NAS scope.

PDU Session Resource on the other hand, is the resource that RAN allocates to implement/represent the PDU session. It is in AS scope.

It is clear that we use PDU session resource and QoS flow when it relates to the procedures and messages. However there are some remaining FFS in TS 38.413 and in TS 38.300, there are text using PDU session instead of PDU session resource setup/modify, etc. This should be cleaned up.

Proposal 1: RAN3 to agree the text proposals.

Text proposal for TS 38.300 is included in [1].

# 3	Proposals

Proposal 1: RAN3 to agree the attached text proposals.

Text proposal for TS 38.300 is included in [1].

# 4	Reference

- [1]	R3-173942	PDU session vs PDU session resource, Ericsson

# Text Proposal for NGAP TS 38.413 v0.3.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

## 8.2	PDU Session Management Procedures

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>