3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-173975

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	10.10.2.5

Source:	Ericsson

Title:	gNB-CU and gNB-DU configuration update

Document for:	pCR for TS 38.473

# Introduction

At the previous RAN3 meeting the gNB-CU and gNB-DU configuration update functions were discussed. The following agreements were captured in the Chairman’s notes [1]:

- F1AP shall support the update of configuration information between the CU and the DU (direction is FFS).

- F1AP support for configuration information update shall use Class 1 procedure.

In [2] we discuss the need of introducing the configuration update procedures in both directions and we discuss the structure of the gNB-DU Configuration Update and the gNB-CU Configuration Update messages. In [2] we also provide a stage 2 text proposal for TS 38.401. In this contribution, we translate the proposals in [2] into a stage 3 text proposal for TS 38.473.

Proposal 1	RAN3 is kindly asked to agree with the text proposal in Annex I.

# References

Chairman’s notes, RAN WG3 meeting #97, Berlin, Germany, August 2017.

R3-173974, Configuration Update, Ericsson

# Annex 1: TP for TS 38.473

Start of Text Proposal 1 for TS 38.473

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-DU Served Cells to Add |  | 1.. <maxCellingNBDU> |  | Complete list of added cells served by the gNB-DU | GLOBAL | reject |
| > Served cell information | O |  | 9.3.X |  | - | - |
| > gNB-DU system information | O |  |  | System information owned by gNB-DU (details FFS) |  |  |
| gNB-DU Served Cells to Modify |  | 1.. <maxCellingNBDU> |  | Complete list of modified cells served by the gNB-DU | GLOBAL | reject |
| > Old Cell ID | M |  |  | Old NR Cell Identifier |  |  |
| > Served cell information | O |  | 9.3.X |  | - | - |
| > gNB-DU system information | O |  |  | System information owned by gNB-DU (details FFS) |  |  |
| gNB-DU Served Cells to Delete |  | 1.. <maxCellingNBDU> |  | Complete list of modified cells served by the gNB-DU | GLOBAL | reject |
| > Old Cell ID | M |  |  | Old NR Cell Identifier |  |  |

End of Text Proposal 1 for TS 38.473

Start of Text Proposal 2 for TS 38.473

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cells to be Activated List |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | GLOBAL | reject |
| > Cell ID | M |  |  | NR Cell Identifier (without gNB ID) | - | - |
| > PCI | O |  |  | Request a new value for the physical cell ID | YES | ignore |
| > gNB-CU System Information | O |  |  | System information owned by gNB-CU (details FFS) | - | - |
| Cells to be Deactivated List |  | 1.. <maxCellingNBDU> |  | List of cells to be deactivated | GLOBAL | reject |
| > Cell ID | M |  |  | NR Cell Identifier (without gNB ID) | - | - |

End of Text Proposal 2 for TS 38.473

Start of Text Proposal 2 for TS 38.473

#### 9.3.X		Served Cell Information

This IE contains cell configuration information of a cell that the gNB-CU may need for correct operation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell ID | M |  |  | NR Cell Identifier (without gNB ID) | YES | reject |
| PCI | M |  |  | Physical Cell ID |  |  |
| TAC | M |  | OCTECT STRING(2) | Tracking Area Code |  |  |
| Broadcast PLMNs |  |  |  | Broadcast PLMNs | – | – |
| >PLMN Identity | M |  |  |  | – | – |
| CHOICE NR-Mode-Info | M |  |  |  | – | – |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – | – |
| >TDD |  |  |  |  | – | – |
| >>TDD Info |  | 1 |  |  | – | – |
| Number of Antenna Ports | O |  |  |  | YES | ignore |
| PRACH Configuration | O |  |  |  | YES | ignore |

End of Text Proposal 2 for TS 38.473